|  | # Written Assessment Guide # for # “Perfom Basic Maintenance of Emboridery Machine” |
| --- | --- |
|  | Date |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: |  | CS Code: | Level: 2 | Version: |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Perfom Basic Maintenance of Emboridery Machine |  | Assessment Date (DD/MM/YY): |  |  |
| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |  |  |  |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |
| --- | --- |
| Written Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |

| Title of Qualification: | CS Code: | Level: | Version: |
| --- | --- | --- | --- |
| Competency Standard Title: Perfom Basic Maintenance of Emboridery Machine | Assessment Date (DD/MM/YY): |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| - Which oil do we use for the lubrication of machine? | We use special white oil for thelubrication |
| - What parts of the machine need cleaning ? | Shuttle Head of machine (needle bar) Machine card Machine frame (paindograph) Belt |
| - How do we adjust the needle and needle bar? | Loose the needle screw to adjust the needle Put the needle in the needle bar by keeping short groove of the needle on the backside Adjust needle long groove on the front side Tight the screw on the needle bar |
| - How do you change the motor belt? | - Locate the safety cover on the motor belt - Unscrew the safety cover of the motor belt - Replace the motor belt carefully - Put the safety cover with screws |
| - What are the tools we use for basic maintenance of machine? | L key set Screw driver set Plier Nose Plier Ranch spanner |