|  | # Assessment Evidence Guide # for # “Machine Embroidery” LEVEL-IV |
| --- | --- |
|  | Date |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: Machine Embroider Level-III |  | CS Code: | Level: 3 | Version: |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Carry Out Different Types of Embroidery Stitches by Free Motion Machine Perform Surface Decorative on Machine Embroidery Perform Embellishment for Embroidery on Machine Perform Basic Maintenance of Embroidery Machine Perform Embroidery on Computer Embroidery Machine Perform Troubleshooting of Embroidery Machine |  | Assessment Date (DD/MM/YY): Time: 4 Hours |  |  |
| Candidate Details | Name: Registration/Roll Number: |  |  |  |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment): - Assessment Task 1: Carry out any 3 type of stitches from each of the following assigned by the assessor; - Flat Stitches - Zigzag Stitches - Assessment Task 2: Candidate is required to carry out decorative work on the machine as per the mock-up given by the assessor - Assessment Task 3: Candidate is required perform embellishment by mirror and sequin as per the mock-up given by the assessor - Assessment Task 4: Candidate is required to perform basic maintenance of embroidery machine as per instructions of the assessor - Assessment Task 5: Candidate is required perform embroidery on computerized embroidery machine as per given DST design …………. And complete: - Knowledge assessment test (Written or Oral) - Portfolios at the time of assessment (if any) |  |  |  |
|  | During a practical assessment, under observation by an assessor, you will complete: Assessment Task 1: Candidate is required to carry out any 3 type of stitches from each of the following assigned by the assessor; - Flat Stitches - Zigzag Stitches Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Gather and Cut the material in required quantity and sizes Performance Criteria 5: Check that the materials to be used are free from faults Performance Criteria 6: Transfer traced design on given material Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements Performance Criteria 11: Perform basting to start embroidery on the fabric/material Performance Criteria 12: Perform the flat embroidery stitch following the traced/ transferred design lines Performance Criteria 13: Perform embroidery using required type of flat stitches (Running, Back & Stem, Herringbone, Skip, long & short, shade), while ensuring firmness of embroidery stitches Performance Criteria 14: Preform the zigzag embroidery stitch i.e (satin, jump, round, E (chawal), shade, fancy) following the traced/ transferred design lines Performance Criteria 15: Carry out embroidery using required type of zigzag stitches, while ensuring firmness of embroidery stitches Performance Criteria 16: Lock the embroidery work with final stitch while avoiding tightness Performance Criteria 17: Control the quality of embroidery according to quality standards and the artwork specifications Performance Criteria 18: Perform finishing procedures includes clipping, stain removal and ironing of final product |  |  |  |
| Minimum Evidence Required | Assessment Task 2: Candidate is required to carry out decorative work on the machine as per the mock-up given by the assessor Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Gather and Cut the material in required quantity and sizes Performance Criteria 5: Check that the materials to be used are free from faults Performance Criteria 6: Transfer traced design on given material Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements Performance Criteria 11: Estimate material consumption for required applique work Performance Criteria 12: Fuse applique material [if required] using ironing/pinning/basting process while maintaining surface evenness Performance Criteria 13: Cut material from design outline or Cut the material with seam allowance for rali work, whichever required according to design Performance Criteria 14: Tack applique patch with machine embroidery stitches on given material at designated position Performance Criteria 15: Iron the material to remove plication Performance Criteria 16: Perform cutting of the material to be quilted Performance Criteria 17: Pin and sew material with quilt backing sheet by avoid puckering under the given material Performance Criteria 18: Burn ribbon from starting and ending points, if required Performance Criteria 19: Attach ribbon with sew-in procedure by following design Performance Criteria 20: Develop ribbon ornaments (flower, petal etc.) by twisting and folding techniques as per design requirements Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications Performance Criteria 22: Perform required finishing procedures (clipping and stain removal) on the final product …………………………….. |  |  |  |
|  | Assessment Task 3: Candidate is required perform embellishment by mirror and sequin as per the mock-up given by the assessor Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Gather and Cut the material in required quantity and sizes Performance Criteria 5: Check that the materials to be used are free from faults Performance Criteria 6: Transfer traced design on given material Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements Performance Criteria 11: Ensure bobbin is moving anti clock wise when placed in bobbin case Performance Criteria 12: Apply the sequin on the traced design by avoiding breakage of sequins Performance Criteria 13: Attach mirror with glue/adhesive on the traced design Performance Criteria 14: Attach mirror on the traced design with jump stitch carefully Performance Criteria 15: Check the tension post, if thread is tight or loose Performance Criteria 16: Tighten or loose the tension post screw as required Performance Criteria 17: Adjust the bobbin case for thread tension, if required Performance Criteria 18: Tight or loose the screw of bobbin with screw driver as required Performance Criteria 19: Rewind or change the bobbin if thread is finished Performance Criteria 20: Attach the needle in the needle bar by keeping cut side of the needle on the correct side according to the machine, if required Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications Performance Criteria 22: Perform required finishing procedures (clipping and stain removal) on the final product …………………………….. |  |  |  |
|  | Assessment Task 4: Candidate is required to perform basic maintenance of embroidery machine as per instructions of the assessor Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Loose the needle clump screw to adjust the needle Performance Criteria 5: Adjust the needle in the needle bar by keeping grooves of the needle as per machine requirement Performance Criteria 6: Unscrew the shuttle by moving shaft accordingly Performance Criteria 7: Set timing of the main shaft as per machine specification Performance Criteria 8: Tight screws of the shuttle and needle plate for closing Performance Criteria 9: Locate the safety cover on the motor belt Performance Criteria 10: Unscrew the safety cover of the motor belt Performance Criteria 11: Replace the motor belt carefully Performance Criteria 12: Put the safety cover with screws ………………………….. |  |  |  |
|  | Assessment Task 5: Candidate is required perform embroidery on computerized embroidery machine as per given DST design Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Plug in and copy the file in USB folder Performance Criteria 5: Perform framing on computer embroidery machine Performance Criteria 6: Apply reinforcement material as per design if required Performance Criteria 7: Transfer the design file in the computer embroidery machine through data transfer device Performance Criteria 8: Select the required format of the design (DST file format) on the machine screen Performance Criteria 9: Perform threading of the machine and wind the bobbin Performance Criteria 10: Ensure bobbin is moving anti clock wise when placed in bobbin case Performance Criteria 11: Fix and tighten the fabric/material on the machine frame while avoiding plication Performance Criteria 12: Set the range of the design on the material/fabric Performance Criteria 13: Start the machine while ensuring quality of the embroidery Performance Criteria 14: Check the tension post, if thread is tight or loose Performance Criteria 15: Tighten or loose the tension post screw as required Performance Criteria 16: Adjust the bobbin case for thread tension, if required Performance Criteria 17: Tight or loose the screw of bobbin with screw driver as required Performance Criteria 18: Rewind or change the bobbin if thread is finished Performance Criteria 19: Attach the needle in the needle bar by keeping cut side of the needle on the correct side according to the machine, if required Performance Criteria 20: Reinstall the software in computerized machine in case of error. Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications Performance Criteria 22: Dissemble the fabric/material from the frame Performance Criteria 23: Perform required finishing procedures (clipping, stain removal, reinforcement material and adhesive/acrylic) on the final product …………………………….. |  |  |  |
|  | Portfolios required at the time of assessment (if any) for Performance criteria 1 for the evaluation of portfolio: Diary log of completed work of Different Types of Embroidery Stitches by Free Motion Machine Performance criteria 2 for the evaluation of portfolio: Diary log of completed work of Embellishment for Embroidery on Machine Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Basic Maintenance of Embroidery Machine Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Embroidery on Computer Embroidery Machine Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Troubleshooting of Embroidery Machine …………………………….. |  |  |  |

Continued on following page

Assessors Judgment Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |  |  |  |  |  |  |  |
| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |  |
| Activity |  | Method |  |  |  |  | Result |  |
| Nature of Activity |  | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |  |

| Each Assessment Task (with performance criteria) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assessment Task 1: Candidate is required to carry out any 3 type of stitches from each of the following assigned by the assessor; - Flat Stitches - Zigzag Stitches |  |  | Description of assessment task 1 |  |  |  |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
|  | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
| - 4 | Performance Criteria 4: Gather and Cut the material in required quantity and sizes |  |  |  |  |  |
|  | Performance Criteria 5: Check that the materials to be used are free from faults |  |  |  |  |  |
|  | Performance Criteria 6: Transfer traced design on given material |  |  |  |  |  |
|  | Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication |  |  |  |  |  |
|  | Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected |  |  |  |  |  |
|  | Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product |  |  |  |  |  |
|  | Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements |  |  |  |  |  |
|  | Performance Criteria 11: Perform basting to start embroidery on the fabric/material |  |  |  |  |  |
|  | Performance Criteria 12: Perform the flat embroidery stitch following the traced/ transferred design lines |  |  |  |  |  |
|  | Performance Criteria 13: Perform embroidery using required type of flat stitches (Running, Back & Stem, Herringbone, Skip, long & short, shade), while ensuring firmness of embroidery stitches |  |  |  |  |  |
|  | Performance Criteria 14: Preform the zigzag embroidery stitch i.e (satin, jump, round, E (chawal), shade, fancy) following the traced/ transferred design lines |  |  |  |  |  |
|  | Performance Criteria 15: Carry out embroidery using required type of zigzag stitches, while ensuring firmness of embroidery stitches |  |  |  |  |  |
|  | Performance Criteria 16: Lock the embroidery work with final stitch while avoiding tightness |  |  |  |  |  |
|  | Performance Criteria 17: Control the quality of embroidery according to quality standards and the artwork specifications |  |  |  |  |  |
|  | Performance Criteria 18: Perform finishing procedures includes clipping, stain removal and ironing of final product |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Each Assessment Task (with performance criteria) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assessment Task 2: Candidate is required to carry out decorative work on the machine as per the mock-up given by the assessor |  |  | Description of assessment task 2 |  |  |  |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
|  | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
| - 4 | Performance Criteria 4: Gather and Cut the material in required quantity and sizes |  |  |  |  |  |
|  | Performance Criteria 5: Check that the materials to be used are free from faults |  |  |  |  |  |
|  | Performance Criteria 6: Transfer traced design on given material |  |  |  |  |  |
|  | Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication |  |  |  |  |  |
|  | Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected |  |  |  |  |  |
|  | Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product |  |  |  |  |  |
|  | Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements |  |  |  |  |  |
|  | Performance Criteria 11: Estimate material consumption for required applique work |  |  |  |  |  |
|  | Performance Criteria 12: Fuse applique material [if required] using ironing/pinning/basting process while maintaining surface evenness |  |  |  |  |  |
|  | Performance Criteria 13: Cut material from design outline or Cut the material with seam allowance for rali work, whichever required according to design |  |  |  |  |  |
|  | Performance Criteria 14: Tack applique patch with machine embroidery stitches on given material at designated position |  |  |  |  |  |
|  | Performance Criteria 15: Iron the material to remove plication |  |  |  |  |  |
|  | Performance Criteria 16: Perform cutting of the material to be quilted |  |  |  |  |  |
|  | Performance Criteria 17: Pin and sew material with quilt backing sheet by avoid puckering under the given material |  |  |  |  |  |
|  | Performance Criteria 18: Burn ribbon from starting and ending points, if required |  |  |  |  |  |
|  | Performance Criteria 19: Attach ribbon with sew-in procedure by following design |  |  |  |  |  |
|  | Performance Criteria 20: Develop ribbon ornaments (flower, petal etc.) by twisting and folding techniques as per design requirements |  |  |  |  |  |
|  | Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications |  |  |  |  |  |
|  | Performance Criteria 22: Perform required finishing procedures (clipping and stain removal) on the final product |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |
| Assessment Task 3: Candidate is required perform embellishment by mirror and sequin as per the mock-up given by the assessor |  |  | Description of assessment task 3 |  |  |  |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - 3 | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
|  | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 4 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
|  | Performance Criteria 4: Gather and Cut the material in required quantity and sizes |  |  |  |  |  |
|  | Performance Criteria 5: Check that the materials to be used are free from faults |  |  |  |  |  |
|  | Performance Criteria 6: Transfer traced design on given material |  |  |  |  |  |
|  | Performance Criteria 7: Fix and tighten the fabric/material on the hand frame/frame(adda) while avoiding plication |  |  |  |  |  |
|  | Performance Criteria 8: Ensure that correct component parts/fabric/material for embroidery are selected |  |  |  |  |  |
|  | Performance Criteria 9: Ensure the materials used meet the specification matching embroidery Artwork and Design sample of the embroidered product |  |  |  |  |  |
|  | Performance Criteria 10: Set-up layout of work area meeting ergonomics and material flow requirements |  |  |  |  |  |
|  | Performance Criteria 11: Ensure bobbin is moving anti clock wise when placed in bobbin case |  |  |  |  |  |
|  | Performance Criteria 12: Apply the sequin on the traced design by avoiding breakage of sequins |  |  |  |  |  |
|  | Performance Criteria 13: Attach mirror with glue/adhesive on the traced design |  |  |  |  |  |
|  | Performance Criteria 14: Attach mirror on the traced design with jump stitch carefully |  |  |  |  |  |
|  | Performance Criteria 15: Check the tension post, if thread is tight or loose |  |  |  |  |  |
|  | Performance Criteria 16: Tighten or loose the tension post screw as required |  |  |  |  |  |
|  | Performance Criteria 17: Adjust the bobbin case for thread tension, if required |  |  |  |  |  |
|  | Performance Criteria 18: Tight or loose the screw of bobbin with screw driver as required |  |  |  |  |  |
|  | Performance Criteria 19: Rewind or change the bobbin if thread is finished |  |  |  |  |  |
|  | Performance Criteria 20: Attach the needle in the needle bar by keeping cut side of the needle on the correct side according to the machine, if required |  |  |  |  |  |
|  | Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications |  |  |  |  |  |
|  | Performance Criteria 22: Perform required finishing procedures (clipping and stain removal) on the final product |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 4: Candidate is required to perform basic maintenance of embroidery machine as per instructions of the assessor |  |  | Description of assessment task 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - 1 | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
| - 2 | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
| - 4 | Performance Criteria 4: Loose the needle clump screw to adjust the needle |  |  |  |  |  |
|  | Performance Criteria 5: Adjust the needle in the needle bar by keeping grooves of the needle as per machine requirement |  |  |  |  |  |
|  | Performance Criteria 6: Unscrew the shuttle by moving shaft accordingly |  |  |  |  |  |
|  | Performance Criteria 7: Set timing of the main shaft as per machine specification |  |  |  |  |  |
|  | Performance Criteria 8: Tight screws of the shuttle and needle plate for closing |  |  |  |  |  |
|  | Performance Criteria 9: Locate the safety cover on the motor belt |  |  |  |  |  |
|  | Performance Criteria 10: Unscrew the safety cover of the motor belt |  |  |  |  |  |
|  | Performance Criteria 11: Replace the motor belt carefully |  |  |  |  |  |
|  | Performance Criteria 12: Put the safety cover with screws |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 5: Candidate is required perform embroidery on computerized embroidery machine as per given DST design |  |  | Description of assessment task 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - 1 | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
| - 2 | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
| - 4 | Performance Criteria 4: Plug in and copy the file in USB folder |  |  |  |  |  |
|  | Performance Criteria 5: Perform framing on computer embroidery machine |  |  |  |  |  |
|  | Performance Criteria 6: Apply reinforcement material as per design if required |  |  |  |  |  |
|  | Performance Criteria 7: Transfer the design file in the computer embroidery machine through data transfer device |  |  |  |  |  |
|  | Performance Criteria 8: Select the required format of the design (DST file format) on the machine screen |  |  |  |  |  |
|  | Performance Criteria 9: Perform threading of the machine and wind the bobbin |  |  |  |  |  |
|  | Performance Criteria 10: Ensure bobbin is moving anti clock wise when placed in bobbin case |  |  |  |  |  |
|  | Performance Criteria 11: Fix and tighten the fabric/material on the machine frame while avoiding plication |  |  |  |  |  |
|  | Performance Criteria 12: Set the range of the design on the material/fabric |  |  |  |  |  |
|  | Performance Criteria 13: Start the machine while ensuring quality of the embroidery |  |  |  |  |  |
|  | Performance Criteria 14: Check the tension post, if thread is tight or loose |  |  |  |  |  |
|  | Performance Criteria 15: Tighten or loose the tension post screw as required |  |  |  |  |  |
|  | Performance Criteria 16: Adjust the bobbin case for thread tension, if required |  |  |  |  |  |
|  | Performance Criteria 17: Tight or loose the screw of bobbin with screw driver as required |  |  |  |  |  |
|  | Performance Criteria 18: Rewind or change the bobbin if thread is finished |  |  |  |  |  |
|  | Performance Criteria 19: Attach the needle in the needle bar by keeping cut side of the needle on the correct side according to the machine, if required |  |  |  |  |  |
|  | Performance Criteria 20: Reinstall the software in computerized machine in case of error. |  |  |  |  |  |
|  | Performance Criteria 21: Control the quality of embroidery according to quality standards and the artwork specifications |  |  |  |  |  |
|  | Performance Criteria 22: Dissemble the fabric/material from the frame |  |  |  |  |  |
|  | Performance Criteria 23: Perform required finishing procedures (clipping, stain removal, reinforcement material and adhesive/acrylic) on the final product |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Portfolio (if any) |  | Description of portfolio |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Current Sufficient Authentic Valid Reliable |  |  |  |  |  |
| Portfolio meet the following performance standards: |  |  | Yes | No | Remarks |
| - 3 | Performance criteria 1 for the evaluation of portfolio: Diary log of completed work of Different Types of Embroidery Stitches by Free Motion Machine |  |  |  |  |
| - 4 | Performance criteria 2 for the evaluation of portfolio: Diary log of completed work of Embellishment for Embroidery on Machine |  |  |  |  |
|  | Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Basic Maintenance of Embroidery Machine |  |  |  |  |
|  | Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Embroidery on Computer Embroidery Machine |  |  |  |  |
|  | Performance criteria 3 for the evaluation of portfolio: Diary log of completed work of Troubleshooting |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |