|  | # Assessment Evidence Guide # for # “Perform Basic Maintenance of Embroidery Machine” Level IV |
| --- | --- |
|  | Date: 28-02-2020 |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: |  | CS Code: | Level: 4 | Version: |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Perform Basic Maintenance Of Embroidery Machine |  | Assessment Date (DD/MM/YY): |  |  |
| Candidate Details | Name: Registration/Roll Number: |  |  |  |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment): - Assessment Task 1: Perform cleaning and oiling of machine for smooth operation - Assessment Task 2: Perform needle and shuttle adjustment and also change the belt of the machine …………. And complete: - Knowledge assessment test (Written or Oral) - Portfolios at the time of assessment (if any) |  |  |  |
| Minimum Evidence Required | During a practical assessment, under observation by an assessor, you will complete: - Assessment Task 1: Perform cleaning and oiling of machine for smooth operation Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Clean the shuttle with blower or oil Performance Criteria 5: Clean the head of the machine Performance Criteria 6: Clean the machine card through air blower carefully Performance Criteria 7: Perform dusting of the machine Performance Criteria 8: Clean the oil pump passage and stainer if available Performance Criteria 9: Shower oil on the needle bar and shuttle with shower gun as required Performance Criteria 10: Check oil gauge of the machine if available …………………………….. |  |  |  |
|  | Assessment Task 2: Perform needle and shuttle adjustment and also change the belt of the machine Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs Performance Criteria 4: Loose the needle screw to adjust the needle Performance Criteria 5: Put the needle in the needle bar by keeping short groove of the needle on the backside Performance Criteria 6: Adjust needle long groove on the front side Performance Criteria 7: Tight the screw on the needle bar Performance Criteria 8: Open the needle plate by unscrewing Performance Criteria 9: Unscrew the shuttle by moving shaft accordingly Performance Criteria 10: Set timing of the main shaft as per machine specification Performance Criteria11: Tight screws of the shuttle and needle plate for closing Performance Criteria 12: Locate the safety cover on the motor belt Performance Criteria 13: Unscrew the safety cover of the motor belt Performance Criteria 14: Replace the motor belt carefully Performance Criteria 15: Put the safety cover with screws …………………………….. |  |  |  |

Continued on following page

Assessors Judgment Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |  |  |  |  |  |  |  |
| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |  |
| Activity |  | Method |  |  |  |  | Result |  |
| Nature of Activity |  | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |  |

| Each Assessment Task (with performance criteria) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assessment Task 1: Perform cleaning and oiling of machine for smooth operation |  |  | Description of assessment task 1 |  |  |  |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - 1 | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
| - 2 | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
|  | Performance Criteria 4: Clean the shuttle with blower or oil |  |  |  |  |  |
|  | Performance Criteria 5: Clean the head of the machine |  |  |  |  |  |
|  | Performance Criteria 6: Clean the machine card through air blower carefully |  |  |  |  |  |
| - 4 | Performance Criteria 7: Perform dusting of the machine |  |  |  |  |  |
|  | Performance Criteria 8: Clean the oil pump passage and stainer if available |  |  |  |  |  |
|  | Performance Criteria 9: Shower oil on the needle bar and shuttle with shower gun as required |  |  |  |  |  |
|  | Performance Criteria 10: Check oil gauge of the machine if available |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 2: Perform needle and shuttle adjustment and also change the belt of the machine |  |  | Description of assessment task 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - 1 | Performance Criteria 1: Maintain personal hygiene and wear personal protective equipment |  |  |  |  |  |
| - 2 | Performance Criteria 2: Segregate and disposing hazardous and non-hazardous waste as per standard operating procedures |  |  |  |  |  |
| - 3 | Performance Criteria 3: Receive and carry out instructions from supervisor and reports problems as per organizational SOPs |  |  |  |  |  |
|  | Performance Criteria 4: Loose the needle screw to adjust the needle |  |  |  |  |  |
|  | Performance Criteria 5: Put the needle in the needle bar by keeping short groove of the needle on the backside |  |  |  |  |  |
|  | Performance Criteria 6: Adjust needle long groove on the front side |  |  |  |  |  |
| - 4 | Performance Criteria 7: Tight the screw on the needle bar |  |  |  |  |  |
| - 5 | Performance Criteria 8: Open the needle plate by unscrewing |  |  |  |  |  |
| - 6 | Performance Criteria 9: Unscrew the shuttle by moving shaft accordingly |  |  |  |  |  |
| - 7 | Performance Criteria 10: Set timing of the main shaft as per machine specification |  |  |  |  |  |
| - 8 | Performance Criteria11: Tight screws of the shuttle and needle plate for closing |  |  |  |  |  |
| - 9 | Performance Criteria 12: Locate the safety cover on the motor belt |  |  |  |  |  |
| - 10 | Performance Criteria 13: Unscrew the safety cover of the motor belt |  |  |  |  |  |
| - 11 | Performance Criteria 14: Replace the motor belt carefully |  |  |  |  |  |
| - 12 | Performance Criteria 15: Put the safety cover with screws |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |