|  | # Written Assessment Guide # For # National Vocational Qualification Level-3 in Jewellery CAD-CAM |
| --- | --- |
|  | March 2020 |
|  | National Vocational & Technical Training Commission Government of Pakistan |

| Title of Qualification: National Vocational Qualification Level-3 in Jewellery CAD-CAM |  | CS Code: | Level: 03 | Version: 01 |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Prepare Drawing of Basic Jewellery Article Manually Create Computer Aided Drawing of Basic Level Jewellery Create Computer Aided Drawing of Intermediate Level Jewellery Create Computer Aided Drawing of Advance Level Jewellery Produce Prototype of Jewellery Article using 3D Printer |  | Assessment Date (DD/MM/YY): |  |  |
| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |  |  |  |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |
| --- | --- |
| Written Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |

| Title of Qualification: National Vocational Qualification Level-3 in Jewellery CAD-CAM | CS Code: | Level: 03 | Version: 01 |
| --- | --- | --- | --- |
| Competency Standard Title: Prepare Drawing of Basic Jewellery Article Manually Create Computer Aided Drawing of Basic Level Jewellery Create Computer Aided Drawing of Intermediate Level Jewellery Create Computer Aided Drawing of Advance Level Jewellery Produce Prototype of Jewellery Article using 3D Printer | Assessment Date (DD/MM/YY): |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| Differentiate between 2D and 3D. | Two dimensional forms have width and height. Three dimensional form has width ,Length and height |
| Define Jewellery. | Jewellery is a decorative ornament to worn on body which may be an abstract or depict a design theme having proportion, rhythm, function and wearable features. |
| Give few names of Precious Metals. | Silver, Gold, Platinum, Palladium etc. |
| Give few names of Non Precious Metals. | Brass, Copper, Bronze, Lead ,Zinc etc. |
| Give few names of natural occurring gem stones. | Diamond, Ruby, Emerald, Sapphire, Aquamarine, Tourmaline, Opal, Pearl, Agate, Amethyst, Mozanite, Zirconia, Coral etc. |
| Give few types of line. | Horizontal, vertical, perpendicular, parallel, Zigzag, Spiral, Curved, Diagonal lines. |
| Define Hatching. | These are the combination of horizontal , vertical or both and diagonal lines used to create texture, value, section and the illusion of form |
| Define Motif. | It is defined as a recurrent of image, idea or symbol that develops or explains the theme. |
| Define Article. | It is defined as a combination of motives in repetitive order with or without moving links that shows functional, aesthetic and wearable features. |
| Define two main measuring systems. | American System (S.I) and British System (B.S) |
| Name the views of Orthographic projections. | Top View, Front View, and Side View. |
| Write the names of any three Types of Dimensioning. | Parallel, Superimposed Running, Chain, Combined and Radii Dimensions |
| Define Isometric View. | It is a two-dimensional representation for viewing a three-dimensional object. The angles to draw the isometric projections are 30 and 60 degree. |
| Differentiate between CAD and CAM. | Computer Aided Design & Computer Aided Manufacturing |
| Write the name View ports in software. | Top, Front, Side, Perspective |
| Write the name of basic jewellery motifs /articles. | Ring, Pendants ,Earrings, Cufflinks, Bangles |
| Write the names of major commands in Curves menus of Jewellery CAD software. | Single Line, Polyline, Interp Curve, Circle, Arc, Rectangle, and Curve on surface etc. |
| Write the names of major commands in Surface menus of Jewellery CAD software. | Sweep1, Sweep2, Loft, Offset Surface, Patch etc. |
| What are the methods of precision and accuracy through software? | Grid Snap, O-snap |
| What are the main shapes of gems? | Round, Princess, Marquise, Pear, Radiant etc. |
| What are the basic types of Stone Settings? | Prong, Bezel, Flush, Channel, Pave settings. |
| Write the names of major commands in Transform menus of Jewellery CAD software. | Move, Array, Scale, Shear, Orient, Mirror, Rotate etc. |
| Differentiate between Carat and Karat. | Carat is a weight unit for gems and Karat measures the purity of gold. |
| Write the names of few Precious & semi-precious metals. | Gold, Silver, Titanium, Copper etc. |
| Define Rendering Phenomenon. | Effect of light on article to highlight its materials in photo realistic environment. |
| Write the names of rendering plugin in software. | V-Ray |
| Write the names of few picture file format(s). | jpeg, bmp, tga etc. |
| Write the names few colours/tones of jewellery metals or alloy. | Yellow gold, Rose gold, Rhodium plated gold, White gold etc. |
| What are the basic types of shank? | Traditional , Euro Style, Taper, Crossover/Bypass, Split , Cathedral Shanks etc. |
| Write down the names of Jewellery Manufacturing Processes. | Designing/Drafting ,Wax Carving , Casting ,Finishing ,Polishing ,Mould Making ,Electroplating ,Stone Setting |
| Write the name of operation used for material subtraction and addition. | Boolean Operations |
| Write the name of different Boolean operations. | Boolean Difference/Subtraction , Boolean Union , Boolean Intersection |
| Write the name of few intermediate types of Stone Settings. | Pave, Cluster, Tension setting |
| Write the names of 05 commands of Transform Menu. | Scale ,Shear, Orient ,Move, Mirror etc. |
| What is the function of command “Array Polar”? | It creates multiple copies of an object in circular pattern from 0 to 360 degree. |
| What is the difference between “Helix and Spiral” command? | Helix creates spring form with same diameter at start and end Spiral creates spring form with different diameter at start and end |
| What is the difference between commands, Scale-1D, Scale-2D, Scale-3D? | Scale 1D: It increases or decrease any one dimension i.e. Length or Width or Thickness Scale 2D: It increases or decrease any two(2) dimensions i.e. Length x Width or Length x Thickness or Width x Thickness. Scale 3D: It increases or decrease all three (3) dimensions together. |
| What is the function of command “Flow along surface”? | It transfers any object or design to target surface at a certain height in space. |
| How can we make customize back ground in rendering? | Any HD picture is imported in ground plane to declare it customize back ground and environment. |
| Write the name of at least 05 ground planes. | Tile ,Wood ,Architecture, Fabric |
| Define Casting process of Jewellery? | Shaping of an object by pouring of molten metal in to mould. |
| Write down the function of command “Pave Builder”. | This command lies in Gem menu and load cluster of gems with specified micro prongs |
| Write down the function of command “Smart Flow”. | It lies in Transform Menu and transforms the 3D objects, gems, Curves on the specified surface in space simultaneously by working on base surface. |
| Write down the function of command “Cage Edit”. | It lies in Transform Menu and creates deformation and bends on 3D solid object by activating its control points through bounding box. |
| Write down major extensions of CAM files. | STL,SLC |
| What are the advance types of Stone Settings. | Illusion, Invisible ,Cluster, Tension settings |
| Write few names of advance commands of surface menu. | Loft ,Revolve ,Rail Revolve, Profile Sweep , Curve Network, Blend Surface etc. |
| Define cabochon. | The shape cabochon refers to a piece of gemstone (or other material) which has been shaped and polished, instead of faceted. |
| How can we make customize back ground in rendering? | We can import HD picture though back ground bitmap. |
| What commands are used for Fish Tail Setting? | Micro Prong Cutters, Bright Cut Channel |
| Write few names of advance commands of Curve menu. | Create UV ,Apply UV ,Project ,Pullback, Helix, Curve from 2 views, Angle ISO-Curve, Duplicate Edge etc. |
| Write few names of advance commands of Tools menu. | Custom Rail, Profile End Cape, Centre Line Builder, Pattern Builder, Object on Curve etc. |
| Write few names of advance commands of Solid Menu. | Move Face, Array Hole ,Chamfer Edge, Fillet Edge, Rib etc. |
| Write few names of advance commands of Transform Menu. | Cage Edit, Smart Flow, Flow along Surface ,Splop, Maelstrom |
| Write the name of few moveable parts of bangles and bracelets. | Clasp, Open Box, S-Hook, Clutch, Figure & Safety etc. |
| Define Casting process of Jewellery. | Shaping of an object by pouring of molten metal in to mould. |
| Write down the function of command “Smart Flow”. | It lies in Transform Menu and transforms the 3D objects, gems, Curves on the specified surface in space simultaneously by working on base surface. |
| Write down the function of command “Cage Edit”. | It lies in Transform Menu and creates deformation and bends on 3D solid object by activating its control points through bounding box. |
| Write down major extensions of CAM files. | STL,SLC |
| What are the advance types of Stone Settings? | Illusion, Invisible ,Cluster, Tension settings |
| What commands are used for Fish Tail Setting? | Micro Prong Cutters, Bright Cut Channel |
| Write few names of advance commands of Curve menu. | Create UV ,Apply UV ,Project ,Pullback, Helix, Curve from 2 views, Angle ISO-Curve, Duplicate Edge etc. |
| Write few names of advance commands of Tools menu. | Custom Rail, Profile End Cape, Centre Line Builder, Pattern Builder, Object on Curve etc. |