| DACUM Facilitator: BASHIR AGHA (Principal GJTMC, Quetta) Co-Facilitator: Muhammad Salman Butt (CEO Aspire Consult) | # Summative Assessment Evidence Guide # For # National Vocational Qualification Level-3 in Jewellery Electroplating |
| --- | --- |
|  | March, 2020 |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: National Vocational Qualification Level-3 in Jewellery Electroplating |  | CS Code: |  | Level: 03 | Version: 01 |
| --- | --- | --- | --- | --- | --- |
| Competency Standard Titles: COMPLY WITH PERSONAL HEALTH AND SAFETY GUIDELINES PERFORM PRE-TREATMENT OF JEWELLERY ARTICLE PERFORM ELECTROPLATING OF JEWELLERY ARTICLE PERFORM POST-TREATMENT OF PLATED ARTICLE RECOVER PRECIOUS METALS DEVELOP ENTREPRENEURIAL SKILLS |  |  | Assessment Date (DD/MM/YY): |  |  |
| Candidate Details | Name: Registration/Roll Number: |  |  |  |  |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment): Assessment Task 1: Comply with personal health and safety guidelines during all assessment tasks Assessment Task 2: Perform Pre-treatment of Jewellery Article as assigned by the assessor Assessment Task 3: Perform electroplating of jewellery article using any of the following applicable methods as assigned by assessor Assessment Task 4: Perform post-treatment of plated article using any of the following applicable methods as assigned by assessor And complete: - Knowledge assessment test (Written or Oral) - Portfolios at the time of assessment (if any) |  |  |  |  |

| Minimum Evidence Required | During a practical assessment, under observation by an assessor, you will complete all of the assessment task as per instructions by assessor: Assessment Task 1: Comply with personal health and safety guidelines during all assessment tasks P1: Select personal protective equipment in terms of type and quantity according to work orders. P2. Wear PPE according to job requirements P3. Maintain cleanliness and hygiene P4. Identify and segregate the hazardous and non-hazardous waste materials that need to be disposed, and use proper disposal containers to dispose-off hazardous waste as per procedure Assessment Task 2: Perform Pre-treatment of Jewellery Article as assigned by the assessor P1: Check for any surface defects including marks, scratches and roughness P2. Segregate jewellery articles according to quality P3. Perform buffing to polish the surface of the jewellery article P4. Inspect for faulty hinges and soldered joints. P5. Clean Jewellery article using method(s) assigned by assessor (assessor may assign multiple methods if needed for the operation) Option-1: Perform steam cleaning - Setup steamer for cleaning process. - Clean jewellery article with steam ensuring the articles are free of any deposits Option-2: Perform ultrasonic cleaning - Prepare solution for ultrasonic cleaning. - Adjust temperature and frequency parameters. - Fix the article in jig and clean jewellery article using ultrasonic machine for required time - Rinse article with water to remove cleaning media. - Inspect cleaned surface of the article. Option-3: Perform alkali cleaning - Prepare recipe of the alkali cleaning solution as per jewellery metal. - Mix ingredients to make alkaline solution for cleaning - Label solution container mentioning the ingredients and hazards of the solution. - Fix the article in jig and clean jewellery article using alkali cleaning bath for required time - Rinse article with distilled water to remove cleaning media. Option-4: Perform electrolytic cleaning - Prepare electrolytic cleaning solution as per recipe. - Connect jewellery article with electrode in electrolytic cleaning apparatus. - Adjust electric current and voltage parameters. - Clean article for required time - Rinse article with distilled water to remove cleaning media. Option-5: Perform acid activation of the surface - Prepare recipe of the acidic cleaning solution as per jewellery metal. - Mix ingredients to make acidic solution for cleaning - Label solution container mentioning the ingredients and hazards of the solution. - Fix the article in jig and clean jewellery article using acidic cleaning bath for required time - Rinse article with distilled water to remove cleaning media. P6. Perform Electroless plating on complex jewellery article (optional as per requirements of jewellery article) - Prepare recipe of the Electroless plating solution and mix ingredients as per jewellery metal. - Fix the article in jig and perform Electroless plating of jewellery article as per requirement - Rinse article with distilled water to remove cleaning media P7. Perform masking for multi-tone plating (optional as per requirements of jewellery article) - Prepare masking paint as per requirement of the jewellery article. - Perform masking on required portion of jewellery article - Hang the article for drying after masking Assessment Task 3: Perform electroplating of jewellery article using any of the following applicable methods as assigned by assessor Option-1: Perform electroplating of jewellery article - Set operating parameters (temperature, pH, voltage, and current density) as per requirement of the article. - Adjust anode/cathode surface area ratio. - Connect electrodes with power supply. - Immerse jewellery article in electroplating bath. - Perform electroplating with provided metal and rinse article with distilled water to remove electrolyte. Option-2: Perform alloy plating - Set operating parameters (temperature, pH, voltage, and current density) as per requirement of the article. - Adjust anode/cathode surface area ratio - Connect electrodes with power supply - Immerse jewellery article in alloy plating bath. - Perform alloy plating with provided metal and rinse article with distilled water to remove electrolyte. Option-3: Perform/ Demonstrate pen plating - Set operating parameters of pen plating unit. - Dip the tip of plating pen into electroplating solution. - Mark the parts of jewellery article with the help of plating pen’s tip where plating is required. - Rinse article with distilled water to remove electrolyte. Assessment Task 4: Perform post-treatment of plated article using any of the following applicable methods as assigned by assessor Option-1: Apply inorganic protective coating - Remove masking by solvent and perform ultrasonic cleaning if required. - Prepare inorganic protective coating solution as per recipe - Apply protective coating - Cure protective coating by air drying / heat drying as per provided equipment Option-2: Apply organic protective coating - Remove masking by solvent and perform ultrasonic cleaning if required. - Prepare organic protective coating solution as per recipe - Apply protective coating - Cure protective coating by Ultra Violet/ heat drying as per provided equipment Option-3: Apply electrophoretic composite coating - Remove masking by solvent and perform ultrasonic cleaning if required. - Prepare electrophoretic composite coating solution as per recipe - Setup workstation for electrophoretic composite coating - Perform electrophoretic protective coating - Cure protective coating by heat drying |
| --- | --- |
|  | Portfolios required at the time of assessment (if any) P1: Diary log of work completed on complying with personal health and safety guidelines P2: Diary log of work completed on recovering precious metal P3: Diary log of work completed on developing entrepreneurship skills |

Continued on following page

Assessors Judgment Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |  |  |  |  |  |  |  |
| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |  |
| Activity |  | Method |  |  |  |  | Result |  |
| Nature of Activity |  | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |  |

| Assessment Task 1 |  |  | Description of assessment task 1 Comply with personal health and safety guidelines during all assessment tasks |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1 | Select personal protective equipment in terms of type and quantity according to work orders. |  |  |  |  |  |
| 2 | Wear PPE according to job requirements |  |  |  |  |  |
| 3 | Maintain cleanliness and hygiene |  |  |  |  |  |
| 4 | Identify and segregate the hazardous and non-hazardous waste materials that need to be disposed, and use proper disposal containers to dispose-off hazardous waste as per procedure |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 2 |  |  | Description of assessment task 2 Perform Pre-treatment of Jewellery Article as assigned by the assessor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1 | Check for any surface defects including marks, scratches and roughness |  |  |  |  |  |
| 2 | Segregate jewellery articles according to quality |  |  |  |  |  |
| 3 | Perform buffing to polish the surface of the jewellery article |  |  |  |  |  |
| 4 | Inspect for faulty hinges and soldered joints. |  |  |  |  |  |
| 5 | Clean Jewellery article using method(s) assigned by assessor (assessor may assign multiple methods if needed for the operation) |  |  |  |  |  |
|  | Option-1: Perform steam cleaning - Setup steamer for cleaning process. - Clean jewellery article with steam ensuring the articles are free of any deposits |  |  |  |  |  |
|  | Option-2: Perform ultrasonic cleaning - Prepare solution for ultrasonic cleaning. - Adjust temperature and frequency parameters. - Fix the article in jig and clean jewellery article using ultrasonic machine for required time - Rinse article with water to remove cleaning media. - Inspect cleaned surface of the article. |  |  |  |  |  |
|  | Option-3: Perform alkali cleaning - Prepare recipe of the alkali cleaning solution as per jewellery metal. - Mix ingredients to make alkaline solution for cleaning - Label solution container mentioning the ingredients and hazards of the solution. - Fix the article in jig and clean jewellery article using alkali cleaning bath for required time - Rinse article with distilled water to remove cleaning media. |  |  |  |  |  |
|  | Option-4: Perform electrolytic cleaning - Prepare electrolytic cleaning solution as per recipe. - Connect jewellery article with electrode in electrolytic cleaning apparatus. - Adjust electric current and voltage parameters. - Clean article for required time - Rinse article with distilled water to remove cleaning media. |  |  |  |  |  |
|  | Option-5: Perform acid activation of the surface - Prepare recipe of the acidic cleaning solution as per jewellery metal. - Mix ingredients to make acidic solution for cleaning - Label solution container mentioning the ingredients and hazards of the solution. - Fix the article in jig and clean jewellery article using acidic cleaning bath for required time - Rinse article with distilled water to remove cleaning media. |  |  |  |  |  |
| 6 | Perform Electroless plating on complex jewellery article (optional as per requirements of jewellery article) |  |  |  |  |  |
|  | Prepare recipe of the Electroless plating solution and mix ingredients as per jewellery metal. |  |  |  |  |  |
|  | Fix the article in jig and perform Electroless plating of jewellery article as per requirement |  |  |  |  |  |
|  | Rinse article with distilled water to remove cleaning media |  |  |  |  |  |
| 7 | Perform masking for multi-tone plating (optional as per requirements of jewellery article) |  |  |  |  |  |
|  | Prepare masking paint as per requirement of the jewellery article. |  |  |  |  |  |
|  | Perform masking on required portion of jewellery article |  |  |  |  |  |
|  | Hang the article for drying after masking |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 3 |  |  | Description of assessment task 3 Perform electroplating of jewellery article using any of the following applicable methods as assigned by assessor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| Option-1 | Perform electroplating of jewellery article |  |  |  |  |  |
|  | Set operating parameters (temperature, pH, voltage, and current density) as per requirement of the article. |  |  |  |  |  |
|  | Adjust anode/cathode surface area ratio. |  |  |  |  |  |
|  | Connect electrodes with power supply. |  |  |  |  |  |
|  | Immerse jewellery article in electroplating bath. |  |  |  |  |  |
|  | Perform electroplating with provided metal and rinse article with distilled water to remove electrolyte. |  |  |  |  |  |
| Option-2 | Perform alloy plating |  |  |  |  |  |
|  | Set operating parameters (temperature, pH, voltage, and current density) as per requirement of the article. |  |  |  |  |  |
|  | Adjust anode/cathode surface area ratio |  |  |  |  |  |
|  | Connect electrodes with power supply |  |  |  |  |  |
|  | Immerse jewellery article in alloy plating bath. |  |  |  |  |  |
|  | Perform alloy plating with provided metal and rinse article with distilled water to remove electrolyte. |  |  |  |  |  |
| Option-3 | Perform/ Demonstrate pen plating |  |  |  |  |  |
|  | Set operating parameters of pen plating unit. |  |  |  |  |  |
|  | Dip the tip of plating pen into electroplating solution. |  |  |  |  |  |
|  | Mark the parts of jewellery article with the help of plating pen’s tip where plating is required. |  |  |  |  |  |
|  | Rinse article with distilled water to remove electrolyte. |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 4 |  |  | Description of assessment task 4 Perform post-treatment of plated article using any of the following applicable methods as assigned by assessor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| Option-1 | Apply inorganic protective coating |  |  |  |  |  |
|  | Remove masking by solvent and perform ultrasonic cleaning if required. |  |  |  |  |  |
|  | Prepare inorganic protective coating solution as per recipe |  |  |  |  |  |
|  | Apply protective coating |  |  |  |  |  |
|  | Cure protective coating by air drying / heat drying as per provided equipment |  |  |  |  |  |
| Option-2 | Apply organic protective coating |  |  |  |  |  |
|  | Remove masking by solvent and perform ultrasonic cleaning if required. |  |  |  |  |  |
|  | Prepare organic protective coating solution as per recipe |  |  |  |  |  |
|  | Apply protective coating |  |  |  |  |  |
|  | Cure protective coating by Ultra Violet/ heat drying as per provided equipment |  |  |  |  |  |
| Option-3 | Apply electrophoretic composite coating |  |  |  |  |  |
|  | Remove masking by solvent and perform ultrasonic cleaning if required. |  |  |  |  |  |
|  | Prepare electrophoretic composite coating solution as per recipe |  |  |  |  |  |
|  | Setup workstation for electrophoretic composite coating |  |  |  |  |  |
|  | Perform electrophoretic protective coating |  |  |  |  |  |
|  | Cure protective coating by heat drying |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Portfolio (if any) |  | Description of portfolio Diary log of completed work |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Current Sufficient Authentic Valid Reliable |  |  |  |  |  |
| Portfolio meet the following performance standards: |  |  | Yes | No | Remarks |
| 1 | Diary log of work completed on complying with personal health and safety guidelines |  |  |  |  |
| 2 | Diary log of work completed on recovering precious metal |  |  |  |  |
| 3 | Diary log of work completed on developing entrepreneurship skills |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |