|  | # Written Assessment Guide # For National Vocational Qualification Level-3 in Jewellery Electroplating |
| --- | --- |
|  | Date |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: National Vocational Qualification Level-3 in Jewellery Electroplating |  | CS Code: | Level: 03 | Version: 01 |
| --- | --- | --- | --- | --- |
| Competency Standard Title: PERFORM ELECTROPLATING OF JEWELLERY ARTICLE |  | Assessment Date (DD/MM/YY): |  |  |
| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |  |  |  |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |
| --- | --- |
| Written Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |

| Title of Qualification: National Vocational Qualification Level-3 in Jewellery Electroplating | CS Code: | Level: 03 | Version: 01 |
| --- | --- | --- | --- |
| Competency Standard Title: PERFORM ELECTROPLATING OF JEWELLERY ARTICLE | Assessment Date (DD/MM/YY): |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| What is electroplating? | The process that uses electric current to reduce dissolved metal cations so that they form a thin coherent metal coating on an electrode. |
| What is purpose of electroplating? | Major purposes of electroplating are; - Achieve required appearance - Corrosion Protection - Add Engineering or mechanical properties to Jewellery |
| List down the major parts of electroplating unit. | - Cathode - Anode - Electrolyte - D.C supply - Electroplating bath |
| What are important parameters of electroplating? | - Control of electrolyte formulation and pH - Control of anode surface area and position - Electrical conditions - Temperature of electrolyte - Agitation |
| What is the purpose of using additives in electroplating bath? | Major purposes of additives are; - Brightening - Levelling - Grain refining |
| Is Jewellery article used as cathode or anode? And why? | Jewellery article is used as cathode to coat it with selected (anode) metal. |
| How are cathode and anode connected in electroplating? | Cathode (Jewellery Article) is connected to Negative (-ve) terminal whereas Anode (Metal electrode) is connected to Positive (+ve) terminal of electrical supply. |
| What is current density? | Current density is the current supplied per unit square area of article |
| What is specific gravity? | It tells the density of solution with respect to water. |
| What is the unit of thickness used to measure plated layer? | Thickness is usually measured in Microns. |
| What is the composition of nickel plating bath? | - Nickel sulphate - Nickel chloride - Boric acid - Water |
| What is composition of copper plating bath? | - Copper salt - Water - Additives |
| What is the composition of silver-plating bath? | Silver salt and conducting salt with water |
| What is the use of Hull cell? | Hull cell is used for qualitative analysis of bath used for plating |