|  | Assessment Evidence Guide For Level 4 “Networking and Cyber Security “ (Summative Assessment) |
| --- | --- |
|  | Date 13-17 November, 2023 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | NVC Level 4 - Networking and Cyber Security |
| --- | --- |
| Competency Standard | - Perform Network Connectivity and Communications - Develop Scalable Addressing Schemes - Perform Basic Network switching - Choose the right network for improved bandwidth and efficiency - Configure and secure WLANs - Configure routing protocols and static routes on routers - Implement Network security protocol and practices - Configure WAN Services and protocols - Analyse Security Threats and Vulnerabilities - Manage security risk and assessment - Perform Network Security - Implement Cryptography and Access Control - Perform Digital Forensic on sample data |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment) details given as: Networking: Perform the following as per given Annexure-A - Design Network diagram as shown in the Annexure A - Configure Static Trunk between "SW A and SW B” and Static Trunk between “SW C and SW D” as per Annexure A - Configure DHCP Services over PAK Router and Configure Inter-VLAN Routing over UAE Router as per Annexure A - Configure Routing Protocol as shown as Annexure A - Configure Loop Back Interfaces as per Annexure A - Configure BGP and Perform Convergence Cyber Security : Perform the following as per given Annexure-B - Set up a vulnerable web application - Set Up Metasploitable 2 Linux Virtual Machine on VMware workstation - access the web applications, using web browser as per diagram - Launch SQL injection attack - Launch and verify cross site scripting attack on target web application |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Task 1 Design Network diagram as shown in the Annexure A - Select the suitable simulator to design the network - Place the components as per given details in annexure A Task 2 Configure Static Trunk between "SW A and SW B” and Static Trunk between “SW C and SW D” as per Annexure A - Access Switches and enter in global configuration - Configure Trunk Interface on SW-A and SW-B - Configure Trunk Interface on SW-C and SW-D - Test VLAN Traffic over Trunk Task 3 Configure DHCP Services over PAK Router, and Configure Inter-VLAN Routing over UAE Router as per Annexure A - Create Two sub interface and assign IPs as per annexure A - Create DHCP pool over PAK Router. - Exclude IP Addresses of Sub interfaces over UAE Router as per annexure - Verify the routing tables Task 4 Configure Routing Protocol as shown as Annexure A - Configure OSPF 1 over all Routers and keep in mind about the areas as per annexure - Configure Router PAK, R2 and R5 for the participation of DR and BDR Election - Configure Router PAK, R4 and R7 for the participation of DR and BDR Election - Configure Virtual link between R2 and R4 Task 5 Configure Loop Back Interfaces as per annexure A - Create Loop 0 over PAK router and IP 100.1.1.1/32 - Create Loop 0 over UAE router and IP 200.1.1.1/32 Task 6 Configure BGP and Perform Convergence - Configure BGP Neighbor relationship between PAK and BGP using Loop back IPs - Make sure BGP Table is established - Test and verify BGP Cyber Security : Perform the following as per given Annexure-B Task 1 Set up a vulnerable web application - Set Up Metasploitable 2 Linux Virtual Machine on VMware workstation - access the web applications, using web browser as per diagram Task 2 Launch SQL injection attack - Open browser, enter URL localhost/dvwa/login.php. - Login using the username "admin" and password as "password" - Set the DVWA security to LOW. - Click on SQL injection on the left-menu - On User ID field, enter "1" and click Submit. It will print the ID, First_name, and Surname on the screen - Use always true scenario to extract the First_names and Surnames from the database using input: %' or '1'='1' Task 3: Launch and verify cross site scripting attack on target web application. - Configure Stored XSS on DVWA with low security - Run DVWA warmup , type just the basic exploit into the textbox “Message”. - Write: <script>alert('You have been hacked!');</script> - Set Stored XSS on DVWA with high security - Increase the maxlength attribute of the name - Type <h1>StackZero</h1> - Press Sign Guestbook button - Verify the exploit by a random comment (the field is mandatory), - Ensure XSS exploit the page reloads successful |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | NVC Level 4 - Networking and Cyber Security |
| Competency Standards | - Perform Network Connectivity and Communications - Develop Scalable Addressing Schemes - Perform Basic Network switching - Choose the right network for improved bandwidth and efficiency - Configure and secure WLANs - Configure routing protocols and static routes on routers - Implement Network security protocol and practices - Configure WAN Services and protocols - Analyze Security Threats and Vulnerabilities - Manage security risk and assessment - Perform Network Security - Implement Cryptography and Access Control - Perform Digital Forensic on sample data |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Task 1 Design Network diagram as shown in the Annexure A - Select the suitable simulator to design the network - Place the components as per given details in annexure A Task 2 Configure Trunk between "SW A and SW B” and Trunk between “SW C and SW D” as per Annexure A - Access Switches and enter in global configuration - Configure Trunk Interface on SW-A and SW-B - Configure Trunk Interface on SW-C and SW-D - Test VLAN Traffic over Trunk Task 3 Configure DHCP Services over PAK Router, and Configure Inter-VLAN Routing over UAE Router as per Annexure A - Create Two sub interface and assign IPs as per annexure A - Create DHCP pool over PAK Router. - Exclude IP Addresses of Sub interfaces over UAE Router as per annexure - Verify the routing tables Task 4 Configure Routing Protocol as shown as Annexure A - Configure OSPF 1 over all Routers and keep in mind about the areas as per annexure - Configure Router PAK, R2 and R5 for the participation of DR and BDR Election - Configure Router PAK, R4 and R7 for the participation of DR and BDR Election - Configure Virtual link between R2 and R4 Task 5 Configure Loop Back Interfaces as per annexure A - Create Loop 0 over PAK router and IP 100.1.1.1/32 - Create Loop 0 over UAE router and IP 200.1.1.1/32 Task 6 Configure BGP and Perform Convergence - Configure BGP Neighbor relationship between PAK and BGP using Loop back IPs - Make sure BGP Table is established - Test and verify BGP Cyber Security : Perform the following as per given Annexure-B Task 1 Set up a vulnerable web application - Set Up Metasploitable 2 Linux Virtual Machine on VMware workstation - access the web applications, using web browser as per diagram Task 2 Launch SQL injection attack Launch and verify cross site scripting attack on target web application - Open browser, enter URL localhost/dvwa/login.php. - Login using the username "admin" and password as "password" - Set the DVWA security to LOW. - Click on SQL injection on the left-menu - On User ID field, enter "1" and click Submit. It will print the ID, First_name, and Surname on the screen - Use always true scenario to extract the First_names and Surnames from the database using input: %' or '1'='1' Task 3: Launch and verify cross site scripting attack on target web application. - Configure Stored XSS on DVWA with low security - Run DVWA warmup , type just the basic exploit into the textbox “Message”. - Write: <script>alert('You have been hacked!');</script> - Set Stored XSS on DVWA with high security - Increase the maxlength attribute of the name - Type <h1>StackZero</h1> - Press Sign Guestbook button - Verify the exploit by a random comment (the field is mandatory), - Ensure XSS exploit the page reloads successful |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the suitable simulator to design the network |  |  |
| - Place the components as per given details in annexure A |  |  |
| - Access Switches and enter in global configuration |  |  |
| - Configure Trunk Interface on SW-A and SW-B |  |  |
| - Configure Trunk Interface on SW-C and SW-D |  |  |
| - Test VLAN Traffic over Trunk |  |  |
| - Create Two sub interface and assign IPs as per annexure A |  |  |
| - Create DHCP pool over PAK Router |  |  |
| - Exclude IP Address of Sub interfaces as per annexure |  |  |
| - Verify the routing tables |  |  |
| - Configure OSPF 1 over all Routers and keep in mind about the areas as per annexure |  |  |
| - Configure Router PAK, R2 and R5 for the participation of DR and BDR Election |  |  |
| - Configure Router PAK, R4 and R7 for the participation of DR and BDR Election |  |  |
| - Configure Virtual link between R2 and R4 |  |  |
| - Create Loop 0 over PAK router and IP 100.1.1.1/32 |  |  |
| - Create Loop 0 over UAE router and IP 200.1.1.1/32 |  |  |
| - Configure BGP Neighbor relationship between PAK and BGP using Loop back IPs |  |  |
| - Make sure BGP Table is established |  |  |
| - Test and verify BGP |  |  |
| Cyber Security |  |  |
| - Set Up Metasploitable 2 Linux Virtual Machine on VMware workstation |  |  |
| - access the web applications, using web browser as per diagram |  |  |
| - Open browser, enter URL localhost/dvwa/login.php |  |  |
| - Login using the username "admin" and password as "password" |  |  |
| - Set the DVWA security to LOW. |  |  |
| - Click on SQL injection on the left-menu |  |  |
| - On User ID field, enter "1" and click Submit. |  |  |
| - Use always true scenario to extract the First_names and Surnames from the database using input: %' or '1'='1' |  |  |
| - Configure Stored XSS on DVWA with low security |  |  |
| - Run DVWA warmup , type just the basic exploit into the textbox “Message”. |  |  |
| - Write: <script>alert('You have been hacked!');</script> |  |  |
| - Set Stored XSS on DVWA with high security |  |  |
| - Increase the maxlength attribute of the name |  |  |
| - Type <h1>StackZero</h1> |  |  |
| - Press Sign Guestbook button |  |  |
| - Verify the exploit by a random comment (the field is mandatory), |  |  |
| - Ensure XSS exploit the page reloads successful |  |  |

Candidate’s Signature__________________ Assessor’s Signature______________

Date: ______________________________

Assessors Judgment Guide

| Qualification | NVC Level 4 - Networking and Cyber Security |
| --- | --- |
| Competency Standard | - Perform Network Connectivity and Communications - Develop Scalable Addressing Schemes - Perform Basic Network switching - Choose the right network for improved bandwidth and efficiency - Configure and secure WLANs - Configure routing protocols and static routes on routers - Implement Network security protocol and practices - Configure WAN Services and protocols - Analyse Security Threats and Vulnerabilities - Manage security risk and assessment - Perform Network Security - Implement Cryptography and Access Control - Perform Digital Forensic on sample data |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Perform leak testing, evacuation, and gas charging of refrigerator as per given requirement. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Selected the suitable simulator to design the network |  |  |  |  |  |
|  | Placed the components as per given details in annexure A |  |  |  |  |  |
|  | Accessed Switches and enter in global configuration |  |  |  |  |  |
|  | Configured Trunk Interface on SW-A and SW-B |  |  |  |  |  |
|  | Configured Trunk Interface on SW-C and SW-D |  |  |  |  |  |
|  | Tested VLAN Traffic over Trunk |  |  |  |  |  |
|  | Created Two sub interface and assign IPs as per annexure A |  |  |  |  |  |
|  | Created DHCP pool over PAK Router |  |  |  |  |  |
|  | Excluded IP Address of Sub interfaces as per annexure |  |  |  |  |  |
|  | Verified the routing tables |  |  |  |  |  |
|  | Configured OSPF 1 over all Routers and keep in mind about the areas as per annexure |  |  |  |  |  |
|  | Configured Router PAK, R2 and R5 for the participation of DR and BDR Election |  |  |  |  |  |
|  | Configured Router PAK, R4 and R7 for the participation of DR and BDR Election |  |  |  |  |  |
|  | Configured Virtual link between R2 and R4 |  |  |  |  |  |
|  | Created Loop 0 over PAK router and IP 100.1.1.1/32 |  |  |  |  |  |
|  | Created Loop 0 over UAE router and IP 200.1.1.1/32 |  |  |  |  |  |
|  | Configured BGP Neighbor relationship between PAK and BGP using Loop back IPs |  |  |  |  |  |
|  | Make sure BGP Table is established |  |  |  |  |  |
|  | Tested and verify BGP |  |  |  |  |  |
|  | Cyber Security portion |  |  |  |  |  |
|  | Set up a vulnerable web application |  |  |  |  |  |
|  | Launched SQL injection attack |  |  |  |  |  |
|  | Open browser, enter URL localhost/dvwa/login.php. |  |  |  |  |  |
|  | Login using the username "admin" and password as "password" |  |  |  |  |  |
|  | Set the DVWA security to LOW. |  |  |  |  |  |
|  | Clicked on SQL injection on the left-menu |  |  |  |  |  |
|  | On User ID field, entered "1" and clicked Submit. |  |  |  |  |  |
|  | Use always true scenario to extract the First_names and Surnames from the database using input: %' or '1'='1' |  |  |  |  |  |
|  | Configured Stored XSS on DVWA with low security |  |  |  |  |  |
|  | Run DVWA warmup , type just the basic exploit into the textbox “Message”. |  |  |  |  |  |
|  | Write: <script>alert('You have been hacked!');</script> |  |  |  |  |  |
|  | Set Stored XSS on DVWA with high security |  |  |  |  |  |
|  | Increased the maxlength attribute of the name |  |  |  |  |  |
|  | Type <h1>StackZero</h1> |  |  |  |  |  |
|  | Pressed Sign Guestbook button |  |  |  |  |  |
|  | Verified the exploit by a random comment (the field is mandatory), |  |  |  |  |  |
|  | Ensured XSS exploit the page reloads successful |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocation Certificate Level 02 Junior Technician in HVACR |
| --- | --- |
| Competency Standard | - Perform Network Connectivity and Communications - Develop Scalable Addressing Schemes - Perform Basic Network switching - Choose the right network for improved bandwidth and efficiency - Configure and secure WLANs - Configure routing protocols and static routes on routers - Implement Network security protocol and practices - Configure WAN Services and protocols - Analyse Security Threats and Vulnerabilities - Manage security risk and assessment - Perform Network Security - Implement Cryptography and Access Control - Perform Digital Forensic on sample data |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define end, intermediary and host devices |  |  |
|  | Define wired and wireless network |  |  |
|  | Define VLSM and differentiate between Subnetting and VLSM |  |  |
|  | . Define Inter Vlan Routing |  |  |
|  | Define IDS and Security zones |  |  |
| 6 | Where are Access Control Lists (ACLs) commonly applied in networking, and what is their primary function ? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________

Annexure A

Annexure B

| Answer Key |  |
| --- | --- |
|  | Define end, intermediary and host devices |
|  | End Devices: Definition: Devices that directly interact with users for the desired end result. Examples: Computers, smartphones, printers, and servers. Intermediary Devices: Definition:Devices that facilitate communication between end devices and ensure data is properly routed through the network.Examples: Routers, switches, and bridges. Host Devices: Definition: Host devices are any hardware devices that has the capability of permitting access to a network via a user interface, specialized software, network address, protocol stack, or any other means. Examples: include, but are not limited to, computers, personal electronic devices, thin clients, and multi-functional devices. |
|  | Define wired and wireless network |
|  | - Wi Wired and wireless networks are two fundamental types of computer networks that facilitate communication and data transfer between devices. - Wired Network: - Definition: A wired network uses physical cables or wires to connect devices and enable communication between them. - Examples: Ethernet, Power line, and Fiber-optic networks. - Wireless Network: - Definition: A wireless network allows devices to communicate without the need for physical cables by using radio waves or infrared signals. - Examples: Wi-Fi, Bluetooth, and cellular networks. |
|  | Define VLSM and differentiate between Subnetting and VLSM |
|  | VLSM stands for Variable Length Subnet Masking. It is a technique used in networking to allocate IP address spaces more efficiently by allowing different subnets to have different subnet mask lengths, thereby using address space more effectively for avoiding wastage of Addresses space Subnetting, on the other hand, is the process of dividing a larger IP network into smaller, more manageable sub-networks. It involves creating subnets with fixed-size subnet masks. In summary: VLSM allows for subnets within subnets, enabling more flexibility in allocating IP addresses. Subnetting is the general practice of dividing a network into smaller sub-networks, but it typically involves using fixed-size subnet masks for simplicity. VLSM is a specific form of subnetting that introduces variable-sized subnet masks for more efficient use of IP address space. |
|  | Define Inter Vlan Routing |
|  | Inter-VLAN routing is a technique that allows communication between different VLANs on a network. It typically involves a router or Layer 3 switch to facilitate the exchange of data packets between VLANs, enabling segmented and efficient network traffic management |
|  | Define IDS and Security zones |
|  | IDS (Intrusion Detection System) is a security tool that monitors network or system activities for suspicious behavior or patterns, alerting administrators to potential security threats. Security zones are network segments or areas with distinct security requirements and policies, designed to control and monitor traffic flow, ensuring a secure and organized network architecture. |
|  | Where are Access Control Lists (ACLs) commonly applied in networking, and what is their primary function? |
|  | ACLs are commonly applied at the network layer of routers. Their primary function is to control and manage traffic by permitting or denying packets based on specified criteria such as source or destination IP addresses |