|  | # Written Assessment Guide # for # “Garments Manufacturing Technology” # (Garments Supervisor) # L5 |
| --- | --- |
|  | Date |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: National Diploma level 5 in Garments Manufacturing Technology Garments Supervisor |  | CS Code: | Level: 05 | Version: |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Identify Process improvement Techniques for garment industry |  | Assessment Date (DD/MM/YY): |  |  |
| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |  |  |  |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |
| --- | --- |
| Written Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |

| Title of Qualification: Garment Manufacturing Technology | CS Code: | Level: | Version: |
| --- | --- | --- | --- |
| Competency Standard Title: Supervise Packing and dispatching of finished garments | Assessment Date (DD/MM/YY): |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| - What is the lean manufacturing? | Lean manufacturing is a methodology that focuses on minimizing waste within manufacturing systems while simultaneously maximizing productivity |
| - What are best lean manufacturing tools for garment industry? | - 5S stands for the 5 steps of this methodology: Sort, Set in Order, Shine, Standardize, Sustain. These steps involve going through everything in a space, deciding what's necessary and what isn't, putting things in order, cleaning, and setting up procedures for performing these tasks on a regular basis. - KAIZEN is a Japanese word which consists of two terms i.e.KAI means “CHANGE” and ZEN means “GOOD”. So over all KAIZEN means “Changes done for better improvement in a management system of companies”. In simple manner KAIZEN means continuous improvement by including all members, Higher authorities, Managers, Workers and each and everyone related to it with practical implementation of ideas. • One of the Lean tools ( TPS). • Focus on Gradually change , a lot of little improvements add up to huge gains in productivity (elimination of waste) and quality. • Importance of EVERYONE buying into the concept and the vision - Visualization : Visual Management systems enable factory workers to be well informed about production procedures, status and other important information for them to do their jobs as effectively as possible. Large visual displays are generally much more effective means of communication to workers on the factory floor than written reports and guidelines and therefore should be used as much as possible. When it comes to improving compliance with a process, visual presentation helps the team better understand a complicated process including the correct sequence of events, the correct way to perform each action, internal and external relationships between actions, and other factors - Visual Displays - Charts, metrics, procedures and process documentation which are reference information for production workers. For example, trend chart of yield performance, % variation of defect rate, month-to-date shipping volume status, etc. - Visual Controls – Indicators intended to control or signal actions to group members. This may include production status information, quality tracking information, etc. For example, color-coded panel for temperature or speed setting control limits that help an operator quickly identify process is out of the control range. Kanban cards are another example of visual controls - Visual process indicators – These communicate the correct production processes or flow of materials. For example, this would include the use of painted floor areas for non-defective stock and scrap or indicators for the correct flow of materials on the factory floor. |
| - What wastages of lean in garment manufacturing? | - Seven wastes of lean manufacturing: - 7 types of wastes and non-value adding activities are as following. ... - 1. Transportation. When work is transferred from one place to another is a non-value added activity. ... - 2. Excess inventory. ... - 3. Excess motion. ... - 4. Waiting. ... - 5. Over Production. ... - 6. Over processing. ... - 7. Defects. |
| - What is Standard allowed Minutes (SAM)? | Standard Allowed Minute (SAM) is used to measure a task or work content of a garment. ... This 'SAM' term is widely used by industrial engineers and garment production people in the garment manufacturing industry. |
| - How the SAM is calculated in garment industry? | Select one operation for which you want to calculate SAM. Standard allowed minutes (SAM) = (Basic minute + Bundle allowances + machine and personal allowances). Add bundle allowances (10%) and machine and personal allowances (20%) to basic time. Now you got Standard Minute value SAM And basic minutes (B.M) are calculated as B.M =Average single cycle time * Rating Factor |
| - What is the time study? | Time study is a work measurement technique commonly used by apparel engineers to determine the rate at which a specific operation is performed. The objective of a time study is to develop and check production standards. An operation is studied by time analysts while a style is in production. |
| - What is JIT(Just in time) in garment industry ? | JIT means producing the right part in right quantity, at the right time, thus reducing manufacturing waste. The objective is to produce a continuous flow of value so that the customer can pull. JIT supports quick response to customer, better sense of takt time and abnormality control. |
| - What are the advantages of KAIZEN? | 1. Continuous improvement. 2. People learn from their mistakes. 3. People find ways to make things better. 4. Goods and services also improve over time. 5. Leads to improved morale, better products, and satisfied customers and reduced cost. 6. Improves quality. 7. Increases efficiency. 8. Improves customer satisfaction. 9. Value can be added using kaizen in service industries. 10. Applies to all non-manufacturing fields. 11. Tools specifically adapted for this purpose. |