Assessors Judgment Guide

| Qualification | National Diploma Level-5 in Telecommunication Technology Telecommunication Fundamentals |
| --- | --- |
| Competency Standard(s) | 0714-E&A-11: Demonstrate Demodulation, Multiplexing & De Multiplexing of Information Signal |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task | - Perform Amplitude Modulation of Information Signal - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier - Perform Demodulation of De-multiplexed AM Modulated Signal |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  | Yes | No | Remarks |
| - Perform Amplitude Modulation of Information Signal |  |  |  |  |  |
| - Calibrate CRO. |  |  |  |  |  |
| - Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |  |  |  |
| - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. |  |  |  |  |  |
| - Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
| - Repeat the above steps on three different AM Transmitter Trainers to get three different AM modulated signals. |  |  |  |  |  |
| - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier |  |  |  |  |  |
| - Apply the three AM modulated signals to TDM Multiplexer. |  |  |  |  |  |
| - Observe the multiplexed output on channel 1 of CRO. |  |  |  |  |  |
| - Apply to multiplexed output to the De-multiplexer. |  |  |  |  |  |
| - Observe output for first signal on channel 1 of CRO and compare the output signal with the relative modulated signal by observing it on channel 2 of CRO. |  |  |  |  |  |
| - Repeat the step 2.4 to De-multiplex all the signals one by one. |  |  |  |  |  |
| - Perform Demodulation of De-multiplexed AM Modulated Signal |  |  |  |  |  |
| - Apply the AM Modulated signals and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainers one by one. |  |  |  |  |  |
| - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
| Compare the Message signal to the Demodulated signal. |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Diploma Level-5 in Telecommunication Technology Telecommunication Fundamentals |
| --- | --- |
| Competency Standard(s) | Demonstrate Demodulation, Multiplexing & De Multiplexing of Information Signal |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question: Define Modulation. |  |  |
|  | Candidate’s response |  |  |
|  | What are different types of modulation? |  |  |
|  | What is the difference between TDM and TDMA? |  |  |
|  | Why do we use multiplexing? |  |  |
|  | Why do we need modulation? |  |  |
|  | Can audio signals be transmitted with antennas without using any modulation technique? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Diploma Level-5 in Telecommunication Technology Telecommunication Fundamentals |
| Competency Standards | Demonstrate Demodulation, Multiplexing & De Multiplexing of Information Signal |
| Assessment Task | - Perform Amplitude Modulation of Information Signal - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier - Perform Demodulation of De-multiplexed AM Modulated Signal |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Perform Amplitude Modulation of Information Signal |  |  |
| - Calibrate CRO. |  |  |
| - Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |
| - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. |  |  |
| - Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |
| - Repeat the above steps on three different AM Transmitter Trainers to get three different AM modulated signals. |  |  |
| - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier |  |  |
| - Apply the three AM modulated signals to TDM Multiplexer. |  |  |
| - Observe the multiplexed output on channel 1 of CRO. |  |  |
| - Apply to multiplexed output to the De-multiplexer. |  |  |
| - Observe output for first signal on channel 1 of CRO and compare the output signal with the relative modulated signal by observing it on channel 2 of CRO. |  |  |
| - Repeat the step 2.4 to De-multiplex all the signals one by one. |  |  |
| - Perform Demodulation of De-multiplexed AM Modulated Signal |  |  |
| - Apply the AM Modulated signals and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainers one by one. |  |  |
| - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |
| - Compare the Message signal to the Demodulated signal. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | National Diploma Level-5 in Telecommunication Technology Telecommunication Fundamentals |
| --- | --- |
| Competency Standard(s) | Demonstrate Demodulation, Multiplexing & De Multiplexing of Information Signal |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Perform Amplitude Modulation of Information Signal - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier - Perform Demodulation of De-multiplexed AM Modulated Signal |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Perform Amplitude Modulation of Information Signal - Calibrate CRO. - Generate Low Frequency Message signal and observe on channel 1 of CRO. - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. - Connect the output of the modulator to channel 2 of CRO and observe the signal. - Repeat the above steps on three different AM Transmitter Trainers to get three different AM modulated signals. - Perform Time Division Multiplexing (TDM) and De-multiplexing of the AM modulated signal generated earlier - Apply the three AM modulated signals to TDM Multiplexer. - Observe the multiplexed output on channel 1 of CRO. - Apply to multiplexed output to the De-multiplexer. - Observe output for first signal on channel 1 of CRO and compare the output signal with the relative modulated signal by observing it on channel 2 of CRO. - Repeat the step 2.4 to De-multiplex all the signals one by one. - Perform Demodulation of De-multiplexed AM Modulated Signal - Apply the AM Modulated signals and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainers one by one. - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. - Compare the Message signal to the Demodulated signal. |
| Minimum Evidence Required |  |