Instruction Sheet for the Candidate

| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Basic Electrical Work on Telecom site |
| --- | --- |
| Competency Standard | 0714-E&A-22: Make Cable/Wire Joints |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Make Cross/Twist joint - Make Straight/Married joint - Make T- Joint - Make Rat tail joint - Make Britannia joint |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Make Cross/Twist joint - Select the cable. - Strip the wire according to 50mm. - Twist the conductors. - Solder the conductor - Insulate the joint Make Straight/Married joint - Select the cable. - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor. - Insulate the joint. Make T- Joint - Select the cable. - Remove the insulation of cable 1 to 50mm from where a connection is required. - Separate conductors of cable 1 equally. - Take another 12 mm stripped wire 2. - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2. - Solder the joint. Make Rat tail joint - Select the single conductor wires. - Strip both the wires to 5mm. - Twist the conductor. - Insulate the joint. - Solder the joint Make Britannia joint - Select the cable. - Strip both cables to 75mm. - Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm and wrap around in both directions to 6mm. - Solder the joint. - Insulate the joint |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Basic Electrical Work on Telecom site |
| Competency Standard | Make Cable/Wire Joints |
| Purpose of Assessment | Formative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the cable. |  |  |
| - Strip the wire according to 50mm. |  |  |
| - Twist the conductors. |  |  |
| - Solder the conductor |  |  |
| - Insulate the joint |  |  |
| - Strip wire to 75mm according to joint requirement. |  |  |
| - Intermingle the conductors to 60mm into each other. |  |  |
| - Twist 60mm conductors leaving behind 15mm of each cable. |  |  |
| - Solder the conductor. |  |  |
| - Insulate the joint. |  |  |
| - Remove the insulation of cable 1 to 50mm from where a connection is required. |  |  |
| - Separate conductors of cable 1 equally. |  |  |
| - Take another 12 mm stripped wire 2. |  |  |
| - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2. |  |  |
| - Solder the joint. |  |  |
| - Select the single conductor wires. |  |  |
| - Strip both the wires to 5mm. |  |  |
| - Twist the conductor. |  |  |
| - Insulate the joint. |  |  |
| - Solder the joint |  |  |
| - Strip both cables to 75mm. |  |  |
| - Bend the tips of both cable right angle about to 6mm. |  |  |
| - Hold the two cables overlap 50mm with tips in opposite direction. |  |  |
| - Take another bare conductor of 1mm and wrap around in both directions to 6mm. |  |  |
| - Solder the joint. |  |  |
| - Insulate the joint |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Basic Electrical Work on Telecom site |
| --- | --- |
| Competency Standard | Make Cable/Wire Joints |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select the cable. |  |  |  |  |  |
|  | Strip the wire according to 50mm. |  |  |  |  |  |
|  | Twist the conductors. |  |  |  |  |  |
|  | Solder the conductor |  |  |  |  |  |
|  | Insulate the joint |  |  |  |  |  |
|  | Strip wire to 75mm according to joint requirement. |  |  |  |  |  |
|  | Intermingle the conductors to 60mm into each other. |  |  |  |  |  |
|  | Twist 60mm conductors leaving behind 15mm of each cable. |  |  |  |  |  |
|  | Solder the conductor. |  |  |  |  |  |
|  | Insulate the joint. |  |  |  |  |  |
|  | Remove the insulation of cable 1 to 50mm from where a connection is required. |  |  |  |  |  |
|  | Separate conductors of cable 1 equally. |  |  |  |  |  |
|  | Take another 12 mm stripped wire 2. |  |  |  |  |  |
|  | Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2. |  |  |  |  |  |
|  | Solder the joint. |  |  |  |  |  |
|  | Select the single conductor wires. |  |  |  |  |  |
|  | Strip both the wires to 5mm. |  |  |  |  |  |
|  | Twist the conductor. |  |  |  |  |  |
|  | Insulate the joint. |  |  |  |  |  |
|  | Solder the joint |  |  |  |  |  |
|  | Strip both cables to 75mm. |  |  |  |  |  |
|  | Bend the tips of both cable right angle about to 6mm. |  |  |  |  |  |
|  | Hold the two cables overlap 50mm with tips in opposite direction. |  |  |  |  |  |
|  | Take another bare conductor of 1mm and wrap around in both directions to 6mm. |  |  |  |  |  |
|  | Solder the joint. |  |  |  |  |  |
|  | Insulate the joint |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Diploma Level-5 in Telecommunication Technology |
| --- | --- |
| Competency Standard | Make Cable/Wire Joints for Single Phase Wiring |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Differentiate between cable and wire. |  |  |
|  | Enlist different types of joints used for cables. |  |  |
|  | What do you mean by Cross/Twist joint? |  |  |
|  | What is Straight/Married joint? |  |  |
|  | What is T- Joint? |  |  |
|  | What is Rat tail joint? |  |  |
|  | What is Britannia joint? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________