Assessors Judgment Guide

| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Optimize Telecom Network Level -5 |
| --- | --- |
| Competency Standard(s) | 0714-E&A-72: Perform 2G Drive/Walk Test |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Perform following tasks - Perform 2G Drive test on provided site. - Perform 2G Performance Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Mapped site coordinates in Navigation software |  |  |  |  |  |
|  | Plotted site sectors according to data sheet provided. |  |  |  |  |  |
|  | Ensured sector’s azimuth as per plan. |  |  |  |  |  |
|  | Verified physical sector azimuth with site data provided. |  |  |  |  |  |
|  | Connected cell phone with data collection software to Confirm sector identities are serving as per plan. |  |  |  |  |  |
|  | Confirmed Electrical and Mechanical tilts are perfectly matched with plan. |  |  |  |  |  |
|  | Confirmed scrambling code was serving corresponding to your current location |  |  |  |  |  |
|  | Confirmed cell ID was serving corresponding to your current location. |  |  |  |  |  |
|  | Locked Cell phone to 2G band. |  |  |  |  |  |
|  | Initiated call from cell phone. |  |  |  |  |  |
|  | Run call setup script for short calls. |  |  |  |  |  |
|  | Started drive test. |  |  |  |  |  |
|  | Stopped drive test when successfully handover to neighbouring site |  |  |  |  |  |
|  | Moved to selected point as per define. |  |  |  |  |  |
|  | Started log recording and execute short call setup and took screenshot of CSSR stop log recording. |  |  |  |  |  |
|  | Performed throughput test (HSPA + HSDPA). |  |  |  |  |  |
|  | Performed cell reselection test. |  |  |  |  |  |
|  | Performed IRAT test |  |  |  |  |  |
|  | Made performance test report. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Optimize Telecom Network Level -5 |
| --- | --- |
| Competency Standard(s) | Perform 2G Drive/Walk Test |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define cellular network Sectors, |  |  |
|  | Walk test techniques, AutoCAD software and sector coverage Map. |  |  |
|  | Explain concept of Electrical and mechanical tilts and sector identities. |  |  |
|  | Drive test techniques and Handovers inter/intra. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Optimize Telecom Network Level -5 |
| Competency Standards | Perform 2G Drive/Walk Test |
| Assessment Task | - Perform 2G Drive test on provided site. - Perform 2G Performance Test |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Mapp site coordinates in Navigation software |  |  |
| - Plot site sectors according to data sheet provided. |  |  |
| - Ensure sector’s azimuth as per plan. |  |  |
| - Verify physical sector azimuth with site data provided. |  |  |
| - Connect cell phone with data collection software to Confirm sector identities are serving as per plan. |  |  |
| - Confirm Electrical and Mechanical tilts are perfectly matched with plan. |  |  |
| - Confirm scrambling code was serving corresponding to your current location |  |  |
| - Confirm cell ID was serving corresponding to your current location. |  |  |
| - Lock Cell phone to 2G band. |  |  |
| - Initiate call from cell phone. |  |  |
| - Run call setup script for short calls. |  |  |
| - Startdrive test. |  |  |
| - Stop drive test when successfully handover to neighbouring site |  |  |
| - Move to selected point as per define. |  |  |
| - Start log recording and execute short call setup and took screenshot of CSSR stop log recording. |  |  |
| - Perform throughput test(HSPA + HSDPA). |  |  |
| - Perform cell reselection test. |  |  |
| - Perform IRAT test |  |  |
| - Make performance test report. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | National Diploma Level-5 in Telecommunication Technology National vocational certificate of Optimize Telecom Network Level -5 |
| --- | --- |
| Competency Standard(s) | Perform 2G Drive/Walk Test |
| Purpose of Assessment | Formative assesment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the 4Hrs (for practical demonstration & assessment): - Perform 2G Drive test on provided site. - Perform 2G Performance Test |
| Time: 4Hrs | During a practical assessment, under observation by an assessor, you are required to - Perform 2G Drivetest on provided site. - Map site coordinates in Navigation software - Plot site sectors according to data sheet provided. - Ensure sector’s azimuth as per plan. - Verify physical sector azimuth with site data provided. - Connect cell phone with data collection software to Confirm sector identities are serving as per plan. - Confirm Electrical and Mechanical tilts are perfectly matched with plan. - Confirm scrambling code is serving corresponding to your current location. - Confirm cell ID is serving corresponding to your current location. - Lock Cell phone to 2G band. - Initiate call from cell phone. - Run call setup script for short calls. - Initiate Download and Upload script - Start drive test. - Stop drive test when successfully handover to neighbouring site - Perform 2G Performance Test - Move to near point as per define. - Start log recording and execute short call setup and take screenshot of CSSR stop log recording. - Perform throughput test(HSPA + HSDPA). - Perform cell reselection test. - Perform IRAT test - Make performance test report. |
| Minimum Evidence Required |  |

Tools and Equipment

- Laptop/Tab supported cell phone and monitoring software - Compass - Data collection software supported cell phones.