Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard(s) | Make water connection |
| Purpose of Assessment | Formative |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): 1. Make water connection |
| Time: | During a practical assessment, under observation by an assessor, you are required to 1. Identify work place 2. Dig a pit on connection site to main water supply pipe. 3. Mount the clamp over the main water supply pipe 4. Set the rubber seal in the clamp 5. Set the ferrule at the rubber seal under the clamp 6. Tighten the screws of the clamp with screw wrench 7. Mark hole in the main pipe with center punch and hammer 8. Connect the socket on the ferrule 9. Connect the house pipe in the socket 10. Fit the sluice/gate and non-return valve in the house water supply line 11. Explain safety precautions during plumbing task. 12. Differentiate between plumbing and plumber 13. State steps for using drill machine. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Competency Standards | Make water connection |
| Assessment Task | 1 - Make water connection |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify work place |  |  |
| - Dig a pit on connection site to main water supply pipe. |  |  |
| - Mount the clamp over the main water supply pipe |  |  |
| - Set the rubber seal in the clamp |  |  |
| - Set the ferrule at the rubber seal under the clamp |  |  |
| - Tighten the screws of the clamp with screw wrench |  |  |
| - Mark hole in the main pipe with center punch and hammer |  |  |
| - Connect the socket on the ferrule |  |  |
| - Connect the house pipe in the socket |  |  |
| - Fit the sluice/gate and non-return valve in the house water supply line |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard(s) | Make water connection |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Make water connection |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified work place |  |  |  |  |  |
|  | Dug a pit on connection site to main water supply pipe. |  |  |  |  |  |
|  | Mounted the clamp over the main water supply pipe. |  |  |  |  |  |
|  | Set the rubber seal in the clamp |  |  |  |  |  |
|  | Set the ferrule at the rubber seal under the clamp |  |  |  |  |  |
|  | Tighten the screws of the clamp with screw wrench |  |  |  |  |  |
|  | Marked hole in the main pipe with center punch and hammer |  |  |  |  |  |
|  | Connected the socket on the ferrule |  |  |  |  |  |
|  | Connected the house pipe in the socket |  |  |  |  |  |
|  | Fitted the sluice/gate and non-return valve in the house water supply line |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard(s) | Make water connection |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | State different tools and equipment for making new connection for water supply |  |  |
|  | Define connection procedure for pipes |  |  |
|  | Define cutting procedures |  |  |
|  | Define threading procedures |  |  |
|  | Define PPEs |  |  |
|  | Enlist uses of different tools used in water connection |  |  |
|  | Define clamps |  |  |
|  | Name the types of clamps |  |  |
|  | What are the uses of valve while taking connection |  |  |
|  | Differentiate between branch and main pipe. |  |  |

Feedback to the Candidate

Candidate’s Signature____________________ Assessor’s Signature __________________