Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Purpose of Assessment | Summative Assessment - Plumbing |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Install plumbing fixtures (Wash hand Basin, English WC and Urinal) - Knowledge Assessment |
| Time: 05 Hrs | During a practical assessment, under observation by an assessor, you are required to “ Install plumbing fixtures (Wash hand Basin, English WC and Urinal) according to given sample (Annexure-A)” demonstrating the following criteria: |
| Minimum Evidence Required | - Identify safety requirements for installation - Identify tools for work - Identify Fixtures - Perform measurements - Mark out Installation point - Turn the water supply off of the concerned section - Mark and drill pilot holes for the floor fixing screws. - Mark, drill and plug the wall holes for basin fixing screw - Install tee stop cock at the end of water supply pipe (hot and cold) - Connect the flexible rubber connection pipes with pillar cocks or mixture - Fit the couplene in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. - Re-position the pedestal and screw it into proper place. - Position the basin and check its level then screw the basin to wall. - Inject silicon sealant for sealing. - Tighten the pedestal screws. - Turn on the water supply and check for leaks. - Compact and Level the floor at the place of installation of water closet. - Place P-trap at compacted and levelled surface. - Place the W.C pot on the trap in leveled surface and provide support to it. - Fill the P-trap with cement and cloth bandage soaked in cement solution. - Connect soil pipe to P-trap - Connect tee with soil pipe outside the toilet wall. - Joint the vent pipe with small end of tee. - Connect larger end of soil pipe to septic tank. - Attach flush pipe in to the pot hole. - Drill holes at the required place and height for flushing cistern. - Install flushing with screws and joint it with flush pipe. - Fill the earth to required level. - Finish conglomerate floor or tile flooring - Install adequate support for wall hangers and install wall hangers if needed. - Drill the holes at two feet height - Determine the position for waste pipe nipple. - Apply thread sealant tape. - Install female collar to the waste pipe. - Place the gasket on the female collar with the beveled surface facing away from the collar. - Position the urinal on the wall hangers and screwed them. - Secure the urinal bottle trap and waste pipe to female collar using the screws. - Install the flush valve according to manufacturer instructions. - Connect the water supply and complete waste pipe connection. - Turn on water supply. - Flush and check the entire installation for leakage |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Purpose of Assessment | Summative Assessment - PLUMBING |
| Assessment Task | Install plumbing fixtures (Wash hand Basin, English WC and Urinal) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify safety requirements for installation |  |  |
| - Identify tools for work |  |  |
| - Identify Fixtures |  |  |
| - Perform measurements |  |  |
| - Mark out Installation point |  |  |
| - Turn the water supply off of the concerned section |  |  |
| - Mark and drill pilot holes for the floor fixing screws. |  |  |
| - Mark, drill and plug the wall holes for basin fixing screw |  |  |
| - Install tee stop cock at the end of water supply pipe (hot and cold) |  |  |
| - Connect the flexible rubber connection pipes with pillar cocks or mixture |  |  |
| - Fit the couplene in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. |  |  |
| - Re-position the pedestal and screw it into proper place. |  |  |
| - Position the basin and check its level then screw the basin to wall. |  |  |
| - Inject silicon sealant for sealing. |  |  |
| - Tighten the pedestal screws. |  |  |
| - Turn on the water supply and check for leaks. |  |  |
| - Compact and Level the floor at the place of installation of water closet. |  |  |
| - Place P-trap at compacted and levelled surface. |  |  |
| - Place the W.C pot on the trap in leveled surface and provide support to it. |  |  |
| - Fill the P-trap with cement and cloth bandage soaked in cement solution. |  |  |
| - Connect soil pipe to P-trap |  |  |
| - Connect tee with soil pipe outside the toilet wall. |  |  |
| - Joint the vent pipe with small end of tee. |  |  |
| - Connect larger end of soil pipe to septic tank. |  |  |
| - Attach flush pipe in to the pot hole. |  |  |
| - Drill holes at the required place and height for flushing cistern. |  |  |
| - Install flushing with screws and joint it with flush pipe. |  |  |
| - Fill the earth to required level. |  |  |
| - Finish conglomerate floor or tile flooring |  |  |
| - Install adequate support for wall hangers and install wall hangers if needed. |  |  |
| - Drill the holes at two feet height |  |  |
| - Determine the position for waste pipe nipple. |  |  |
| - Apply thread sealant tape. |  |  |
| - Install female collar to the waste pipe. |  |  |
| - Place the gasket on the female collar with the beveled surface facing away from the collar |  |  |
| - Position the urinal on the wall hangers and screwed them. |  |  |
| - Secure the urinal bottle trap and waste pipe to female collar using the screws. |  |  |
| - Install the flush valve according to manufacturer instructions. |  |  |
| - Connect the water supply and complete waste pipe connection. |  |  |
| - Turn on water supply. |  |  |
| - Flush and check the entire installation for leakage |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Purpose of Assessment | Summative Assessment - PLUMBING |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | ( Plumbing Public Health and Water Supply ) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified safety requirements for installation |  |  |  |  |  |
|  | Identified tools for work |  |  |  |  |  |
|  | Identified Fixtures |  |  |  |  |  |
|  | Performed measurements |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Turned the water supply off of the concerned section |  |  |  |  |  |
|  | Marked and drilled pilot holes for the floor fixing screws. |  |  |  |  |  |
|  | Marked, drilled and plugged the wall holes for basin fixing screw |  |  |  |  |  |
|  | Installed tee stop cock at the end of water supply pipe (hot and cold) |  |  |  |  |  |
|  | Connected the flexible rubber connection pipes with pillar cocks or mixture |  |  |  |  |  |
|  | Fitted the couplene in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. |  |  |  |  |  |
|  | Re-positioned the pedestal and screw it into proper place. |  |  |  |  |  |
|  | Positioned the basin and check its level then screw the basin to wall. |  |  |  |  |  |
|  | Injected silicon sealant for sealing. |  |  |  |  |  |
|  | Tighten the pedestal screws. |  |  |  |  |  |
|  | Turned on the water supply and check for leaks. |  |  |  |  |  |
|  | Compacted and Level the floor at the place of installation of water closet. |  |  |  |  |  |
|  | Placed P-trap at compacted and levelled surface. |  |  |  |  |  |
|  | Placed the W.C pot on the trap in leveled surface and provide support to it. |  |  |  |  |  |
|  | Filled the P-trap with cement and cloth bandage soaked in cement solution. |  |  |  |  |  |
|  | Connected soil pipe to P-trap |  |  |  |  |  |
|  | Connected tee with soil pipe outside the toilet wall. |  |  |  |  |  |
|  | Jointed the vent pipe with small end of tee. |  |  |  |  |  |
|  | Connected larger end of soil pipe to septic tank. |  |  |  |  |  |
|  | Attached flush pipe in to the pot hole. |  |  |  |  |  |
|  | Drilled holes at the required place and height for flushing cistern. |  |  |  |  |  |
|  | Installed flushing with screws and joint it with flush pipe. |  |  |  |  |  |
|  | Filled the earth to required level. |  |  |  |  |  |
|  | Finished conglomerate floor or tile flooring |  |  |  |  |  |
|  | Installed adequate support for wall hangers and installed wall hangers if needed. |  |  |  |  |  |
|  | Drilled the holes at two feet height |  |  |  |  |  |
|  | Determined the position for waste pipe nipple. |  |  |  |  |  |
|  | Applied thread sealant tape. |  |  |  |  |  |
|  | Installed female collar to the waste pipe. |  |  |  |  |  |
|  | Placed the gasket on the female collar with the beveled surface facing away from the collar |  |  |  |  |  |
|  | Positioned the urinal on the wall hangers and screwed them. |  |  |  |  |  |
|  | Secured the urinal bottle trap and waste pipe to female collar using the screws. |  |  |  |  |  |
|  | Installed the flush valve according to manufacturer instructions. |  |  |  |  |  |
|  | Connected the water supply and complete waste pipe connection. |  |  |  |  |  |
|  | Turned on water supply. |  |  |  |  |  |
|  | Flushed and checked the entire installation for leakage |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Purpose of Assessment | Summative Assessment - PLUMBING |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Enlist the use of valve while taking connection |  |  |
|  | Define branch and main pipe |  |  |
|  | Name the method for different type of pipe joints |  |  |
|  | What are the specifications for cold and hot water connections |  |  |
|  | Define gravity and pressure supply of water |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________