Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Perform Water Quality Sampling (Surface Water) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Prepare work area Collect sample for Microbiological analysis - Take Direct Sample - Take Grab sample |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare work area Collect sample for Microbiological analysis - Determine the extent of the sampling effort, the sampling methods to be employed, and which equipment and supplies are needed. - Obtain necessary sampling and monitoring equipment. - Decontaminate or pre clean equipment, and ensure that it is in working order Take Direct Sample - Ensure that labelling on the bottle to be filled is correct and that the sample number matches the number on the paperwork (field observation form). - Take the sample from the bank, or wade into the water in waders or boots, minimising disturbance as much as possible. - Don’t uncap the bottle until just before the sample is to be taken. - When filling the bottle (both for rinsing and for the final sample) take the sample upstream and to the side of you. - Uncap the bottle to be filled, immerse the bottle in the water (to a depth of ~15cm) lying it flat with its mouth towards the flow of the water, then slowly move the bottle forwards into the flowing water. - If rinsing is required (make sure you check the standard operating procedure) allow approximately 20 mL of water to enter the bottle. Cap, shake well and pour the rinsate downstream of yourself. - Repeat twice so the bottle is rinsed a total of three times (if required). - Then fill the bottle, again upstream and to the side of yourself at a depth of~15 cm, moving it slowly forwards through the water to the required level (generally the shoulder of the bottle, but always check the sampling and analysis plan). Cap, and store the sample container as required for transport to the laboratory. - If bottles contain preservatives (i.e. for metal analysis), they must not be rinsed and should be filled by decanting from another, rinsed bottle. Take Grab sample - Ensure that labelling on bottle to be filled is correct and that the sample number matches the number on the paperwork (field observation form). - Check that the grab sampler is clean; this should be done at the completion of each day’s sampling, normally by simply rinsing in tap water. - Extend the grab sampler so that it will reach to the point that you wish to sample. - Carefully lower the grab sampler into the water with the mouth of the bottle facing upstream into the flow of water to a depth of ~15 cm. Keep the bottle moving forward into the flow of water while it is filling - Allow the bottle to partially fill, take it out of the water, swirl around, and tip out rinsate downstream of sampling site. - Repeat twice so that the grab sampler has been rinsed three times. - Then, fill the grab sampler at a depth of ~15 cm, with the mouth of the bottle facing upstream, slowly moving the bottle forward into the flow of water. - Use this sample to rinse the labelled sample container three times using the same procedure as for taking a direct sample. - Then, fill the rinsed, labelled sample container from the grab sample bottle. It may be necessary to refill the grab sample bottle to obtain enough water to fill the sample container. If this is necessary, recap the sample container between top-ups. Fill the sample container to the required level (generally the shoulder, but check the sampling and analysis plan, as this may vary). - Take care not to touch the opening of the sample container with any part of the grab pole bottle or your hands as this can easily introduce contaminants to the sample. - If the sample container contains a preservative, be careful when filling to prevent any splash back. - Cap (tightly) and store the sample container as required for transport to the laboratory. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Competency Standard | Perform Water Quality Sampling (Surface Water) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare work area Collect sample for Microbiological analysis - Take Direct Sample - Take Grab sample |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Determine the extent of the sampling effort, the sampling methods to be employed, and which equipment and supplies are needed. |  |  |
| - Obtain necessary sampling and monitoring equipment. |  |  |
| - Decontaminate or pre clean equipment, and ensure that it is in working order |  |  |
| - Ensure that labelling on the bottle to be filled is correct and that the samplenumber matches the number on the paperwork (field observation form). |  |  |
| - Take the sample from the bank, or wade into the water in waders or boots, minimising disturbance as much as possible. |  |  |
| - Don’t uncap the bottle until just before the sample is to be taken. |  |  |
| - When filling the bottle (both for rinsing and for the final sample) take the sample upstream and to the side of you. |  |  |
| - Uncap the bottle to be filled, immerse the bottle in the water (to a depth of ~15cm) lying it flat with its mouth towards the flow of the water, then slowly move the bottle forwards into the flowing water. |  |  |
| - If rinsing is required (make sure you check the standard operating procedure) allow approximately 20 mL of water to enter the bottle. Cap, shake well andpour the rinsate downstream of yourself. |  |  |
| - Repeat twice so the bottle is rinsed a total of three times (if required). |  |  |
| - Then fill the bottle, again upstream and to the side of yourself at a depth of~15 cm, moving it slowly forwards through the water to the required level (generally the shoulder of the bottle, but always check the sampling and analysis plan). Cap, and store the sample container as required for transport to the laboratory. |  |  |
| - If bottles contain preservatives (i.e. for metal analysis), they must not be rinsed and should be filled by decanting from another, rinsed bottle. |  |  |
| - Ensure that labelling on bottle to be filled is correct and that the sample number matches the number on the paperwork (field observation form). |  |  |
| - Check that the grab sampler is clean; this should be done at the completion of each day’s sampling, normally by simply rinsing in tap water. |  |  |
| - Extend the grab sampler so that it will reach to the point that you wish to sample. |  |  |
| - Carefully lower the grab sampler into the water with the mouth of the bottle facing upstream into the flow of water to a depth of ~15 cm. Keep the bottle moving forward into the flow of water while it is filling |  |  |
| - Allow the bottle to partially fill, take it out of the water, swirl around, and tip out rinsate downstream of sampling site. |  |  |
| - Repeat twice so that the grab sampler has been rinsed three times. |  |  |
| - Then, fill the grab sampler at a depth of ~15 cm, with the mouth of the bottle facing upstream, slowly moving the bottle forward into theflow of water. |  |  |
| - Use this sample to rinse the labelled sample container three times using the same procedure as for taking a direct sample. |  |  |
| - Then, fill the rinsed, labelled sample container from the grab sample bottle. It may be necessary to refill the grab sample bottle to obtain enough water to fill the sample container. If this is necessary, recap the sample container between top-ups. Fill the sample container to therequired level (generally the shoulder,but check the sampling and analysis plan, as this may vary). |  |  |
| - Take care not to touch the opening of the sample container with any part of the grab pole bottle or your hands as this can easily introduce contaminants to the sample. |  |  |
| - If the sample container contains a preservative, be careful when filling to prevent any splash back. |  |  |
| - Cap (tightly) and store the sample container as required for transport to the laboratory. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Perform Water Quality Sampling (Surface Water) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare work area Collect sample for Microbiological analysis - Take Direct Sample - Take Grab sample |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Determine the extent of the sampling effort, the sampling methods to be employed, and which equipment and supplies are needed. |  |  |  |  |  |
|  | Obtain necessary sampling and monitoring equipment. |  |  |  |  |  |
|  | Decontaminate or pre clean equipment, and ensure that it is in working order |  |  |  |  |  |
|  | Ensure that labelling on the bottle to be filled is correct and that the samplenumber matches the number on the paperwork (field observation form). |  |  |  |  |  |
|  | Take the sample from the bank, or wade into the water in waders or boots, minimising disturbance as much as possible. |  |  |  |  |  |
|  | Don’t uncap the bottle until just before the sample is to be taken. |  |  |  |  |  |
|  | When filling the bottle (both for rinsing and for the final sample) take the sample upstream and to the side of you. |  |  |  |  |  |
|  | Uncap the bottle to be filled, immerse the bottle in the water (to a depth of ~15cm) lying it flat with its mouth towards the flow of the water, then slowly move the bottle forwards into the flowing water. |  |  |  |  |  |
|  | If rinsing is required (make sure you check the standard operating procedure) allow approximately 20 mL of water to enter the bottle. Cap, shake well andpour the rinsate downstream of yourself. |  |  |  |  |  |
|  | Repeat twice so the bottle is rinsed a total of three times (if required). |  |  |  |  |  |
|  | Then fill the bottle, again upstream and to the side of yourself at a depth of~15 cm, moving it slowly forwards through the water to the required level (generally the shoulder of the bottle, but always check the sampling and analysis plan). Cap, and store the sample container as required for transport to the laboratory. |  |  |  |  |  |
|  | If bottles contain preservatives (i.e. for metal analysis), they must not be rinsed and should be filled by decanting from another, rinsed bottle. |  |  |  |  |  |
|  | Ensure that labelling on bottle to be filled is correct and that the sample number matches the number on the paperwork (field observation form). |  |  |  |  |  |
|  | Check that the grab sampler is clean; this should be done at the completion of each day’s sampling, normally by simply rinsing in tap water. |  |  |  |  |  |
|  | Extend the grab sampler so that it will reach to the point that you wish to sample. |  |  |  |  |  |
|  | Carefully lower the grab sampler into the water with the mouth of the bottle facing upstream into the flow of water to a depth of ~15 cm. Keep the bottle moving forward into the flow of water while it is filling |  |  |  |  |  |
|  | Allow the bottle to partially fill, take it out of the water, swirl around, and tip out rinsate downstream of sampling site. |  |  |  |  |  |
|  | Repeat twice so that the grab sampler has been rinsed three times. |  |  |  |  |  |
|  | Then, fill the grab sampler at a depth of ~15 cm, with the mouth of the bottle facing upstream, slowly moving the bottle forward into the flow of water. |  |  |  |  |  |
|  | Use this sample to rinse the labelled sample container three times using the same procedure as for taking a direct sample. |  |  |  |  |  |
|  | Then, fill the rinsed, labelled sample container from the grab sample bottle. It may be necessary to refill the grab sample bottle to obtain enough water to fill the sample container. If this is necessary, recap the sample container between top-ups. Fill the sample container to therequired level (generally the shoulder,but check the sampling and analysis plan, as this may vary). |  |  |  |  |  |
|  | Take care not to touch the opening of the sample container with any part of the grab pole bottle or your hands as this can easily introduce contaminants to the sample. |  |  |  |  |  |
|  | If the sample container contains a preservative, be careful when filling to prevent any splash back. |  |  |  |  |  |
|  | Cap (tightly) and store the sample container as required for transport to the laboratory. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Perform Water Quality Sampling (Surface Water) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the different method of sampling for Surface Water? |  |  |
|  | What is the depth required for collecting Sample for Surface Water? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________