Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Testing of Calcium & Magnesium by EDTA Titrimetric Method Level 5 |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Prepare samples for testing - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during work |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| - Minimum Evidence Required | Prepare samples for testing - Check the sample label for the requirement of testing Calcium. - Prepare EDTA 0.01M solution - Prepare Calcium standard solution of 10 and 100 ppm . - Identify appropriate test method, Glassware and safety requirements. Prepare samples for testing - Wash all glassware as required for analysis - Set up the apparatus. Fill the titrant (EDTA solution of 0.01 M) in burette accordance with the specified work instructions. - Conduct pre-use and safety checks Perform tests on samples - Prepare sample in 100 ml beaker as per standard. - Maintain the pH (12) of the sample - Add indicator and Shake it till the indicator gets dissolved . - Then titrate it slowly with constant stirring against the EDTA (0.01 M) as per standard. - Multiply the EDTA burette reading with factor, this will be equal to the calcium concentration in the sample. - Analyzed Three replicate for each sample. - Store unused reagents and dispose off wastes as required by relevant regulations and codes. - Calculate Magnesium by using the formula Quality Control Checks - Verify EDTA concentration by titrating with known standards of Ca. - Apply quality control checks before sample analysis. Record the results/ Finalize works - Note down the Results on analyst workbook. - Calculate uncertainty of the results Adopt precautions during work - Maintain the pH of the sample to 12. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Competency Standard | Testing of Calcium & Magnesium by EDTA Titrimetric Method Level 5 |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare samples for testing - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during work |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Check the sample label for the requirement of testing Calcium |  |  |
| - Prepare EDTA 0.01M solution |  |  |
| - Prepare Calcium standard solution of10 or 100 ppm |  |  |
| - Identify appropriate test method, Glassware and safety requirements |  |  |
| - Wash all glassware as required for analysis |  |  |
| - Set up the apparatus. Fill the titrant (EDTA solution of 0.01 M) in burette accordance with the specified work instructions. |  |  |
| - Conduct pre-use and safety checks. |  |  |
| - Prepare sample in 100 ml beaker as per standard. |  |  |
| - Maintain the pH (12) of the sample |  |  |
| - Add indicator and Shake it till the indicator gets dissolved |  |  |
| - Then titrate it slowly with constant stirring against the EDTA (0.01 M) as per standard. |  |  |
| - Multiply the EDTA burette reading with 40, this will be equal to the calcium concentration in the sample |  |  |
| - Analyzed Three replicate for each sample. |  |  |
| - Store unused reagents and dispose off wastes as required by relevant regulations and codes. |  |  |
| - Calculate Magnesium by using the formula |  |  |
| - Verify EDTA concentration by titrating with known standards of Ca. |  |  |
| - Apply quality control checks before sample analysis. |  |  |
| - Note down the Results on analyst workbook. |  |  |
| - Calculate uncertainty of the results |  |  |
| - Maintain the pH of the sample to 12. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Testing of Calcium & Magnesium by EDTA Titrimetric Method |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare samples for testing - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during work |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Check the sample label for the requirement of testing Calcium. |  |  |  |  |  |
|  | Prepare EDTA 0.01M solution |  |  |  |  |  |
|  | Prepare Calcium standard solution of10 or 100 ppm |  |  |  |  |  |
|  | Identify appropriate test method, Glassware and safety requirements. |  |  |  |  |  |
|  | Wash all glassware as required for analysis |  |  |  |  |  |
|  | Set up the apparatus. Fill the titrant (EDTA solution of 0.01 M) in burette accordance with the specified work instructions |  |  |  |  |  |
|  | Conduct pre-use and safety checks. |  |  |  |  |  |
|  | Prepare sample in 100 ml beaker as per standard. |  |  |  |  |  |
|  | Maintain the pH (12) of the sample |  |  |  |  |  |
|  | Add indicator and shake it till the indicator gets dissolved |  |  |  |  |  |
|  | Then titrate it slowly with constant stirring against the EDTA (0.01 M) as per standard. |  |  |  |  |  |
|  | Multiply the EDTA burette reading with 40, this will be equal to the calcium concentration in the sample |  |  |  |  |  |
|  | Analyzed Three replicate for each sample |  |  |  |  |  |
|  | Store unused reagents and dispose off wastes as required by relevant regulations and codes |  |  |  |  |  |
|  | Calculate Magnesium by using the formula |  |  |  |  |  |
|  | Verify EDTA concentration is by titrating with known standards of Ca. |  |  |  |  |  |
|  | Apply quality control checks before sample analysis. |  |  |  |  |  |
|  | Note down the Results on analyst workbook |  |  |  |  |  |
|  | Calculate uncertainty of the results |  |  |  |  |  |
|  | Maintain the pH of the sample to 12. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | Testing of Calcium & Magnesium by EDTA Titrimetric Method |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Why 1M NaOH is added to analyze calcium? |  |  |
|  | Why burette reading after titrating sample for calcium is multiplied by 40,Also write down formula ? |  |  |
|  | Write down standardization procedure for EDTA? |  |  |
|  | How 1 M NaOH is prepared? |  |  |
|  | How many grams of EDTA is required to prepare 0.01M EDTA Solution in 1 litre? |  |  |
|  | Write down procedure for preparation of murexide? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________