Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Perform Demonstrate testing for Total Coliforms , Fecal Coliforms & E-coli (Multiple Tube Fermentation Technique, Method-I) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs.time frame (for practical demonstration & assessment): - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for Coliform - Adopt recommended preparatory method for Fecal Coliform - Adopt recommended preparatory method for E-coli - Perform handling or basic testing of samples for coliforms - Perform handling or basic testing of samples for fecal coliforms - Perform handling or basic testing of samples for E-coli - Apply appropriate confirmation techniques - Maintain Environment and Quality Control - Reporting of results using appropriate units |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare to undertake basic microbiological tests - Prepare sterile environment for undertaking tests. - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. - Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. Adopt recommended preparatory method for Coliform - Use commercially available media for detection of Coliforms following Multiple Tube Fermentation technique. - Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength by weighing an amount as per supplier’s instructions using calibrated analytical balance as presumptive phase. - Use fifteen test tubes method for testing as per standard method. - Add inverted vial (durham tube) in each test tube or alternatively add 0.01 g/L bromocresol purple in media. - Prepare recommended media (brilliant green lactose bile broth (BGLB) for confirmed Coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance as confirmed phase. - Add inverted vial (durham tube) in each test tube and close tubes with metal or heat resistant plastic caps. - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. - Autoclave media and store all sterilized media in refrigerator as per standard before use. Adopt recommended preparatory method for Fecal Coliform - Use commercially available media for detection of fecal coliforms following Multiple Tube Fermentation technique. - Prepare recommended media (EC broth) for confirmation of fecal coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. - Dispense media in fermentation tubes along with inverted fermentation tubes, close tubes with metal or heat resistant plastic caps. - Maintain pH of media as per manufacturer’s instruction after sterilization. - Autoclave media and store all sterilized media in refrigerator as per standards before use. Adopt recommended preparatory method for E-coli - Use commercially available media for detection of E-coli following Multiple Tube Fermentation technique. - Prepare recommended media (LEMB agar) for confirmation of E-coli by weighing an amount as per supplier’s instructions using calibrated analytical balance. - Dispense media in bottle, autoclave and bring at room temperature, dispense in each petri plate and allow solidifying. Perform handling or basic testing of samples for coliforms - For presumptive test, inoculate test tubes of double and single strength of LTB media as per standards. - Incubate test tubes at 35 oC temperature for the growth of coliforms for 24-48 hours as per standards. - Positive test tubes are shown as colour change from purple to yellow and the gas and bubble formation in the test tubes. - Negative test shows no colour change and gas formation. - For confirmation phase, shift positive LTB into Brilliant Green Lactose Bile Broth (BGLB) using metallic loop. - Incubate test tubes at 35 oC for confirmed growth of coliforms into BGLB for 24-48 hours as per standards. - For completed phase and QC check, inoculate 10 % positive test tubes of EC medium, incubate as per standard, note the positive test tube as turbid. - Report results as MPN index/100 ml using MPN table. Perform handling or basic testing of samples for fecal coliforms - For fecal coliform confirmation, Shift a loopful of positive growth from LTB into EC broth, incubate for 24-48 hours at standard required temperature. - Report results of Fecal Coliforms using MPN table. Perform handling or basic testing of samples for E-coli - Streak LEMB agar plate using platinum loop, incubate at 35 oC temperature as per standards. - Growth of dark centred purple colonies, flat, with or without metallic sheen indicates E-coli colonies. Apply appropriate confirmation techniques - Perform microscopic examination as per standards for the confirmation of E-coli which seems rod shape and red in colour during gram staining. - Perform biochemical testing as per standards i.e. IMViC test for the confirmation of E-coli. Maintain Environment and Quality Control - Accept or reject media / sample based on the condition and sterility as per standards. - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. - Use controls i.e. positive and negative controls along with the tests as per standards. - Clean and store equipment in accordance with laboratory guidelines. - Sterilize equipment as required - Store chemicals, media and controls as per standards. - Clean and disinfect area - Dispose contaminated waste properly in accordance with the standards. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Competency Standard | # Perform Demonstrate testing for Total Coliforms , Fecal Coliforms & E-coli (Multiple Tube Fermentation Technique, Method-I) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare to undertake basic microbiological tests - 2. Adopt recommended preparatory method for Coliform - 3. Adopt recommended preparatory method for Fecal Coliform - 4. Adopt recommended preparatory method for E-coli - 5. Perform handling or basic testing of samples for coliforms - 5. Perform handling or basic testing of samples for fecal coliforms - 6. Perform handling or basic testing of samples for E-coli - Apply appropriate confirmation techniques - Maintain Environment and Quality Control - Reporting of results using appropriate units |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare sterile environment for undertaking tests. |  |  |
| - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. |  |  |
| - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |
| - Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. |  |  |
| - Use commercially available media for detection of Coliforms following Multiple Tube Fermentation technique. |  |  |
| - Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength by weighing an amount as per supplier’s instructions using calibrated analytical balance as presumptive phase. |  |  |
| - Use fifteen test tubes method for testing as per standard method. |  |  |
| - Add inverted vial (durham tube) in each test tube or alternatively add 0.01 g/L bromocresol purple in media. |  |  |
| - Prepare recommended media (brilliant green lactose bile broth (BGLB) for confirmed Coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance as confirmed phase. |  |  |
| - Add inverted vial (durham tube) in each test tube and close tubes with metal or heat resistant plastic caps. |  |  |
| - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |
| - Autoclave media and store all sterilized media in refrigerator as per standard before use. |  |  |
| - Use commercially available media for detection of Fecal Coliforms following Multiple Tube Fermentation technique. |  |  |
| - Prepare recommended media (EC broth) for confirmation of Fecal Coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |
| - Dispense media in fermentation tubes along with inverted fermentation tubes, close tubes with metal or heat resistant plastic caps. |  |  |
| - Maintain pH of media as per manufacturer’s instruction after sterilization. |  |  |
| - Autoclave media and store all sterilized media in refrigerator as per standards before use. |  |  |
| - Use commercially available media for detection of E-coli following Multiple Tube Fermentation technique. |  |  |
| - Prepare recommended media (LEMB agar) for confirmation of E-coli by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |
| - Dispense media in bottle, autoclave and bring at room temperature, dispense in each petri plate and allow solidifying. |  |  |
| - For presumptive test, inoculate test tubes of double and single strength of LTB media as per standards. |  |  |
| - Incubate test tubes at 35 oC temperature for the growth of coliforms for 24-48 hours as per standards. |  |  |
| - Positive test tubes are shown as colour change from purple to yellow and the gas and bubble formation in the test tubes. - Negative test shows no colour change and gas formation. |  |  |
| - For confirmation phase, shift positive LTB into Brilliant Green Lactose Bile Broth (BGLB) using metallic loop. |  |  |
| - Incubate test tubes at 35 oC for confirmed growth of coliforms into BGLB for 24-48 hours as per standards. |  |  |
| - For completed phase and QC check, inoculate 10 % positive test tubes of EC medium, incubate as per standard, note the positive test tube as turbid. |  |  |
| - Report results as MPN index/100 ml using MPN table. |  |  |
| - For fecal coliform confirmation, Shift a loopful of positive growth from LTB into EC broth, incubate for 24-48 hours at standard required temperature. |  |  |
| - Report results of Fecal Coliforms using MPN table. |  |  |
| - Streak LEMB agar plate using platinum loop, incubate at 35 oC temperature as per standards. |  |  |
| - Growth of dark centred purple colonies, flat, with or without metallic sheen indicates E-coli colonies. |  |  |
| - Perform microscopic examination as per standards for the confirmation of E-coli which seems rod shape and red in colour during gram staining. |  |  |
| - Perform biochemical testing as per standards i.e. IMViC test for the confirmation of E-coli. |  |  |
| - Accept or reject media / sample based on the condition and sterility as per standards. |  |  |
| - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |
| - Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |
| - Clean and store equipment in accordance with laboratory guidelines. |  |  |
| - Sterilize equipment as required |  |  |
| - Store chemicals, media and controls as per standards. |  |  |
| - Clean and disinfect area |  |  |
| - Dispose contaminated waste properly in accordance with the standards. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Perform Demonstrate testing for Total Coliforms , Fecal Coliforms & E-coli (Multiple Tube Fermentation Technique, Method-I) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for Coliform - Adopt recommended preparatory method for Fecal Coliform - Adopt recommended preparatory method for E-coli - Perform handling or basic testing of samples for coliforms - Perform handling or basic testing of samples for fecal coliforms - Perform handling or basic testing of samples for E-coli - Apply appropriate confirmation techniques - Maintain Environment and Quality Control - Reporting of results using appropriate units |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare sterile environment for undertaking tests. |  |  |  |  |  |
|  | Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. |  |  |  |  |  |
|  | Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |  |  |  |
|  | Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. |  |  |  |  |  |
|  | Use commercially available media for detection of Coliforms following Multiple Tube Fermentation technique. |  |  |  |  |  |
|  | Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength by weighing an amount as per supplier’s instructions using calibrated analytical balance as presumptive phase. |  |  |  |  |  |
|  | Use fifteen test tubes method for testing as per standard method. |  |  |  |  |  |
|  | Add inverted vial (durham tube) in each test tube or alternatively add 0.01 g/L bromocresol purple in media. |  |  |  |  |  |
|  | Prepare recommended media (brilliant green lactose bile broth (BGLB) for confirmed Coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance as confirmed phase. |  |  |  |  |  |
|  | Add inverted vial (durham tube) in each test tube and close tubes with metal or heat resistant plastic caps. |  |  |  |  |  |
|  | Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |  |  |  |
|  | Autoclave media and store all sterilized media in refrigerator as per standard before use. |  |  |  |  |  |
|  | Use commercially available media for detection of Fecal Coliforms following Multiple Tube Fermentation technique. |  |  |  |  |  |
|  | Prepare recommended media (EC broth) for confirmation of Fecal Coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |  |  |  |
|  | Dispense media in fermentation tubes along with inverted fermentation tubes, close tubes with metal or heat resistant plastic caps. |  |  |  |  |  |
|  | Maintain pH of media as per manufacturer’s instruction after sterilization. |  |  |  |  |  |
|  | Autoclave media and store all sterilized media in refrigerator as per standards before use. |  |  |  |  |  |
|  | Use commercially available media for detection of E-coli following Multiple Tube Fermentation technique. |  |  |  |  |  |
|  | Prepare recommended media (LEMB agar) for confirmation of E-coli by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |  |  |  |
|  | Dispense media in bottle, autoclave and bring at room temperature, dispense in each petri plate and allow solidifying. |  |  |  |  |  |
|  | For presumptive test, inoculate test tubes of double and single strength of LTB media as per standards. |  |  |  |  |  |
|  | Incubate test tubes at 35 oC temperature for the growth of coliforms for 24-48 hours as per standards. |  |  |  |  |  |
|  | Positive test tubes are shown as colour change from purple to yellow and the gas and bubble formation in the test tubes. |  |  |  |  |  |
|  | Negative test shows no colour change and gas formation. |  |  |  |  |  |
|  | For confirmation phase, shift positive LTB into Brilliant Green Lactose Bile Broth (BGLB) using metallic loop. |  |  |  |  |  |
|  | Incubate test tubes at 35 oC for confirmed growth of coliforms into BGLB for 24-48 hours as per standards. |  |  |  |  |  |
|  | For completed phase and QC check, inoculate 10 % positive test tubes of EC medium, incubate as per standard, note the positive test tube as turbid. |  |  |  |  |  |
|  | Report results as MPN index/100 ml using MPN table. |  |  |  |  |  |
|  | For fecal coliform confirmation, Shift a loopful of positive growth from LTB into EC broth, incubate for 24-48 hours at standard required temperature. |  |  |  |  |  |
|  | Report results of Fecal Coliforms using MPN table. |  |  |  |  |  |
|  | Streak LEMB agar plate using platinum loop, incubate at 35 oC temperature as per standards. |  |  |  |  |  |
|  | Growth of dark centred purple colonies, flat, with or without metallic sheen indicates E-coli colonies. |  |  |  |  |  |
|  | Perform microscopic examination as per standards for the confirmation of E-coli which seems rod shape and red in colour during gram staining. |  |  |  |  |  |
|  | Perform biochemical testing as per standards i.e. IMViC test for the confirmation of E-coli. |  |  |  |  |  |
|  | Accept or reject media / sample based on the condition and sterility as per standards. |  |  |  |  |  |
|  | Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |  |  |  |
|  | Accept or reject media / sample based on the condition and sterility as per standards. |  |  |  |  |  |
|  | Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |  |  |  |
|  | Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |  |  |  |
|  | Clean and store equipment in accordance with laboratory guidelines. |  |  |  |  |  |
|  | Sterilize equipment as required |  |  |  |  |  |
|  | Store chemicals, media and controls as per standards. |  |  |  |  |  |
|  | Clean and disinfect area |  |  |  |  |  |
|  | Dispose contaminated waste properly in accordance with the standards. |  |  |  |  |  |
|  | Sterilize equipment as required |  |  |  |  |  |
|  | Store chemicals, media and controls as per standards. |  |  |  |  |  |
|  | Clean and disinfect area |  |  |  |  |  |
|  | Dispose contaminated waste properly in accordance with the standards. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Perform Demonstrate testing for Total Coliforms , Fecal Coliforms & E-coli (Multiple Tube Fermentation Technique, Method-I) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define MPN method? |  |  |
|  | Why we choose E-coli as indicator bacteria? |  |  |
|  | What are the aseptic techniques followed during sample collection, transportation and inoculation of samples? |  |  |
|  | How double strength and single strength of media Lauryl tryptose broth is prepared? |  |  |
|  | Define the principle and working of autoclave? |  |  |
|  | Explain presumptive test for detection of Total Coliforms using this diagram? |  |  |
|  | Differentiate between gram positive and gram negative bacteria? |  |  |
|  | How contaminated media is disposed? |  |  |
|  | Write down the procedure of gram staining? |  |  |
|  | Explain Indole test? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________