Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Testing for Total Coliforms and Fecal Coliforms (Membrane Filtration Method, Method-II) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs.time frame (for practical demonstration & assessment): - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for coliform - Adopt recommended preparatory method for fecal coliform - Perform handling or basic testing of samples for coliforms - Perform handling or basic testing of samples for fecal coliforms - Maintain Environment and Quality Control - Reporting of results using appropriate units |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare to undertake basic microbiological tests - Prepare sterile environment for undertaking tests. - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle.Ensure that water sample is appropriate for the test requested as per standards. - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. - Transport sample within 1 hour after collection at < 8 oC but not frozen, for coliforms and fecal coliform analyses, the holding time from collection to analysis is 30 hour. Adopt recommended preparatory method for coliform - Use commercially available media for detection of Coliforms following Membrane Filtration technique. - Prepare recommended media (m-Endo agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. - Heat to boiling the media but do not autoclave. - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. - Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying .Adopt recommended preparatory method for Fecal Coliform - Use commercially available media for detection of fecal coliforms following Membrane Filtration technique. - Prepare recommended media (mFC agar) for confirmation of fecal coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. - Do not autoclave the media, just heat to boiling. - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. - Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying. Perform handling or basic testing of samples for coliforms - For coliform testing, pass recommended volume of water sample as per standards through the membrane filter using sterile membrane filtration assembly. - Place the membrane filter on recommended media for coliforms; incubate petri plates inverted in incubator set at 35oC temperature as per standard for 24-48 hours. - Count coliform colony as a pink to dark red color with metallic sheen .Report result in units as per standard. Perform handling or basic testing of samples for fecal coliforms - Pass recommended volume of water sample through the membrane filter using sterile membrane filtration assembly for fecal coliforms as per standards. - Place the membrane filter on recommended media for fecalcoliforms; incubate petri plates inverted in incubator set at the required temperature as per standard for 24-48 hours. - Count fecal coliform colony as a blue colour .Report result in units as per standard. Maintain Environment and Quality Control - Adopt recommended safety precautions when handling biologically hazardous specimen. - Accept or reject media / sample based on the condition and sterility as per standards. - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. - Use controls i.e. positive and negative controls along with the tests as per standards. - Clean and store equipment in accordance with laboratory guidelines. - Sterilize equipment as required in standard. Reporting of results using appropriate units - Report the results bacterial colonies of Total Coliforms as CFU/100ml. - Report the results of bacterial colonies Fecal Coliforms as CFU/100ml. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Competency Standard | # Testing for Total Coliforms and Fecal Coliforms (Membrane Filtration Method, Method-II) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for coliform - Adopt recommended preparatory method for fecal coliform - Perform handling or basic testing of samples for coliforms - Perform handling or basic testing of samples for fecal coliforms - Maintain Environment and Quality Control - Reporting of results using appropriate units |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare sterile environment for undertaking tests. |  |  |
| - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. |  |  |
| - Ensure that water sample is appropriate for the test requested as per standards. |  |  |
| - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |
| - Transport sample within 1 hour after collection at < 8 oC but not frozen, for coliforms and fecal coliform analyses, the holding time from collection to analysis is 30 hour. |  |  |
| - Use commercially available media for detection of Coliforms following Membrane Filtration technique. |  |  |
| - Prepare recommended media (m-Endo agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. |  |  |
| - Heat to boiling the media but do not autoclave. |  |  |
| - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |
| - Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying. |  |  |
| - Use commercially available media for detection of fecal coliforms following Membrane Filtration technique. |  |  |
| - Prepare recommended media (mFC agar) for confirmation of fecal coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |
| - Do not autoclave the media, just heat to boiling. |  |  |
| - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |
| - Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying. |  |  |
| - For coliform testing, pass recommended volume of water sample as per standards through the membrane filter using sterile membrane filtration assembly. |  |  |
| - Place the membrane filter on recommended media for coliforms; incubate petri plates inverted in incubator set at 35oC temperature as per standard for 24-48 hours. |  |  |
| - Count coliform colony as a pink to dark red color with metallic sheen .Report result in units as per standard. |  |  |
| - Pass recommended volume of water sample (100 ml) through the membrane filter using sterile membrane filtration assembly for fecal coliforms as per standards. |  |  |
| - Place the membrane filter on recommended media for fecal coliforms; incubate petri plates inverted in incubator set at the 44.5 oC temperature as per standard for 24-48 hours. |  |  |
| - Count fecal coliform colony as a blue colour .Report result in units as per standard. |  |  |
| - Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |
| - Accept or reject media / sample based on the condition and sterility as per standards. |  |  |
| - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |
| - Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |
| - Clean and store equipment in accordance with laboratory guidelines. |  |  |
| - Sterilize equipment as required in standard. |  |  |
| - Report the results of bacterial colonies of Total Coliforms as CFU/100ml. |  |  |
| - Report the results of bacterial colonies Fecal Coliforms as CFU/100ml. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Testing for Total Coliforms and Fecal Coliforms (Membrane Filtration Method, Method-II) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for coliform - Adopt recommended preparatory method for fecal coliform - Perform handling or basic testing of samples for coliforms - Perform handling or basic testing of samples for fecal coliforms - Maintain Environment and Quality Control - Reporting of results using appropriate units |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare sterile environment for undertaking tests. |  |  |  |  |  |
|  | Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. |  |  |  |  |  |
|  | Ensure that water sample is appropriate for the test requested as per standards. |  |  |  |  |  |
|  | Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |  |  |  |
|  | Transport sample within 1 hour after collection at < 8 oC but not frozen, for coliforms and fecal coliform analyses, the holding time from collection to analysis is 30 hour. |  |  |  |  |  |
|  | Use commercially available media for detection of Coliforms following Membrane Filtration technique. |  |  |  |  |  |
|  | Prepare recommended media (m-Endo agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. |  |  |  |  |  |
|  | Heat to boiling the media but do not autoclave. |  |  |  |  |  |
|  | Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |  |  |  |
|  | Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying. |  |  |  |  |  |
|  | Use commercially available media for detection of fecal coliforms following Membrane Filtration technique. |  |  |  |  |  |
|  | Prepare recommended media (mFC agar) for confirmation of fecal coliforms by weighing an amount as per supplier’s instructions using calibrated analytical balance. |  |  |  |  |  |
|  | Do not autoclave the media, just heat to boiling. |  |  |  |  |  |
|  | Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |  |  |  |
|  | Bring medium at room temperature after autoclaving, dispense in each petri plate and allow solidifying. |  |  |  |  |  |
|  | For coliform testing, pass recommended volume of water sample as per standards through the membrane filter using sterile membrane filtration assembly. |  |  |  |  |  |
|  | Place the membrane filter on recommended media for coliforms; incubate petri plates inverted in incubator set at 35oC temperature as per standard for 24-48 hours. |  |  |  |  |  |
|  | Count coliform colony as a pink to dark red color with metallic sheen .Report result in units as per standard. |  |  |  |  |  |
|  | Pass recommended volume of water sample (100 ml) through the membrane filter using sterile membrane filtration assembly for fecal coliforms as per standards. |  |  |  |  |  |
|  | Place the membrane filter on recommended media for fecal coliforms; incubate petri plates inverted in incubator set at the 44.5 oC temperature as per standard for 24-48 hours. |  |  |  |  |  |
|  | Count fecal coliform colony as a blue colour .Report result in units as per standard. |  |  |  |  |  |
|  | Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |  |  |  |
|  | Accept or reject media / sample based on the condition and sterility as per standards. |  |  |  |  |  |
|  | Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |  |  |  |
|  | Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |  |  |  |
|  | Clean and store equipment in accordance with laboratory guidelines. |  |  |  |  |  |
|  | Sterilize equipment as required in standard. |  |  |  |  |  |
|  | Report the results of bacterial colonies of Total Coliforms as CFU/100ml. |  |  |  |  |  |
|  | Report the results of bacterial colonies Fecal Coliforms as CFU/100ml. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Competency Standard | # Testing for Total Coliforms and Fecal Coliforms (Membrane Filtration Method, Method-II) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | Define the principle of MF method? |  |  |  |
|  | How the sterilization of equipment and glassware is done using autoclave? |  |  |  |
|  | Write down the procedure for the preparation of m-endo agar? |  |  |  |
|  | Write down the procedure of the preparation of mFC agar? |  |  |  |
|  | Write down the steps for the preparation of agar petriplates? |  |  |  |
|  | What are the colours of colonies of Total Coliforms and Fecal Coliforms on m-Endo agar and mFC agar? |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |