Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Module | Water Testing Technician (Microbiological)-I |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Prepare to undertake basic microbiological tests i.e. Multiple Tube Fermentation technique (MPN)/Membrane Filtration technique (MF Method) as per instructor. - Adopt recommended preparatory method for Total Coliform, Fecal Coliforms &E-coli. - Perform handling or basic testing of samples for Total Coliform, Fecal Coliforms &E-coli. - Perform Microscopy by gram staining - Apply appropriate confirmation techniques using MPN method - Maintain Environment and Quality Control - Report results using appropriate units |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare to undertake basic microbiological tests i.e. Multiple Tube Fermentation technique (MPN) / Membrane Filtration technique (MF Method) as per instructor. - Collect samples for microbiological examination according to standard protocols. - Collect sample in clean, sterile, non-reactive borosilicate glass or plastic bottle. - Ensure volume of water sample is appropriate for test. - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. Adopt recommended preparatory method for Total Coliform, Fecal Coliforms &E-coli following MPN Method. - Use commercially available media for detection of Total Coliform, Fecal Coliforms &E-coli - Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength / brilliant green lactose bile broth (BGLB)/EC broth / LEMB agar as per instructions. Perform handling or basic testing of samples for Total Coliform, Fecal Coliforms & E-coli following MPN Method. - Perform presumptive test for Coliforms using fifteen test tube method - Perform confirmation test for Coliforms - Perform completed test for Coliforms - Perform test for confirmation of fecal coliforms - Perform test for confirmation of E-coli Apply appropriate confirmation techniques - Perform microscopy by gram staining - Perform biochemical testing using MPN technique Adopt recommended preparatory/analytical method for Total Coliform & Fecal Coliforms following MF Method. - Prepare media for m-endo agar & mFC agar for Total Coliforms and Fecal Coliforms by MF Method - Perform handling or basic testing of samples for Total Coliform & Fecal Coliforms following MF Method. - Record results using appropriate units. Maintain Environment and Quality Control - Use controls i.e. positive and negative controls along with the tests as per standards. - Sterilize equipment as required - Clean and disinfect area - Dispose contaminated waste properly in accordance with the standards. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| Module | Water Testing Technician (Microbiological)-I |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Prepare to undertake basic microbiological tests i.e. Multiple Tube Fermentation technique (MPN)/Membrane Filtration technique (MF Method) as per instructor. - Adopt recommended preparatory method for Total Coliform, Fecal Coliforms &E-coli. - Perform handling or basic testing of samples for Total Coliform, Fecal Coliforms &E-coli. - Perform Microscopy by gram staining - Apply appropriate confirmation techniques using MPN method - Maintain Environment and Quality Control - Report results using appropriate units |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Collect samples for microbiological examination according to standard protocols. |  |  |
| - Collect sample in clean, sterile, non reactive borosilicate glass or plastic bottle. |  |  |
| - Ensure volume of water sample is appropriate for test. |  |  |
| - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |
| - Use commercially available media for detection of Total Coliform, Fecal Coliforms &E-coli. |  |  |
| - Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength / brilliant green lactose bile broth (BGLB)/ EC broth / LEMB agar as per supplier’s instructions. |  |  |
| - Perform presumptive test for Coliforms |  |  |
| - Perform confirmation test for Coliforms |  |  |
| - Perform completed test for Coliforms |  |  |
| - Perform test for confirmation of fecal coliforms |  |  |
| - Perform test for confirmation of E-coli |  |  |
| - Perform microscopy by gram staining |  |  |
| - Perform biochemical testing using MPN technique |  |  |
| - Prepare media for m-endo agar & mFC agar for Total Coliforms and Fecal Coliforms by MF Method |  |  |
| - Perform handling or basic testing of samples for Total Coliform & Fecal Coliforms following MF Method. |  |  |
| - Record results using appropriate units. |  |  |
| - Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |
| - Sterilize equipment as required |  |  |
| - Clean and disinfect area |  |  |
| - Dispose contaminated waste properly in accordance with the standards. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Module | Water Testing Technician (Microbiological)-I |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare to undertake basic microbiological tests i.e. Multiple Tube Fermentation technique (MPN)/Membrane Filtration technique (MF Method) as per instructor. - Adopt recommended preparatory method for Total Coliform, Fecal Coliforms &E-coli. - Perform handling or basic testing of samples for Total Coliform, Fecal Coliforms &E-coli. - Perform Microscopy by gram staining - Apply appropriate confirmation techniques using MPN method - Maintain Environment and Quality Control - Report results using appropriate units |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Collect samples for microbiological examination according to standard protocols. |  |  |  |  |  |
|  | Collect sample in clean, sterile, non reactive borosilicate glass or plastic bottle. |  |  |  |  |  |
|  | Ensure volume of water sample is appropriate for test. |  |  |  |  |  |
|  | Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |  |  |  |
|  | Use commercially available media for detection of Total Coliform, Fecal Coliforms &E-coli. |  |  |  |  |  |
|  | Prepare recommended media (Lauryl tryptose broth, LTB) double strength and single strength / brilliant green lactose bile broth (BGLB)/ EC broth / LEMB agar as per supplier’s instructions. |  |  |  |  |  |
|  | Perform presumptive test for Coliforms |  |  |  |  |  |
|  | Perform confirmation test for Coliforms |  |  |  |  |  |
|  | Perform completed test for Coliforms |  |  |  |  |  |
|  | Perform test for confirmation of fecal coliforms |  |  |  |  |  |
|  | Perform test for confirmation of E-coli |  |  |  |  |  |
|  | Perform microscopy by gram staining |  |  |  |  |  |
|  | Perform biochemical testing using MPN technique |  |  |  |  |  |
|  | Prepare media for m-endo agar & mFC agar for Total Coliforms and Fecal Coliforms by MF Method |  |  |  |  |  |
|  | Perform handling or basic testing of samples for Total Coliform & Fecal Coliforms following MF Method. |  |  |  |  |  |
|  | Record results using appropriate units. |  |  |  |  |  |
|  | Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |  |  |  |
|  | Sterilize equipment as required |  |  |  |  |  |
|  | Clean and disinfect area |  |  |  |  |  |
|  | Dispose contaminated waste properly in accordance with the standards. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate-Level-3 in Water Quality & Resource Management (Water Quality Surveyor) |
| --- | --- |
| Module | Water Testing Technician (Microbiological) - I |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain the principle of MF Method? |  |  |
|  | What are different methods of sterilization? |  |  |
|  | Explain positive and negative controls? Give examples. |  |  |
|  | How LEMB Agar plates are prepared? |  |  |
|  | What are the colors of coliorm and fecal coliform colonies in membrane filtration method? |  |  |
|  | Explain the procedure of decontamination of glassware and media? |  |  |
|  | Explain methyl red test? |  |  |
|  | How the colonies of E-coli appear after gram staining? |  |  |
|  | What are the results of confirmed colifomrs in MPN method? |  |  |
|  | Explain the disadvantages of MPN method? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________