Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | Perform Testing/Determination of Fluoride (F) by ISE Meter |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Perform Testing of flouride by ISE Meter - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during result |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Perform Testing of fluoride by ISE Meter - Check the sample label for the requirement of analysis of Fluoride. - prepare flouride control solutions from reference stock solution of 1000 ppm as per requirement - Identify appropriate test method, Glassware, Equipment and safety requirements. Prepare for laboratory testing - Set up test equipment ISE Meter and reagents in accordance with the specified work instructions. - Conduct pre-use and safety checks. - Check the calibration status of equipment. Perform calibration if required. Perform tests on samples - Take specific volume (as per standard) of deionized water in a cuvette and add specific volume of fluoride Buffer (pH: 5.3-5.5) in it. - Check for zero calibration. - Check the concentration of known standards for verification of calibration and efficiency of electrode. - Analyze sample. - Analyze three replicate for each sample and Store unused reagents and dispose of wastes as required by relevant regulations and codes. - Clean and store equipment. Apply Quality Control Checks - Calibrate ISE meter by running known standard - Analyze Method blank after every 10 samples. - Analyze Laboratory Control samples with every batch of 10 samples. - Check repeatability. - Record Daily quality control data in QC Data sheet-I. Record the results/ Finalise work - Report the results on result record form with proper uncertainty and unit. - Clear and restore instrument. Adopt precautions during work - Check calibration. - Check the pH of the buffer solution, it must be in range. - Adopt precautionary measurements for probe handling. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| Competency Standard | Perform Testing/Determination of Fluoride (F) by ISE Meter |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Perform Testing of pH by pH Meter - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during result |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Check the sample label for the requirement of analysis of Fluoride. |  |  |
| - prepare fluoride control solutions from reference stock solution of 1000 ppm as per requirement |  |  |
| - Identify appropriate test method, Glassware, Equipment and safety requirements. |  |  |
| - Set up test equipment ISE Meter and reagents in accordance with the specified work instructions. |  |  |
| - Conduct pre-use and safety checks. |  |  |
| - Check the calibration status of equipment. Perform calibration if required. |  |  |
| - Take specific volume (as per standard) of deionized water in a cuvette and add specific volume of fluoride Buffer (pH: 5.3-5.5) in it. |  |  |
| - Check for zero calibration. |  |  |
| - Check the concentration of known standards for verification of calibration and efficiency of electrode. |  |  |
| - Analyze sample. |  |  |
| - Analyze three replicate for each sample and Store unused reagents and dispose of wastes as required by relevant regulations and codes. |  |  |
| - Clean and store equipment. |  |  |
| - Calibrate ISE meter by running known standard |  |  |
| - Analyze Method blank after every 10 samples. |  |  |
| - Analyze Laboratory Control samples with every batch of 10 samples. |  |  |
| - Check repeatability. |  |  |
| - Record Daily quality control data in QC Data sheet-I. |  |  |
| - Report the results on result record form with proper uncertainty and unit. |  |  |
| - Clear and restore instrument . |  |  |
| - Check calibration. |  |  |
| - Check the pH of the buffer solution, it must be in range. |  |  |
| - Adopt precautionary measurements for probe handling. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | Perform Testing/Determination of Fluoride (F) by ISE Meter |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform Testing of pH by pH Meter - Prepare for laboratory testing - Perform tests on samples - Quality Control Checks - Record the results/ Finalize work - Adopt precautions during result |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Check the sample label for the requirement of analysis of Fluoride. |  |  |  |  |  |
|  | prepare fluoride control solutions from reference stock solution of 1000 ppm as per requirement |  |  |  |  |  |
|  | Identify appropriate test method, Glassware, Equipment and safety requirements. |  |  |  |  |  |
|  | Set up test equipment ISE Meter and reagents in accordance with the specified work instructions. |  |  |  |  |  |
|  | Conduct pre-use and safety checks. |  |  |  |  |  |
|  | Check the calibration status of equipment. Perform calibration if required. |  |  |  |  |  |
|  | Take specific volume (as per standard) of deionized water in a cuvette and add specific volume of fluoride Buffer (pH: 5.3-5.5) in it. |  |  |  |  |  |
|  | Check for zero calibration. |  |  |  |  |  |
|  | Check the concentration of known standards for verification of calibration and efficiency of electrode. |  |  |  |  |  |
|  | Analyze sample. |  |  |  |  |  |
|  | Analyze three replicate for each sample and Store unused reagents and dispose of wastes as required by relevant regulations and codes. |  |  |  |  |  |
|  | Clean and store equipment. |  |  |  |  |  |
|  | Calibrate ISE meter by running known standard |  |  |  |  |  |
|  | Analyze Method blank after every 10 samples. |  |  |  |  |  |
|  | Analyze Laboratory Control samples with every batch of 10 samples. |  |  |  |  |  |
|  | Check repeatability. |  |  |  |  |  |
|  | Record Daily quality control data in QC Data sheet-I. |  |  |  |  |  |
|  | Report the results on result record form with proper uncertainty and unit. |  |  |  |  |  |
|  | Clear and restore instrument. |  |  |  |  |  |
|  | Check calibration. |  |  |  |  |  |
|  | Check the pH of the buffer solution, it must be in range. |  |  |  |  |  |
|  | Adopt precautionary measurements for probe handling. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | Perform Testing/Determination of Fluoride (F) by ISE Meter |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the basic principle of fluoride measurement? |  |  |
|  | What is the effect of high fluoride level in water samples to human body ? |  |  |
|  | What is meant by ISE meter? |  |  |
|  | What is the ratio between fluoride control samples for preparing linear curve? |  |  |
|  | What is the drinking water range of fluoride? |  |  |
|  | Which method is used for measuring fluoride? |  |  |
|  | What are methods for analyzing fluoride? List any two methods. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________