Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Plate Count (Method-III) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs.time frame (for practical demonstration & assessment): - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for Total Plate Count - Perform handling or basic testing of samples for Total Plate count - Maintain Environment and Quality Control |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare to undertake basic microbiological tests - Prepare sterile environment for undertaking tests. - Collect samples for microbiological examination in clean, sterile, non- reactive borosilicate glass or plastic bottle. - Ensure that water sample is appropriate for the test requested as per standards. - When the sample is collected, leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. - Ice samples during transport to the laboratory if they cannot be processed within 1 hour after collection. - Maintain samples in the dark and keep cool with ice at < 8 oC but not frozen. Adopt recommended preparatory method for Total bacterial Count - Use commercially available media for detection of total bacterial count following plate count technique. - Prepare recommended media (Plate count agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. - Sterilize media using autoclave as per standards. - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. - Bring medium at room temperature before use or store in the refrigerator. Perform handling or basic testing of samples for Total bacterial count - Prepare dilutions for polluted water samples. - Pour recommended volume of water sample to be tested into petri plate using auto-pipette by pour plate method. - Pour media into plates, shake well, and allow solidifying and incubating at the recommended temperature for 48 hours. - Count bacterial colonies after incubation and report as CFU/ml as per standards. Maintain Environment and Quality Control - Adopt recommended safety precautions when handling biologically hazardous specimen. - Accept or reject media / sample based on the condition and sterility as per standards. - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. - Use controls i.e. positive and negative controls along with the tests as per standards. - Clean and store equipment in accordance with laboratory guidelines. - Sterilize equipment as required - Store chemicals, media and controls as per standards. - Clean and disinfect area. - Properly dispose contaminated waste in accordance with the standards. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| Competency Standard | # Perform Testing for Total Plate Count (Method-III) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for Total Plate Count - Perform handling or basic testing of samples for Total Plate count - Maintain Environment and Quality Control |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare sterile environment for undertaking tests. |  |  |
| - Collect samples for microbiological examination in clean, sterile, non- reactive borosilicate glass or plastic bottle. |  |  |
| - Ensure that water sample is appropriate for the test requested as per standards. |  |  |
| - When the sample is collected, leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |
| - Ice samples during transport to the laboratory if they cannot be processed within 1 hour after collection. |  |  |
| - Maintain samples in the dark and keep cool with ice at < 8 oC but not frozen. |  |  |
| - Use commercially available media for detection of total bacterial count following plate count technique. |  |  |
| - Prepare recommended media (Plate count agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. |  |  |
| - Sterilize media using autoclave as per standards. |  |  |
| - Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |
| - Bring medium at room temperature before use or store in the refrigerator. |  |  |
| - Prepare dilutions for polluted water samples. |  |  |
| - Pour recommended volume of water sample to be tested into petri plate using auto-pipette by pour plate method. |  |  |
| - Pour media into plates, shake well, and allow solidifying and incubating at the recommended temperature for 48 hours. |  |  |
| - Count bacterial colonies after incubation and report as CFU/ml as per standards. |  |  |
| - Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |
| - Accept or reject media / sample based on the condition and sterility as per standards. |  |  |
| - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |
| - Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |
| - Clean and store equipment in accordance with laboratory guidelines. |  |  |
| - Sterilize equipment as required |  |  |
| - Store chemicals, media and controls as per standards. |  |  |
| - Clean and disinfect area. |  |  |
| - Properly dispose contaminated waste in accordance with the standards. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Plate Count (Method-III) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare to undertake basic microbiological tests - Adopt recommended preparatory method for Total Plate Count - Perform handling or basic testing of samples for Total Plate count - Maintain Environment and Quality Control |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare sterile environment for undertaking tests. |  |  |  |  |  |
|  | Collect samples for microbiological examination in clean, sterile, non- reactive borosilicate glass or plastic bottle. |  |  |  |  |  |
|  | Ensure that water sample is appropriate for the test requested as per standards. |  |  |  |  |  |
|  | When the sample is collected, leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |  |  |  |
|  | Ice samples during transport to the laboratory if they cannot be processed within 1 hour after collection. |  |  |  |  |  |
|  | Maintain samples in the dark and keep cool with ice at < 8 oC but not frozen. |  |  |  |  |  |
|  | Use commercially available media for detection of total bacterial count following plate count technique. |  |  |  |  |  |
|  | Prepare recommended media (Plate count agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. |  |  |  |  |  |
|  | Sterilize media using autoclave as per standards. |  |  |  |  |  |
|  | Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. |  |  |  |  |  |
|  | Bring medium at room temperature before use or store in the refrigerator. |  |  |  |  |  |
|  | Prepare dilutions for polluted water samples. |  |  |  |  |  |
|  | Pour recommended volume of water sample to be tested into petri plate using auto-pipette by pour plate method. |  |  |  |  |  |
|  | Pour media into plates, shake well, and allow solidifying and incubating at the recommended temperature for 48 hours. |  |  |  |  |  |
|  | Count bacterial colonies after incubation and report as CFU/ml as per standards. |  |  |  |  |  |
|  | Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |  |  |  |
|  | Accept or reject media / sample based on the condition and sterility as per standards. |  |  |  |  |  |
|  | Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |  |  |  |
|  | Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |  |  |  |
|  | Clean and store equipment in accordance with laboratory guidelines. |  |  |  |  |  |
|  | Sterilize equipment as required |  |  |  |  |  |
|  | Store chemicals, media and controls as per standards. |  |  |  |  |  |
|  | Clean and disinfect area. |  |  |  |  |  |
|  | Properly dispose contaminated waste in accordance with the standards. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Plate Count (Method-III) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Write down the procedure of sample collection for TPC? |  |  |
|  | Enlist different steps for the preparation of plate count agar media? |  |  |
|  | How the plates of plate count agar are prepared? |  |  |
|  | Why the dilution of water samples is prepared? |  |  |
|  | Define pour plate method? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________