Instruction Sheet for the Candidate

| Qualification | # National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Coliforms and E-coli (IDEXX Method, Method-IV) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs.time frame (for practical demonstration & assessment): - Prepare to undertake basic microbiological tests - Adopt recommended method for handling and testing of samples - Maintain Environment and Quality Control |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Prepare to undertake basic microbiological tests - Prepare sterile environment for undertaking tests. - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. - Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. Adopt recommended method for handling and testing of samples - Use commercially available media for detection of coliforms & E-coli following Colilert IDEXX quanti tray method. - Use recommended techniques to prepare samples for testing. - Add recommended volume of water sample in sterile mixing bottle and one package of the colilert reagent. - Transfer sample/reagent mixture from the mixing bottle into a quanti-tray. - Seal tray in IDEXX sealer, label tray on the back with sample code, date and time. - Incubate tray at the temperature set by the standard. - Count the number of small and large positive wells and report results using IDEXX MPN table to find the most probable number for Total Coliform. - Obtain E. coli results by placing the wells under a black light and counting the number of fluorescent wells. Maintain Environment and Quality Control - Adopt recommended safety precautions when handling biologically hazardous specimen. - Accept or reject media / sample based on the condition and sterility as per standards. - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. - Use controls i.e. positive and negative controls along with the tests as per standards. - Clean and store equipment in accordance with laboratory guidelines. - Sterilize equipment as required - Store chemicals, media and controls as per standards. - Clean and disinfect area. - Properly dispose contaminated waste in accordance with the standards. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | # National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| Competency Standard | # Perform Testing for Total Coliforms and E-coli (IDEXX Method, Method-IV) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare to undertake basic microbiological tests - Adopt recommended method for handling and testing of samples - Maintain Environment and Quality Control |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare sterile environment for undertaking tests. |  |  |
| - Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. |  |  |
| - Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |
| - Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. |  |  |
| - Use commercially available media for detection of coliforms & E-coli following Colilert IDEXX quanti tray method. |  |  |
| - Use recommended techniques to prepare samples for testing. |  |  |
| - Add recommended volume of water sample in sterile mixing bottle and one package of the colilert reagent. |  |  |
| - Transfer sample/reagent mixture from the mixing bottle into a quanti-tray. |  |  |
| - Seal tray in IDEXX sealer, label tray on the back with sample code, date and time. |  |  |
| - Incubate tray at the temperature set by the standard. |  |  |
| - Count the number of small and large positive wells and report results using IDEXX MPN table to find the most probable number for Total Coliform. |  |  |
| - Obtain E. coli results by placing the wells under a black light and counting the number of fluorescent wells. |  |  |
| - Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |
| - Accept or reject media / sample based on the condition and sterility as per standards. |  |  |
| - Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |
| - Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |
| - Clean and store equipment in accordance with laboratory guidelines. |  |  |
| - Sterilize equipment as required |  |  |
| - Store chemicals, media and controls as per standards. |  |  |
| - Clean and disinfect area. |  |  |
| - Properly dispose contaminated waste in accordance with the standards. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | # National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Coliforms and E-coli (IDEXX Method, Method-IV) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare to undertake basic microbiological tests - Adopt recommended method for handling and testing of samples - Maintain Environment and Quality Control |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare sterile environment for undertaking tests. |  |  |  |  |  |
|  | Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. Ensure volume of water sample is appropriate for test. |  |  |  |  |  |
|  | Leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. |  |  |  |  |  |
|  | Transport sample within 1 hour after collection at < 8 oC but not frozen, holding time from collection to analysis is 30 hour. |  |  |  |  |  |
|  | Use commercially available media for detection of coliforms & E-coli following Colilert IDEXX quanti tray method. |  |  |  |  |  |
|  | Use recommended techniques to prepare samples for testing. |  |  |  |  |  |
|  | Add recommended volume of water sample in sterile mixing bottle and one package of the colilert reagent. |  |  |  |  |  |
|  | Transfer sample/reagent mixture from the mixing bottle into a quanti-tray. |  |  |  |  |  |
|  | Seal tray in IDEXX sealer, label tray on the back with sample code, date and time. |  |  |  |  |  |
|  | Incubate tray at the temperature set by the standard. |  |  |  |  |  |
|  | Count the number of small and large positive wells and report results using IDEXX MPN table to find the most probable number for Total Coliform. |  |  |  |  |  |
|  | Obtain E. coli results by placing the wells under a black light and counting the number of fluorescent wells. |  |  |  |  |  |
|  | Adopt recommended safety precautions when handling biologically hazardous specimen. |  |  |  |  |  |
|  | Accept or reject media / sample based on the condition and sterility as per standards. |  |  |  |  |  |
|  | Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. |  |  |  |  |  |
|  | Use controls i.e. positive and negative controls along with the tests as per standards. |  |  |  |  |  |
|  | Clean and store equipment in accordance with laboratory guidelines. |  |  |  |  |  |
|  | Sterilize equipment as required |  |  |  |  |  |
|  | Store chemicals, media and controls as per standards. |  |  |  |  |  |
|  | Clean and disinfect area. |  |  |  |  |  |
|  | Properly dispose contaminated waste in accordance with the standards. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level-4 in “Water Quality and Resource Management” (Lab Analyst) |
| --- | --- |
| Competency Standard | # Perform Testing for Total Coliforms and E-coli (IDEXX Method, Method-IV) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define the principle of Colilert Method? |  |  |
|  | Write down different steps of IDEXX Colilert method? |  |  |
|  | How the positive results of Total Coliforms & E-coli are visible in contaminated sample? |  |  |
|  | How the results of wells are counted? |  |  |
|  | Write down the unit of results reported? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________