Instruction Sheet for the Candidate

| Qualification | National Diploma- Level-5 in “Water Quality and Resource Management” (Technician-Supervisor) |
| --- | --- |
| Competency Standard | Hydraulics and Irrigation Engineering |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Find pressure by Piezometer and/or manometer - Find out Hydrostatic pressure and center of pressure on vertically Immersed surface. - Calculate hydraulic and energy gradient for pipe lines. - Calculate flow in open channels. - Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages - Draw the plan of a multipurpose irrigation project - Draw cross section of dams - Find design discharge of Karif and Rabi crops for given area - Draw the alignment of a canal, minor distributary and watercourses on contour map - Prepare drawings of an Aqueduct crossing - Prepare drawings of a pipe outlet |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |

| Minimum Evidence Required | Find pressure by Piezometer and/or manometer - Identify pressure gauge - Measure pressure by Piezometer and/or Manometer Find out Hydrostatic pressure and center of pressure on vertically Immersed surface. - Identify apparatus - Immerse apparatus in water tank - Identify the formulae for hydrostatic pressure and center of pressure. - Find the area of immersed surface - Find moment of inertia of the figure - Calculate hydrostatic pressure on surface - Find position of centre of pressure Calculate hydraulic and energy gradient for pipe lines. - Prepare pipeline design charts using standard formulae. - Identify the limitations of formulae. - Identify variations in roughness coefficients. - Calculate the pressure in pipeline systems using the hydraulic gradient line. - Calculate the pipe discharge from reservoirs. Calculate flow in open channels. - Identify methods used for measuring flows in open channels. - Identify instruments to be used - Use formulae for calculating flows in open channels. - Find the velocity by current meter and velocity rod - Find discharge. Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw map of Pakistan. - Show rivers on map. - Draw barrages, main canals, head works and link canals Draw the plan of a multipurpose irrigation project - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw plan of project. - Show all details on plans Draw cross section of dams - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw x-section of dam. - Label all necessary details on plan. Find design discharge of Karif and Rabi crops for given area - Identify area of crops (Kharif or Rabi) - Identify delta of each crop - Calculate duty - Estimate total water requirement Draw the alignment of a canal, minor distributary and watercourses on contour map - Arrange drawing instruments - Draw a contour map with two meter interval - Draw alignment of canal, distributary and water course on some scale Prepare drawings of an Aqueduct crossing - Identify cross drainage works(aqueduct, super passage, level crossing, tail escape) - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of aqueduct Prepare drawings of a pipe outlet - Identify outlets - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of pipe outlet |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Diploma- Level-5 in “Water Quality and Resource Management” (Technician-Supervisor) |
| Competency Standard | Hydraulics and Irrigation Engineering |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Find pressure by Piezometer and/or manometer - Find out Hydrostatic pressure and center of pressure on vertically Immersed surface. - Calculate hydraulic and energy gradient for pipe lines. - Calculate flow in open channels. - Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages - Draw the plan of a multipurpose irrigation project - Draw cross section of dams - Find design discharge of Karif and Rabi crops for given area - Draw the alignment of a canal, minor distributary and watercourses on contour map - Prepare drawings of an Aqueduct crossing - Prepare drawings of a pipe outlet |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify pressure gauge |  |  |
| - Measure pressure by Piezometer and/or Manometer |  |  |
| - Identify apparatus |  |  |
| - Immerse apparatus in water tank |  |  |
| - Identify the formulae for hydrostatic pressure and center of pressure. |  |  |
| - Find the area of immersed surface |  |  |
| - Find moment of inertia of the figure |  |  |
| - Calculate hydrostatic pressure on surface |  |  |
| - Find position of centre of pressure |  |  |
| - Prepare pipeline design charts using standard formulae. |  |  |
| - Identify the limitations of formulae. |  |  |
| - Identify variations in roughness coefficients. |  |  |
| - Calculate the pressure in pipeline systems using the hydraulic gradient line. |  |  |
| - Calculate the pipe discharge from reservoirs. |  |  |
| - Identify methods used for measuring flows in open channels. |  |  |
| - Identify instruments to be used |  |  |
| - Use formulae for calculating flows in open channels. |  |  |
| - Find the velocity by current meter and velocity rod |  |  |
| - Find discharge. |  |  |
| - Arrange drawing instruments |  |  |
| - Select drawing sheet |  |  |
| - Set sheet on drawing table as per standards. |  |  |
| - Draw map of Pakistan. |  |  |
| - Show rivers on map. |  |  |
| - Draw barrages, main canals, head works and link canals |  |  |
| - Arrange drawing instruments |  |  |
| - Select drawing sheet |  |  |
| - Set sheet on drawing table as per standards. |  |  |
| - Draw plan of project. |  |  |
| - Show all details on plans |  |  |
| - Arrange drawing instruments |  |  |
| - Select drawing sheet |  |  |
| - Set sheet on drawing table as per standards. |  |  |
| - Draw x-section of dam. |  |  |
| - Label all necessary details on plan. |  |  |
| - Identify area of crops (Kharif or Rabi) |  |  |
| - Identify delta of each crop |  |  |
| - Calculate duty |  |  |
| - Estimate total water requirement |  |  |
| - Arrange drawing instruments |  |  |
| - Draw a contour map with two meter interval |  |  |
| - Draw alignment of canal, distributary and water course on some scale |  |  |
| - Identify cross drainage works(aqueduct, super passage, level crossing, tail escape) |  |  |
| - Arrange drawing instruments |  |  |
| - Select drawing sheet |  |  |
| - Set sheet on drawing table as per standards |  |  |
| - Select scale as per requirement |  |  |
| - Make drawing of aqueduct |  |  |
| - Identify outlets |  |  |
| - Arrange drawing instruments |  |  |
| - Select drawing sheet |  |  |
| - Set sheet on drawing table as per standards |  |  |
| - Select scale as per requirement |  |  |
| - Make drawing of pipe outlet |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Diploma- Level-5 in “Water Quality and Resource Management” (Technician-Supervisor) |
| --- | --- |
| Competency Standard | Hydraulics and Irrigation Engineering |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Find pressure by Piezometer and/or manometer - Find out Hydrostatic pressure and center of pressure on vertically Immersed surface. - Calculate hydraulic and energy gradient for pipe lines. - Calculate flow in open channels. - Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages - Draw the plan of a multipurpose irrigation project - Draw cross section of dams - Find design discharge of Karif and Rabi crops for given area - Draw the alignment of a canal, minor distributary and watercourses on contour map - Prepare drawings of an Aqueduct crossing - Prepare drawings of a pipe outlet |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify pressure gauge |  |  |  |  |  |
|  | Measure pressure by Piezometer and/or Manometer |  |  |  |  |  |
|  | Identify apparatus |  |  |  |  |  |
|  | Immerse apparatus in water tank |  |  |  |  |  |
|  | Identify the formulae for hydrostatic pressure and center of pressure. |  |  |  |  |  |
|  | Find the area of immersed surface |  |  |  |  |  |
|  | Find moment of inertia of the figure |  |  |  |  |  |
|  | Calculate hydrostatic pressure on surface |  |  |  |  |  |
|  | Find position of centre of pressure |  |  |  |  |  |
|  | Prepare pipeline design charts using standard formulae. |  |  |  |  |  |
|  | Identify the limitations of formulae. |  |  |  |  |  |
|  | Identify variations in roughness coefficients. |  |  |  |  |  |
|  | Calculate the pressure in pipeline systems using the hydraulic gradient line. |  |  |  |  |  |
|  | Calculate the pipe discharge from reservoirs. |  |  |  |  |  |
|  | Identify methods used for measuring flows in open channels. |  |  |  |  |  |
|  | Identify instruments to be used |  |  |  |  |  |
|  | Use formulae for calculating flows in open channels. |  |  |  |  |  |
|  | Find the velocity by current meter and velocity rod |  |  |  |  |  |
|  | Find discharge. |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Select drawing sheet |  |  |  |  |  |
|  | Set sheet on drawing table as per standards. |  |  |  |  |  |
|  | Draw map of Pakistan. |  |  |  |  |  |
|  | Show rivers on map. |  |  |  |  |  |
|  | Draw barrages, main canals, head works and link canals |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Select drawing sheet |  |  |  |  |  |
|  | Set sheet on drawing table as per standards. |  |  |  |  |  |
|  | Draw plan of project. |  |  |  |  |  |
|  | Show all details on plans |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Select drawing sheet |  |  |  |  |  |
|  | Set sheet on drawing table as per standards. |  |  |  |  |  |
|  | Draw x-section of dam. |  |  |  |  |  |
|  | Label all necessary details on plan. |  |  |  |  |  |
|  | Identify area of crops (Kharif or Rabi) |  |  |  |  |  |
|  | Identify delta of each crop |  |  |  |  |  |
|  | Calculate duty |  |  |  |  |  |
|  | Estimate total water requirement |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Draw a contour map with two meter interval |  |  |  |  |  |
|  | Draw alignment of canal, distributary and water course on some scale |  |  |  |  |  |
|  | Identify cross drainage works(aqueduct, super passage, level crossing, tail escape) |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Select drawing sheet |  |  |  |  |  |
|  | Set sheet on drawing table as per standards |  |  |  |  |  |
|  | Select scale as per requirement |  |  |  |  |  |
|  | Make drawing of aqueduct |  |  |  |  |  |
|  | Identify outlets |  |  |  |  |  |
|  | Arrange drawing instruments |  |  |  |  |  |
|  | Select drawing sheet |  |  |  |  |  |
|  | Set sheet on drawing table as per standards |  |  |  |  |  |
|  | Select scale as per requirement |  |  |  |  |  |
|  | Make drawing of pipe outlet |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Diploma- Level-5 in “Water Quality and Resource Management” (Technician-Supervisor) |
| --- | --- |
| Competency Standard | Hydraulics and Irrigation Engineering |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | State development, scope and significance of Hydraulics in civil engineering |  |  |
|  | Define density, specific weight, specific volume, specific gravity, surface tension, viscosity compressibility, cohesion, and adhesion. |  |  |
|  | State pressure, intensity of pressure, pressure head, Pascal's law and its simple applications |  |  |
|  | Distinguish among atmospheric pressure, gauge pressure and absolute pressure. |  |  |
|  | Describe measurement of fluid pressure by Piezometer tube and different manometers. |  |  |
|  | Define hydro kinematics |  |  |
|  | Distinguish path lines and stream lines. |  |  |
|  | Distinguish the type of flow in pipes i.e. steady and unsteady flow, uniform and non-uniform flow, turbulent flow. |  |  |
|  | State the total head of flowing liquid and total energy. |  |  |
|  | Explain the major and minor losses of head of water flowing through pipes. |  |  |
|  | Define irrigation. State history of irrigation. |  |  |
|  | State scope, merits and demerits of irrigation |  |  |
|  | Explain various sources of irrigation water. |  |  |
|  | Enlist factors affecting water requirement |  |  |
|  | Define the terms crop period, base period, Kharif-Rabi ratio, core watering, cash crop, crop rotation, and delta. |  |  |
|  | Explain basic methods of water distribution to field i.e. Surface irrigation method, Furrow method, Contour method, Flooding method. |  |  |
|  | Explain Sprinkler irrigation method |  |  |
|  | Explain Drip or Trickle irrigation method. |  |  |
|  | Define canal falls. State the basic requirement of canal fall. |  |  |
|  | Describe the types of canal falls. |  |  |
|  | Describe drainage outlet and tail escape. |  |  |
|  | Define outlet. Explain the characteristics of outlets. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________