National Vocational Level-4 in “Water Quality and Resource Management” (Lab Analyst)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

ACKNOWLEDGEMENT

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government organizations, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Program (TSSP)co-funded by European Union, Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who led the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills, Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ,

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Fayyaz Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-5 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equip the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks especially to Syed Javed Hassan, Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

- Table of Contents

1.	Introduction	4

2.	Purpose of the Qualification	4

3.	Date of Validation	4

4.	Date of Review	5

5.	Codes of Qualifications	5

6.	Members of Qualification Development Committee	6

7.	Members of Qualification Validation Committee	7

8.	Entry Requirements	8

9.	Detail of Qualification and its Competency Standards	8

1.	Water and Waste Water Testing (Physico-chemical Parameter)-II	11

1021CS-01.	Testing/Determination of Bicarbonate (HCO3) by Titrimetric Method	11

1021CS-02.	Testing/Determination of Carbonate (CO3) by Titrimetric Method	13

1021CS-03.	Testing/Determination of Chloride (Cl) by Titrimetric/Argentometric Method	15

1021CS-04.	Testing/Determination of Chlorine (Cl2) by DPD/Chlorine Meter Method	17

1021CS-05.	Testing/Determination of Fluoride (F) by ISE Meter	19

2.	Water Quality Testing for Microbiological Contamination-II	21

1021CS-06.	Perform Testing for Total Plate Count (Method-III)	21

1021CS-07.	Perform Testing for Total Coliforms and E-coli (Method-IV)	24

3.	Quality Control Operations	27

1021CS-08.	Analyze Method Blank	27

1021CS-09.	Perform Calibration and Calibration Verification	28

1021CS-010.	Analyze Laboratory Control Samples (LCS)	29

1021CS-011.	Analyze Audit Blind Sample	30

1021CS-012.	Perform Correctness of Results	31

1021CS-013.	Determine the Method Detection Limits	33

1021CS-014.	Develop Control Charts	34

4.	Water Resources of Pakistan	36

1021CS-015.	Identify Water Resources of Pakistan	36

5.	Soft Skills	38

1021CS-016.	Manage the meetings	38

1021CS-017.	Manage workforce planning	40

1021CS-018.	Undertake project work	42

1021CS-019.	Apply interpersonal skills	44

1021CS-020.	Work safely in an office environment	46

1021CS-021.	Maintain professionalism in workplace	47

- Introduction

Water quality and treatment is process that improves the quality of water to make it more acceptable for specific end-use. The end use may be drinking, industrial water supply, irrigation, recreation water or many other uses, including being safely returned to the environment. Water treatment removes contaminants and undesirable components, or reduces their concentration so that the water becomes fit for its desired end-use. This treatment is crucial to human health and allows humans to benefit from both drinking and irrigation use.

In fact, no sphere of life may be identified that does not include contribution of the water. Thus, the importance of this diploma may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Water Quality and Treatment Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading organizations of water sector (PCRWR, WASA, PCSIR, PHED etc.)in the country.

- Purpose of the Qualification

The purpose of these qualification is to set high professional standards for Water Quality and Resource Management. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the graduates

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for water sector

- Enable the existing workforce to capacitate themselves in new water technologies

- Date of Validation

The level 5 of National DAE qualification on Water Quality and Resource Management has been validated by the Qualifications Development Committee (QDC) members on 10-12thJune, 2019and will remain valid for ten years i.e.12th June, 2029

- Date of Review

The level 5 of National DAE qualification on Water Quality and Resource Management has been validated by the Qualifications Development Committee (QDC) members on 10-12thJune, 2019and shall be reviewed after three years i.e. 12th June, 2022

- Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification |  |
| --- | --- |
| Code | Description |
| 1021CS (1) | National Vocational Certificate Level-I & II in “Water Quality and Resource Management” (Assistant) |
| 1021CS (2) | National Vocational Certificate Level-III in “Water Quality and Resource Management” (Water Quality Surveyor) |
| 1021CS (3) | National Vocational Certificate Level-IV in “Water Quality and Resource Management” (Lab Analyst) |
| 1021CS (4) | National Vocational Certificate Level-V in “Water Quality and Resource Management (Supervisor/ Water Treatment Technician) |

- Members of Qualification Development Committee

| S# | Name - Designation | Organization |
| --- | --- | --- |
|  | Ms. Lubna Naheed Bukhari | PRCWR, Islamabad |
|  | Dr. Hifza Rasheed, Director | PRCWR, Islamabad |
|  | Engr. Ahmed Shamsher, Deputy Secretary | PHED,KPK |
|  | Muhammad Adnan, Deputy Director | IMC, Islamabad |
|  | Dr. Asad Ghufran | IIUI, Islamabad |
|  | Mr. Azizullah Khan | WASA Rawalpindi |
|  | Dr. Anwer Baig | NUST |
|  | Miss Saiqa Imran | PRCWR, Islamabad |
|  | Mr. Shafiqur Rehman | PRCWR, Islamabad |
|  | Mr. Ali | PHED, Balochistan |
|  | Mr. Abdul Latif | PHED, Sindh |
|  | Mr. Tanveer Malik | Bin Qutab, Islamabad |
|  | Muhammad Yasir, Assistant Director | SS&C NAVTTC, Islamabad |
|  | Ms. Saba Sadiq, DACUM Facilitator | University of Lahore, Lahore |

- Members of Qualification Validation Committee

| S# | Name - Designation | Organization |
| --- | --- | --- |
|  | Dr.Muhammad Aslam Tahir, Chairman | PRCWR,Islamabad |
|  | Dr.Manzoor Ahmed Malik, Director | PRCWR,Islamabad |
|  | Engr.Faizan-ul-Hasan, Director | PRCWR,Islamabad |
|  | Dr.Hifza Rasheed, Director | PRCWR,Islamabad |
|  | Dr.Muhammad Arshad Malik, Assistant Professor | IIUI,Islamabad |
|  | Syed Musstajab Akbar Shah Eqani, Assistant Professor | COMSATS Islamabad |
|  | Engr.Ahmed Shamsher, Deputy Secretary | PHED,KPK |
|  | Muhammad Adnan, Deputy Director | IMC,Islamabad |
|  | Muhammad Zaheer, Assistant Director | NTB,Islamabad |
|  | Muhammad Yasir, Assistant Director | SS&C NAVTTC,Islamabad |
|  | Ms. Samreen Iqbal, Senior Scientist officer | Qarshi Industries, Islamabad |
|  | Ms. Saba Sadiq, DACUM Facilitator | University of Lahore, Lahore |

- Entry Requirements

The entry for National Vocational Certificate level 4, in Water Quality and Resource Management are

- A person having National Vocational Certificate level-3, in Water Quality and Resource Management

- Detail of Qualification and its Competency Standards

| NVQF (Competency Standards) | Level | Category | Estimated Hours |  |  | CH |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th. | Pr. | T H |  |
| Level 4 |  |  |  |  |  |  |  |
| 11. Water and waste water Testing (Physico-chemical Parameter)-II Water Testing Analyst (Chemical) |  |  |  |  |  |  |  |
| Perform Testing/Determination of Bicarbonate (HCO3) by Titrimetric Method | 4 | Technical | 15 | 62 | 77 | 7.7 |  |
| Perform Testing/Determination of Carbonate (CO3) by Titrimetric Method | 4 | Technical | 15 | 62 | 77 | 7.7 |  |
| Perform Testing/Determination of Chloride (Cl) by Titrimetric/Argentometric Method | 4 | Technical | 15 | 62 | 77 | 7.7 |  |
| Perform Testing/Determination of Chlorine (Cl2) by DPD/Chlorine Meter Method | 4 | Technical | 15 | 62 | 77 | 7.7 |  |
| Perform Testing/Determination of Fluoride (F) by ISE Meter | 4 | Technical | 15 | 63 | 78 | 7.8 |  |
| Total |  |  | 75 | 314 | 386 | 38.6 |  |
| 12. Water Quality Testing for Microbiological Contamination-II Water Testing Analyst (Microbiological) |  |  |  |  |  |  |  |
| Perform Testing for Total Plate Count (Method-III) | 4 | Technical | 20 | 60 | 80 | 8 |  |
| Perform Testing for Total Coliforms and E-coli (Method-IV) | 4 | Technical | 20 | 60 | 80 | 8 |  |
| Total |  |  | 40 | 120 | 160 | 16 |  |
| 13. Quality Control Operations (Quality Control Technician) |  |  |  |  |  |  |  |
| Analyze Method Blank | 4 | Technical | 15 | 60 | 75 | 7.5 |  |
| Perform Calibration and Calibration Verification | 4 | Technical | 15 | 60 | 75 | 7.5 |  |
| Analyze Laboratory Control Samples (LCS) | 4 | Technical | 15 | 60 | 75 | 7.5 |  |
| Analyze Audit Blind Sample | 4 | Technical | 15 | 60 | 75 | 7.5 |  |
| Perform Correctness of Results | 4 | Technical | 15 | 65 | 80 | 8 |  |
| Determine the Method Detection Limits | 4 | Technical | 15 | 60 | 75 | 7.5 |  |
| Develop Control Charts | 4 | Technical | 15 | 50 | 65 | 6.5 |  |
| Total |  |  | 105 | 415 | 520 | 52 |  |
| 14. Water Resources of Pakistan (Water Resources Technician) |  |  |  |  |  |  |  |
| Identify Water Resources of Pakistan | 4 | Technical | 6 | 20 | 26 | 2.6 |  |
| Total |  |  | 6 | 20 | 26 | 2.6 |  |
| 15. Soft skills |  |  |  |  |  |  |  |
| Manage the meetings with customers | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Manage workforce planning | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Undertake project work | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Apply interpersonal skills | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Work safely in an office environment | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Maintain professionalism in the workplace | 4 | Generic | 6 | 12 | 18 | 1.8 |  |
| Total |  |  | 36 | 72 | 108 | 10.8 |  |
| Grand Total |  |  | 262 | 938 | 1200 | 120.0 |  |

## Water and Waste Water Testing (Physico-chemical Parameter)-II

### Testing/Determination of Bicarbonate (HCO3) by Titrimetric Method

Overview: This competency standards covers the skill and knowledge required toPrepare samples for testing, Prepare for laboratory testing, Perform tests on samples, Quality Control Checks, Record the results/ Finalise work and Adopt precautions during work

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare samples for testing | - Check the sample label for the requirement of analysis of Bicarbonate. - keeps the sample at room temperature for few minutes. - Check for the availability of HCl solutions otherwise prepare as per standard procedure - Identify appropriate test method, Glassware and safety requirements. |
| - Prepare for laboratory testing | - Wash all glassware with washing acid and then rinse thoroughly with deionized water. - Standardize the HCl 0.02 N Solution with Sodium Carbonate solution of 0.05 N - Set up the burette on burette stand properly. Fill the HCl solution of 0.02 N in burette accordance with the specified work instructions. - Conduct pre-use and safety checks. |
| - Perform tests on samples | - To 10 mL of sample few drops of mixed indicator is added and is titrated against 0.02 N HCl - Take 25 ml burette with 0.05ml resolution for titration, for low alkalinity less than 50mg/l 10ml burette with 0.02ml resolution need to be used. - Stop the titration when sample color changed to pink. - The total Alkalinity is calculated by using following formula HCO3 alkalinity, mg CaCO3/L; =; A × N × 50000. ml sample. Where: A = ml standard acid used N = Normality of the standard acid. - analyzed Three replicate for each sample. - Store unused reagents and dispose of wastes as required by relevant regulations and codes. - Clean and store equipment. |
| - Quality Control Checks | - Method blank is analyzed after every 10 samples - Laboratory Control samples of 5 & 100 ppm analyzed with every batch of 10 samples. - check repeatability by reanalysing a pre-analyzed sample once a day - Daily quality control data is recorded in QC Data sheet-I. - Results of laboratory control sample are regularly displayed in the form of control charts. |
| - Record the results/ Finalise work | - Note down the Results on analyst workbook. - Calculate uncertainty of theresults - Record the results on result record form and submit to reporting section - Clear and restore work area. |
| - Adopt precautions during work | - Sodium Carbonate used for standardization should be dried before making solution at 2500C for 4 hours. - Apparatus should be thoroughly washed before analysis. - Titration must be done carefully and quickly to avoid loss of dissolved gasses during titration - Dissolved gases can, directly or indirectly contribute to alkalinity; therefore the gases must neither be lost nor gained during sampling, storage and titration |

Knowledge and Understanding

Understanding the basic principle of Bicarbonate testing i.e.  determined by titration of the sample with a standard solution of a strong mineral acid.

  - Identifying excessive HCO3 level in water samples

  - Planning and organizing for testing.

  - Using appropriate instrument and glassware

  - Understanding and applying procedures for HCO3 testing in water

  - Determining and reporting accurate and relevant results from testing

Tools & Equipment

- Burette

- Hydrochloric Acid solution (0.1N)

- Hydrochloric Acid solution (0.02N)

- Sodium carbonate solution (0.05N)

- Bromocresol green + Methyl red (Mixed indicator)

- Deionized water

- Glass Beaker (Class A)

- Volumetric Flask 100 ml

- Auto-pipette 10 ml

### Testing/Determination of Carbonate (CO3) by Titrimetric Method

Overview: This competency standards covers the skill and knowledge required to prepare samples for testing, Prepare for laboratory testing, Perform tests on samples, Quality Control Checks, Record the results/ Finalize work and Adopt precautions during work

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare samples for testing | - Check the sample label for the requirement of analysis of Bicarbonate. - keeps the sample at room temperature for few minutes. - Check for the availability of HCl solutions otherwise prepare as per standard procedure - Identify appropriate test method, Glassware and safety requirements. |
| - Prepare for laboratory testing | - Wash all glassware with washing acid and then rinse thoroughly with deionized water. - Standardize the HCl 0.02 N Solution with Sodium Carbonate solution of 0.05 N - Set up the burette on burette stand properly. Fill the HCl solution of 0.02 N in burette accordance with the specified work instructions. - P3: Conduct pre-use and safety checks. |
| - Perform tests on samples | - Take 10 ml sample add 2 or 3 drops of phenolphthalein pink color indication shows the pH range 8.3 or above. - Titration is done against HClwith 10ml burette of 0.02ml resolution as per standard. - Color changes from pink to transparent color. - Alkalinity determined is called as Phenolphthalein Alkalinity (P) as it gives color at pH 8.3. Carbonate Alkalinity as CaCO3 mg/L = 2P - analyzed Three replicate for each sample. - Store unused reagents and dispose of wastes as required by relevant regulations and codes. - Clean and store equipment. |
| - Quality Control Checks | - Method blank is analyzed after every 10 samples - Laboratory Control samples analyzed with every batch of 10 samples. - check repeatability by reanalysing a pre-analyzed sample once a day - Daily quality control data is recorded in QC Data sheet-I. - Results of laboratory control sample are regularly displayed in the form of control charts. |
| - Record the results/ Finalize work | - Note down the Results on analyst workbook. - Calculate uncertainty of theresults - Record the results on result record form and submit to reporting section - Clear and restore work area. |
| - Adopt precautions during work | - Sodium Carbonate used for standardization should be dried before making solution at 2500C for 4 hours. - Apparatus should be thoroughly washed before analysis. - Titration must be done carefully and quickly to avoid loss of dissolved gasses during titration. - Dissolved gases can, directly or indirectly contribute to alkalinity; therefore the gases must neither be lost nor gained during sampling, storage and titration |

Knowledge and Understanding

Understanding the basic principle of carbonate testing i.e.  Determined by titration of the sample with a standard solution of a strong mineral acid.

  - Identifying excessive CO3 level in water samples

  - Planning and organizing for testing.

  - Using appropriate instrument and glassware

  - Understanding and applying procedures for CO3 testing in water

  - Determining and reporting accurate and relevant results from testing

  - If pH of the sample is above 8.3, it indicates the presence of carbonate in the sample.

Tools & Equipment

- Burette

- Hydrochloric Acid solution (0.1N)

- Hydrochloric Acid solution (0.02N)

- Sodium carbonate solution (0.05N)

- Phenolphthalein indicator

- Deionized water

- Glass Beaker (Class A)

- Volumetric Flask 100 ml

- Auto-pipette 10 ml

### Testing/Determination of Chloride (Cl) by Titrimetric/Argentometric Method

Overview: This competency standards covers the skill and knowledge required toPrepare samples  for testing, Prepare for laboratory testing, Perform tests on samples, Quality Control Checks, Record the results/ Finalize work and Adopt precautions during work

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare samples for testing | - Check the sample label for the requirement of analysis of Bicarbonate. - keeps the sample at room temperature for few minutes. - Check for the availability of AgNO3solutions 0.0141N otherwise prepare as per standard procedure - Identify appropriate test method, Glassware and safety requirements. |
| - Prepare for laboratory testing | - Wash all glassware with washing acid and then rinse thoroughly with deionized water. - Standardize silver nitrate with sodium chloride solution - Set up the burette on burette stand properly. Fill the Ag NO3 solution of 0.02 N in burette accordance with the specified work instructions. - Conduct pre-use and safety checks. |
| - Perform tests on samples | - Take 20 ml Sample in beaker; add few drops of K2CrO4 in it. - Titrate it against 0.0141N AgNO3with 25ml burette of 0.05ml resolution - The burette reading is then noted when sample color changes to pinkish yellow - Calculation: mg Cl -1/L=; (A – B) × N ×1000 ×35.6. Where: A = mL titration for sample B = mL titration for blank C = Normality of AgNO3; ml of sample. - analyzed Three replicate for each sample. - Store unused reagents and dispose of wastes as required by relevant regulations and codes. - Clean and store equipment. |
| - Quality Control Checks | - Method blank is analyzed after every 10 samples - Laboratory Control samples of 5 & 100 ppm analyzed with every batch of 10 samples. - check repeatability by reanalysing a pre-analyzed sample once a day - Daily quality control data is recorded in QC Data sheet-I. - Results of laboratory control sample are regularly displayed in the form of control charts. |
| - Record the results/ Finalize work | - Note down the Results on analyst workbook. - Calculate uncertainty of theresults - Record the results on result record form and submit to reporting section - Clear and restore work area. |
| - Adopt precautions during work | - Silver nitrate stock solution should be stored in amber colored bottle. - The glass ware should be properly washed and cleaned before analysis - Care should be taken during weighing of silver nitrate. - The sample should be free of interfering radicals otherwise there presence may introduce errors in the chloride estimation. - The tip of the electrode must be in level with tip of the other slaved |

Knowledge and Understanding

- Understanding the basic principle of Chloride testing i.e.  Determined by titration of the sample with silver nitrate solution silver chloride is precipitated quantitatively before red silver chromate is formed.

- Identifying excessive Chloride level in water samples

  - Planning and organizing for testing.

  - Using appropriate instrument and glassware

  - Understanding and applying procedures for Chloride testing in water

  - Determining and reporting accurate and relevant results from testing

- The purpose of this method is to characterize the quality of potable water. Chlorides may impart salty taste of water. High concentration of chlorides may indicate pollution by sewage or industrial wastes or by the intrusion of seawater or saline water.

Tools & Equipment

- Burette

- Standard Silver Nitrate: (0.0141 N)

- Sodium chloride (0.0141N)

- Potassium chromate Indicator solution      Deionized water

- Glass Beaker (Class A)

- Volumetric Flask 100 ml

- Auto-pipette 10 ml

### Testing/Determination of Chlorine (Cl2) by DPD/Chlorine Meter Method

Overview: This competency standards covers the skill and knowledge required toPrepare samples for testing, Prepare for laboratory testing, Perform tests on samples, Quality Control Checks, Record the results/ Finalize work and Adopt the precautions during testing work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare samples for testing | - Check the sample label for the requirement of Chlorine testing. - Keeps the sample at room temperature for few minutes. - Check for the availability of required reagents. - Identify appropriate test method, equipment and safety requirements. |
| - Prepare for laboratory testing | - Set up test equipment Chlorine meter and/or reagents in accordance with the specified work instructions. - Conduct pre-use and safety checks. - Check the calibration status of equipment. Perform calibration if required. |
| - Perform tests on samples | - Turn on the meter as per following the work instructions. - Select free or total chlorine mode as per requirement of test - Add to cuvet 5ml of reagent B by mean of 5ml syringe or auto pipette or as per requirement of method. - Fill the cuvet up to the 10 ml mark by adding 5ml of sample. - Replace the cap and shake gently. - Place the cuvet in to the holder and ensure that the notch on the cap is positioned securely in to the groove. - Press “ZERO” Wait for a few seconds, instrument will be zeroed - Remove the cuvet only in case of total chlorine measurement add three drops of reagent C to the cuvet - Add the contents of one packet of DPD reagent to the cuvet - Replace the cap and shake gently. - Place the cuvet in to the holder wait for one minute in case of free chlorine or 2 minutes and thirty seconds in case of total chlorine or as per requirement of method. Run the sample the display will showreading in ppm. - Analyzed three replicate for each sample. - Store unused reagents and dispose off wastes as required by relevant regulations and codes. - Clean and store equipment. |
| - Quality Control Checks | - Method blank is analyzed after every 10 samples - Laboratory Control samples analyzed with every batch of 10 samples. - Daily quality control data is recorded in QC Data sheet-I. - Results of laboratory control sample are regularly displayed in the form of control charts. |
| - Record the results/ Finalise work | - Note down the Results on analyst workbook. - Calculate uncertainty of theresults - Record the results on result record form and submit to reporting section - Clear and restore work area. |
| - Adopt the precautions during testing work | - Do not touch the cuvet walls with hands. - Do not let the test sample stand too long after reagent is added accuracy may be lost. - Whenever the cuvet is placed into the measurement cell, it must be free of fingerprints, oil or dirt. - To obtain the representative sample, gently but thoroughly, mix it before sample is taken. - Each time the cuvet is used, tighten the cap to the same degree. |

Knowledge and Understanding

- Demonstration of an ability to prepare water samples and perform quality tests according to specified standards and parameters relevant to water quality standards including:

  - Understanding the basic principle of spectrometric Chlorine measurement i.e. determination of intensity of DPD complex photometric measurement according to the free or combined chlorine in the water sample

  - Identifying chlorine conc. In accordance to the requirement and hazards in water samples

  - Planning and organizing testing assignment

  - Using appropriate testing equipment and personal protective clothing and equipment

  - Understanding and applying procedures for testing

  - Determining and reporting accurate and relevant results from testing

Tools & Equipment

- Reagent B for free Chlorine determination

- Reagent C for combined chlorine determination

- DPD Reagent

- Deionized water

- Glass Beaker (Class A)

- Volumetric Flask 100 ml

- Auto-pipette 10 ml

- Lint free tissue

### Testing/Determination of Fluoride (F) by ISE Meter

Overview: This competency standards covers the skill and knowledge required topreparesamples for testing, Prepare for laboratory testing, Perform tests on samples, Quality Control Checks, Record the results/ Finalize work and adopt the precautions during testing work

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare samples for testing | - Check the sample label for the requirement of analysis of Fluoride. - keeps the sample at room temperature for few minutes. - Check for the availability of Fluoride Standard solution of required concentration otherwise prepare as per standard procedure from reference stock solution of 1000 ppm as per requirement - Identify appropriate test method, Glassware, Equipment and safety requirements. |
| - Prepare for laboratory testing | - Set up test equipment ISE Meter and/or reagents in accordance with the specified work instructions. - Conduct pre-use and safety checks. - Check the calibration status of equipment. Perform calibration if required. - Conduct pre-use and safety checks. |
| - Perform tests on samples | - Take volume as per standard of deionized water in a cuvet and add specific volume of fluoride Buffer (pH: 5.3-5.5) in it. - Dip the electrode in the solution with constant stirring. This will show zero concentration - Note the concentration of known standards for verification of calibration and efficiency of electrode. - Then take sample and analyze it as described above. - analyzed Three replicate for each sample. - Store unused reagents and dispose of wastes as required by relevant regulations and codes. - Clean and store equipment. |
| - Quality Control Checks | - Method blank is analyzed after every 10 samples - A Laboratory Control sample is analyzed with every batch of 10 samples. - check repeatability by reanalysing a pre-analyzed sample once a day - Daily quality control data is recorded in QC Data sheet-I. - Results of laboratory control sample are regularly displayed in the form of control charts. |
| - Record the results/ Finalise work | - Note down the Results on analyst workbook. - Calculate uncertainty of theresults - Record the results on result record form and submit to reporting section - Clear and restore work area. |
| - Adopt the precautions during testing work | - First check the reliability of the instrument by running known standards. - The pH of the buffer solution must be in range of 5.3-5.5 for the accurate work. - Alkalinity, iron, aluminum and free chlorine may interfere during the analysis of fluoride. - It should be kept in mind that at alkaline pH the hydroxide ions also interfere the electrode response. - Use samples and standards at the same temperature. |

Knowledge and Understanding

- Demonstration of an ability to prepare water samples and perform quality tests according to specified standards and parameters relevant to water quality standards including:

  - Understanding the basic principle of Fluoride measurement i.e. determination through lanthanum fluoride crystal across which a potential is established by fluoride solutions. The crystal contacts the sample solution at one face and an internal reference solution at the other. The fluoride electrode measures the ion activity of fluoride which depends on the total ionic strength and on pH, and on fluoride complexing species.

  - Identifying fluoride conc. in accordance to the requirement and hazards in water samples

  - Planning and organizing testing assignment

  - Using appropriate testing equipment and personal protective clothing and equipment

  - Understanding and applying procedures for testing

  - Determining and reporting accurate and relevant results from testing.

Tools & Equipment

- ISE Meter

- Auto Pipette

- Fluoride Standard

- Fluoride Buffer

- Glass Beaker (Class A)

- Volumetric Flask 100 ml

## Water Quality Testing for Microbiological Contamination-II

### Perform Testing for Total Plate Count (Method-III)

Overview: This competency standards covers the skill and knowledge required to Prepare to undertake basic microbiological tests, Adopt recommended preparatory method for coliform, Adopt recommended preparatory method for fecal coliform , Perform handling or basic testing of samples for coliforms , Perform handling or basic testing of samples for fecal coliforms, Maintain Environment ,Quality Control and Document control.

| Competency units | Performance Criteria |
| --- | --- |
| CU 1: Prepare to undertake basic microbiological tests | P1: Prepare sterile environment for undertaking tests. P2: Collect samples for microbiological examination in clean, sterile, non-reactive borosilicate glass or plastic bottle. P3: Ensure that water sample is appropriate for the test requested as per standards. P4: When the sample is collected, leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination. P5: Ice samples during transport to the laboratory if they cannot be processed within 1 hour after collection. P6: Maintain samples in the dark and keep cool with ice at < 8 oC but not frozen. |
| CU 2: Adopt recommended preparatory method for Total Plate Count | P1: Use commercially available media for detection of total bacterial count following plate count technique. P2: Prepare recommended media (Plate count agar) by weighing the amount as per manufacturers’ instruction using calibrated analytical balance. P3: Sterilize media using autoclave as per standards. P4: Maintain pH of media after sterilization as per manufacturer’s instruction after sterilization. P5: Bring medium at room temperature before use or store in the refrigerator. |
| CU3: Perform handling or basic testing of samples for Total Plate Count | P1: Prepare dilutions for polluted water samples. P2: Pour recommended volume of water sample to be tested into petri plate using auto-pipette. P3: Pour media into plates, shake well, and allow solidifying and incubating at the recommended temperature for 48 hours. P4: Count bacterial colonies after incubation and report as CFU/ml as per standards. |
| CU4: Maintain Environment and Quality Control | P1: Adopt recommended safety precautions when handling biologically hazardous specimen. P2: Accept or reject media / sample based on the condition and sterility as per standards. P3: Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. P4: Use controls i.e. positive and negative controls along with the tests as per standards. P5: Clean and store equipment in accordance with laboratory guidelines. P6: Sterilize equipment as required P7: Store chemicals, media and controls as per standards. P8: Clean and disinfect area. P9: Properly dispose contaminated waste in accordance with the standards. |

Knowledge and Understanding

- Understanding of the individuality of bacteria by selecting the most significant media, reagents and tests to use in studying the cultural, and biochemical activities of bacteria.

- Determination of number of live cultural heterotrophic bacteria in water and measuring changes during water treatment.

- Total viable count gives a quantitative estimate of the concentration of microorganisms such as bacteria in a sample. The count represents the number of colony forming units (cfu) per ml of the sample.

- Procedures for the use of laboratory testing equipment.

- Microbiological test procedures and related calculations.

- Maintenance and storage of reference material of ATCC loops to use as positive and negative controls.

- Disposal of contaminated media and growth cultures.

Tools & Equipment

- Pre sterilized sampling bottles

- Insulated sample boxes with coolants to maintain temperature of sample at 4oC.

- Microbiological testing laboratory equipment i.e., biological safety cabinet, pH metre, analytical balance, Equipment to maintain the required temperature for bacterial growth i.e. incubators and water bath

- Equipment to sterilize and decontaminate the media and glassware i.e. autoclave.

- Temperature recording equipment i.e. thermometers NIST traceable.

### Perform Testing for Total Coliforms and E-coli (Method-IV)

Overview: This competency standards covers the skill and knowledge required Prepare to undertake basic microbiological tests, Adopt recommended  method for handling and testing of samples, Maintain Environment, Quality Control and  Document control.

| Competency units | Performance Criteria |
| --- | --- |
| CU 1: Prepare to undertake basic microbiological tests | P1: Prepare sterile environment for undertaking tests. P2: Collect samples for microbiological examination in clean, sterile, non reactive borosilicate glass or plastic bottle. P3: Ensure that water sample is appropriate for the test requested as per standards. P4: When the sample is collected, leave ample air space in the bottle at least 2.5 cm to facilitate mixing by shaking before examination as per standards. P5: Ice samples during transport to the laboratory if they cannot be processed within 1 hour after collection. P6: Maintain samples in the dark and keep cool with ice at < 8 oC but not frozen as per standards.. P7: For coliforms and E-coli analyses, the holding time from collection to analysis is 30 hour. |
| CU 2: Adopt recommended method for handling and testing of samples | P1: Use commercially available media for detection of coliforms & E-coli following Colilert IDEXX quanti tray method. P2: Use recommended techniques to prepare samples for testing. P3: Add recommended volume of water sample in sterile mixing bottle and one package of the colilert reagent. P4: Transfer sample/reagent mixture from the mixing bottle into a quanti-tray. P5: Seal tray in IDEXX sealer, label tray on the back with sample code, date and time. P6: Incubate tray at the temperature set by the standard. P7: Count the number of small and large positive wells and report results using IDEXX MPN table to find the most probable number for Total Coliform. P8: Obtain E. coli results by placing the wells under a black light and counting the number of fluorescent wells. |
| CU 3: Maintain Environment and Quality Control | P1: Adopt recommended safety precautions when handling biologically hazardous specimen. P2: Accept or reject media / sample based on the condition and sterility as per standards. P3: Check and maintain sterility, temperature and humidity of rooms and equipment as per standards. P4: Use controls i.e. positive and negative controls along with the tests as per standards. P5: Clean and store equipment in accordance with laboratory guidelines. P6: Sterilise equipment as required as per standards. P7: Store chemicals, media and controls as per standards. P8: Clean and disinfect area P9: Properly dispose contaminated waste in accordance with the standards. |
| CU 4: Document control | P1: Document test and control results in recognised format. P2: Ensure traceability of sample from receipt to reporting of results. |

Knowledge and Understanding

- Understanding of the individuality of bacteria by selecting the most significant media, reagents and tests to use in studying the cultural, and biochemical activities of bacteria.

- Colilert uses the patented Defined Substrate Technology® (DST®) to simultaneously detect total coliforms and E. coli. Two nutrient-indicators, ONPG and MUG, are the major sources of carbon in Colilert and can be metabolized by the coliform enzyme β-galactosidase and the E. coli enzyme β-glucuronidase, respectively.

- Procedures for the use of laboratory testing equipment.

- Microbiological test procedures and  related calculations

- Maintenance and storage of reference material of ATCC loops of E-coli and Staphylococcus aureas.

- Different microbiological testing technique for faecal indicator bacteria.

- Disposal of contaminated media and growth cultures.

Tools & Equipment

- Pre sterilized sampling bottles

- Insulated sample boxes with coolants to maintain temperature of sample at 4oC.

- Microbiological laboratory equipment i.e. IDEXX sealer, IDEXX tray, UV box.

- Equipment to maintain the required temperature for bacterial growth i.e. incubators and water bath

- Equipment to sterilize and decontaminate the media and glassware i.e. autoclave.

- Temperature recording equipment i.e. thermometers NIST traceable.

## Quality Control Operations

### Analyze Method Blank

Overview: This competency standards covers the skill and knowledge required toPrepare Method Blank Sample, Analyze Method Blank and Perform the Evaluation.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare Method Blank Sample | - Prepare method blank from reagent water and all reagents that normally are in contact with the sample during the entire analytical procedure - Take the De-ionized water and add all the reagents which are used in the sample preparation |
| - Analyse Method Blank | - Analyse Method blank with every analytical batch in order to determine the contribution of reagents and the preparative analytical steps to error in the measurement. - Set the instrument according to equipment instructions - Take an analyse free sample through the analysis using the same reagents, glassware, and instrumentation - Analyse the method blank sample on the instrument at the start of the sample analysis and also at different intervals. |
| - Perform the Evaluation | - Evaluates The method blank results for the presence of contamination. - In case of unacceptable contamination present in the method blank, Identify the sources of contaminations and take corrective actions. - Re-prepared and re-analysed the sample as the results is suspected if analyse in the method blank is greater than Method Detection Limit. |

Knowledge and Understanding

  - The Quality control procedures

  - Procedure for Preparation of the method blank sample

Tools & Equipment

  - comply with QC standards

  - Inspect own work

  - Identify and apply relevant QC requirements.

  - Type A Glassware

  - Autopipett

  - Deionized Water

### Perform Calibration and Calibration Verification

Overview: This competency standards covers the skill and knowledge required toPerform instrument calibration, Perform initial calibration (IC) and Perform Continued calibration verification (ccv).

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform instrument calibration | - Prepare the standard solutions - Perform instrument calibration by the use of reference standards or in-house standards. - performs instruments calibration according to manufacturer instructions |
| - Perform initial calibration (IC) | - Perform initial calibration with in-house standards or standard solutions made from the certified reference materials at levels appropriate for the analysis. - Prepare stock standard solution - Prepare minimum three desired standards solution at levels appropriate for the analysis or calibration range - Set the instrument according to equipment instructions - Analyse these standards solutions on the instrument to generate the standard curve. |
| - Perform Continued calibration verification (ccv). | - Perform Continued calibration verification (ccv) at specified intervals throughout the analytical sequence. - Verified the calibration again through the analysis of independently prepared standard solutions - Also used the pre analysed sample for calibration verification. |

Knowledge and Understanding

  - Procedures for standard solution preparation

  - Procedures for calibration of instruments.

Tools & Equipment

  - comply with instrument instructions

  - Type A Glassware

  - Autopipett

  - Deionized Water

### Analyze Laboratory Control Samples (LCS)

Overview: This competency standards covers the skill and knowledge required toPrepare the laboratory control sample, Analyze the Lab Control Sample and Corrective Actions

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare the laboratory control sample | - Prepare the laboratory control sample from solution of Certified Reference Material in de-ionized water as per standards. - PreparefirstStock solution from certified standards - Prepare then sub stock standard of desired concentration. - Use fallowing dilution formula for preparation of stock and sub stock standards C1V1 = C2V2 Where C1= initial concentration V1=Volume required C2=concentration required V2=final volume - Use all glassware of Type-A, - Use calibrated autopipette |
| - Analyse the Lab Control Sample | - Analyse the Lab Control in each batch after every 10 samples. - Plot the result on control chart - If the result of the Lab Control Sample with the limits of control chart go for analysis - If the result of the Lab Control Sample exceeds the limits of control chart take corrective actions |
| - Corrective Actions | - Check to see if the LCS was prepared and analysed according to the method. - Check calibration of the instrument - Recalibrate and reanalyse LCS and samples |

Knowledge and Understanding

  - The Quality control procedures

  - Procedure for Preparation of the Lab Control sample

Tools & Equipment

  - comply with QC standards

  - Inspect own work

  - Identify and apply relevant QC requirements.

  - Glassware

  - Autopipett

  - Certified reference Standards

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare Audit Blind Sample | - Prepare the Audit blind sample for performance evaluation of lab analysts. - Prepare lab control sample of the desired parametersas fallowing - Prepare the stock standard solution from the certified reference material. - Prepare the desired concentration of the desired parameters from stock solution |
| - Analyse the Audit Blind Sample Sample | - Set the instrument according to equipment instructions. - Analyse the laboratory control sample as per standard method. - The laboratory analyst analyse the LCS sample and submits the results. |
| - Evaluation of the Audit Blind Sample results | - Calculate per cent recoveries for each parameter. - The per cent recovery is calculated as: Recovery (%) =found value x 100 True value - Evaluatethe results of LCS “SATISFACTORY” (if found in the acceptance range) and “UNSATISFACTORY”. (if not found in the acceptance range) |

### Analyze Audit Blind Sample

Overview: This competency standards covers the skill and knowledge required toprepare audit blind sample and analyze the audit.

Knowledge and Understanding

- The Quality control procedures

- Procedures for the evaluation of the competence

Tools & Equipment

  - comply with QC/QA standards

  - inspect own work

  - identify and apply relevant QC/QA requirements

### Perform Correctness of Results

Overview: This competency standards covers the skill and knowledge required toAnalyze the Sample, Perform Anion- cation balance, Check the measure EC and ions sum and Check Measured TDS to EC ratio.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Analyse the Sample | P1.Analyse the sample for pH, Electrical conductivity and total Dissolved solids P2Analyse the sample for major Anions and Cations. P3. Calculate the TDS by sum of the concentration of all the constituents in mg/l P4. Calculate the electrical conductivity as per the standard method. |
| CU2.Perform Anion- cation balance | P1. Perform the anion-cation balance by the following formula: % difference=100x∑Cations-∑Anions ∑Cations + ∑Anions P2. The anion-cation sum expressed as milliequilent per litter must balance P3.The acceptance criteria is as fallows Anion Sum (meq/L) Acceptable difference 0-3.0 + 0.2meq/L 3.0-10.0 + 0.2% 10.0-800 + 5% |
| CU3.Check the measure EC and ions sum | P1.Measure the electrical conductivity of the sample P2.sum the anions and cations P3.Both the anions and cations sum should be approximately 1/100 of the measured EC value P4. If either sum does not meet this criteria, re analyse the sample P5. The acceptance criteria is as fallows 100xanions(or cations) sum, meq/L= (0.9 – 1.1) EC |
| CU4.Check Measured TDS to EC ratio | P1. Measure the electrical conductivity of the sample P2.Measure the TDS of the sample P3. Calculate the ratio of the TDS and EC as Measured TDS/conductivity= 0.55 – 0.7 P4.The measured TDS to EC ratio should be between 0.55 and 0.7 P5. If it is outside these limits thee either measured TDS or measure conductivity is suspect; reanalyse the sample |

Knowledge and Understanding

  - The Quality control procedures

Tools & Equipment

  - comply with QC/QA standards

  - Inspect own work

  - Identify and apply relevant QC/QA requirements

### Determine the Method Detection Limits

Overview: This competency standards covers the skill and knowledge required to Prepare Standard Solutions, Analyze the Standard Solution and Calculate the Method Detection Limit and Evaluate the MDL results.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1:Prepare Standard Solutions | P1. In order to calculate MDL prepare solution of concentration approximately four times the estimated limit. P2. Prepare standard stock solution P3.Prepare desired standard concentration solution from the stock solution. |
| CU2:Analyse the Standard Solution and Calculate the Method Detection Limit | P1 Set the instrument according to equipment instructions P2.Analyse the prepared solution 10 times over the course of several days. P3. Calculate the standard deviation by the usual method. P4. Calculate the MDL by substituting standard deviation in the following equation: Method Detection Limit (MDL) = t value x Std. deviation Where t = degree of freedom (n-1) |
| CU3.Evaluate the MDL results | P1. If the MDL is much higher than the estimated detection limit, it may indicate problems with the procedure or with the analytical equipment, improperly made reagents and expiration dates for chemicals. P2.Reprepare all the standards P3. Re analyse the standard and calculate the method detection limit. |

Knowledge and Understanding

  - The Quality control procedures

  - Procedure for Determination of the MDL

Tools & Equipment

  - comply with QC/QA standards

  - inspect own work

  - identify and apply relevant QC/QA requirements

### Develop Control Charts

Overview: This competency standards covers the skill and knowledge required toPrepare Lab Control Sample for Control Chart, Analyze the Lab Control Sample, Set the Control Limitsand Interpret the Control chart data.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Prepare Lab Control Sample for Control Chart | P1. Considering the measurement range of various parameter/methods Prepare lab control samples by following steps P2. Prepare Stock solution from certified reference material/in-house standards. Desired standard concentration is prepared from the stock solution by using the following formula C1V1 = C2V2 C1 = Initial Concentration V1 = Required Volume C2 = Concentration Required V2 = Final Volume |
| CU2:Analyse the Lab Control Sample | P1: Analyse the Lab control sample of each parameter 30 times by different analysts on different days. P2. Calculate the Mean as ∑ of results No. of Observations P3: calculate the Standard Deviation (SD) as Xi= No of Observations x¯=Mean Value N= No of readings |
| CU:3 Set the Control Limits | Set the Control Limits as fallowing P1. Upper control limit UCL = x¯ + 3 s P2: Upper warning limit UWL = x + 2 s P3: Lower warning limit LWL = x - 2 s P4: Lower control limit LCL = x - 3 s |
| CU:4 Interpret the Control chart data | P1. Control Limits: If one measurement exceeds the CL repeat the analyses immediately. If the repeat measurement is within the CL, continue the analyses. If it exceeds the CL, discontinue the analyses, identify the root cause and correct the problem. P2. Warning Limits: If two out of three successive points exceeds the WL, analyse another sample. If the next point is within the WL, continue analyses, if the next point exceeds the WL, evaluate potential bias and correct the problem. P3. Standard Deviation: If four out of five successive points exceeds 1s, or are in decreasing or in increasing order indicate the mean shift. Analyse another lab control sample for confirmation. If the next point is less than 1s, or changes the order, continue analyses, otherwise discontinue analyses and correct the problem. P4. Trending: If seven successive points are on the same side of the central line, indicate that there has been a shift in the mean; work can continue, however the cause of the shift needs to be identified and the limits re-set if justified |

Knowledge and Understanding

  - The Quality control procedures

  - Procedure for Preparation of the Control Chart

Tools & Equipment

  - comply with QC/QA standards

  - inspect own work

  - identify and apply relevant QC/QA requirements

## Water Resources of Pakistan

### Identify Water Resources of Pakistan

Overview: This unit describes the skills and knowledge required to understand and ultimately manage water resources of Pakistan.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Demonstrate water and land potential of Pakistan | Identify physiographic features Identify climate and meteorology Differentiate various kind of geology and soils Identify land resources including irrigated areas, rain-fed areas, hill torrent areas, desert areas, and wastelands Determine water resources |
| Demonstrate irrigation and drainage system | Identify Irrigation System of Pakistan Understand Historic Irrigated Agriculture Classify Development after Independence Recognize Indus Basin Irrigation System (IBIS) - Recognize Irrigation Systems Outside Indus Plains including: Rod Kohi/Bandat (Flood Irrigation) - Medium/Small Dams - Sailaba (Falling Flood) Irrigation |
| Demonstrate water conservation technologies and practices | Demonstrate technologies for Irrigated Areas Demonstrate Watercourse Improvement Demonstrate Improved Farm Layout Demonstrate Laser Land Levelling Demonstrate Improved Irrigation Application Methods Demonstrate Skimming Wells DemosntrateUse of Low Quality Groundwater for Agriculture Demonstrate Irrigation Scheduling Demonstrate Zero Tillage Technology Demonstrate Technologies for Barani (rainfed) Areas - Small/Mini Dams - Dug Wells Demonstrate Pressurized Irrigation System Demonstrate Technologies for Mountainous Areas Northern Areas |
| Demonstrate rainwater harvesting techniques | Demonstrate various types of rainwater harvesting techniques Prepare design of RWH Prepare cost estimates for RWH Systems Implement RWH systems |

## Soft Skills

### Manage the meetings

Overview: This unit describes the skills and knowledge required to manage a range of meetings including overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes.

| Competency Unit | Performance Criteria |
| --- | --- |
| 1. Prepare for meetings | P1. Develop an agenda in line with the stated meeting purpose P2. Ensure the style and structure of the meeting are appropriate to its purpose P3. Identify meeting participants and notify them in accordance with organizational procedures P4. Confirm meeting arrangements in accordance with the requirements of meeting P5. Dispatch meeting working papers to participants within designated timelines |
| 2. Conduct meetings | P1. Conduct meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements P2. Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes P3. Ensure meeting facilitation enables participation, discussion, problem-solving and resolution of issues P4: Record minutes of meeting in accordance with organizational requirements. P4.Brief other minute-taker on method for recording meeting minutes in accordance with organizational requirements and conventions for type of meeting |
| 3. Follow up meetings | P1. Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions P2. Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements P3.Report outcomes of meetings as required, within designated timelines |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes:

- Outline meeting terminology, structures, arrangements

- Outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings

- Describe options for meetings including In-person/physical, teleconferencing, web-conferencing and using webcams

- Identify the relevant organizational procedures and policies regarding meetings, chairing and minutes including identifying organizational formats for minutes and agendas.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

A candidate who demonstrates competency in this unit must be able to provide evidence of the ability to manage meetings. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments. Demonstrated evidence is required of the ability to:

- Apply conventions and procedures for formal and informal meetings including:

- Developing and distributing agendas and working papers

- Identifying and inviting relevant meeting participants

- organizing and confirming meeting arrangements

- running the meeting and following up

  - organize, take part in and chair a meeting

  - record and store meeting documentation

  - Follow organizational policies and procedures

### Manage workforce planning

Overview: This unit describes the skills and knowledge required to manage planning in relation to an organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify workforce | P1. Review current data on staff turnover and demographics P2. Assess factors that may affect workforce supply P3. Develop organization’s requirement for skilled workforce |
| - Develop workforce objectives and strategies | P1. Review organizational strategy and establish aligned objectives for modification P2. Prepare strategies to address unacceptable staff turnover, if required P3. Define objectives to retain required skilled labor P4. Define objectives for workforce diversity and cross-cultural management P5. Obtain agreement and endorsement for objectives and establish targets P6. Develop contingency plans to cope with extreme situations |
| - Implement initiatives to support workforce planning objectives | P1. Implement action to support agreed objectives for recruitment, training, redeployment and redundancy P2. Develop and implement strategies to assist workforce to deal with organizational dynamics P4. Implement succession planning model to ensure desirable workers are developed and retained P5. Implement programs to ensure workplace is an employer of choice |
| - Monitor and evaluate workforce trends | P1. Evaluate workforce plan against patterns in exiting employee and workforce changes P2. Monitor labor supply trends for areas of high turnover in external environment P3. Monitor effects of labor trends on demand for labor P4. Survey organizational climate to gauge worker satisfaction P5. Refine objectives and strategies in response to national and international changes and make recommendations in response to global trends. P6. Regularly review government policy on labor jobs according to labor rights. P7. Evaluate effectiveness of change processes against agreed objectives |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes:

  - Explain current information about external labor supply relevant to the specific industry or skill requirements of the organization

  - Outline industrial relations relevant to the specific industry

  - Describe labor force analysis and forecasting skills

Critical Evidence(s) Required

The candidate needs to produce the following Critical Evidence(s) in order to be competent in this competency standard:

  - Review relevant trends and supply and demand factors that will impact on an organization’s workforce

  - Develop a workforce plan that includes relevant research and specific strategies to ensure access to a skilled and diverse workforce.

### Undertake project work

Overview: This unit describes the skills and knowledge required to undertake a straightforward project or a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Define project | P1. Assess project scope and other relevant documentation P2. Identify project stakeholders P3. Seek clarification of discrepancies from delegating authority related to project and project parameters P4. Determine and access available resources to undertake project |
| - Develop project plan | P1. Develop project feasibility report P2. Develop project plan in line with the project parameters P3. Develop and approve project budget P4. Formulate risk management plan for project, including Workplace Health and Safety (WHS) |
| - Control and monitor project | P1. Ensure project team members are clear about their responsibilities and the project requirements P2. Ensure outcomes and documented time lines of the project are met P3. Maintain required recordkeeping systems throughout the project P4. Implement and monitor plans of project finances and resources P5. Prepare project progress reports as required to stakeholders P6. Monitor risk management as required to ensure project outcomes are met |
| - Finalize the project | P1. Assess project scope and other relevant documentation P2. Identify project stakeholders P3. Seek clarification of discrepancies from delegating authority related to project and project parameters P4. Determine and access available resources to undertake project |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes:

- Give examples of project management tools and how they contribute to a project

- Outline types of documents and other sources of information commonly used in defining the parameters of a project

  - Explain processes for identifying and managing risk in a project

  - Explain the organization’s procedures and processes that are relevant to managing a project including:

    - lines of authority and approvals

    - quality assurance

    - human resources

    - budgets and finance

    - recordkeeping

    - reporting

- Outline the legislative and regulatory context of the organization in relation to project work, including workplace health and safety (WHS) requirements

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

Use project management tools to develop and implement a project plan including:

- deliverables

- work breakdown

- budget and allocation of resources

- timelines

- risk management

- recordkeeping and reporting

### Apply interpersonal skills

Overview: This unit describes the skills and knowledge required to use advanced and specialized communication skills in the client-counselor relationship.

| Competency Unit | Performance Criteria |
| --- | --- |
| 1. Communicate effectively | P1. Identify communication barriers and use strategies to overcome these barriers in the client-counselor relationship P2. Facilitate the client-counselor relationship through selection and use of micro skills P3. Observe and respond to non-verbal communication cues P4. Integrate case note taking with minimum distraction |
| 2. Use specialized counseling interviewing skills | P1. Select and use communication skills according to the sequence of a counseling interview P2. Identify points at which specialized counseling interviewing skills are appropriate for inclusion P3. Use specialized counseling communication techniques based on their impacts and potential to enhance client development and growth P4. Identify and respond appropriately to strong client emotional reactions |
| 3. Evaluate own communication | P1. Reflect on and evaluate own communication with clients P2. Recognize the effect of own values and beliefs on communication with clients P3. Identify and respond to the need for development of own skills and knowledge |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes:

- Legal and ethical considerations for communication in counseling practice, and how these are applied in individual practice:

  - codes of conduct/practice

  - discrimination

  - human rights

  - practitioner/client boundaries

  - privacy, confidentiality and disclosure

  - rights and responsibilities of workers, employers and clients

  - work role boundaries responsibilities and limitations of the counselor role

  - workplace health and safety

- Communication techniques and micro-skills including:

  - attending behaviors active listening, reflection of content feeling, summarizing

  - questioning skills open, closed, simple and compound questions

  - client observation skills

  - noting and reflecting skills

  - providing client feedback

- Components of the communication process including:

  - encoder

  - decoder

- Primary factors that impact on the communication process including:

  - context

  - participants

  - rules

  - messages

  - channels

  - noise

  - feedback

- Communication barriers and resolution strategies, including:

  - environmental

  - physical

  - individual perceptions

  - cultural issues

  - language

  - age issues

  - disability

- Observational techniques including:

  - facial expressions

  - non-verbal behavior

  - posture

  - silence

- Ways including:

  - visual in which different people absorb information

  - auditory

  - kinesthetic

- Impacts of trauma and stress on the communication process, including on:

  - concentration and attention

  - memory

  - Intelligence

  - use of verbal and written language

  - use of body language

  - challenging within the counseling session

- Self-evaluation practices, including:

  - how to recognize own biases

  - Impact of own values on the counseling relationship

### Work safely in an office environment

Overview: This unit describes the performance outcomes, skills and knowledge required to participate in workplace occupational health and safety (OHS) processes to protect workers own health and safety, and that of others.

| Competency Unit | Performance Criteria |
| --- | --- |
| 1. Work safely | P1. Follow established safety procedures when conducting work P2. Carry out pre-start systems and equipment checks in accordance with workplace procedures |
| 2. Implement workplace safety requirements | P1. Identify designated persons for reporting queries and concerns about safety in the workplace P2. Identify existing and potential hazards in the workplace, report them to designated persons and record them in accordance with workplace procedures P3. Identify and implement workplace procedures and work instructions for controlling risks P4. Report emergency incidents and injuries to designated persons P5. Maintain emergency contact list |
| 3. Participate in OHS consultative processes | P1. Contribute to workplace meetings, inspections or other consultative activities P2. Raise OHS issues with designated persons in accordance with organizational procedures P3. Take actions to eliminate workplace hazards or to reduce risks |
| 4. Follow safety procedures | P1. Identify and report emergency incidents P2. Follow organizational procedures for responding to emergency incidents P3. Check of safety tools |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

  - Explain responsibilities of employers and employees under relevant health and safety regulation

  - Describe emergency procedures including procedures for fires, accidents and evacuation

  - Outline commonly used hazard signs and safety symbols.

### Maintain professionalism in workplace

Overview: This unit describes the skills and knowledge required to use advanced and specialized communication skills in the client-counselor relationship.

| Competency Unit | Performance Criteria |
| --- | --- |
| 1. Respect work timeframes | P1. Demonstrate punctuality in meeting, set working hours and times. P2. Utilize working hours only for working and follow company regulations. P3. Complete work tasks within deadlines according to order of priority P4. Perform extra ordinary during working hours |
| 2. Maintain personal appearance and hygiene | P1. Clean hair, body and nails regularly. P2. Wear suitable cloths for the workplace, and respect local and cultural contexts P3. Meet specific company dress code requirements P4. Keep smiling and have positive body language during working hours |
| 3. Maintain adequate distance with colleagues and clients | P1. Respect personal space of colleagues and clients with reference to local customs and cultural contexts. P2. Avoid cross transmission of infections (especially through respiration). |
| 4. Work in an ethical manner | P1. Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. P2. Use company resources in accordance with company ethical standards. P4. Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. P5. Instruct co-workers on ethical, lawful and reasonable directives. P6. Share company values/practices with co-workers using appropriate behavior and language. P7. Report work incidents/situations and/or resolved in accordance with company protocol/guidelines. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes:

- Application of good manners and right conduct

- Basic practices for oral and personal hygiene

- Common products used for oral and personal hygiene

- Outline the company code of conduct/values

- Outline the Company regulations, performance and ethical standards

- Work responsibilities/job functions

- Communication skills

- Workplace hygiene standards