| ### Assessment material ## EVIDENCE GUIDE | MASON |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Qualification Mason CS Code: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Level: 2 ## Credit: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Version: 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## contents | - Assessment Summary and Record - Candidate Assessment - Assessor Judgment Guide - List of required tools/equipment, material and context of assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## ASSESSMENT and assessor DETAILS | Competent |  |  |  |  | Not Yet Competent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessment |  | Re-Assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessor’s Name |  |  |  |  |  |  |  |  |  | Assessor’s Code |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessor’s Signature |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |
|  | DD MM YYYY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## candidate details | Candidate’s Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | First Name |  | Last Name |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Father’s Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Institute Name and District |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CNIC/BFORM # |  |  |  |  |  |  |
|  | Registration Number issued by Assessment Body |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gender |  |  |  |  |  | MaleFemale Transgender |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Candidate's Consent |  |  |  |  |  | I agree to the time and date of the assessment and am aware of the requirements of the assessment. I fully understand my rights of appeal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Candidate’s Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## assessment results summary form | You can use this coversheet as an Assessment Results Summary Form. Simply post a photocopy of this completed coversheet to NAVTTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## NAVTTC office only | 1. date form received: | DD MM YYYY | 2. date entered into database: DD MM YYYY |

| 1 | ### assessment summary & record |
| --- | --- |

| ## activity | ## METHOD |  |  |  | ## desired outcomes | ## result |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NATURE OF ACTIVITY | WRITTEN | oral | Portfolio | OBSERVATION | desired outcomes for successful assessment of competency standard: Mason | competent | not yet competent |
| Practical Skill Demonstration |  |  |  |  | - Perform layout according to drawings/instructions - Lay bricks/blocks according to instruction/ drawings - Carryout concrete work according to instruction/ drawings - Carryout Plaster and pointing according to instruction/ drawings - Lay drains |  |  |
| Knowledge Assessment |  |  |  |  | - Answer all questions your Assessor may have during the practical assessment. |  |  |
| Other Requirements |  |  |  |  | - N.A |  |  |

| 2 | ### candidate assessment |
| --- | --- |
| Candidate's Name………………………………………………..... Father’s Name ………………………………………………………………….. |  |
| all work assessed in this COMPETENCY STANDARD must be your own work. |  |

GUIDANCE TO CANDIDATE

To meet this standard you are required to complete the following tasks within five hours timeframe: - Construct a brick masonry wall size 4’ x9’’ x1’ for open drain of size 4.0’x1.0’x1.0’ with cement sand ratio (1:6), PCC base (1:4:8) of thickness 3’’ and plaster the walls of the drain with cement mortar (1:6).Thickness of plaster is 3/4” - Pointing of brick wall panel 2’x2’ with cement sand ratio 1:2

| ## ACTIVITies | ## candidate response |
| --- | --- |
| - Complete practical task of “Mason” under observation by an assessor | - During a practical assessment, under observation by an assessor, I will correctly : - Construct a brick masonry walls of size 4’x0.75’x1’ for open drain of size 4.0’x1.0’x1.0’ with cement sand ratio (1:6), horizontal mortar layer of ½ inch. PCC base (1:4:8) of thickness 3’’and plaster the walls of the drain with cement mortar (1:6).Thickness of plaster is ¾ ” - Draw a centre line of length 4’ with reference to given start point to perform a Layout of open drain of size 4.0’x1.0’x1.0’ - Draw offset lines on either side of the centre line at distance of 1.125’ from the centre line. - If the mixing process done by the labour of cement, sand and crush is according to the ratio of 1:4:8. - Fix the formwork done by the labour is according to the size of base of open drain(4’x2.5’x0.25’) - Lay down the P.C.C (1:4:8) at the specified site according to the size of base of open drain(4’x2.5’x0.25’) - Compact the P.C.C with tamping rod of 5/8” diameter - Finish the P.C.C concrete surface with wooden float. - Prepare cement mortar according to ratio (1:6) for the construction of masonry wall 4’x9’’x1’of an open drain 4.0’x1.0’x1.0’. - Soak the brick if required. - Lay cement mortar of thickness 0.5 inch for the construction of a wall (4’X9’’ X 1’) of open drain on both sides. - Lay bricks in layers by using English bond to construct a wall 4’X 9’’ X 1’ - Maintain vertical alignment of wall size 4’x9’’x1’ by using plumb bob - Maintain horizontal alignment of wall size 4x9’’x1’ by using string and pins - Finnish the surface of walls to remove extra mortar by using wire brush - Apply cement slurry on brick wall panel to ensure adhesion of mortar with brick wall. - Apply cement mortar ¾ inch thick on brick walls of size open drain and level it with the help of trowel/straighter - Finish the surface of wall 4x1’by using float. - Adapt the safety measures as per work requirement. - Pointing of brick wall panel 2’x2’ with cement sand ratio 1:2 - If the brick wall panel 2’x2’ is cured. - Set the mixing process done by the labour of cement and sand is according to the ratio of 1:2 - Align the grooves of brick wall by using chisel to prepare the surface for pointing of brick wall panel 2’x2’ - Fill the gaps of joints of brick layers wall panel 2’x2’ by using cement sand mortar (1:2) - Remove the excess mortar with the help of wire brush and leave a face joint that is just lower than the face of the wall. - Important Note: The following general performance criteria will be followed and performed in all the tasks (where applicable) by the candidate in order to meet this standard. - Follow health and safety procedure as per the requirements of given task - Prepare work station as per the requirements of task - Select tools/ equipment as per the requirements of task - Report to supervisor in case of any contingency after taking necessary actions - Complete the work by following sequence of operations - Perform task(s) within standard timeframe |
| 2. Other requirements | - [N/A] |
| 3. Answer any questions your assessor may have during the practical assessment | - My answers to questions are correct and demonstrate my understanding of the topics and their application. |

| 3 | ### assessor judgement guide |
| --- | --- |
| Candidate’s Name ………………………………………………… Father’s Name.…………………………………………………….. |  |
| ## INSTRUCTIONS FOR ASSESSOR | This section contains minimum evidence requirements. Oral questioning may be used to clarify candidate understanding of the topic and its application. |

| ## Activities | ## minimum evidence required | ## Yes | ## No | ## assessor comments |
| --- | --- | --- | --- | --- |
| 1. Complete practical task of Mason under observation by an assessor | During a practical assessment, under observation by an assessor, the candidate correctly carried out the following tasks: |  |  |  |
| Construct a brick masonry walls of size 4’x0.75’x1’ for open drain of size 4.0’x1.0’x1.0’ with cement sand ratio (1:6), PCC base (1:4:8) of thickness 3’’and plaster the walls of the drain with cement mortar (1:6).Thickness of plaster is ¾ ” | Drawn a centre line of length 4’ with reference to given start point to perform a Layout of open drain of size 4.0’x1.0’x1.0’. |  |  |  |
|  | Drawn offset lines on either side of the centre line at distance of 1.125’ from the centre line. |  |  |  |
|  | Did the mixing process done by the labour of cement, sand and crush is according to the ratio of 1:4:8. |  |  |  |
|  | Ensured whether the formwork done by the labour is according to the size of base of open drain (4’x2.5’x0.25’). |  |  |  |
|  | Laid the P.C.C (1:4:8) at the specified site according to the size of base of open drain (4’x2.5’x0.25’). |  |  |  |
|  | Compact the P.C.C with tamping rod of 5/8” diameter. |  |  |  |
|  | Finished the P.C.C concrete surface with wooden float. |  |  |  |
|  | Prepared a cement mortar according to ratio (1:6) for the construction of masonry wall 4’x0.75’x1’of an open drain 4.0’x1.0’x1.0’. |  |  |  |
|  | Soaked the brick if required |  |  |  |
|  | Laid cement mortar of thickness 0.5 inch for the construction of a wall (4’X 0.75’ X 1’) of open drain both sides. |  |  |  |
|  | Laid bricks in layers by using English bond to construct a wall 4’X 0.75’ X 1’. |  |  |  |
|  | Checked vertical alignment of wall size 4’x9’’x1’ by using plumb bob |  |  |  |
|  | Checked horizontal alignment of wall size 4x9’’x1’ by using string and pins |  |  |  |
|  | Finished the surface of walls to remove extra mortar by using wire brush. |  |  |  |
|  | Applied cement slurry on brick wall panel to ensure adhesion of mortar with brick wall. |  |  |  |
|  | Applied cement mortar ¾ inch thick on brick walls of open drain and level it with the help of trowel. |  |  |  |
|  | Finished the surface of plaster by using float |  |  |  |
|  | Adapted the safety measures as per work requirement. |  |  |  |
| Pointing of brick wall panel 2’x2’ with cement sand ratio 1:2 | - Cured the brick wall panel 2’x2’ is. |  |  |  |
|  | - Did the mixing process done by the labour of cement and sand is according to the ratio of 1:2. |  |  |  |
|  | - Aligned the grooves of brick wall by using chisel to prepare the surface for pointing of brick wall panel 2’x2’. |  |  |  |
|  | Filled the courses of brick wall panel 2’x2’ by using cement sand mortar (1:2). |  |  |  |
|  | Removed the excess mortar with the help of wire brush and leave a face joint that is just lower than the face of the wall. |  |  |  |
| General performance criteria | Follow health and safety procedure as per the requirements of given task |  |  |  |
|  | Prepare work station as per the requirements of task |  |  |  |
|  | Select tools/ equipment as per the requirements of task |  |  |  |
|  | Report to supervisor in case of any contingency after taking necessary actions |  |  |  |
|  | Complete the work by following sequence of operations perform task(s) within standard timeframe |  |  |  |

| 3. Answer any questions the assessor may have during the practical assessment | Candidate’s answers to questions are correct and demonstrate understanding of the topics and their application. Assessor to document below all questions asked and candidate answers. Use extra sheets if required and attach. |  |  |  |
| --- | --- | --- | --- | --- |

| 4 | ### List of TOOLS, equipment, material and context of assessment |
| --- | --- |

| INSTRUCTIONS | This section contains information regarding; - Context of the assessment - List of required tools and equipment. - List of consumable items required during the service |
| --- | --- |
| - Context of Assessment | This task will be performed in real time/simulated environment. |

| - List of tools and equipment required (for five candidates) |  |  |
| --- | --- | --- |
| S. No | Items | Quantity |
| 1 | Trowel | 05 |
| 2 | Brick Hammer | 05 |
| 3 | Spirit Level | 05 |
| 4 | String (length= 10 m) | 05 |
| 5 | Pins | 20 |
| 6 | Straight Edge | 05 |
| 7 | Plumb Bob | 05 |
| 8 | Mason Square | 05 |
| 9 | Steel tape | 05 |
| 10 | Hammer | 05 |
| 11 | Chisel | 05 |
| 12 | Wheel Barrow | 05 |
| 13 | Wooden float | 05 |
| 14 | Gloves | 10 |
| 15 | Shovel | 05 |
| 16 | Steel Pan | 10 |
| 17 | Wire Brush | 05 |
| 18 | Safety Helmet | 05 |

| - List of consumable items required (for five candidates) |  |  |
| --- | --- | --- |
| S. No | Items | Quantity |
| 1 | Cement | 350 Kg |
| 2 | sand | 2000 Kg |
| 3 | Crush | 600 Kg |
| 4 | bricks | 500 No |
| 5 | Lime powder | 25 Kg |