| ### Assessment material ## EVIDENCE GUIDE | Mason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Qualification Mason CS Code: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Level: 3 ## Credit: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Version: 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## contents | - Assessment Summary and Record - Candidate Assessment - Assessor Judgment Guide - List of required tools/equipment, material and context of assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## ASSESSMENT and assessor DETAILS | Competent |  |  |  |  | Not Yet Competent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessment |  | Re-Assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessor’s Name |  |  |  |  |  |  |  |  |  | Assessor’s Code |  |  |  |  |  |  |  |  |  |  |  |
|  | Assessor’s Signature |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |
|  | DD MM YYYY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## candidate details | Candidate’s Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | First Name |  | Last Name |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Father’s Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Institute Name and District |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CNIC/BFORM # |  |  |  |  |  |  |
|  | Registration Number issued by Assessment Body |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gender |  |  |  |  |  | MaleFemale Transgender |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Candidate's Consent |  |  |  |  |  | I agree to the time and date of the assessment and am aware of the requirements of the assessment. I fully understand my rights of appeal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Candidate’s Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## assessment results summary form | You can use this coversheet as an Assessment Results Summary Form. Simply post a photocopy of this completed coversheet to NAVTTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## NAVTTC office only | 1. date form received: | DD MM YYYY | 2. date entered into database: DD MM YYYY |

| 1 | ### assessment summary & record |
| --- | --- |

| ## activity | ## METHOD |  |  |  | ## desired outcomes | ## result |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NATURE OF ACTIVITY | WRITTEN | oral | Portfolio | OBSERVATION | desired outcomes for successful assessment of competency standard: Mason | competent | not yet competent |
| Practical Skill Demonstration |  |  |  |  | - Perform stone work according to instructions/drawing and specification. - Erect scaffolding according to instructions/drawing - Fix doors and windows according to instructions/ drawing - Carry out tiling work according to instructions/drawings |  |  |
| Knowledge Assessment |  |  |  |  | - Answer all questions your Assessor may have during the practical assessment. |  |  |
| Other Requirements |  |  |  |  | - N.A |  |  |

| 2 | ### candidate assessment |
| --- | --- |
| Candidate's Name………………………………………………..... Father’s Name ………………………………………………………………….. |  |
| all work assessed in this COMPETENCY STANDARD must be your own work. |  |

GUIDANCE TO CANDIDATE

To meet this standard you are required to complete the following tasks within Five hours timeframe: - Construct stone wall of size 4’x4’ of 13” thickness with cement mortar at ratio 1:6 at site. - Carry out tiling work on the surface of 3’x3’ wall with tile size 1’x1’ with bond. - Develop a platform of size 6’x2’x4’ by using scaffolding for fixing of window frame at a height 7’ from ground level. The size of the window is 3’x3’.

| ## ACTIVITies | ## candidate response |
| --- | --- |
| - Complete practical task of “Mason” under observation by an assessor | - During a practical assessment, under observation by an assessor, I will correctly : - Construct stone wall of size 4’x4’ of 13” thickness with cement mortar at ratio 1:6 at site. - Carry out dressing of stone to make them uniform size. - Confirm whether the mixing process done by the labour of cement and sand is according to the ratio of 1:6. - Lay cement sand mortar on the bed of stone wall with thickness of 5 cm. - Align the stone on the cement sand mortar layer to construct stone wall of size 4’x4’ of 13” thickness. - Lay the second layer of stone to ensure that vertical joints are staggered. - Maintain vertical alignment of wall by using plumb bob. - Fill the joints in the wall with small pieces of stones. - Fill the grooves by grouting the cement sand mortar (1:6) to strengthen it. - Carry out tiling work on the surface of 3’x3’ wall with tile size 1’x1’ with bond - Ensure the levels of the surface of 3’x3’ wall with hand level. - Prepare the surface of the wall 3’x3’ with steel wire brush - Chip the surface of the wall to make bond of tile with wall. - Prepare adhesive to be used within 30 minutes - Apply adhesive of 1/4” on the surface of the tile and place it at desired location - Lay down tiles starting from the centre of the wall 3’x3’ by using tile spacer for uniform joints and to avoid sliding - Tap in all tiles with a rubber hammer to ensure solid bond and level plane. - Fill the grooves and joints of the tiles on the surface of wall 3’x3’ with a grout after getting some strength. - Remove the excess grout by using damp sponge. - Finish the surface and joints of tiles. - Develop a platform of size 6’x2’x4’ by using scaffolding for fixing of window frame at a height 7’ from ground level. The size of the window is 3’x3’ - Fix vertical poles at required position with cross couplers / knots. - Fix horizontal poles with couplers/knots. - Lay the planks on horizontal poles to make temporary working platform. - Attach the guardrails to avoid risk of fall. - Inspect thoroughly the scaffolding to ensure safety. - Prepare opening size for window frame of size 3’x3’ in the wall at a height 7’ from the ground. - Fix the hinges of the frame with the help of PCC (1:2:4) to fix window frame of size 3’x3’ in the wall at a height 7’ from the ground. - Position the horizontal and vertical alignment of the window frames after completion of concreting operation. - Finnish the frame hinge area by using cement mortar (1:6). - Adapt the safety measures as per work requirement. - Important Note: The following general performance criteria will be followed and performed in all the tasks (where applicable) by the candidate in order to meet this standard. - Follow health and safety procedure as per the requirements of given task - Prepare work station as per the requirements of task - Select tools/ equipment as per the requirements of task - Report to supervisor in case of any contingency after taking necessary actions - Complete the work by following sequence of operations Perform task(s) within standard timeframe |
| 2. Other requirements | - [N.A] |
| 3. Answer any questions your assessor may have during the practical assessment | - My answers to questions are correct and demonstrate my understanding of the topics and their application. |

| 3 | ### assessor judgement guide |
| --- | --- |
| Candidate’s Name ………………………………………………… Father’s Name.…………………………………………………….. |  |
| ## INSTRUCTIONS FOR ASSESSOR | This section contains minimum evidence requirements. Oral questioning may be used to clarify candidate understanding of the topic and its application. |

| ## Activities | ## minimum evidence required | ## Yes | ## No | ## assessor comments |
| --- | --- | --- | --- | --- |
| 1. Complete practical task of Mason under observation by an assessor | During a practical assessment, under observation by an assessor, the candidate correctly carried out the following tasks: |  |  |  |
| Construct stone wall of size 4’x4’ of 13” thickness with cement mortar at ratio 1:6 at site. | Carried out dressing of stone to make them same height and of 13” thickness. |  |  |  |
|  | Confirmed whether the mixing process done by the labour of cement and sand is according to the ratio of 1:6. |  |  |  |
|  | Laid cement sand mortar on the bed of stone wall with thickness of 5 cm. |  |  |  |
|  | Aligned the stone on the cement sand mortar layer to construct stone wall of size 4’x4’ of 13” thickness. |  |  |  |
|  | Laid the second layer of stone to ensure that vertical joints are staggered. |  |  |  |
|  | Checked vertical alignment of wall by using plumb bob. |  |  |  |
|  | Filled the joints in the wall with small pieces of stones. |  |  |  |
|  | Filled the grooves by grouting the cement sand mortar (1:6) to strengthen it. |  |  |  |
| Carry out tiling work on the surface of 3’x3’ wall with tile size 1’x1’ with bond | Ensured the levels of the surface of 3’x3’ wall with hand level. |  |  |  |
|  | - Prepared the surface of the wall 3’x3’ with steel wire brush. |  |  |  |
|  | Chipped the surface of the wall to make bond of tile with wall |  |  |  |
|  | Prepared adhesive to be used within 30 minutes. |  |  |  |
|  | Applied adhesive of 1/4” on the surface of the tile and place it at desired location. |  |  |  |
|  | Laid tiles starting from the centre of the wall 3’x3’ by using tile spacer for uniform joints and to avoid sliding. |  |  |  |
|  | Tapped in all tiles with a rubber hammer to ensure solid bond and level plane. |  |  |  |
|  | Filled the grooves and joints of the tiles on the surface of wall 3’x3’ with a grout after getting some strength. |  |  |  |
|  | Removed the excess grout by using damp sponge. |  |  |  |
|  | Finished the surface and joints of tiles. |  |  |  |
| Develop a platform of size 6’x2’x4’ by using scaffolding for fixing of window frame at a height 7’ from ground level. The size of the window is 3’x3’. | Fixed vertical poles at required position with cross couplers / knots. |  |  |  |
|  | Fixed horizontal poles with couplers/knots. |  |  |  |
|  | Laid the planks on horizontal poles to make temporary working platform. |  |  |  |
|  | Attached the guardrails to avoid risk of fall. |  |  |  |
|  | Inspected thoroughly the scaffolding to ensure safety. |  |  |  |
|  | Prepared opening size for window frame of size 3’x3’ in the wall at a height 7’ from the ground. |  |  |  |
|  | Fixed the hinges of the frame with the help of PCC (1:2:4) to fix window frame of size 3’x3’ in the wall at a height 7’ from the ground. |  |  |  |
|  | Positioned the horizontal and vertical alignment of the window frames after completion of concreting operation. |  |  |  |
|  | Finished the frame hinge area by using cement mortar (1:6). |  |  |  |
|  | Adapted safety measures as per work requirement. |  |  |  |
| General Performance Criteria | Follow health and safety procedure as per the requirements of given task |  |  |  |
|  | Prepare work station as per the requirements of task |  |  |  |
|  | Select tools/ equipment as per the requirements of task |  |  |  |
|  | Report to supervisor in case of any contingency after taking necessary actions |  |  |  |
|  | Complete the work by following sequence of operations perform task(s) within standard timeframe |  |  |  |
| 2. Other requirements | N/A |  |  |  |

| 3. Answer any questions the assessor may have during the practical assessment | Candidate’s answers to questions are correct and demonstrate understanding of the topics and their application. Assessor to document below all questions asked and candidate answers. Use extra sheets if required and attach. |  |  |  |
| --- | --- | --- | --- | --- |

| 4 | ### List of TOOLS, equipment, material and context of assessment |
| --- | --- |

| INSTRUCTIONS | This section contains information regarding; - Context of the assessment - List of required tools and equipment. - List of consumable items required during the service |
| --- | --- |
| - Context of Assessment | This task will be performed in real time/simulated environment. |

| - List of tools and equipment required (for five candidates) |  |  |
| --- | --- | --- |
| S. No | Items | Quantity |
| 1 | Trowel | 05 |
| 2 | Tile Cutter | 05 |
| 3 | Spirit Level | 05 |
| 4 | String (length= 10 m) | 05 |
| 5 | Pins | 20 |
| 6 | Straight Edge | 05 |
| 7 | Plumb Bob | 05 |
| 8 | Mason Square | 05 |
| 9 | Steel tape | 05 |
| 10 | Steel Pan | 05 |
| 11 | Spacer | 100 |
| 12 | Wheel Barrow | 05 |
| 13 | Shovel | 05 |
| 14 | Wire Brush | 10 |
| 15 | Stone Hammer | 05 |
| 16 | Sponge | 05 |
| 17 | Scaffolding Key set | 05 |
| 18 | Adjustable Screw wrench | 05 |
| 19 | Cleaning cloth (length 1m) | 05 |
| 20 | Safety Helmet | 05 |
| 21 | Safety Shoes | 10 |
| 22 | Safety Goggles | 05 |

| - List of consumable items required (for five candidates) |  |  |
| --- | --- | --- |
| S. No | Items | Quantity |
| 1 | Tiles 1’x1’ | 45 Sft |
| 2 | Tile Bond | 5 Bags |
| 3 | Tile Bond | 5 Boxes |
| 4 | Stone | 125 Cft |
| 5 | Cement | 50 kg |
| 6 | Sand | 300kg |
| 7 | MS Pipe (Length = 15’) | 20 No |
| 8 | MS Pipe (length= 10’) | 20 No |
| 9 | MS Joints | 50 No |
| 10 | Window Frame (3’x3’) | 5 No |
| 11 | Wooden Planks (10’x1’x 1.125”) | 15 |