|  | Assessment Evidence Guide For “Automobile Supervisor” Level-4 Summative Assessment |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standards | - Perform Overhauling of Engine - Repair Manual Transmission System - Repair Automatic Transmission System - Maintenance of Hybrid Electric Vehicle - Advance Vehicle driving assistance systems - Drive Vehicle with Manual and Automatic Transmission |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration Number____________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): B. Measure the piston size, replace the counter shaft bearing of Manual transmission, change the automatic transmission fluid as per SOP, service the battery terminals of HEV, check the traction control system of the vehicle, and perform a final test drive after completing all tasks. |
| Time: 05 Hrs | During a practical assessment, under observation by an assessor, you are required to demonstrating the following criteria: |
| Minimum Evidence Required | - Remove engine pistons and check the condition of pistons piston rings and connecting rods. - Measure the taper of engine cylinders. - Measure the piston size with the help of micrometer. - Check the condition of cylinder block. - Disassembly and Inspection of manual gear box casing/housing. - Replace the counter shaft bearings as per recommendations. - Inspection of manual transmission gears, bearings, and seals for wear and damage. - Disassemble the automatic transmission fluid sump and wash the fluid strainer / filter. - Change transmission oil as per recommendations. - Take off the electrical system from the main switch. - Disengage the main wire grip from the HEV battery panel. - Inspect and clean the HEV battery terminals if corrosion is observed with the battery cleaner. - Visual inspection of Traction Control System wiring, connectors, and sensors for any visible damage, wear, or corrosion. - Testing wheel speed sensors with a multimeter or oscilloscope. - Checking brake fluid levels and the brake system for leaks. - Test driving to evaluate TCS performance under controlled conditions. - Read fault codes and remove/reset all systems. Using Diagnostic OBD-II |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Supervisor Level-4 |
| Competency Standards | - Perform Overhauling of Engine - Repair Manual Transmission System - Repair Automatic Transmission System - Maintenance of Hybrid Electric Vehicle - Advance Vehicle driving assistance systems. - Drive Vehicle with Manual and Automatic Transmission |
| Assessment Task | B. Measure the piston size, replace the counter shaft bearing of Manual transmission, change the automatic transmission fluid as per SOP, service the battery terminals of HEV, check the traction control system of the vehicle, and perform a final test drive after completing all tasks. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Remove engine pistons and check the condition of pistons piston rings and connecting rods. |  |  |
| - Measure the taper of engine cylinders. |  |  |
| - Measure the piston size with the help of micrometer. |  |  |
| - Check the condition of cylinder block. |  |  |
| - Disassemble and inspect manual gear box casing/housing. |  |  |
| - Replace the counter shaft bearings as per recommendations. |  |  |
| - Inspect manual transmission gears, bearings, and seals for wear or damage. |  |  |
| - Disassemble the automatic transmission fluid sump and wash the fluid strainer / filter. |  |  |
| - Change transmission oil as per recommendations. |  |  |
| - Take off the electrical system from the main switch. |  |  |
| - Disengage the main wire grip from the HEV battery panel. |  |  |
| - Inspect and clean the HEV battery terminals if corrosion is observed with the battery cleaner. |  |  |
| - Perform visual inspection of Traction Control System wiring, connectors, and sensors for any visible damage, wear, or corrosion. |  |  |
| - Test wheel speed sensors with a multimeter or oscilloscope. |  |  |
| - Check brake fluid levels and the brake system for leaks. |  |  |
| - Test drive to evaluate TCS performance under controlled conditions. |  |  |
| - Read fault codes and remove/reset all systems. Using Diagnostic OBD-II. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standards | - Perform Overhauling of Engine - Repair Manual Transmission System - Repair Automatic Transmission System - Maintenance of Hybrid Electric Vehicle - Advance Vehicle driving assistance systems. - Drive Vehicle with Manual and Automatic Transmission |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | B. Measure the piston size, replace the counter shaft bearing of Manual transmission, change the automatic transmission fluid as per SOP, service the battery terminals of HEV, check the traction control system of the vehicle, and perform a final test drive after completing all tasks. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes |  | No | Remarks |  |
|  | Removed engine pistons and checked the condition of pistons piston rings and connecting rods. |  |  |  |  |  |  |  |
|  | Measured the taper of engine cylinders. |  |  |  |  |  |  |  |
|  | Measured the piston size with the help of micrometer. |  |  |  |  |  |  |  |
|  | Checked the condition of cylinder block. |  |  |  |  |  |  |  |
|  | Disassembled and inspect manual gear box casing/housing. |  |  |  |  |  |  |  |
|  | Replaced the counter shaft bearings as per recommendations. |  |  |  |  |  |  |  |
|  | Inspected manual transmission gears, bearings, and seals for wear or damage. |  |  |  |  |  |  |  |
|  | Disassembled the automatic transmission fluid sump and wash the fluid strainer / filter. |  |  |  |  |  |  |  |
|  | Changed transmission oil as per recommendations. |  |  |  |  |  |  |  |
|  | Took off the electrical system from the main switch. |  |  |  |  |  |  |  |
|  | Disengaged the main wire grip from the HEV battery panel. |  |  |  |  |  |  |  |
|  | Inspected and cleaned the HEV battery terminals if appear the corrosion on it’s with the battery cleaner. |  |  |  |  |  |  |  |
|  | Visually inspected Traction Control System wiring, connectors, and sensors for any visible damage, wear, or corrosion. |  |  |  |  |  |  |  |
|  | Tested wheel speed sensors with a multimeter or oscilloscope. |  |  |  |  |  |  |  |
|  | Checked brake fluid levels and the brake system for leaks. |  |  |  |  |  |  |  |
|  | Tested driving to evaluate TCS performance under controlled conditions. |  |  |  |  |  |  |  |
|  | Read fault codes and remove/reset all systems. Using Diagnostic OBD-II |  |  |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |  |  |

Knowledge Assessment

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standards | - Perform Overhauling of Engine - Repair Manual Transmission System - Repair Automatic Transmission System - Maintenance of Hybrid Electric Vehicle - Advance Vehicle driving assistance systems. - Drive Vehicle with Manual and Automatic Transmission |
| Candidate Details | Name: __________________________________________________________ Registration Number: __________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Why is torque important when crankshaft and bearings installation is carried out? |  |  |
|  | How should the timing belt/chain and gears be aligned during reassembly? |  |  |
|  | Why is it necessary to adjust valve timing and clearances in reassembly? |  |  |
|  | What is the sequence of removing major components (cylinder head, intake, exhaust)? |  |  |
|  | What is the function of clutch system in manual transmission? |  |  |
|  | Why is it important to perform a road test in different drive positions during a transmission diagnosis? |  |  |
|  | Why is it necessary to change the transmission oil and clean the oil sump/strainer? |  |  |
|  | What regular maintenance does a hybrid vehicle require? |  |  |
|  | How often should the hybrid battery be checked? |  |  |
|  | Which component in a cruise control system adjusts the vehicle speed? |  |  |
|  | What happens to the cruise control system when the brake pedal is pressed? |  |  |
|  | Which component detects wheel slip for the traction control system? |  |  |
|  | How does TCS interact with the vehicle’s brakes to maintain control? |  |  |
|  | What is the purpose of the clutch pedal in a manual transmission vehicle? |  |  |
|  | When should you use the "R" (Reverse) gear in an automatic transmission? |  |  |
|  | What is the Driving Principle? |  |  |
|  | How does an automatic transmission shift gear? |  |  |
|  | When should you use your turn signals? |  |  |
|  | How should the timing belt/chain and gears be aligned during reassembly? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________