National Competency Standards Level-4 for Automobile

Occupation (Automobile Supervisor)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORDS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Automobile Supervisor Level 4. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director   (NAVTTC)

Table of Contents

1.	Introduction	4

1.1. Date of development:	4

1.2. Date of review & validation:	4

1.3. Entry requirement:	4

2.	Code of Qualification	4

3.	Summary Of Competency Standards	5

4.	Detail of Competency Standards	6

0716MSA1701. Perform Overhauling of Engine	6

0716MSA1702. Repair Manual Transmission System	10

0716MSA1703. Repair Automatic Transmission System	14

0716MSA1704. Basic Maintenance of Hybrid Electric Vehicle	16

0716MSA1705. Advanced Vehicle Driving Assistance Systems	19

0716MSA1706. Drive Vehicle with Manual& Automatic Transmission	23

5.	List of Tools and Equipment	26

6.	List of Consumables	27

7.	Members of Qualification Development & Review Committee	28

# Introduction

Automobile Technology is a branch of engineering which deals with everything about automobiles and practices to propel them.  Automobile can also be defined as a vehicle which can move by itself, for example Car, jeep, bus, truck, scooter, etc. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Automobile Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the Auto Mobile industry of the country.

| ## 1.1. Date of development: | February 2018 |
| --- | --- |
| ## 1.2. Date of review & validation: | July, 2024 |
| ## 1.3. Entry requirement: | The entry requirement for National Vocational Certificate Level-4 in Automobile Supervisor is a Person having Level-3 Certificate of Automobile Assistant Supervisor |

# Code of Qualification

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| Code | Qualification Title |
| --- | --- |
| 0716MSA15 | National Vocational Certificate Level-2 in Automobile Technology (Automobile Mechanic) |
| 0716MSA16 | National Vocational Certificate Level-3 in Automobile Technology (Automobile Assistant Supervisor) |
| 0716MSA17 | National Vocational Certificate Level-4 in Automobile Technology (Automobile Supervisor) |

# Summary Of Competency Standards

| Code | Competency Standards | Category | Level | Th | Pr | Total | Credit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0716MSA1701 | Perform Overhauling of Engine | Technical | 4 | 45 | 210 | 255 | 25.5 |
| 0716MSA1702 | Repair Manual Transmission System | Technical | 4 | 40 | 192 | 232 | 23.2 |
| 0716MSA1703 | Repair Automatic Transmission System | Technical | 4 | 45 | 192 | 237 | 23.7 |
| 0716MSA1704 | Maintenance of Hybrid Electric Vehicle | Technical | 4 | 50 | 180 | 230 | 23 |
| 0716MSA1705 | Advance Vehicle driving assistance systems | Technical | 4 | 45 | 90 | 135 | 13.5 |
| 0716MSA1706 | Drive Vehicle with Manual and Automatic Transmission | Technical | 4 | 15 | 96 | 111 | 11.1 |
| Total |  |  |  | 240 | 960 | 1200 | 120 |

# Detail of Competency Standards

## 0716MSA1701. Perform Overhauling of Engine

Overview

This competency standard is developed to provide skills and knowledge to Overhaul Engine (Petrol and Diesel), in accordance with the Manufacturer Manual. You will be able to diagnose fault/s and other maintenance issues of automobile engines while ensuring safe use of tools equipment and materials.

| Competency Units | Performance Criteria |
| --- | --- |
| - Inspect Engine of the vehicle | - Carryout compression test to check the engine performance according to standards - Identify leakages in engine for oil/ coolant by following standard procedure - Drain engine oil and coolant as per standard procedure |
| - Remove Engine from the Vehicle | - Disconnect relevant electrical connections of engine as per standard procedure - Remove ancillary components of engine as per standard procedure - Remove engine from the vehicle for overhauling according to standard procedure |
| - Dismantle the components of the engine | - Mount engine on engine overhauling trolley as standards - Dismantle engine components to identify faults as per Manufacturer Repair Manual - Mark engine components and identify their fixing order or the place - Clean all parts of engine for repair as per standard procedure |
| - Repair the Engine | - Check engine cylinder head (Cam Shaft, Valve Set and Valve Seat) to repair/replace, if required - Take measurements for machining of engine components (Crank Shaft, Cam Shaft and Engine Block) as per standards - Reassemble engine with specified torque as per standard procedures |
| - Re-fit Engine in the Vehicle | - Refit engine on vehicle, fill engine oil and radiator coolant to required level - Check supply lines from fuel, air, and coolant etc. - Reconnect relevant engine electrical connections of battery and wire harness - Vehicle test run as per SOPs and final adjustments if necessary; ensure proper working condition |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS)

- Repair Manual

- Mechanism of Engine

- Use	of hand tools, torque wrench, special service tools for removal/adjustment

- Measuring tools and equipment (Pressure Gauges, Micrometer, Vernier Caliper, Multi-meters and scanners)

- Lifting equipment

- Engine components

- Measurement of wear in Crank Shaft, and in Cam Shaft, Engine Bore, Piston, Bearings, Bushes and Shims

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Identify faults of automatic transmission system and repair or replace faulty  parts / components according to SOPs

List of Tools and Equipment

| S. No. | Items |
| --- | --- |
| 1. | Vehicle with workshop manual |
| 2. | Wrench |
| 3. | Compression Gauge. |
| 4. | Vacuum Gauge |
| 5. | Radiator Pressure Tester |
| 6. | Tool Kit (Hand tools) |
| 7. | Hydraulic jack |
| 8. | Engine hoist |
| 9. | Safety stands. |
| 10. | Wheel Spanner |
| 11. | Two post lift |
| 12. | Working bench |
| 13. | Washing Tray |
| 14. | Valve spring compressor tool |
| 15. | Vernier Caliper |
| 16. | Micrometer |
| 17. | Scale |
| 18. | Feeler Gauge. |
| 19. | Repair manual |
| 20. | Torque wrench |
| 21. | Piston Ring Compressor Tool. |
| 22. | Valve spring compressor tool |
| 23. | Engine Scanner. |

## 0716MSA1702. Repair Manual Transmission System

Overview

This competency standard is designed to provide skills and knowledge to repair manual transmission of vehicle by Auto Mechanic, in accordance with the Manufacturer Manual. You will be able to diagnose faults related to transmission system of vehicle and repair faulty part/s according to set standards.

| Competency Units | Performance Criteria |
| --- | --- |
| - Diagnose Faults in Manual Transmission System | - Check clutch mechanism (paddle, linkage etc.), hydraulic clutch fluid conditions and leakage, according to specified standards, and diagnose faults, if any - Check gear mechanism (linkage bushes and bearings, shafts of gears, differential etc.), oil level and leakage according to the specified standards, and diagnose faults, if any - Check Gear box mounts and diagnose faults, if any - Disassemble transmission to diagnose faults, if any |
| - Repair Manual Transmission System | - Maintain or change hydraulic clutch paddle fluid of clutch according to specified level - Repair or replace any defected part/s according to specified standards, after performing vehicle test on road for checking slipping, abnormal noise, vibrations etc. - Replace clutch set (clutch and pressure plate, clutch release bearing) and repair related components according to specified standards, if required - Repair or replace faulty part/s of transmission gear box according to specified standards |
| - Repair auxiliary transmission components | - Repair or replace faulty parts of propeller shafts according to specified standards - Repair or replace faulty components of differential according to specified standards - Repair or replace faulty axle according to specified standards - Repair or replace faulty seals and washers according to specified standards - Reassemble and install a transmission gearbox in a vehicle according to Manufacturer Manual |
| - Disassemble Propeller shaft, Differential and Drive axle | - Identify key components of the propeller shaft (e.g., universal joints, slip yoke, center support bearing), differential (e.g., ring gear, pinion gear, spider gears), and drive axle (e.g., axle shafts, bearings, seals). - Disassemble the propeller shaft, differential, and drive axle, following manufacturer guidelines and workshop manuals. - Perform detailed inspections of disassembled components, checking for signs of wear, cracks, corrosion, or other forms of damage. - Evaluate the condition of components and determines whether they are suitable for reuse, require reconditioning, or need replacement, providing recommendations based on industry standards and manufacturer specifications. |
| - Transaxle and 4-wheel drive | - Identify key components of the transaxle like (gears, synchronizers, bearings, seals) and 4-wheel drive system like ( transfer case, drive shafts, differentials, locking hubs). - Identify symptoms of common transaxle and 4-wheel drive issues, such as unusual noises, difficulty shifting gears, leaks, or loss of power to wheels. - Utilize diagnostic tools (e.g., OBD-II scanner, pressure gauges, endoscopes) to identify faults in the transaxle and 4-wheel drive system, such as worn gears, faulty synchronizers, or transfer case problems. - Replace or repairs defective components within the transaxle and 4-wheel drive system, ensuring all parts are within manufacturer specifications and tolerances. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS)

- Manufacturer Repair Manual

- Uses of measuring tools (Vernier Caliper, Micro-meter, Feeler Gauge etc.)

- Gear Ratio

- Torque Wrench

- Lubricants

- Sealing Materials

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Identify faults of manual transmission system and repair or replace faulty parts/ components according to SOPs

  - Identify faults of propeller shaft, Differential and drive axle, transaxle and 4-wheel drive systems.

List of Tools and Equipment

| S. No. | Items |
| --- | --- |
| 1 | Vehicle |
| 2 | stethoscope |
| 3 | Complete tool kit |
| 4 | Universal joint cross |
| 5 | Sealant gun |
| 6 | Seal extractor |
| 7 | Bearing puller |
| 8 | Gear puller |
| 9 | Safety stand/ two post lift |
| 10 | Air compressor |
| 11 | Creeper |
| 12 | Trolley jack |
| 13 | SST (Special service tools) |
| 14 | Snap ring pliers |

## 0716MSA1703. Repair Automatic Transmission System

Overview

This competency standard is developed to provide skills and knowledge to repair Automatic Transmission System and ancillary components by Auto Mechanic, in accordance with the Manufacturer Manual. You will be able to diagnose faults in Automatic Transmission System of the vehicle and repair the same while using standard procedures.

| Competency Units | Performance Criteria |
| --- | --- |
| - Inspection of Automatic Transmission components/Parts | - Observe Construction and operation of different types of automatic Transmission (AGS, CVT, Conventional AT) - Identification of function and purpose of each component - Identification of Different automatic transmission fluids/oils |
| - Diagnose Automatic System Faults in Transmission | - Carry out road test at different speeds for smooth operations of torque converter and gear shifting according to Manufacturer standard - Check automatic transmission mounts for faults if any - Check automatic transmission fluid (ATF) level, according to standard specification and identify leaks if any - Check automatic transmission solenoid by using automotive scanner and identify faults if any - Check electrical controls and Hydraulic pressure of automatic transmission for faults if any |
| - Disassemble Automatic Transmission System and Ancillary Components | - Remove automatic transmission and disassemble it to check for worn-out/ faulty part/s and replace them as per Manufacturer Repair Manual - Check automatic transmission performance by ensuring proper linkages and controls as per standards - Reassemble automatic transmission and refit to the vehicle according to standards - Perform Calibration of Automatic transmission after service |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS) precautions

- Manufacturer Repair Manual

- Fluid types of automatic transmission

- Use of hand tools, torque wrench, special service tools for removal/ adjustment

- Use of measuring tools and equipment (Pressure Gauges, Multi-meters, Scanners)

- Use of lifting equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify faults of Automatic Transmission System and repair or replace faulty parts/ components according to SOPs

List of Tools and Equipment

| S. No. | Items |
| --- | --- |
| 1 | Multimeter |
| 2 | stethoscope |
| 3 | Complete tool kit |
| 4 | Scanner |
| 5 | Sealant gun |
| 6 | Seal extractor |
| 7 | Bearing puller |
| 8 | Gear puller |
| 9 | Safety stand/ two post lift |
| 10 | Air compressor |
| 11 | Creeper |
| 12 | Trolley jack |
| 13 | SST (Special service tools) |
| 14 | Snap ring |

## 0716MSA1704. Basic Maintenance of Hybrid Electric Vehicle

Overview

This competency standard is developed to provide skills and knowledge to Inspect Hybrid Electric vehicle, Diagnose Hybrid system Powertrain and Maintain Hybrid System powertrain

| Competency Units | Performance Criteria |
| --- | --- |
| - Inspect Hybrid Electric vehicle | - Adhere to all safety protocols when working with high-voltage systems, including wearing appropriate personal protective equipment (PPE), using insulated tools, and following lockout/tagout procedures. - Conducts regular maintenance tasks such as checking and replacing the battery cooling system fluid, inspecting and servicing the regenerative braking system, and maintaining the internal combustion engine and electric motor. |
| - Diagnose Hybrid system Powertrain | - Identify symptoms of hybrid powertrain issues, such as reduced fuel efficiency, loss of power, unusual noises, warning lights, or errors displayed on the vehicle's diagnostic system. - Follow systematic troubleshooting procedures to isolate faults, including checking for proper voltage levels, continuity, and communication between hybrid system components. - Analyze diagnostic data to determine the underlying cause of hybrid powertrain issues, such as a faulty inverter, degraded battery pack, or malfunctioning electric motor. |
| - Maintain Hybrid System powertrain | - Ensures the hybrid system is de-energized before performing any maintenance work, using proper techniques to disconnect the high-voltage battery and secure the vehicle - Uses diagnostic tools to monitor the health and performance of the hybrid system, identifying potential issues early and performing preventive maintenance to avoid costly repairs. - Replace components according the standard procedures. - Reviews maintenance records regularly to identify recurring issues or patterns, using this information to adjust maintenance schedules and improve overall system reliability and performance. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS) precautions

- Manufacturer Repair Manual

- Hybrid Powertrain system functions

- Standard maintenance procedure on Hybrid System

- Use of scanning tool to measure performance of Hybrid Powertrain

- Use of lifting equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify faults of Hybrid Electric vehicle

- Perform basic maintenance on HEV

List of Tools and Equipment

| S. No. | Items |
| --- | --- |
| 1 | Multimeter |
| 2 | Universal scanner |
| 3 | Complete tool kit |
| 4 | SST (Special service tools) |
| 5 | Test Bulb |
| 6 | Seal extractor |
| 7 | PPEs |
| 8 | Torque wrench |
| 9 | Safety stand/ two post lift |
| 10 | Wire Strippers |
| 11 | Crimping Tools |
| 12 | Trolley jack |
| 13 | Soldering Iron and Solder |

## 0716MSA1705. Advanced Vehicle Driving Assistance Systems

Overview

This competency standard is designed to provide skills and knowledge to Inspect cruise control system, Inspect Traction control system and Diagnose and Troubleshooting of ADAS (Advance Driver Assistance System) System

| Competency Units | Performance Criteria |
| --- | --- |
| - Inspect cruise control system | - Demonstrates understanding of different types of cruise control systems, including traditional cruise control and adaptive cruise control, highlighting their specific components and functions. - Identifies symptoms of cruise control system issues, such as the system not engaging, disengaging unexpectedly, or failing to maintain the set speed. - Utilize diagnostic tools (e.g., OBD-II scanner, multimeter) to read fault codes, monitor system performance, and test individual components for proper operation. - Follows systematic troubleshooting procedures to isolate faults, including checking the integrity of electrical connections, verifying the functionality of the speed sensor and brake switch, and testing the control module and throttle actuator - Performs necessary maintenance tasks, including cleaning or replacing sensors, repairing or replacing damaged wiring, and ensuring all components are securely mounted and properly connected. - Tests the cruise control system after inspection and maintenance to confirm that it engages, maintains speed, and disengages correctly, ensuring the system operates safely and reliably under various driving conditions |
| - Inspect Traction control system | - Accurately identifies key ABS components, including wheel speed sensors, hydraulic control unit (HCU), electronic control unit (ECU), and ABS modulator. - Identifies symptoms of common ABS problems, such as ABS warning lights, unusual noises during braking, or reduced braking performance. |
| - Diagnose and Troubleshooting of ADAS (Advance Driver Assistance System) System | - Identify key ADAS components, including sensors (e.g., radar, lidar, cameras, ultrasonic sensors), control modules, actuators, and user interface elements. - Explains the function and interaction of each component within the ADAS, detailing how they contribute to features such as adaptive cruise control, lane-keeping assist, automatic emergency braking, and blind-spot monitoring. - Identify symptoms of ADAS issues, such as inaccurate or non-functional driver assistance features, warning lights, or error messages on the vehicle’s dashboard. - Utilizes diagnostic tools (e.g., OBD-II scanner, specialized ADAS calibration equipment, multimeter) to read fault codes, monitor system performance, and test individual components. - Conducts thorough testing of the ADAS after repairs to confirm that all features are functioning correctly, using both static and dynamic tests to verify system performance in various conditions. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS) precautions

- In-depth knowledge of ADAS features such as adaptive cruise control, lane-keeping assist, automatic emergency braking, blind-spot monitoring, parking assist, and traffic sign recognition.

- Work of different sensors (radar, lidar, cameras, ultrasonic) detect and interpret data from the vehicle’s surroundings.

- Ability to use diagnostic tools such as OBD-II scanners, specialized ADAS calibration equipment, and multimeters to identify and troubleshoot faults.

- Understanding the importance of sensor calibration and alignment to ensure accurate ADAS operation.

- Knowledge of how software and firmware updates can impact ADAS functionality.

- Knowledge of safety protocols and best practices for working with ADAS, including the handling of sensitive electronic components and high-voltage systems.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Understanding of systematic troubleshooting approaches to isolate faults, identify root causes, and implement effective solutions.

- Ability to apply problem-solving skills to diagnose and resolve complex ADAS issues.

List of Tools and Equipment

| S. No. | Items |
| --- | --- |
| 1 | Multimeter |
| 2 | Universal scanner |
| 3 | Complete tool kit |
| 4 | SST (Special service tools) |
| 5 | Test Bulb |
| 6 | Seal extractor |
| 7 | PPEs |
| 8 | Torque wrench |
| 9 | Safety stand/ two post lift |
| 10 | Wire Strippers |
| 11 | Crimping Tools |
| 12 | Trolley jack |
| 13 | Soldering Iron and Solder |
| 14 | ADAS-specific Diagnostic Software |
| 15 | Calibration Equipment |

## 0716MSA1706. Drive Vehicle with Manual& Automatic Transmission

Overview

This competency standard is designed to Drive Car with Manual Transmission, Drive Car with Automatic Transmission and 	Drive Four Stroke Motorcycle

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Drive Car with Manual Transmission | - Drive Car with Manual Transmission and follow the following - Adjust mirrors - Adjust seats - Adjust Steering - Know the Different pedals & their uses. - Proper using of Clutch. - Use proper gear Shifting & Gear position. - Proper use of indicator. - Know the Different pedals & their uses. - Parking in proper place. |
| - Drive Car with Automatic Transmission | - Drive Car with Automatic Transmission and follow the following - Adjust mirrors - Adjust seats - Adjust Steering - Know the use of Brake & Accelerator. - Shifting of gear lever in proper position - Proper use of indicator. - Parking in proper place. |
| - Drive Four Stroke Motorcycle | - Drive Four Stroke Motorcycle and follow the following - Adjust back view mirror - Start motorcycle in proper sequence. - Proper use of front wheel brake - Proper use of rear wheel brake - Proper use of clutch - Proper use of indicator while turning - Proper use of horn - Park in proper place - Wear helmet before driving motorcycle |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

K-1 Describes the following

- Adjustment of mirrors

- Adjustment of seats

- Adjustment of Steering

- Different pedals & their uses.

- Proper using of Clutch.

- Gear Shifting & Gear position.

- Proper use of indicator.

- Parking in proper place.

K-2 Explain the following

- Adjust back view mirror

- Starting of motorcycle in proper sequence.

- Proper use of front wheel brake

- Proper use of rear wheel brake

- Proper use of clutch

- proper use of indicator while turning

- Proper use of horn

- Park in proper place

Critical Evidence(s) Required

- Capable to drive car with manual transmission

- Capable to drive four stroke motorcycle

List of Tools and Equipment

1	Vehicle Equipped with manual transmission

2	Vehicle Equipped with Automatic transmission

3	Motorcycle 125cc

# List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Vehicle with workshop manual | 1 |
| 2 | Adjustable Wrench | 25 |
| 3 | Compression Gauge. | 25 |
| 4 | Vacuum Gauge | 25 |
| 5 | Radiator Pressure Tester | 5 |
| 6 | Tool Kit (Hand tools) | 5 |
| 7 | Hydraulic jack | 5 |
| 8 | Engine hoist | 5 |
| 9 | Safety stands. | 5 |
| 10 | Wheel Spanner | 5 |
| 11 | Two post lift | 5 |
| 12 | Working bench | 5 |
| 13 | Washing Tray | 5 |
| 14 | Valve spring compressor tool | 5 |
| 15 | Vernier Caliper | 25 |
| 16 | Micrometer | 25 |
| 17 | Scale | 25 |
| 18 | Feeler Gauge. | 25 |
| 19 | Repair manual | 25 |
| 20 | Piston Ring Compressor Tool. | 25 |
| 21 | Engine Scanner. | 2 |
| 22 | stethoscope | 5 |
| 23 | Complete tool kit | 5 |
| 24 | Universal joint cross | 5 |
| 25 | Sealant gun | 5 |
| 26 | Seal extractor | 25 |
| 27 | Bearing puller | 25 |
| 28 | Gear puller | 25 |
| 29 | Safety stand/ two post lift | 2 |
| 30 | Air compressor | 2 |
| 31 | Creeper | 25 |
| 32 | Trolley jack | 5 |
| 33 | SST (Special service tools) | 5 |
| 34 | Snap ring pliers | 25 |
| 35 | Multimeter | 25 |
| 36 | Scanner | 5 |
| 37 | Snap ring | 25 |
| 38 | Universal scanner | 2 |
| 39 | Test Bulb | 25 |
| 40 | PPEs | 25 each |
| 41 | Torque wrench | 5 |
| 42 | Wire Strippers | 25 |
| 43 | Crimping Tools | 25 |
| 44 | Soldering Iron and Solder | 25 |
| 45 | ADAS-specific Diagnostic Software | 1 |
| 46 | Calibration Equipment | 5 |
| 47 | Vehicle Equipped with Manual transmission | 1 |
| 48 | Vehicle Equipped with Automatic transmission | 1 |
| 49 | Motorcycle 125cc | 1 |

# List of Consumables

| S.no | Consumable | Quantity |
| --- | --- | --- |
| 1 | Cotton rag | 2 kg |
| 2 | Kerosene oil | 5 liters |
| 3 | Petrol | 8 liters |
| 4 | Overhauling Kit | 5 |
| 5 | valve set | 5 |
| 6 | valve seal set | 5 |
| 7 | Piston ring set | 5 |
| 8 | Piston set | 5 |
| 9 | Thrust washers | 10 |
| 10 | valve guide set | 10 |
| 11 | Timing belt and bearing | 5 |
| 12 | Alternator and AC belt | 5 |
| 13 | Engine oil | 5 liters |
| 14 | Engine coolant | 10 liters |
| 15 | Oil filter | 5 |
| 16 | Air filter | 5 |
| 17 | Clutch set | 4 |
| 18 | Silicon tube | 40 |
| 19 | Gear oil | 10 liters |
| 20 | Axle seal set | 10 |
| 21 | Hose pipe | 5 |
| 22 | Fly wheel bearing | 5 |
| 23 | Clutch cables | 10 |
| 24 | Pressure plate | 4 |
| 25 | Clutch bearings | 5 |
| 26 | Cotton gloves | 30 |
| 27 | Dust cover of drive shaft | 10 |
| 28 | Transmission oil SAE 75w85 | 8 liters |
| 29 | Solenoid valve ring | 10 |
| 30 | Spool valve | 5 |
| 31 | Automatic transmission mounts | 10 |
| 32 | A/T seal | 5 |

# Members of Qualification Development & Review Committee

The Qualifications Development, Validation and Review Committees consisted of following members:

| S. # | Name | Designation |  |  | Organization |
| --- | --- | --- | --- | --- | --- |
| 1 | Ijaz Hamid | Chief Instructor (Auto & Diesel) |  |  | GCT, Railway Road, Lahore - PTEVTA |
| 2 | Syed Salman Nasir Ali Shah | Deputy Manager |  |  | PTEVTA |
| 3 | Mehwish Aisha Ahsan | CBT Expert/Assessor |  |  | Lahore Punjab |
| 4 | Adeel Ahmad | Assistant Manager (Tech) |  |  | Lahore Transport Company, Lahore |
| 5 | Rehman Ali | Assistant Manager |  |  | Al-Haj Faw Motors (Pvt.) Ltd. |
| 6 | Abdul Basit | Technical Advisor |  |  | Toyota Garden Motors, Lahore |
| 7 | Aamir Javed | Service Manager |  |  | Suzuki Khalid Motors, Lahore |
| 8 | Syed Kazim Hussain | Technical Advisor |  |  | Toyota Township Motors, Lahore |
| 9 | Atif Mahmood | Service Manager |  |  | Suzuki Mini Motors, Lahore |
| 10. | Muhammad Abdul Wasay | Assistant Manager |  |  | Al-Haj Faw Motors (Pvt.) Ltd. |
| Qualifications Validation Committee |  |  |  |  |  |
| 1. | Ijaz Hamid |  | Chief Instructor (Auto & Diesel) | GCT, Railway Road, Lahore - PTEVTA |  |
| 2. | Mian Atique |  | CEO | Rehman Motors Workshop, Model Town, Lahore |  |
| 3 | Syed Aamir Ali Wasti |  | Director | Honda Kizan, Lahore |  |
| 4 | Muhammad Naveed Malik |  | CEO | Vision Plus, Automotive Equipment Solutions, Lahore |  |
| 5 | Haji Muhammad Arshad |  | CEO | Honda, Raheem Yar Khan Motors |  |
| 6 | Abdul Waheed |  | CEO | Honda Pitspot, Johar Town Motors |  |
| 7 | M. Mumtaz Husain |  | AM (After Sale Service) | Honda Atlas Cars, Lahore |  |
| 8 | Jawad Haider |  | Technical Advisor | Suzuki, Lahore |  |
| Qualification Review Committee |  |  |  |  |  |
| 1 | Engr. Tehseen Khalid |  | Lecturer | GCT Peshawar |  |
| 2 | Mr. Muhammad Boota Saleem |  | Sr. Instructor | VTI Okara PVTC-Punjab |  |
| 3 | Engr. Umair Ahmed Qureshi |  | Sr. Instructor | GVTI Bhimber / AJK TEVTA |  |
| 4 | Engr. Karam Ali Janwari |  | Sr. Instructor | VTC Khairpur / STEVTA |  |
| 5 | Mr. Muhammad Shoaib |  | Sr. Instructor | GTTI Kasur / PTEVTA |  |
| 6 | Engr. Inayat ur Rehman |  | DACCUM Facilitator | Ex-.KPK-TEVTA |  |
| 7 | Muhammad Ishaq |  | DD SS & C | Islamabad |  |
| 8 | Asim Jamal |  | DD SS & C | Islamabad |  |