|  | Assessment Evidence Guide For National Competency Standards CNC MACHINIST Level IV Formative Assessment 0715MMT3604 Perform CNC Lathe Operation |
| --- | --- |
|  | Date 04-09 November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3604 Perform CNC Lathe Operation |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Understand and operate the control panel - Calculate coordinates of job - Mount and align the job (Tool and work offset) - Prepare Manual Program - Prepare canned cycle program - Perform Simulation (e.g. Swansoft/CIMCO etc) Simulators - Feed the Program in machine for production Activity: Manufacture the workpiece according to the specifications outlined in Annexure A, using CNC lathe machine. This can be achieved through manual programming or by utilizing CAD/CAM software. |
| Time: 4 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Perform machine zero positioning to ensure accurate reference points. - Move the turret along the X and Z axes using the Jog/Manual/MPG keys. - Utilize program editing keys such as Program, Edit, MDI, Insert, Alter, EOB, Cancel, and Delete for adjustments. - Adjust the feed and spindle speed using the Override keys. - Calculate the turning job's coordinates based on the drawing. - Secure the job on the CNC Lathe machine's table. - Properly set and align the tool in the tool holder. - Perform work reference setting (tool offsetting) for each tool. - Demonstrate the use of G-codes and M-codes in CNC Lathe operations. - Develop a program in accordance with the drawing. - Create a threading program based on the drawing. - Insert the part program into the simulation software. - Run the program simulation to detect and correct potential errors. - Input the program into the CNC machine using PROGRAM, EDIT, INSERT, ALTER, and EOB keys. - Run the program in single-block mode to ensure proper execution. - Execute the program for production operations. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | CNC MACHINIST Level IV |
| Competency Standard | 0715MMT3604 Perform CNC Lathe Operation |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Understand and operate the control panel - Calculate coordinates of job - Mount and align the job (Tool and work offset) - Prepare Manual Program - Prepare canned cycle program - Perform Simulation (e.g. Swansoft/CIMCO etc) Simulators - Feed the Program in machine for production Activity: Manufacture the workpiece according to the specifications outlined in Annexure A, using CNC lathe machine. This can be achieved through manual programming or by utilizing CAD/CAM software. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Perform machine zero positioning to ensure accurate reference points. |  |  |
| - Move the turret along the X and Z axes using the Jog/Manual/MPG keys. |  |  |
| - Utilize program editing keys such as Program, Edit, MDI, Insert, Alter, EOB, Cancel, and Delete for adjustments. |  |  |
| - Adjust the feed and spindle speed using the Override keys. |  |  |
| - Calculate the turning job's coordinates based on the drawing. |  |  |
| - Secure the job on the CNC Lathe machine's table. |  |  |
| - Properly set and align the tool in the tool holder. |  |  |
| - Perform work reference setting (tool offsetting) for each tool. |  |  |
| - Demonstrate the use of G-codes and M-codes in CNC Lathe operations. |  |  |
| - Develop a program in accordance with the drawing. |  |  |
| - Create a threading program based on the drawing. |  |  |
| - Insert the part program into the simulation software. |  |  |
| - Run the program simulation to detect and correct potential errors. |  |  |
| - Input the program into the CNC machine using PROGRAM, EDIT, INSERT, ALTER, and EOB keys. |  |  |
| - Run the program in single-block mode to ensure proper execution. |  |  |
| - Execute the program for production operations. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3604-Perform CNC Lathe Operation |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | • Understand and operate the control panel • Calculate coordinates of job • Mount and align the job (Tool and work offset) • Prepare Manual Program • Prepare canned cycle program • Perform Simulation • Feed the Program in machine for production Activity: Manufacture the workpiece according to the specifications outlined in Annexure A, using CNC lathe machine. This can be achieved through manual programming or by utilizing CAD/CAM software. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Machine zero positioning is performed to establish accurate reference points. |  |  |  |  |  |
|  | The turret is moved along the X and Z axes using Jog, Manual, or MPG keys. |  |  |  |  |  |
|  | Program editing keys, such as Program, Edit, MDI, Insert, Alter, EOB, Cancel, and Delete, are used for necessary adjustments. |  |  |  |  |  |
|  | Feed and spindle speed are adjusted through the Override keys. |  |  |  |  |  |
|  | Turning job coordinates are calculated based on the drawing. |  |  |  |  |  |
|  | The job is securely mounted on the CNC Lathe machine's table. |  |  |  |  |  |
|  | The tool is properly set and aligned in the tool holder. |  |  |  |  |  |
|  | Work reference setting (tool offsetting) is conducted for each tool. |  |  |  |  |  |
|  | G-codes and M-codes are used to manage CNC Lathe operations. |  |  |  |  |  |
|  | A program is developed in accordance with the drawing specifications. |  |  |  |  |  |
|  | A threading program is created based on the drawing. |  |  |  |  |  |
|  | The part program is inserted into the simulation software. |  |  |  |  |  |
|  | The program simulation is run to identify and correct potential errors. |  |  |  |  |  |
|  | The program is input into the CNC machine using PROGRAM, EDIT, INSERT, ALTER, and EOB keys. |  |  |  |  |  |
|  | The program is initially run in single-block mode to verify correct execution. |  |  |  |  |  |
|  | The program is executed for production operations. |  |  |  |  |  |
|  | The manufactured part was checked and inspected. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3604-Perform CNC Lathe Operation |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define CNC lathe. |  |  |
|  | List three common types of lathe tools. |  |  |
|  | Name four main components of a CNC lathe. |  |  |
|  | What role does a chuck play in a CNC lathe? |  |  |
|  | Why is coolant used in CNC lathes? |  |  |
|  | What are G and M codes in CNC? |  |  |
|  | Define what CAD and CAM stand for. |  |  |

Annexure A

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________