|  | Assessment Evidence Guide For CNC MACHINIST Level IV Formative Assessment 0715MMT3605 Perform CNC Milling Operations |
| --- | --- |
|  | Date 04-09 November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3605 CNC Perform CNC Milling Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Mounting the Workpiece - Program Generation - Running Simulation - Feeding the Program into the Machine - Performing CNC Milling Activity: Create a CAD drawing and corresponding CAM program, then produce the part on the CNC milling machine according to the specifications in Annexure A. |
| Time: 5 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Mount the workpiece according to job specifications. - Use appropriate fixtures to ensure dimensional accuracy. - Verify tool and workpiece alignment (co-axiality, concentricity). - Implement safety measures to prevent accidental operation. - Import the 3D model correctly into the selected CAM software. - Define the reference point and material for machining, ensuring the XYZ zero point aligns with the work offset. - Apply necessary machining features and toolpath strategies. - Generate the part program correctly using the CNC machine’s post-processor. - Ensure correct G/M codes, coordinates, and feed rates are used. - Save and document the part program properly for the machine. - Load the part program successfully into the simulation platform. - Verify tool movements against the defined sequence. - Identify and modify any errors as needed. - Ensure synchronization between the CNC control unit and the program. - Select the correct part program for execution. - Confirm all safety operation protocols. - Adjust feeds, speeds, and machine overrides correctly. - Set the machine to execution mode and start the machining sequence. - Monitor movements during operation. - Inspect the final product for accuracy and precision against design specifications. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | CNC MACHINIST Level-IV |
| Competency Standard | ## 0715MMT3605 CNC Perform CNC Milling Operations |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Mounting the Workpiece - Program Generation - Running Simulation - Feeding the Program into the Machine - Performing CNC Milling Activity: Create a CAD drawing and corresponding CAM program, then produce the part on the CNC milling machine according to the specifications in Annexure A. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Mount the workpiece according to job specifications. |  |  |
| - Use appropriate fixtures to ensure dimensional accuracy. |  |  |
| - Verify tool and workpiece alignment (co-axiality, concentricity). |  |  |
| - Implement safety measures to prevent accidental operation. |  |  |
| - Import the 3D model correctly into the selected CAM software. |  |  |
| - Define the reference point and material for machining, ensuring the XYZ zero point aligns with the work offset. |  |  |
| - Apply necessary machining features and toolpath strategies. |  |  |
| - Generate the part program correctly using the CNC machine’s post-processor. |  |  |
| - Ensure correct G/M codes, coordinates, and feed rates are used. |  |  |
| - Save and document the part program properly for the machine. |  |  |
| - Load the part program successfully into the simulation platform. |  |  |
| - Verify tool movements against the defined sequence. |  |  |
| - Identify and modify any errors as needed. |  |  |
| - Ensure synchronization between the CNC control unit and the program. |  |  |
| - Select the correct part program for execution. |  |  |
| - Confirm all safety operation protocols. |  |  |
| - Adjust feeds, speeds, and machine overrides correctly. |  |  |
| - Set the machine to execution mode and start the machining sequence. |  |  |
| - Monitor movements during operation. |  |  |
| - Inspect the final product for accuracy and precision against design specifications. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _____________________________

Assessors Judgment Guide

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3605 Perform CNC Milling Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Mounting the Workpiece - Program Generation - Running Simulation - Feeding the Program into the Machine - Performing CNC Milling Activity: Create a CAD drawing and corresponding CAM program, then produce the part on the CNC milling machine according to the specifications in Annexure A. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Is appropriate personal protective equipment (PPE) being worn? |  |  |  |  |  |
|  | Has the CNC machine been inspected for visible issues before operation? |  |  |  |  |  |
|  | Are electrical connections and cables checked for safety and proper insulation? |  |  |  |  |  |
|  | Is the workpiece mounted according to job specifications? |  |  |  |  |  |
|  | Are appropriate fixtures used to ensure dimensional accuracy? |  |  |  |  |  |
|  | Is the tool and workpiece alignment verified (co-axiality, concentricity)? |  |  |  |  |  |
|  | Have safety measures been implemented to prevent accidental operation? |  |  |  |  |  |
|  | Is the 3D model correctly imported into the selected CAM software? |  |  |  |  |  |
|  | Is the reference point and material defined for machining? |  |  |  |  |  |
|  | Have the necessary machining features and toolpath strategies been applied? |  |  |  |  |  |
|  | Is the part program generated correctly using the CNC machine's post-processor? |  |  |  |  |  |
|  | Are the correct G/M codes, coordinates, and feed rates used? |  |  |  |  |  |
|  | Is the part program saved and documented properly for the machine? |  |  |  |  |  |
|  | Is the part program successfully loaded into the simulation platform? |  |  |  |  |  |
|  | Are the tool movements verified against the defined sequence? |  |  |  |  |  |
|  | Have any errors been identified and modified as needed? |  |  |  |  |  |
|  | Is there proper synchronization between the CNC control unit and the program? |  |  |  |  |  |
|  | Is the machine switched to receiving mode before loading the program? |  |  |  |  |  |
|  | Is the correct part program selected for execution? |  |  |  |  |  |
|  | Are all safety operation protocols confirmed? |  |  |  |  |  |
|  | Are feeds, speeds, and machine overrides adjusted correctly? |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Drawing Annexure, A

Knowledge Assessment

| Qualification | CNC MACHINIST Level IV |
| --- | --- |
| Competency Standard | 0715MMT3605-Perform CNC Milling Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | Define CNC |  |  |  |
|  | What is a work offset in CNC? |  |  |  |
|  | List three common types of milling cutters |  |  |  |
|  | Enlist common G-codes used for work offset. |  |  |  |
|  | Define RPM. |  |  |  |
|  | What are G and M codes in CNC? |  |  |  |
|  | Define CAM. |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |