|  | Assessment Evidence Guide For “Machinist” Level-2 Summative Assessment |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Machinist (Level – 2) |
| --- | --- |
| Competency Standards | - Develop Basic Communication Skills - Maintain Safe Work Environment - Maintain Tools & Equipment - Carry Out Calculations and Prepare Estimates - Carryout General Maintenance - Carryout Bench Work - Perform Basic Grinding Operations - Perform Drilling Machine Operation - Perform Turning Operations - Perform Shaping Operations - Workshop Projects-I |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. OR - Perform shaper operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10 |
| Time: 5 Hrs | During a practical assessment, under observation by an assessor, you are required to demonstrating the following criteria: |
| Minimum Evidence Required | A. Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. - Interpret drawing - Follow safety protocols - Perform facing operation to achieve the length of 65 mm. - Turn the job to a diameter 23 mm up to a length of 22 mm. - Turn the first step turning diameter 11 mm upto 4 mm length. - Perform taper turning to reduce diameter from 16 mm to 12 mm for a length of 21 mm. - Turn a second step to achieve 8 mm diameter for a length of 4 mm. - Cut M10 external V-thread having pitch 1.5 mm for a 14mm length. B. Perform shaper operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10. - Set up the shaper machine mounting appropriate tool. - Mount the workpiece in shaper machine vice. - Shape the job size as 60mm x 48mm x 12mm. - Align the tool at 21mm from the top edge for slot cutting. - Cut a groove of 6mm width and 4mm depth. - Drill two holes as per drawing. - Clean the hole through reamer. - Cut internal threads of M10 with a Tap. - Deburr all edges of the workpiece - Clean the work area - Turn off the machine |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machinist (Level 02) |
| Competency Standards | - Develop Basic Communication Skills - Maintain Safe Work Environment - Maintain Tools & Equipment - Carry Out Calculations and Prepare Estimates - Carryout General Maintenance - Carryout Bench Work - Perform Basic Grinding Operations - Perform Drilling Machine Operation - Perform Turning Operations - Perform Shaping Operations. - Workshop Projects-I |
| Assessment Task | - Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. OR - Perform shaper operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. |  |  |
| - Interpret drawing |  |  |
| - Follow safety protocols |  |  |
| - Perform facing operation to achieve the length of 65 mm. |  |  |
| - Turn the job to a diameter 23 mm up to a length of 22 mm. |  |  |
| - Turn the first step turning diameter 11 mm upto 4 mm length. |  |  |
| - Perform taper turning to reduce diameter from 16 mm to 12 mm for a length of 21 mm. |  |  |
| - Turn a second step to achieve 8 mm diameter for a length of 4 mm. |  |  |
| - Cut M10 external V-thread having pitch 1.5 mm for a 14mm length. |  |  |
| - Perform an operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10. |  |  |
| - Set up the shaper machine mounting appropriate tool. |  |  |
| - Mount the workpiece in shaper machine vice. |  |  |
| - Shape the job size as 60mm x 48mm x 12mm. |  |  |
| - Align the tool at 21mm from the top edge for slot cutting. |  |  |
| - Cut a groove of 6mm width and 4mm depth. |  |  |
| - Drill two holes as per drawing. |  |  |
| - Clean the hole through reamer. |  |  |
| - Cut internal threads of M10 with a Tap. |  |  |
| - Deburr all edges of the workpiece |  |  |
| - Clean the work area |  |  |
| - Turn off the machine |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machinist (Level 02) |
| --- | --- |
| Competency Standards | - Develop Basic Communication Skills - Maintain Safe Work Environment - Maintain Tools & Equipment - Carry Out Calculations and Prepare Estimates - Carryout General Maintenance - Carryout Bench Work - Perform Basic Grinding Operations - Perform Drilling Machine Operation - Perform Turning Operations - Perform Shaping Operations. - Workshop Projects-I |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. OR - Perform shaper operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| - Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing Annex-A. |  |  |  |  |  |  |
|  | Interpret drawing |  |  |  |  |  |
|  | Follow safety protocols |  |  |  |  |  |
|  | Perform facing operation to achieve the length of 65 mm. |  |  |  |  |  |
|  | Turn the job to a diameter 23 mm up to a length of 22 mm. |  |  |  |  |  |
|  | Turn the first step turning diameter 11 mm upto 4 mm length. |  |  |  |  |  |
|  | Perform taper turning to reduce diameter from 16 mm to 12 mm for a length of 21 mm. |  |  |  |  |  |
|  | Turn a second step to achieve 8 mm diameter for a length of 4 mm. |  |  |  |  |  |
|  | Cut M10 external V-thread having pitch 1.5 mm for a 14mm length. |  |  |  |  |  |
| B. Perform shaper operation as per drawing of Annex-B to shape a workpiece with a 6mm slot and 2 holes of M10. |  |  |  |  |  |  |
|  | Set up the shaper machine mounting appropriate tool. |  |  |  |  |  |
|  | Mount the workpiece in shaper machine vice. |  |  |  |  |  |
|  | Shape the job size as 60mm x 48mm x 12mm. |  |  |  |  |  |
|  | Align the tool at 21mm from the top edge for slot cutting. |  |  |  |  |  |
|  | Cut a groove of 6mm width and 4mm depth. |  |  |  |  |  |
|  | Drill two holes as per drawing. |  |  |  |  |  |
|  | Clean the hole through reamer. |  |  |  |  |  |
|  | Cut internal threads of M10 with a Tap. |  |  |  |  |  |
|  | Deburr all edges of the workpiece |  |  |  |  |  |
|  | Clean the work area |  |  |  |  |  |
|  | Turn off the machine |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |

Knowledge Assessment

| Qualification | Machinist (Level 02) |
| --- | --- |
| Competency Standards | - Develop Basic Communication Skills - Maintain Safe Work Environment - Maintain Tools & Equipment - Carry Out Calculations and Prepare Estimates - Carryout General Maintenance - Carryout Bench Work - Perform Basic Grinding Operations - Perform Drilling Machine Operation - Perform Turning Operations - Perform Shaping Operations. - Workshop Projects-I |
| Candidate Details | Name: __________________________________________________________ Registration Number: __________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________ |

| Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application. |  |  |  |
| --- | --- | --- | --- |
| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
|  | How many tools can mount in lathe tool post at a time? |  |  |  |
|  | What is the difference between a tap and a die? |  |  |  |
|  | What is the primary function of a tailstock in a lathe? |  |  |  |
|  | How many mm in 1 inch? |  |  |  |
|  | What is the function of coolant in machining operations? |  |  |  |
|  | Enlist 5 major parts of Shaper machine |  |  |  |
|  | What is meant by Twist Drill? |  |  |  |
|  | What is meant by precision measuring tool? |  |  |  |
|  | Where scriber used in machine shop? |  |  |  |
|  | Define Thread Pitch Gauge. |  |  |  |
|  | Define Guide Ways. |  |  |  |
|  | Enlist 3 PPEs used in Mechanical workshop. |  |  |  |
| Feedback to the Candidate |  |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature __________________ |  |  |  |  |  |

Annex-A

Perform machining operations (Facing, Turning, Step Turning, Taper Turning, and Threading).

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.

Annex-B

Perform an operation as per drawing to shape a workpiece with a 6mm slot and 2 holes of M10.

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.