National Vocational Certificate Level-2, Machinist (06 Months)

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1.	Introduction	1

1.1 Date of Development	1

1.2 Date of Review and Validation	1

1.3 Entry Requirement	1

1.4 Minimum qualification for teachers	1

1.5 Medium of instruction	1

1.6 Duration of Course	1

1.7 Overall objectives of training program	1

2.	Scheme of Studies	3

3.	Overview of the Curriculum for Level-2 Machinist	4

4.	Detail of Modules.	6

0715MMT3401 Develop Basic Communication Skills	6

0715MMT3402  Maintain Safe Work Environment	11

0715MMT3403  Maintain Tools & Equipment	13

0715MMT3404  Carry Out Calculations and Prepare Estimates	16

0715MMT3405  Carryout General Maintenance	19

0715MMT3406  Carryout Bench Work	22

0715MMT3407 Perform Basic Grinding	29

0715MMT3408  Perform Drilling Machine Operations	32

0715MMT3409 Perform Basic Lathe Machine Operations	35

0715MMT3410 Perform Shaping Operations	44

0715MMT3411 Workshop Projects	47

5.	List of Personal Protective Equipment	51

6.  List of Tools, Equipment, Machines and Consumables	52

6.	Members of Curriculum Development Committee,	77

# Introduction

Machinist is a multidimensional trade person specialized in machine shop, machine and related cutting & measuring tools and equipment or the maintenance and repair of existing machinery. In order to meet the demand of machine shop, National Vocational and Technical Training Commission (NAVTTC) in collaboration with TVET Sector Support Program (TVET – SSP) has developed National Vocational Qualifications comprising generic, functional and technical competency standards for Machinist occupation. It encompasses a broad range of knowledge and skills related to machining processes, workshop techniques, quality control, safety procedures, and environmental sustainability. The demand for skilled professionals in this field is steadily growing as the industry seeks individuals who can contribute to innovation, process improvement and safe handling of equipment. It aims to establish a framework that defines the essential knowledge, skills, and attitudes required for individuals to perform effectively in this field. These standards will serve as a benchmark for training providers, employers, and learners, ensuring that the workforce is equipped with the competencies necessary to meet industry requirements and contribute to Pakistan's economic growth. Green skills incorporation in TVET aims to build capacity of skilled workforce, which is aware of environmental hazards and accordingly adopts more sustainable and environment friendly practices at the workplace.

| ## 1.1 Date of Development |  |
| --- | --- |
| ## 1.2 Date of Review and Validation |  |
| ## 1.3. Entry Requirement | At least Middle or Grade 8 |
| ## 1.4. Minimum qualification for teachers | - DAE in Mechanical Technology with one-year relevant experience, computer literate OR - Intermediate with two years certificate (G-II) in machinist with 06 years’ experience, computer literate - Must be able to communicate effectively. |
| ## 1.5. Medium of instruction | Urdu, local language, English |
| ## 1.6. Duration of Course | Total. 600 hours. Theory. 120 hours (20 %) Practical. 480 hours (80 %) |

  - Overall objectives of training program

The Machinist qualification of level 2 consists of 20 % Theory and 80 % Practical details and having the following Modules. Details are as follows:

- 0715MMT3401 Develop Basic Communication Skills

- 0715MMT3402 Maintain Safe Work Environment

- 0715MMT3403 Maintain Tools & Equipment

- 0715MMT3404 Carry Out Calculations and Prepare Estimates

- 0715MMT3405 Carryout General Maintenance

- 0715MMT3406 Carryout Bench Work

- 0715MMT3407 Perform Basic Grinding Operations

- 0715MMT3408 Perform Drilling Machine Operation

- 0715MMT3409 Perform Turning Operations

- 0715MMT3410 Perform Shaping Operations

- 0715MMT3411 Workshop Projects-I

# Scheme of Studies

| Module Code | Name of Competency | NVQF Level | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. | Total |  | Th. | Pr. | Total |
| 0715MMT3401 | Develop Basic Communication Skills | 2 | Generic | 2 | 3 | 5 | 0.5 | 0.13 | 0.19 | 0.31 |
| 0715MMT3402 | Maintain Safe Work Environment | 2 | Generic | 5 | 6 | 11 | 1.1 | 0.31 | 0.38 | 0.69 |
| 0715MMT3403 | Maintain Tools & Equipment | 2 | Technical | 9 | 12 | 21 | 2.1 | 0.56 | 0.75 | 1.31 |
| 0715MMT3404 | Carry Out Calculations and Prepare Estimates | 2 | Technical | 6 | 6 | 12 | 1.2 | 0.38 | 0.38 | 0.75 |
| 0715MMT3405 | Carryout General Maintenance | 2 | Technical | 6 | 6 | 12 | 1.2 | 0.38 | 0.38 | 0.75 |
| 0715MMT3406 | Carryout Bench Work | 2 | Technical | 20 | 63 | 83 | 8.3 | 1.25 | 3.94 | 5.19 |
| 0715MMT3407 | Perform Basic Grinding Operations | 2 | Technical | 12 | 60 | 72 | 7.2 | 0.75 | 3.75 | 4.50 |
| 0715MMT3408 | Perform Drilling Machine Operation | 2 | Technical | 10 | 18 | 28 | 2.8 | 0.63 | 1.13 | 1.75 |
| 0715MMT3409 | Perform Turning Operations | 2 | Technical | 25 | 150 | 175 | 17.5 | 1.56 | 9.38 | 10.94 |
| 0715MMT3410 | Perform Shaping Operations | 2 | Technical | 13 | 120 | 133 | 13.5 | 0.81 | 7.50 | 8.31 |
| 0715MMT3411 | Workshop Projects-I | 2 | Technical | 12 | 36 | 48 | 4.8 | 0.75 | 2.25 | 3.00 |
| Total |  |  |  | 120 | 480 | 600 | 60 | 7.5 | 30.0 | 37.5 |

# Overview of the Curriculum for Level-2 Machinist

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of modules |
| --- | --- | --- | --- | --- |
| 0715MMT3401 Develop Basic Communication Skills | - Adopt effective listening skills - Develop Nonverbal Communication Skills - Develop verbal communication Skills - Develop Written Communication Skills - Adopt Functional English - Develop Basic IT Skills | 2 | 3 | 5 |
| 0715MMT3402 Maintain Safe Work Environment | - Identify Hazards at Workplace - Observe Occupational Safety and Health (OSH) - Reduce Waste Material for Sustainable Machining. | 5 | 6 | 11 |
| 0715MMT3403 Maintain Tools & Equipment | - Maintain measuring tools - Maintain marking tools - Maintain general tools - Maintain different gauges - Maintain cutting tools | 9 | 12 | 21 |
| 0715MMT3404 Carry Out Calculations and Prepare Estimates | - Identify various measuring units and systems. - Identify Marking instruments - Perform estimation of materials. - Convert dimensions into weight. | 6 | 6 | 12 |
| 0715MMT3405 Carryout General Maintenance | - Perform General Housekeeping & Maintenance - Perform Preventive Maintenance - Perform Maintenance of Tooling | 6 | 6 | 12 |
| 0715MMT3406 Carryout Bench Work | - Carry-Out Sawing - File the Workpiece - Carry-Out Drilling - Produce Threads on the workpiece with Tap and Die - Perform Chipping & Scrapping - Perform Hand Polishing - Perform Hand Reaming | 20 | 63 | 83 |
| 0715MMT3407 Perform Basic Grinding Operations | - Perform off-hand grinding - Sharp single point cutting tool on grinding - Sharp Drill bit on grinding | 12 | 60 | 72 |
| 0715MMT3408 Perform Drilling Machine Operation | - Produce holes using drilling machine - Perform counter boring and counter sinking - Perform machine reaming | 10 | 18 | 28 |
| 0715MMT3409 Perform Basic Lathe Machine Operations | - Perform centering operations - Perform facing Operations - Perform turning operations - Perform drilling and boring operations - Perform step turning operations - Perform knurling Operations - Taper turning by tail stock off-set method - Taper turning by swiveling the compound rest. - Taper turning by telescopic taper turning attachment - Perform Internal and External threading Operations | 25 | 150 | 175 |
| 0715MMT3410 Perform Shaping Operations | - Prepare Materials for Shaping Operations - Select Tools and Equipment - Set Shaper Machine for Operations | 13 | 120 | 133 |
| 0715MMT3411 Workshop Projects-I | - Select The Relevant Project. - Identify The Right Material. - Select The Right Machining Process and Sequence. - Make The Project. - Ensure Dimensional Accuracy and Quality as Per Drawing or Standard. | 12 | 36 | 48 |
| TOTAL |  | 120 | 480 | 600 |

Detail of Modules.

## 0715MMT3401 Develop Basic Communication Skills

Objective: This module covers the knowledge and  skills required to identifies the competencies required to apply communications skills in accordance with the organization’s guidelines and procedures. You are expected to work in a team to achieve common organizational goals and avoid conflicts. This competency standard would also enable you to use basic computer skills to communicate effectively and prepare work related documents.

Duration: 45 Hours					Theory: 09 Hours								Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Adopt effective listening skills | Trainee will be able to: - Actively listens to others without interrupting and demonstrates patience and respect. - Ask questions to enhance understanding of the speaker's message. - Interprets information to confirm understanding. - Maintains eye contact and provide non-verbal cues (nodding, facial expressions) to show attention. - Limits distraction and focuses attention on the speaker during conversations or presentations. | - Effective listening skills - Principles of effective and interactive communication. - 7 C’s of communication - Procedure engages fully with the speaker - Avoiding Interruptions - After ending the class switch off the lights and fans. Practical Activity - Mock Interview between Trainees | Theory: 01Hrs. Practical: 03Hrs. Total: 04Hrs. | - Chairs - Table | - Class Room |
| Develop Nonverbal Skills Communication | Trainee will be able to: - Demonstrates appropriate body language, such as posture and gestures, to show confidence. - Adapts nonverbal cues to align with the machining environment's technical context and cultural norms. - Interprets and responds to nonverbal signals from colleagues and supervisors to enhance understanding and collaboration. | - Body Language - Practice and Self-Awareness - Be aware of cultural differences - After ending the class and workshop switch off the lights and fans. - After ending the sessions save the signs chart in cabinet. Practical Activity - Perform some safety signals. | Theory: 01Hrs. Practical: 03Hrs. Total: 04Hrs. | - Different Safety signs charts | - Class Room - Workshop |
| Develop verbal communication Skills | Trainee must be able to: - Speaks clearly and confidently when discussing machining tasks and procedures. - Uses technical terms and industry-specific language that are easy to understand for colleagues and supervisors. - Explain ideas and instructions in a logical and organized manner. - Listens actively and responds appropriately during conversations and meetings. - Engages in collaborative discussions to solve machining-related problems. | - verbal communication - Effective negotiation skills. - Conflict resolution strategies. - After ending the class switch off the lights and fans. Practical activity: - Demonstrate communication methods. | Theory: 01Hrs. Practical: 03Hrs. Total: 4Hrs. | - 4 size paper 01 No. for each trainee. - Dice 01 Nos. | - Class Room |
| Develop Written Communication Skills | Trainee must be able to: - Write clear and simple technical notes and reports related to machining tasks. - Uses understandable language and technical terms appropriate for machinists. - Includes diagrams or sketches to clarify written instructions or reports. | - Practice writing technical reports logs - Create and interpret technical drawings. - After ending the class switch off the lights and fans. - After ending the session set your chairs and tables Practical activity: - Write down communication methods. | Theory: 01 Hrs. Practical-3 Hrs. Total- 4 Hrs. | - A4 size paper 01 No for each trainee. | - Class Room |
| Adopt Functional English | Trainee must be able to: - Understands basic English used in everyday work situations. - Communicates clearly and effectively in English with colleagues and supervisors. | - Grasp of industry-specific terms and phrases. - Terminology related to machinery, processes, materials, and safety protocols. - After ending the class switch off the lights and fans. - After ending the session set your chairs and tables Practical activity: - Group discussion about communication resources | Theory: 01Hrs. Practical: 03Hrs. Total: 04Hrs. | - Chairs - Tables | - Class Room |
| Develop Basic IT Skills | - Trainee must be able to: Create folders and files and learn major commands of operating system/windows. - Use the internet for sending/receiving emails and connecting through social or other media. - Interpret data in Microsoft Word and Excel. | - Functional English - Basic IT Skill - Input / output devices of computer - Basic computer skills - MS Word, MS Excel - use of internet - sending and receiving emails - Procedure a preparing of documents and work-related reports. - After use the computer shutdown the PC and make sure switch off. - After ending the labs switch off the lights and fans. Practical activity - Write an application/letter in MS Word. - Prepare annual result sheet in MS Excel | Theory: 04Hrs. Practical: 02Hrs. Total: 25Hrs. | - A4 size paper 02 No for each trainee. - 25 computers - Multimedia - Printer | - Lab with 25 capacities of computers |

## 0715MMT3402 Maintain Safe Work Environment

Objective: This module covers the knowledge and  skills required to Identify Occupational safety and health at workplace in accordance with the organization’s approved guidelines and procedures. You will be expected to identify and use Personnel Protective Equipment (PPE) according to the job requirement and potential hazards at workplace and also reduce waste material for sustainable machining in good manners.

Duration: 30	Hours							Theory: 06 Hours					Practice: 24	Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify Hazards at Workplace | Trainee will be able to: - Interpret hazard symbols correctly. - Identify risks and potential hazards at the workplace. - Identify actions to minimize the risk. - Adopt health and safety precautions in the workplace. | - Familiarization with Symbols - Risk Assessment - Hazard Categories - Reporting Systems - Control Measures - Risk Hierarchy - Emergency Procedures - Safety Protocols - Personal Protective Equipment (PPE) - Procedure for storage of PPEs Practical activity - Inspect the entire workshop and identify hazards. | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - Safety Helmet - Safety Goggle - Note Book | - Workshop |
| Observe Occupational Safety and Health (OSH) | Trainee will be able to: - Identify the types of hazards that are most likely to cause harm to health and safety. - Interpret Health and safety precautions and regulations. - Identify emergency procedures to deal with any accident or injury. - Follow properly Personal Protective Equipment. - Follow safe methods of handling heavy loads. - Deal with problems which are within your control, and report those that cannot be resolved to concerned officer. | - Personal Protective Equipment. - Check list avoid accident - CPR cardiopulmonary resuscitation - artificial breath restoration - Fire and its types - Fire extinguishers and types of fire extinguishers - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity - Preparing accidental report in workshop - Restore the breath (CPR and mouth to mouth) - Lift the weight using Correct method - Perform firefighting exercise. | Theory: 02Hrs. Practical: 12Hrs. Total: 14Hrs. | - A4 size paper 01 No for each trainee - Specific weight 20 kg - Fire extinguishers - PPEs | - Workshop/ open area |
| Reduce Waste Material for Sustainable Machining. | Trainee must be able to: - Identify opportunities to minimize scrap and material waste during machining operations. - Follow procedures to reuse or recycle materials wherever possible. | - Waste reduction and environmental sustainability - Machining Processes - Material Selection - Tooling and Equipment Optimization - Process Optimization - Design - Manufacturability - Material Segregation, Recycling Procedures, Inventory Management and employ training Practical activity: - Identify and segregate the waste material. - Enlist the methods of recycling the waste material. | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - Dustbins - Scrape materials | - Workshop |

0715MMT3403 Maintain Tools & Equipment

Objective: This module covers the knowledge and  skills required to ensure the effective, efficient, and safe operation of tools and equipment by preserving their accuracy, functionality, and longevity.

Duration:  15 Hours						Theory: 03 Hours						Practice: 12 	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Maintain measuring tools | Trainee will be able to: - Identify the specific measuring tools used in machining operations. - Perform inspections to identify wear or damage. - Calibrate measuring tools according to established procedures and standards. - Clean measuring tools thoroughly after each use to prevent wear. - Store measuring tools in designated locations to prevent damage or misplacement. | - Types of measuring tools - Tool Functions - applications of tool - Tool Characteristics Study - Wear and Damage Indicators - Inspection Procedures - Documentation wear or damage - Calibration Procedures - Standards and Specifications - Calibration Records. - Cleaning Techniques - Preventive Maintenance - Tool Handling - Storage Solutions - Tool Organization - Environmental Conditions Study Practical activity: - Perform Tool Inspection Exercise. | Theory: 01Hrs. Practical: 03Hrs. Total: 04Hrs. | - Measuring tools | - Workshop |
| Maintain marking tools | Trainee must be able to: - Identify the marking tools essential for machining tasks. - Inspect marking tools and detect any signs of wear. - Clean marking tools thoroughly to ensure clear and precise marking. - Store marking tools in designated areas to prevent damage or loss. | - Types of marking tools - Inspection of marking tools. - Cleaning procedure. - Importance of proper storage Practical activity: - Check exercise of Marking Tool Maintenance | Theory: 0.5Hrs Practical: 03Hrs Total: 3.5Hrs | - Marking Tool | - Workshop |
| Maintain general tools | Trainee must be able to: - Inspect each tool for wear, functionality, and safety before use. - Clean tools thoroughly after each use to maintain performance. - Store tools in designated, organized locations for easy access and to prevent damage. | - Comprehensive inspection of tools - Proper cleaning techniques - Principles of proper tool storage. Practical activity: - Perform Tool Maintenance Inspection exercise | Theory: 0.5Hrs. Practical: 1.5Hrs. Total: 02 Hrs. | - Measuring tools - Marking tools - Cutting tools | - Class Room - Workshop |
| Maintain different gauges | Trainee must be able to - Identify and categorize various types of gauges used in machining. - Inspect gauges regularly to ensure accurate measurement capabilities. - Clean gauges meticulously to remove debris and ensure precision. - Store gauges in designated areas to prevent contamination and damage. | - Types of gauges - Inspections of gauges - Cleaning procedures - Importance of proper storage Practical activity: - Perform Gauge Accuracy Inspection exercise | Theory: 0.5Hrs. Practical: 1.5Hrs. Total: 02Hrs. | - Different Gauge assorted sizes | - Workshop |
| Maintain cutting tools | Trainee must be able to: - Identify and classify different types of cutting tools essential for machining. - Inspect cutting tools systematically to detect wear, damage, or dullness. - Clean cutting tools thoroughly to remove chips, debris, and contaminants. - Store cutting tools in specified locations to prevent damage and ensure accessibility. | - Types of cutting tools - Inspections of cutting tools - Cleaning techniques for different cutting tools - Principles of proper storage Practical activity: - Perform cutting Tool Sharpening Exercise. | Theory: 0.5Hrs. Practical: 03Hrs. Total: 3.5Hrs. | - Cutting Tool | - Workshop |

0715MMT3404 Carry out Calculations and Prepare Estimates

Objective: This module covers the knowledge and  skills required to identifies the competencies required to prepare estimates in accordance with client’s guidelines. You will be expected to estimate, ensuring cost effectiveness, conforming to standards and regulations. The underpinning knowledge regarding calculations and estimation will be sufficient to provide the basis for your work.

Duration: 15 Hours							Theory: 3 Hours							Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify various measuring units and systems. | Trainee will be able to: - Recognizing and listing different types of measuring units such as metric, imperial, and specialized units. - Categorizing measuring systems including linear, angular, and volumetric systems. - Explaining the principles and applications of each measuring unit and system in industrial settings. - Distinguishing between primary and secondary measuring instruments used for accurate measurements. - Applying measurement unit conversions between different systems as required for machining tasks. | - Understanding units - Familiarity with units - Knowledge of unit’s microns, angstroms, and others - Measuring length or distance - Measuring angles - Measuring volume - Primary Instruments - Secondary Instruments - Conversions between metric and imperial units - Practical Application Practical activity: - Interpret measuring systems. | Theory: 0.5Hrs. Practical: 03 Hrs. Total: 3.5Hrs. | Measuring tools | - Class Room - Workshop |

| Identify Marking instruments | Trainee will be able to: - Select appropriate measuring instruments for precise measurements. - Calibrate measuring instruments for accuracy. - Operate micrometers, calipers, and depth gauges proficiently. - Apply scribers and layout tools accurately on work pieces. - Ensure accurate measurement and marking on materials. - Demonstrate effective use of instruments in practical applications. | - Instrument Types - Criteria for Selection - Calibration Procedures - Checking Accuracy - Micrometers, calipers and depth gauge - Scribers and Layout Tools - Verification Techniques - Error Checking - Hands-on Practice - Problem-Solving Practical activity: - Perform Marking Instruments Identification Exercise | Theory: 0.5Hrs. Practical: 03 Hrs. Total: 3.5Hrs. | Different types of tools | - Class Room - Workshop |
| --- | --- | --- | --- | --- | --- |
| Perform estimation of materials. | Trainee must be able to: - Identify requirements of the material(s) to be utilized for the work in accordance with the job specifications and drawing. - Quantify the material as per drawing. - Use appropriate methods of calculation of cost of material and accessories keeping in view the current market prices. | - Understanding Job Specifications - Material Properties - Reading Technical Drawings - Measurement Techniques - Estimation Methods - Pricing Information - Cost Calculation Methods - Budgeting - Developing skills to perform accurate and efficient material estimations to minimize waste and cost. Practical activity: - Perform Material Estimation Activity | Theory: 01Hrs Practical: 03Hrs Total: 04Hrs | A4 size paper 02 Nos for each trainee | - Class Room - Workshop |
| Convert dimensions into weight. | Trainee must be able to: - Identify dimensions from technical drawings or specifications. - Calculate weight using appropriate conversion factors or formulas. - Convert linear dimensions (length, width, height) into volume. - Verify weight conversions through practical exercises and real-world scenarios. | - Reading Technical Drawings - Interpreting Specifications - Conversion Factors and Formulas - Volume Calculation - Weight Calculation - Volume formulas and units - Hands-on Activities - Real-world Scenarios - Error Checking - Practical Adjustments Practical activity: Perform Weight Calculation of a mild steel block | Theory: 01Hrs. Practical: 03Hrs. Total: 04Hrs. | Mild steel block | - Class Room - Workshop |

0715MMT3405 Carryout General Maintenance

Objective: This module covers the knowledge and  skills required to identified the competencies required to perform maintenance functions by a machinist in accordance with the organization’s approved guidelines and procedures. You will be expected to perform preventive maintenance of machines and tools as well as general housekeeping and maintenance of tools and machines. Your underpinning knowledge will be sufficient to provide you the basis for your work.

Duration:  30 Hours							Theory: 6 Hours						Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform General Housekeeping & Maintenance | Trainee will be able to: - Apply appropriate methods and techniques for cleanliness and maintenance of machines and tools. - Clean and maintain all workplace tools and machines as per housekeeping checklists or instructions given. - Prepare a checklist for daily cleanliness of the workplace. - Respond appropriately to safety hazards on all bench-work tools and machines. - Place all the tools and material in proper place to ensure safe work. - Prepare Specific guidelines and checklists to conduct maintenance and housekeeping of machines and tools. | - Guidelines and checklists to conduct maintenance and housekeeping - Procedure of cleanliness of machine and workplace. - Check list of oiling and lubrication. - Procedure storing tools and material - Guidelines and checklists to conduct maintenance - Housekeeping of machines and tools - Maintain history record - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity: - Perform general housekeeping of machine tools and accessories as per given instructions. - Perform general maintenance of lathe and milling machine. | Theory: 02Hrs. Practical: 09Hrs. Total: 11Hrs. | - Machinery maintenance tool kit - Cotton rags - Lubricating Oil and Oil Gun - Air Compressor - Steel Amirah and Tooling Cabinet - Maintenance trolley - PPEs - Kerosene Oil - Notice Register and Stationary - Grease and Grease gun - WD-40 | - Class Room - Workshop |
| Perform Preventive Maintenance | Trainee will be able to: - Read and interpret maintenance schedule carefully. - Prepare oiling and greasing chart (daily, weekly as per machine requirement). - Prepare machine history record covering date of installation, condition, oiling and maintenance. - Inspect and assess the general condition of an assigned machine on regular basis. - Observe problems and carry out routine maintenance as per given instructions and schedules. - Identify faulty/damaged/ worn-out parts and take appropriate steps to replace them. | - Personal protective equipment (PPEs) and workplace safety - Machine operations - Read machine manuals - scope of preventive maintenance - Preparation of check list - Identify faulty/damaged - Maintaining history record - Writing report to authority. - Observation maintenance problems. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity: - Interpret daily checking of assigned machines on regular basis. - Prepare check list for preventive maintenance of assigned machines. | Theory: 02Hrs Practical: 09Hrs. Total: 11Hrs. | - PPEs - A4 size paper 02 Nos for each trainee | - Class Room - Workshop |
| Perform Maintenance of Tooling | Trainee must be able to: Clean and maintain all bench-work tools and machines as per housekeeping checklists or instructions provided. - Prepare check list for daily cleanliness of the workplace. - Respond appropriately to safety hazards on all bench-work tools and machines. - Identify all the tools and material in proper place to ensure safe work. - Adopt methods and techniques for cleanliness and maintenance of tools. | - Guidelines and checklists - Procedure of storing the tools and materials - Techniques of cleanliness and maintenance of tools. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity: - Store all cutting tools to maintain their sharpness as per given instructions. - Reshape and clean the cutting tools as per prescribed method. | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - Grease and Grease Gun - WD-40 Spray - Thinner/ - Kerosene oil and lubricating oil - Machinery Installation and Maintenance Manual - Cotton Rags - Machinery Maintenance Tool kit - File brush - Drill angle gauge and tool angle gauge - Tool grinder - PPEs | - Class Room - Workshop |

## 0715MMT3406 Carryout Bench Work

Objective: This module covers the knowledge and  skills required to module identifies the competencies you need to perform bench work operations using different tools and equipment, in accordance with approved procedures. You will be expected to perform sawing, filing, threading and reaming using hand tools. You will be required to operate the tools and equipment safely by complying the organizational safety policy and approved procedures. Your underpinning knowledge will be sufficient to provide you the basis for your work.

Duration: 105	Hours						Theory: 21 Hours						Practice: 84 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Carry-Out Sawing | Trainee will be able to: - Select the appropriate blade and set it in a hacksaw frame according to the job requirement. - Select the appropriate marking tool(s), mark the work piece, and clamp it in the vice firmly as per standard practices. - Adopt sawing methods and techniques that are safe and suitable to produce the work piece as needed. - Follow the marked line during the sawing of the work piece to ensure accuracy. | - Personal protective equipment (PPEs) and workplace safety - Mechanical Properties and strength of materials - Interpret basic drawings - Method and use of Measurement systems - Use of marking tools - Types of Hacksaw blade and Hacksaw frame. - Usage of bench working tools - Procedure of sawing operation - Importance of clamping work piece - Procedure of safe clamping - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity: - Mark the flat work piece for three sawing and cut with Hand hacksaw as per drawing. (adopt safety method) | Theory: 03Hrs. Practical: 09Hrs. Total: 12Hrs. | - Hand hacksaws with blades, complete sets (assorted range) with standard accessories - Bench vices - Work bench and stools - Measuring and marking tools - PPEs - Mild Steel Flat | - Workshop |
| File the Work piece | Trainee will be able to: - Select appropriate file and marking tool(s) according to the job requirement. - Mark the work piece and clamp it in the vice firmly as per standard practices. - Adopt filing method and technique which is safe and suitable to produce the work piece as needed. - Follow drawing dimensions and surface finish of filed work pieces to ensure accuracy and precision. | - Safety mechanisms and protocols - Files and its types. - Method of marking - Procedure of safe clamping - Usage of bench work tools and equipment - Procedure of Filling operation - Needle file set - use of measuring tools - Safety precaution. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical activity: - Clamp work piece as required. - File the rectangle work piece with flat file (rough and smooth for finishing) as per given sizes. (adopt safety method) | Theory: 06Hrs. Practical: 30Hrs. Total: 36Hrs. | - Bench vices - Work bench and stools - Measuring and marking tools - All kind of Files (assorted range) - Needle file set - PPEs - Mild Steel Flat | - Workshop |
| - Carry-Out Drilling | Trainee must be able to: - Set up drill machine for producing holes in the work piece according to the job requirement. - Select drill bit and marking tools according to the material and job requirement, respectively. - Mark the work piece according to the drawing and clamp it in the vice firmly as per standard practices. - Adopt the proper drilling method (manual/auto-feed, applying coolants) which is safe and suitable to produce the hole in the work piece. - Follow drawing dimensions of drilled hole to ensure accuracy and precision. | - Safety mechanisms and protocols - Drilling and types of drill machines. - Drill machines functions. - Calculate cutting speed, feed and RPM on drilling machine - Marking and layout - Procedure clamping the work piece - Procedure of drilling operation - Usage of measuring tools - Types of Drills. - Nomenclature of drill bits - Drilling operations - Types and functions of coolants - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical Activity: - Mark and Centre punch the filled rectangle work piece and drill for internal thread and reaming. (adopt safety method) - Perform counter-boring and counter-sinking operation on the work piece according to the drawing | Theory: 03Hrs. Practical: 02Hrs. Total: 15Hrs. | - Machine vices - Pillar drilling machines - Drill chuck with Key - Drill bits (assorted range) - Center drills (assorted range) - Measuring and marking tools - PPEs - Mild Steel Flat | - Workshop |
| Produce Threads on the work piece with Tap and Die | Trainee must be able to: - Select tap and die according to the type of thread to be produced on work piece as per drawing. - Clamp the work piece in the clamping device firmly as per standard practices. - Ensure the tap and die alignment as per prescribed standard. - Make thread with die and follow the appropriate sequence in case of using taps. - Ensure the safety and dimensional accuracy of threads on the work piece as per drawing. | - Safety mechanisms and protocols - Types of threads. - Techniques for Threading. - Types of taps & dies - Calculation of drill size for internal threading - Procedure of tap and die operation - Tap and die alignment. - lubricants - Procedure of inspection - Usage of measuring tools - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments). Practical Activity: - Cut Internal threading on drilled hole work piece as per drawing. (adopt safety method) - Cut external thread on shaft as per drawing. (adopt safety method) | Theory: 03Hrs. Practical: 09Hrs. Total: 12Hrs. | - Machine vices and bench vice - Pillar drilling machines - Drill chuck with Key - Drill bits (assorted range) - Measuring and marking tools - Dies and Taps Set with handle (assorted range) - Thread Plug gauge and thread Ring gauge (assorted range) - State cutting oil - PPEs - Mild Steel Flat - Bright bar round | - Workshop |
| Perform Chipping & Scrapping | Trainee must be able to: - Identify and select appropriate chipping and scraping tools for the task. - Safely and effectively remove unwanted material from work pieces using chipping techniques. - Skillfully scrape surfaces to achieve smooth finishes and precise dimensions. - Inspect work pieces for remaining imperfections and ensure cleanliness. - Dispose of waste materials responsibly and maintain work area cleanliness. | - Safety mechanisms and protocols - Method of marking - Procedure of clamping - Usage of bench work tools - Procedure of hand chipping operation - Types scrapping operation - Procedure of hand scraping operation - Safety precaution. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments) Practical Activity: - Perform flat scrapping operation on rectangular work piece as per given sizes. | Theory: 02Hrs. Practical: 02Hrs. Total: 14Hrs. | - Measuring and marking tools - All kind of scrappers (assorted range) - PPEs - Mild Steel Flat | - Workshop |
| Perform Hand Polishing | Trainee must be able to: - Select appropriate polishing tools and abrasives based on material and finish requirements. - Prepare work pieces by cleaning and smoothing surfaces before polishing. - Apply polishing compounds evenly and consistently to achieve the desired surface finish. - Execute polishing techniques such as circular motions and varying pressures to achieve uniformity. | - Safety mechanisms and protocols - Method of marking - Procedure of clamping - Usage of bench work tools - Procedure of hand polishing operation - Usage of different filling operation - Usage of appropriate emery papers - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments) Practical Activity: - Perform Hand Polishing on rectangular work piece as per given sizes. | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - Bench vices - Work bench and stools - Measuring and marking tools - All kind of Files (assorted range) - Needle file set - Emery papers different numbers - PPEs - Mild Steel Flat | - Workshop |
| Perform Hand Reaming | Trainee must be able to: - Select a reamer according to the work piece requirement. - Clamp the work piece in the clamping device firmly as per standard practices. - Ensure the reamer alignment as prescribed standard. - Produce reamed hole by following safety and prescribed methods. | - Safety mechanisms and protocols - Usage Hand Reaming. - Knowledge of different kind of reamers - Calculation of drill size for reaming - Method of marking - Procedure of clamping the work piece - Procedure of hand reaming operation - Ensure reamer alignment. - Usage of lubricants - Measuring tools - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, storage of tools and instruments) Practical Activity: - Reamed on drilled hole work piece as per drawing. (adopt safety method) | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - Machine vices - Pillar drilling machines - Drill chuck with Key - Drill bits (assorted range) - Center drills (assorted range) - Measuring and marking tools - Reamers (assorted range) - Plug gauge (assorted range) - PPEs - Mild Steel Flat | - Workshop |

## 0715MMT3407 Perform Basic Grinding

Objective: This module covers the knowledge and  skills required to identifies the competencies you need to perform grinding machine operations in accordance with approved procedures. You will be expected to perform different types of grinding which include off-hand, tools and drill bit grinding You will be required to operate the grinding machine safely by complying the organizational safety policy and approved procedures. Your underpinning knowledge will be sufficient to provide you the basis for your work.

Duration: 60 Hours						Theory: 12 Hours						Practice: 48 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform off-hand grinding | Trainee will be able to: - Select the proper size and shape of grinding wheel. - Hold the work piece firmly against the rotating wheel by placing it on the tool rest. - Provide coolant at intervals to avoid over heating of the job. - Follow safety measure - Produce component according to work operations. - Observe personal and workplace safety. | - Pedestal grinding machine and its parts. - Safety mechanisms and protocols - Measuring with protractor or gauges - Pedestal grinding machine - Function of grinding machine. - Procedure of wheel dressing - Importance of coolant. - Setting of tool rest. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform wheel dressing and tool rest adjustment. | Theory: 03Hrs. Practical: 09Hrs. Total:12Hrs. | - Pedestal grinding machine along with standard accessories - drill grinding attachments - Diamond dressing - Grinding wheel balancing stand with standard accessories - PPEs - Maintenance tool kit, general repairing tool kit and hands tool kit | - Work shop |
| Sharp single point cutting tool on grinding | Trainee will be able to: - Select the proper size and shape of grinding wheel. - Hold the work piece firmly against the rotating wheel by placing it on the tool rest. - Provide coolant at intervals to avoid over heating of the job. - Follow safety measure - Sharp the tool according to work requirements. - Observe personal and workplace safety. | - Safety mechanisms and protocols - Types of cutting tools and their angles - Procedure of single point cutting tool grinding. - Measuring with protractor or gauges - Importance of coolant. - Standing position on grinding machine - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform tool grinding on pedestal grinding machine - Right hand roughing tool, - Right hand side tool, - Centre punch, - V-shape tools for threading, - Scriber, - Flat chisel, - Round nose tool. | Theory: 06Hrs. Practical: 30Hrs. Total: 36Hrs. | - Pedestal grinding machine along with standard accessories - drill grinding attachments - Diamond dressing tip with stand - Measuring and marking tools - Bevel protractor - Tool angle gauge - PPEs - Mild steel square bar - Mild steel round bar - Mild steel flat | - Workshop |
| Sharp Drill bit on grinding | Trainee must be able to: - Select the proper size and shape of grinding wheel. - Hold the work piece firmly against the rotating wheel by placing it on the tool rest. - Provide coolant at intervals to avoid over heating of the job. - Follow safety measure - Sharp the Drill bet according to work requirements. - Observe personal and workplace safety. | - Safety mechanisms and protocols - Procedure of drill bit grinding. - Measurement systems with drill angle gauges. - Importance of coolant. - Identify the standing position during grinding. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform drill grinding on pedestal grinding machine | Theory: 03Hrs. Practical: 09Hrs. Total: 12Hrs. | - Measuring and marking tools - drill angle gauge - PPEs - Different sizes of HSS Drills | - Workshop |

## 0715MMT3408 Perform Drilling Machine Operations

Objective: This module covers the knowledge and  skills required to identifies the competencies you need to perform drill machine operations using different tools and equipment, in accordance with approved procedures. You will be performing drilling, counter sinking, counter boring and reaming using machine tools. You will be required to operate the tools and equipment safely by complying the organizational safety policy and approved procedures. Your underpinning knowledge will be sufficient to provide you the basis for your work.

Duration:  15 Hours						Theory: 3 Hours							Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Produce hole using drill machine | Trainee will be able to: - Adopt personal and work place safety. - Set up drilling machine for producing holes according to job requirement. - Manipulate the machine tool controls safely and correctly in line with operational procedures. - Produce components to the required quality and within the specified dimensional accuracy. - Carry out quality sampling checks at suitable intervals. - Shut down the equipment to a safe condition on conclusion of the machining activities. | - Drill machine and its types. - Safety mechanisms and protocols - Clamping procedure of work piece - Calculate cutting speed, feed and RPM drilling machine - Procedure of drilling operation - Function of coolant - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform drilling operation on the work piece according to the drawing | Theory: 01Hrs. Practical: 06Hrs. Total: 07Hrs. | - Machine vices - Pillar drilling machines - Drill chuck with Key - Drill bits (assorted range) - Center drills (assorted range) - Measuring and marking tools - Mild steel flat | - Workshop |
| Perform counter boring and counter sinking | Trainee will be able to: - Select relevant tools according to the information given in engineering drawings and job specifications. - Ensure tooling is correct in terms of size, shape, type, and grade for the work. - Position the work-piece in the drill in such a way that it is aligned, secured and stable during drilling. - Adjust speeds and feeds of drill in accordance with the size, type, and hardness of work-piece material, so that the drill performs optimum cutting without damage to work-piece. | - Safety mechanisms and protocols - Clamping procedure of work piece - Calculate cutting speed, feed and RPM - Usage of measuring tools. Procedure of Counter-boring operation - Procedure of Counter-sinking operation - functions of coolants - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform counter-boring and counter-sinking operation on the work piece according to the drawing | Theory: 01Hrs Practical: 03Hrs Total: 04Hrs | - Machine vices - Pillar drilling machines - Drill chuck with Key - Counter sink (assorted range) - Counter bore (assorted range) - Measuring and marking tools - Mild steel flat | - Work shop |
| Perform machine reaming | Trainee must be able to: - Adopt personal and work place safety. - Clamp work-piece in the vice properly. - Select reamer according to hole size and drawing requirements - Set reamer in the drill chuck according to procedure. - Provide lubricants during reaming for smooth cutting. - Ensure proper alignment of the reamer during operations. | - Safety mechanisms and protocols - Clamping procedure of work piece - Safe techniques for machine Reaming. - Knowledge of different kind of reamer - Calculation of drill size for reaming. - Procedure for reaming operation - Reamer alignment. - Use of lubricants - Usage of measuring tools - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform machine reaming operation on drilled hole work piece as per drawing. | Theory: 01Hrs Practical: 03Hrs Total: 04Hrs | - Machine vices - Pillar drilling machines - Drill chuck with Key - Measuring and marking tools - Reamers (assorted range) - Plug gauge (assorted range) | - Workshop |

0715MMT3409 Perform Basic Lathe Machine Operations

Objective: This module covers the knowledge and  skills required to identified the competencies required to perform lathe machine operations by a machinist in accordance with the organization’s approved guidelines and procedures. You will be expected to perform facing, turning drilling/boring, taper turning, knurling and threading operations using lathe machine. Your underpinning knowledge will be sufficient to provide you the basis for your work.

Duration: 150Hours						Theory: 30 Hours					Practice: 120 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform centering operations | Trainee will be able to: - Select facing tools according to job requirement. - Mount and set the required work-holding devices, work piece and cutting tools. - Follow instructions correctly - Select safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machine tool for centering the job. - Ensure all safety mechanisms are in place. | - Lathe machines and its parts - Types of lathe machines - Lathe machine accessories - Safety mechanisms and protocols - Clamping procedure of work piece - Machine tool settings speed feed rate - Procedure of centering operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform centring operation for facing as per drawing | Theory: 03Hrs Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - Center drill | - Class Room - Workshop |
| Perform facing Operations | Trainee will be able to: - Select facing tools according to job requirement. - Mount and set the required work-holding devices, work piece and cutting tools. - Follow the correct specifications for the part or component to be produced. - Select safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machine tool to achieve the work specification. - Ensure all safety mechanisms are in place. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of facing operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform facing operation as per given drawing | Theory: 3Hrs Practical: 12Hrs Total: 15Hrs | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS | - Workshop |
| Perform turning operation | Trainee must be able to: Obtain and follow work specifications, drawings or sketches to accomplish the work. - Set up and adjust the machine as per work specifications and procedures. - Ensure the components produced have the required quality and within the specified dimensional accuracy. - Shut down the machine and equipment on conclusion of the machining activities. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of turning operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: Perform turning operation as per prescribed drawing | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS | - Workshop |
| Perform drilling and boring operations | Trainee must be able to: - Select drill or boring tools according to drawings. - Mount and set the required work-holding devices, work piece and cutting tools. - Adjust the RPM of machine according to the cutting speed. - Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Observe personal and workplace safety. | - Safety mechanisms and protocols - Clamping procedure of work piece - Machine tool settings speed feed rate - Procedure of drilling operation - Procedure of drilling operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform drilling and boring operation as per drawing | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Twist drill bits - Boring tools - Center drills | - Workshop |
| Perform step turning operations | Trainee must be able to: - Mount and set the required work-holding devices, work piece and cutting tools. - Select and adjust appropriate speeds and feeds of turning machine. - Produce a component which matches the work specifications using appropriate methods and techniques. - Check quality of the component produced at different intervals. - Follow safety precautions to ensure safe work and to avoid any injury. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of step turning operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform step turning operation as per given drawing | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS | - Workshop |
| Perform knurling operations | Trainee must be able to: - Select the knurling tool according to drawing. - Set the tool and work piece in the machine according to procedure. - Adapt methods and techniques to produce proper knurling on work piece. - Select and adjust appropriate speeds and feeds of lathe machine. - Provide coolants during knurling to achieve smooth impression on work piece. - Observe personal and workplace safety. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of knurling operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform knurling operation as per given drawing. | Theory: 03Hrs. Practical:12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - Knurling tool with holder | - Workshop |
| Taper turning by tail stock off-set method | Trainee must be able to: - Loosen tailstock clamp out. - Offset tailstock required amount. - Centre cutting tool. - Setup cutting tool for parallel turning. - Starting at small diameter take excessive cuts until the taper is .05 to .06 in oversize. - Check taper for accuracy using a taper ring gauge. - Finish turns the taper to size and fit required. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of tail stock off set operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform taper turning operation by tail stock as per prescribed drawing. | Theory: 03Hrs. Practical:12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - Revolving / dead center - Dog carrier | - Workshop |
| Taper turning by swiveling the compound rest. | Trainee must be able to: - Remove the binding screw that cross slide to cross feed screw nut. - Tighten the lock screw and set cutting tool on center. - Set work piece in lathe and mark length of taper. - Use binding screw to connect sliding block to side of taper attachment. - Select depth of feed cut by compound rest feed handle. - Take a light cut and recheck taper fit. - Finish turn and fit the taper to gauge. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of taper turning compound slide operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform taper turning operation by swivel compound slide as per prescribed drawing. | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - Right hand side internal boring bar HSS | - Workshop |
| Taper turning by telescopic taper turning attachment | Trainee must be able to: - Clean and oil the guide bar. - Loose lock screws and offset end of guide bar, - Set the bar to required taper in degrees. - Tighten the lock screw and set cutting tool on center. - Set work piece in lathe and mark length of taper and tighten connecting screw on sliding block. - Move carriage until center of attachment is opposite to length of taper. - Lock anchor bracket to lathe bed. - Take a cut and select depth of cut. - Readjust the taper attachment, take a light cut and recheck taper fit. - Finish turn and fit the taper to gauge. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of taper turning attachment operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform taper turning operation by taper turning attachment as per prescribed drawing. | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - Taper turning attachment | - Workshop |
| Perform external and internal threading operation | Trainee must be able to: - Mount and set the required work-holding devices, work piece and cutting tools. - Select and adjust appropriate speeds and feeds of turning machine. - Produce a component which matches the work specifications using appropriate methods and techniques. - Check quality of the component produced at different intervals. - Adopt Proper cutting tool with required dimensions. - Follow safety precautions to ensure safe work and to avoid any injury. | - Safety mechanisms and protocols - Clamping procedure of workpiece - Machine tool settings speed feed rate - Procedure of external threading operation - Procedure of internal threading operation - Describe procedure for inspection of work piece - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Perform V-shape metric internal and external threading operation as per prescribed drawing. - Perform V-shape imperial (BSW) internal and external threading operation as per prescribed drawing. | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Lathe machine along with standard accessories - Measuring and marking tools - PPEs - Mild steel round bar - Right hand side tool HSS - External threading tools - internal threading tools | - Workshop |

## 0715MMT3410 Perform Shaping Operations

Objective: This module covers the knowledge and  skills required to identifies the competencies you need to perform shaping operations on shaper machine in accordance with approved procedures. You will be expected to perform Facing, Step cutting, Squaring, slotting, V shape cutting with point cutting tool. You will be required to operate the shaper machine safely by complying the organizational safety policy and approved procedures. Your underpinning knowledge will be sufficient to provide you the basis for your work

Duration:  90 Hours					Theory: 18 Hours							Practice: 72 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Materials for Shaping Operations | Trainee will be able to: - Interpret drawing and arrange the material accordingly. - Prepare the work piece by required machining (sawing, filing, etc.) and get ready to clamp. - Check and verify the dimensions of work piece for shaping according to the drawing. | - shaping machine and its parts - functions of shaper machine - Interpret basic drawings - Procedure of cutting required material on power hacksaw. - Measurement systems - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Cut the required material on power hacksaw according to the drawing. - Check the dimensions of cut material and deburr it. | Theory: 03Hrs. Practical: 15Hrs. Total: 18Hrs. | - Power hacksaw - Common kinds and sizes of files (assorted range) Measuring and marking tools - Files - PPEs - Mild steel square bar | - Work shop |
| Select Tools and Equipment | Trainee will be able to: - Select the material and shape of the tool(s) according to the job requirements. - Arrange the measuring instruments and holding devices to attain the accuracy of the work as per the prescribed method. | - Types of shaping tools. - Material of shaping tools. - Clamping of Shaping tools - Work holding device. - Measuring tools and angle checking gauge. - Housekeeping of tools and equipment. - Procedure for storage of PPEs, Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: Select the required shaping tool as per given drawing. | Theory: 03Hrs. Practical: 12Hrs. Total: 15Hrs. | - Assorted range - HSS shaping tools - Tungsten carbide Brazed tipped tool - Measuring and marking tools - PPEs | - Class Room - Workshop |
| Set Shaper Machine for Operations | Trainee must be able to: - Clamp the material of work piece and tool into its holding devices as per standard practice. - Maintain a safe distance between the surface of the work piece and the tooltip as per the prescribed method. - Adjust the ram placement and stroke length according to the length of job. - Adjust the parameters of shaping (speed and feed) from control unit as per prescribed method. - Start shaping operation by locating the initial touching point and adjust the depth of cut according to the prescribed procedure. | - Ram and stroke settings. - Calculate Feed and stock per minute - clamping of shaping tool - shaping tool operation - Mounting and dialing of machine vice. - clamping of work piece - Shaping procedure. - Perform Deburring and check size. - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Prepare a square block as prescribed drawing. - Prepare a hexagon as prescribed drawing. - Prepare a V-block as prescribed drawing. | Theory: 12Hrs. Practical: 45Hrs. Total: 57Hrs. | - assorted range - HSS shaping tools - Measuring and marking tools - PPEs - Dial indicator with magnet stand - lubrication oil - Spanners and hammers - Mild steel flat - Mild steel square block | - Class Room - Workshop |

0715MMT3411 Workshop Projects

Objective: This module covers the knowledge and  skills required to workshop project related to a machine shop typically revolves around applying theoretical knowledge to practical tasks, improving technical skills, and achieving specific project goals. Here’s a structured breakdown of common objectives for such projects.

Duration: 45 Hours							Theory: 9 Hours					Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Select The Relevant Project. | Trainee will be able to: - Analyze project requirements and objectives. - Review technical drawings and specifications. - Evaluate feasibility and resources needed. - Confirm project selection with stakeholders. | - Learn techniques for identifying - Interpreting the needs and objectives of a project. - Learn to read and interpret technical drawings - Skills in assessing the feasibility of the project - Communicate project requirements, objectives, - Presentations, reports, and documentation. Practical Activity: - Develop drawing a jaws Couplings with specific dimensions and tolerances. | Theory: 02Hrs. Practical: 06Hrs. Total: 08Hrs. | - A4 size paper 10 Nos for each trainee. | - Class Room |
| Identify The Right Material. | Trainee will be able to: - Assess material properties relevant to the project. - Compare material options based on cost and availability. - Select material that meets technical specifications. - Document material selection process. | - material properties - materials behave under various conditions - Cost analysis. - assess the availability of materials, including lead times - material properties with the technical specifications - methods for testing and verifying material properties - create comprehensive documentation - Prepare reports and presentations Practical Activity: - Arrange The Right Material of jaws Couplings. | Theory: 02Hrs. Practical: 06Hrs. Total: 08 Hrs. | - A4 size paper 05 Nos for each trainee. | - Class Room - Workshop |
| Select The Right Machining Process and Sequence. | Trainee must be able to: - Determine suitable machining processes for the material. - Plan the sequence of machining operations. - Verify compatibility of tools and equipment. - Optimize process for efficiency and quality. | - Material Machinability - Study various machining processes - Types of cutting tools. Practical Activity: - Enlist the operation to be perform of jaws Couplings. | Theory: 02Hrs. Practical: 06Hrs. Total: 08 Hrs. | - A4 size paper 05 Nos for each trainee. | - Class Room |
| Make The Project. | Trainee must be able to: - Plan machining process and sequence. - Arrange tools for each operation. - Monitor progress and adjust, as necessary. - Complete the project within the planned timeframe. | - Setup and Execution - Documentation - Tool Selection - Techniques and Settings - Tool Maintenance - Progress Tracking - Adjustment Techniques - Problem-Solving - Time Management. - Efficiency Improvement - Deadline Adherence - Procedure for storage of PPEs, - Procedure for work completion (disposal of scrap/chips, switching off the machine storage of tools and instruments) Practical Activity: - Prepare a jaws Couplings as per prescribe drawing. | Theory: 02Hrs. Practical: 12Hrs. Total: 14 Hrs | - Lathe machine along with standard accessories - Machine vice and work holding devices - Measuring and marking tools - Right hand side tool HSS - Twist drill bits - Boring tools - Reamers - Grooving and parting-off tools, etc. - (assorted range) - Shaping machine along with standard accessories - Machine vice and work holding devices | - Workshop |
| Ensure Dimensional Accuracy and Quality as Per | Trainee must be able to: - Measure critical dimensions using precision instruments. - Compare measurements to the technical drawing or standard. | - Measurement Tools - Calipers - Micrometers - Types of micrometers - Height Gauges - Dial Indicators - Calibration - Measurement Procedures - Drawing Analysis - Standards and Specifications - Tolerance Checking - Dimensional Verification Practical Activity - Prepare final marking jaws Couplings as per prescribe drawing. | Theory: 01Hrs. Practical: 06Hrs. Total: 07Hrs. | - A4 size paper 02 No for each trainee. - IT Lab - Printer | - IT Lab / - Workshop |

# 5.	List of Personal Protective Equipment

(For A Class Of 25 Students)

| Name of Trade |  | Assistant Machinist Level-2 Machinist |  |
| --- | --- | --- | --- |
| Duration of Course |  | 6 Months |  |
| Sr. # | Description |  | Quantity |
|  | First AID Box |  | 5 Nos |
|  | Fire Extinguisher Cylinder |  | 5 Nos |
|  | Fire Blanket |  | 25 Nos |
|  | Fire Bucket |  | 25 Nos |
|  | Safety Gloves |  | 25 Nos |
|  | Ear Protector |  | 25 Nos |
|  | safety googles |  | 25 Nos |
|  | Safety Helmet |  | 25 Nos |
|  | Safety mask |  | 25 Nos |
|  | Formal Uniform for Work |  | 25 Nos |
|  | Safety Shoes |  | 25 Nos |

# 6.        List of Tools, Equipment, Machines and Consumables

| Sr.# | Name of Tools Items | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | All Kind of Files | FLAT FILE: Length of file (mm) = 300, Type of cut = Double cut. FLAT FILE: Length of file (mm)= 300, Type of cut = Single cut FLAT FILE: Length of file (mm) = 300, Type of cut= Rasp cut HALF ROUND FILE: Length of file (mm)= 300, Type of cut = Double cut ROUND FILE: Length of file (mm) = 200, Type of cut= Double cut TRIANGULAR FILE: Length of file (mm)=150, Type of cut = Double cut. SQUARE FILE: Length of file (mm) = 200, Type of cut = Double cut. KNIFE EDGE FILE: Length of file (mm) = 250, Type of cut = Double cut. NEEDLE FILE SET(Set of 6 pieces): Length of file (mm) = 200, Type of cut = Double cut. All files made of alloy steel, hardened with wooden/Plastic file handle. | 25 each |
| 2 | Aluminum Ladder – 8 To 10 Feet Height | 8 To 10 Feet Height made of best quality aluminum alloy channel with standard supports spacing. | 1 No. |
| 3 | Anvil & Hammers (Assorted Range) | ANVIL Made of cast steel hardened face and horn, square hardy hole. Weight (Kg) = 50 HAMMER (BALL PEEN) Weight of hammer (Kg) = 0.5 HAMMER (CROSS PEEN) Weight of hammer (Kg) = 0.5 HAMMER (STRAIGHT PEEN) Weight of hammer (Kg) = 0.5 All hammer made of Forged steel hardened and ground pein and face. Suitable wooden handle. | 1 No. 25 each |
| 4 | Power Vice | POWER VICE (Pneumatic/Hydraulic Operated) Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm | 1 (Power Vice) |
| 5 | Bearing Pullers | Size = 4, 6 & 8 inches Made of forged steel jaws | 1 each |
| 6 | Bench Vices, Machine Vices, C-Clamps, U-Clamps, Universal Clamping Vices & Swivel Vices | BENCH VICE Made of cast steel/cast iron, hardened jaws, gun metal nut, square threaded spindle. Width of jaws (mm) = 125. Jaws opening (mm) = 150 MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm C-CLAMP Max. Opening = 8 in. Body made of cast iron U-CLAMP Max. Opening = 8 in., Body made of cast iron, UNIVERSAL CLAMPING VICES: Material = Close grain cast Iron, Jaw width = 150 mm, Jaw Height = 50 mm, Max. opening = 400 mm, Compound angle setting swivel bases SWIVEL VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm With swivel base able to rotate up to 360 degrees. | 25 (bench vice) 02 (M/C Vice) 15 Z(C-Clamp) 15 (U-Clamp) 02 (UCV) 02 (Swivel Vice) 10 (Hand Vice) |
| 7 | Cabinets & Steel Almirah (Assorted Range) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5ft Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5ft Material = MS Sheet SWG 22 | 25 each |
| 8 | Center Drills (Assorted Range) | Misc sizes ranging from 1-8 Made of High-Speed Steel | 100 each |
| 9 | Dial Indicator | DIAL INDICATOR: - For surface roughness with magnetic stand, dial zero setting. Range (mm) = 10 Resolution (mm) = 0.01 | 2 Nos. |
| 10 | Diamond Disc Grinding Wheels for Carbide Materials (Assorted Range) | Resin or vitrified bond grinding wheel having diameter, thickness and bore totally compatible with grinding machine | 10 Nos. |
| 11 | Diamond Dressing Tip with Stand | Length: 120 mm Shank Type: Taper Shank dia.: 9mm | 5 Nos. |
| 12 | Dies & Taps Set (Assorted Range) | TAP AND DIE SET: - Consist of suitable taps and dies with tap wrenches and die stokes. System = metric. Size/Capacity (mm) = 4, 6, 8, 10, 12, 14, 16. | 2 sets |
| 13 | Drafting Tables, T-Squares & Set-Squares | Drafting Table Portable (Taper Type) Table size 32'' x 24''Tee Square 600mm Transparent Set Square Transparent medium size 300 mm | 25 Nos. each |
| 14 | Drill Bits (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs | 3 sets |
| 15 | Drill Chuck with Key | DRILL CHUCK Three jaws hardened and ground, Key holes hardened, with Key. Capacity = 1-13 mm | 2 Nos. |
| 16 | Electric Table Lamps, Torch Lights | Electric Table Lamps Tall enough to illuminate large area Bright, even, pleasant light Adjustable brightness and color temperature Well-built, solid, sturdy while flexible Energy Efficient LED Torch Lights Adjustable brightness More than 5 hours run time Rechargeable battery (NiCd) Typical Life: 1000 cycles Size: 10 inches Water Resistant | 2 Nos. 10 Nos. |
| 17 | Fire Extinguisher | ABC dry powder/DCP, portable, wall mounted Capacity = 8 Kg | 2 Nos. |
| 18 | First Aid Box | Suitable size having large and small gauze pads, adhesive bangages, wound cleaning agent, scissors, tweezers, adhesive tape, latex gloves, pocket mask, elastic wraps, splints | 1 No. |
| 19 | Geometry Box with Pencil, Rubber, Sharpener, Eraser & Steel Rule | Instruments Box Special Compass Set with Pencil, Rubber, Sharpener, Eraser & Steel Rule | 25 Nos. |
| 20 | Grease & Greasing Guns (Assorted Range) | GREASE GUN Suitable for soft grease or oil, metallic body funnel and pointed nozzle. Reservoir = medium size | 15 Nos. |
| 21 | Grinding Wheel Balancing Stand with Standard Accessories | Heavy duty ball screw and linear guide Spindle motor: 1.1 kW(2800 rpm) Wheel Grinding Range(Dia.): 75-190mm Wheel Grinding Range(thickness): 25mm X-Y axes wheel stroke: 100mm Grinding Wheel motor rating: 0.5 kW | 1 No. |
| 22 | Grinding Wheels (Surface Grinding Machine, Pedestal Grinding Machine, Cylindrical Grinding Machine (Assorted Size and Shape) | Resin or vitrified bond grinding wheel having diameter, thickness and bore totally compatible with grinding machine | 10 Nos. |
| 23 | Hammers & Shaft Pullers | HAMMER Made of Forged steel hardened and ground pein and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 Shaft Pullers Sizes: 4, 6, 8 inches made of cast iron/steel | 05 Nos. each |
| 24 | Hammers (Assorted Range) | HAMMER Made of Forged steel hardened and ground pein and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 | 5 Nos. each |
| 25 | Hand Hacksaws with Blades, Complete Sets (Assorted Range) with Standard Accessories | HAND HACKSAW FRAME Adjustable frame made of steel tube elliptical section. Suitable for blade 300 mm | 25 Nos. |
| 26 | Height Gauge, Step Gauge, Filler Gauge, Stubs Wire Gauge (Assorted Range) | HEIGHT GAUGE:- Made of high quality steel, rust resistant, base highly polished and lapped, main scale adjustable magnifying glass, with offset scriber carbide point. Working range (mm) = 0 - 300. Step Gauge(90 pcs set) FEELER GAUGE:- No. of Leaves = 100. Thickness (mm) = 0.05 - 1 Stubs Wire Gauge hardened steel Range: 08-36 SWG | 2 Nos. 1 set 5 Nos. 5 Nos. |
| 27 | Knurling Tools (Assorted Range) | Knurling, rough, Medium, fine, revolving rollers Dimension of knurling rollers = 20 x 6 x 6 mm | 5 Nos. |
| 28 | Machine Repairing Tool Kits | Machine Repairing Tool Kits containing Open End Spanner Set, Allen Key Set, measuring tape, Screw Driver Set, LED Flashlight, Hammers, Pliers, Hacksaw with blade, Adjustable wrench, punches, utility knife, super glue, safety gloves, safety glasses, mask, ear plugs | 4 Nos. |
| 29 | Machine Vice Universal Clamping Vices | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Heigt = 50 mm Max. opening = 400 mm UNIVERSAL CLAMPING VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm Compound angle setting swivel bases | 4 Nos. 2 Nos. |
| 30 | Maintenance Box | Made of Iron/hard plastic, shock resistant Size: 400 x 300 x 150 mm | 4 Nos. |
| 31 | Measuring Instruments, Marking Tools & Gauges (Assorted Range) | Scribers (straight and off-set), punches, Try Square, Dividers, Spirit Level, Folding Rule, Measuring Tape, inside calipers, outside calipers, odd leg calipers, steel rule, Vernier Calipers, Digital Vernier Calipers, Micrometer, Thread Pitch Gauge, v-block with clamp, Bevel Protractor, surface gauge, Layout ink/dye, Tool Grinding Gauge, plug gauges, snap gauges, ring gauges. | 2 Sets |
| 32 | Multiple Hand Tools | Hand hack saw, utility knife, scissors, pliers, c-clamp, measuring tape, steel rule, hammers, mallets, screw drivers, level, adjustable wrench, pipe wrench, spanners, Allen keys, socket wrench. | 5 Sets |
| 33 | Overall Combination | Overall Combination Arm pocket, back pocket (optional), chest pocket, elasticated waist band, hip pocket wide side access, stud fastened collar, two-way zip, 65% polyester, 35% cotton | 25 Nos. |
| 34 | Personal Protective Equipment | Overalls, safety helmet, wide brimmed hat, safety footwear, safety glasses and goggles, gloves, respirators, masks, earmuffs and earpieces. | 30 Sets |
| 35 | Plug and Snape Gauge | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm Material: Tool Steel/chrome steel/carbide Sizes: 10, 20, 30 mm according to ANSI Standard | 2 Sets (Each) |
| 36 | Plug Gauge (Assorted Range) | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm | 2 Sets |
| 37 | Prick Punch (Assorted Range & Types) | PRICK PUNCH Made of high-quality steel, hardened and ground point, knurled shank, point angle 60 deg. Length (mm) = 100, 125, 150 Diameter (mm) = 8, 10, 12 | 25(each) |
| 38 | Radius Gauge - Concave & Convex (Assorted Range) | Radius Gauge - Concave & Convex Individually marked, rust proof steel Range: 1-3 mm R---0.25 step & 3.5-7mm R---0.5 step No. of Plates: 30 | 5 Nos. |
| 39 | Reamers (Assorted Range) | High Speed Steel, Straight/Taper shank, straight flute, right hand cut. Sizes: 12, 16, 18, 20,22, 24 mm | 2 Sets |
| 40 | Safety Equipment - Safety Shoes, Safety Gloves, Safety Goggles, Safety Helmet and Ear Plugs | Made of plastic/ glass/leather of best quality to promise safe working in workshop and outdoor environment | 30 Nos. |
| 41 | Scientific Calculator & Multimedia Projector | Scientific Calculator 10 digits display, should be able to process mathematics and statistics functions efficiently Multimedia Projector Display: LCD/LED Pixels:1024 x 768 Brightness: 4000 ANSI lumens minimum Color System: PAL/SECAM/NTSC Computer Compatibility: UXGA/WXGA/SXGA/XGA/SVGA/VGA Projection Method: Front | 30 Nos. 1 Nos. |
| 42 | Shaper Tools (Assorted Range) | SHAPER TOOL Made of high-speed steel cobalt alloy hardened and ground 5/8” x 5/8” x 8” 3/4" x 3/4” x 8” | 10 Each |
| 43 | Smoke Detecting Alarm | Smoke Detecting Alarm (Best Quality) Able to detect smoke easily | 1 Nos. |
| 44 | Spray Guns & Oil Containers | Spray Guns Nozzle Size = 1.4 - 1.8mm Air Inlet = 1/4 inch Type of feed = Gravity Recommended Air Pressure = 2 bar Approx. Paint Capacity = 1000ml Oil Containers Transparent body having graduated marking 10 litre/quart drum body and stretch spout lid with proper sized opening. | 5 Nos. 5 Nos. |
| 45 | Standard Lathe Machine attachments | STANDARD LATHE MACHINE ATTACHMENTS 3-jaw self-centering chuck 4-jaws independent chuck Face plate with slots Square tool post Steady rest Follow rest Longitudinal stop, Mandrill, Lathe Dog Dead centers, Live Centers, Revolving Center Taper sleeve | 10 Sets |
| 46 | Steel Cabinet and Steel Almirah (03 for Each Machine) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5 Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 25 Each |
| 47 | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves etc. | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves made of high quality thermosetting and durable plastic. | 25 Each |
| 48 | Step Gauges Set | Rectangular shape, micro lapped, heat treated, high corrosion resistance steel. No of pieces per set = 103 Size (mm) = 1.01-1.49 Steps = 0.01 mm | 1 Set |
| 49 | Surface Analysis and Inspection Gauges (Assorted Range) | Type: Flat Surface Processing: Paper Finish, Grinding, Shaping, Milling | 1 No. |
| 50 | Surface Plate with Stand (Cast Iron / Graphite) 24 X 24 Inches | CAST IRON SURFACE PLATE Made of close grain cast-iron high-quality finish. Length x Width x Thickness(inch) = 24 x 24 x 1 | 2 Nos. |
| 51 | Threading Tools (Assorted Range) | Made of H.S.S. able to cut BSW and American Standard Threads, Acme thread, square thread Angle: 55 and 60 degrees | 10 Sets |
| 52 | Threads Gauge -Inches / Millimeters (Assorted Range) | Threads Gauge -Inches / Millimeters Size = 4 - 42 TPI and 0.5-10mm No of leaves = 30 Angle (degrees) = 55 & 60 | 5 Nos. |
| 53 | Turning, Parting, Grooving and Forming Tools etc. (Assorted Range) | Made of H.S.S. 200 mm long with square and rectangular cross sections | 10 Sets |
| 54 | Twist Drill Bits and Boring Bars (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs Boring Bars with holders Sizes with H.S.S. tool bits = 6 mm | 3 Sets 10 Sets |
| 55 | Wooden Cabinet, Steel Almirah | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 5 Each |
| 56 | Wooden Cabinets | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood | 5 Nos. |
| 57 | Work-Bench, Work-Table & Stools | Work-Bench L X W X H = 5 X 3 X 2.5 ft. Angle Iron used for legs 64x5mm Wooden Top 40mm thick Work-Table L X W X H = 8 X 4 X 2.5 ft. Stools Four legs made of MS pipe gauge 16SWG 2ft high, 1.25 x 1.25 ft square Wooden seat 25mm thick. | 25 Sets |
| 58 | Universal Milling Machine with Standard Accessories | Table size – 1000x260mm (minimum),Number of T-slots-3 (Minimum),T-slot size -14-16mm,Spindle to table-0-350mm,Number of spindle speeds – 12 steps, Spindle speed range – 50 to 1200 rpm, Swivel of table 45ᵒ,Spindle taper: Horizontal: ISO40,Vertical: ISO40,Table travel Vertical -400-450mm,Table Travel Cross – 280-300mm,Main spindle motor – 3-3.5KW 3Ph,Coolant pump, Machine Weight – 1100 Kg (minimum), without any attachment and accessories. Accessories: • Milling Machine vice, • Tail stock, • Milling arbor 400mmxΦ27mm, • Milling arbor 400mm x Φ22mm, • Stub arbor Φ27mm, • Stub arbor Φ22mm. Standard Accessories: Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set ,Collet holders & collets, L-Key set, Coolant System, Machine light, Centralized lubrication, Table guard,3 Axis Digital Read Out, chip tray for Base, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, • Slotting head,• Universal Head (Vertical),• Rotary Table | 2 Nos, |
| 59 | Standard Milling Machine Attachments (Sets) | Milling Machine vice, Swivel Base vice, Tail stock, Milling arbor 400mmxΦ27mm,Milling arbor 400mm x Φ22mm,Stub arbor Φ27mm,Stub arbor Φ22mm,Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set, Collet holders & collets, L-Key set,3 Axis Digital Read Out, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, Slotting head, Universal Head (Vertical). | 2 Nos. |
| 60 | File Brush | 3 inches | 25 Nos. |
| 61 | Digital Vernier Caliper | 160mm | 02 Nos |
| 62 | Counter sink | 18x60⁰, 12.5x50⁰ | 5 set |
| 63 | Counter Bore | 6, 10, 12 mm | 5 set |
| 64 | Digital Micro Mete | 0-25 mm | 02 Nos |
| 65 | Digital Micro Meter | 25-50 mm | 02 Nos |
| 66 | Chisel Flat, Groove, Cross cut | 8 inches | 05 Each |
| 67 | Number Punch | 4 mm | 02 Sets |
| 68 | Letter Punch | 4 mm | 02 Sets |

| MACHINE TOOLS & ATTACHMENTS |  |  |  |
| --- | --- | --- | --- |
| 1 | Lathe Machine (with standard accessories) | Centre height – 160-200 mm, Maximum swing over bed – 320-400mm, Maximum swing over saddle – 180-190mm, Maximum swing in gap – 460-470mm, Length of gap – 200-250mm, Distance between centers - 750mm, Spindle bore – 38mm, Spindle taper – MT5, Tailstock taper – MT3, Bed width – 180-190mm, Number of spindle speeds – 8, Range of spindle speeds – 70-2000 rpm, Metric threads (26 pitches) – 0.4-7mm, Imperial threads – (34 pitches) – 4-60 TPI, Lead screw dia./pitch – 22/3mm, Travel of compound slide : 75mm, Travel of cross slide – 130mm, Tailstock quill travel – 100mm, Main Motor – 1.5-2 KW 3phase, Coolant Motor, Tool Section – 16mm, Net Weight of machine – 525 to 550 Kg without any attachment and accessories. Standard Accessories: 160mm 3 jaw chuck(Universal), 200mm 4 jaw chuck (Independent) , 250mm face plate, Fixed steady, Traveling steady, Electrically interlocked chuck guard, 4 way indexing tool post stand, Tool post chip guard, Rear splash guard, Spindle foot brake, Thread dial indicator, Coolant system, Low volt machine light, Tool box(Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier). | 10 Nos. |
| 2 | Pedestal Grinder with Tools, Cutting Angle Support | Ball bearing mounted spindle, suitable for tool grinding, protection glass/clear plastic eye shield with lamp, grinding wheel = 250 x 30 x 25 mm, Spindle speed = 2500-3000 rpm, Motor rating 3 ph/440V/50 Hz = 600–750-Watt, Quick angle setting of = 30-90 degree. Attachment: - Wheel dresser star=Standard type / Diamond, Complete in all respect. | 2 Nos. |
| 3 | Surface Grinding Machine | Working surface of Table (W x L) 250 x 500mm Minimum, Table travel (y,x)-250 x 550mm Minimum, Minimum distance spindle centre-table-400mm, Magnetic Chuck-size 200 x 450mm, Longitudinal feeds 5-25m/min, Automatic Cross Feed of Saddle 0.5-10mm, Grinding wheel size- 180 x 13 x 31.75mm, Speed-2800 rpm Minimum, Spindle motor -1.5-2 KW, 3phase, Hydraulic pump motor-0.75 KW, Coolant pump motor, Elevating motor, Cross feed motor, Total rated power- 3KW, 3phase, Maximum table load incl.chuck-120 Kg, Net weight of machine-750 Kg Minimum. STANDARD ACCESSORIES: Wheel balancing base, Wheel balancing arbor, grinding wheel, Diamond dresser fit on the top of machine spindle grinding wheel, Tool box (Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier), Low volt machine light, Built in demagnetizer, Coolant system, 200 x 450mm electro-magnetic chuck. | 2 Nos. |
| 4 | Cylindrical Grinding Machine | Distance between center = 500-600mm, Center height = 120-150 mm, Max dia. grind 100mm, Spindle taper MT4, Maximum transverse Travel = 120-150mm, Swiveling angle 0-10˚ , Spindle speed = 2500-3000 RPM , Tail stock taper MT4, Motor 4Kw, Internal Grinding motor 1-1.25Kw, Work head motor 0.5-1Kw,Hydraulic longitudinal feed, Wheel head manual feed, Wheel head & Work head (Swivel Type). | 2 Nos. |
| 5 | Tool/Cutter Grinding Machine | Suitable to grind all types of tool and cutters, Clamping area of table (mm) =820x 140, Longitudinal table travel (mm) =450, Lateral table travel (mm) = 225,Distance between centers (mm) = 600-650,Height of center (mm) =125-130,Spindle speed (rpm) =2800- 5600,Motor power 3ph = 1 kW, Table angle setting (degrees) =±450,Grinding head tilting on vertical axis (degrees) = ±3600,Grinding head tilting on horizontal axis (degrees) = ±150. Attachments: Face milling cutter grinding attachment (mm) = 450, Radius grinding attachment (mm) = 0-25,Cylindrical grinding attachment (mm) = 250,Internal grinding attachment (mm) =12-50,Indexing attachment = 01,Extended grinding wheel spindle (mm) = 75,Universal vice (mm) = 100x25x75,Drill point grinding attachment (mm) = 3-25,Lathe and planer tool grinding attachment (mm) =10-38,Set of spring collets (mm) = 6, 8, 10, 12 ,16, 20. Accessories : Work head = 1,Left hand tail stock = 1,Right hand tail stock = 1,Diamond wheel dresser = 1,Setoff wheel guards = 1,Universal tooth rest = 1,Center height gauge = 1.Collect wrench = 1,Ejector rod = 1,T- wrench = 1,Pin wrench = 1,Grinding wheel sleeves = 5,Set of grinding wheels different shapes = 7,Spindle extension 100 mm =1,Belt for spindle drive = 1,Suitable tool cabinet = 1,Instruction / operation manual, complete in all respect, COMPLETE AND COMPATIBLE IN ALL RESPECTS, OR EQUIVALENT OF BEST QUALITY. | 1 No. |
| 6 | Centerless Grinding Machine along with Standard Accessories | Speed = 20-220 RPM (Step less Variable), Grinding Wheel = Diameter - 350 mm x 100 Width, Bore Size = 127mm,Grinding Capacity = Dia. - 2mm (min), 70mm (max),Machine Weight = 2200 KG (Approx.),Coolant Pump Motor = 0.12 KW, Regulating Wheel Motor = 0.75 KW DC, Width = 100 mm, Regulating Wheel = Diameter - 230 mm, Swivel Angle = Swivel of grinding dresser - +/- 5 Degree, Regulating Swivel Angle = +/- 5 Degree Horizontal, Dresser Operation = Grinding & Regulating Wheel - Hydraulics, Power Requirement = Grinding Wheel Motor - 5.5 KW, Power Pack Pump = 0.18 KW. | 1 No. |
| 7 | Pillar Drilling Machines | PILLAR DRILL MACHINE: - V-belt drive with 12 spindle speeds, table raise/lower by means of gear and rack. Chuck capacity (mm) = 16 Depth of feed (mm) = 80 Distance pillar to spindle center(mm) = 300 Table size (mm) = 230 x 240. Spindle speeds (rpm) = 220 – 3000. Spindle Morse taper number = MT-2. Motor 3-Ph./440V/50Hz = 0.75KW. Drill Press Vice = 100 mm. Drill chuck with arbor = 1-16mm. Magnet stator switch of on/off | 1 No. |
| 8 | Power Hacksaw Machine | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm,Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm,Width of stand = 350mm,Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY. | 2 No. |
| 9 | Shaper Machine with Standard Accessories | Stroke length = 400-450mm, No. of speeds =2-8, Strokes per mint = 20-130, Vertical Travel of tool slide=150mm or greater, Table automatic feed (5) =0.2 to 2 mm/stroke, Motor rating 3 ph/440v/50 Hz =1.25-1.5 KW, Shaper Vice (Swivel Base) =250-300 mm width of jaws, note: - Along with all standard accessories machine handles, set of spanners. Ellen Key set and complete in all respect. | 2 No. |

List of Consumables

| Sr. No | Name | Qty. Required |
| --- | --- | --- |
| 1 | Cleaning Supplies (E.G., Wipes, Disinfectants) | Sufficient Supply |
| 2 | Safety Posters | 10-15 Nos. |
| 3 | Markers And Highlighters | 20 Nos. |
| 4 | Compostable Bags | 5 Rolls |
| 5 | First Aid Kit | 5 Nos. |
| 6 | Fire Extinguishers | 3 Nos. |
| 7 | Rags And Wipes | 20 Kg |
| 8 | Gloves | 50 Pairs |
| 9 | First Aid Supplies | 5 Kits |
| 10 | Masking Tape | 20 Rolls |
| 11 | WD-40 or Equivalent Lubricant | 2 Cans |
| 12 | Paper Towels | 20 Rolls |
| 13 | Lubricating Oil | 40 Liters |
| 14 | Grease | 1 Kg |
| 15 | Machine Oil | 5 Liters |
| 16 | Trash Bags | 20 Nos. |
| 17 | Disposable Gloves | 50 Pairs |
| 18 | Cleaning Detergent | 1 Kg |
| 19 | Safety Goggles | 50 Nos. |
| 20 | Kerosene Oil | 10 Liter |
| 21 | Lubricating Oil | 30 Liters |
| 22 | Sandpaper | 100 Sheets |
| 23 | Polishing Compound | 1 Nos. |
| 24 | Polishing Cloths | 20 Nos. |
| 25 | Ear Protection | 10 Nos. |
| 26 | Safety Gloves | 50 Pairs |
| 27 | Hand Sanitizer | 3 Bottles |
| 28 | Disposable Masks | 50 Nos. |
| 29 | Hand Grinding Disk | 15 Nos. |
| 30 | Twist Drill Set | 5 Set |
| 31 | File Brush | 5 Nos. |
| 32 | Grinding Wheels (Green and rough) | 4 Nos. |
| 33 | Cutting Oil | 10 Liter |
| 34 | Aprons | 30 Nos. |
| 35 | Surface Grinding Wheels | 1 Nos. |
| 36 | Cylindrical Grinding Wheels | 1 Nos. |
| 37 | Paper Towels | 25 Nos. |
| 38 | Rags And Wipes | 5 Kg |
| 39 | Carbide Tools of Different Shapes and Sizes | 10 Nos. |
| 40 | Tool Bits of Different Shapes and Sizes | 25 Nos. |
| 41 | Chips Cleaning Brushes | 25 Nos. |
| 42 | Shaper Cutting Tools of Different Types and Shapes | 15 Nos. |
| 43 | Milling Inserts Different Types | 10 Nos. |
| 44 | Milling Cutters of Different Types and Shapes | 10 Nos. |

- Members of Curriculum Development Committee,

The Curriculum Development, Committees consisted of the following members:

| Sr. # | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1. | Engr. Inayat ur Rehman | DACUM Facilitator | Retired from KP-TEVTA |
| 2. | Engr. Hafiz Naveed Rashid | Instructor Machinist | GTVC (B) Mansehra KP-TEVTA |
| 3. | Engr. Muhammad Dawood ul Hassan | Jr. Instructor Mechanical | GCT Jhang P-TEVTA |
| 4. | Mr. Faheem Naeem | Instructor Machinist | GTTI Kasur P-TEVTA |
| 5. | Mr. Babar Hussain | Instructor Machinist | COE Mughalpura P-TEVTA |
| 6. | Mr. Asim Jamal | DD SS&C (Coordinator) | NAVTTC HQ. Islamabad |
| 7. | Mr. Muhammad Ishaq | DD :SS&C (Coordinator) | NAVTTC HQ. Islamabad |