|  | Assessment Evidence Guide For “Machinist” Level-3 Summative Assessment |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Machinist Level – 3 |
| --- | --- |
| Competency Standards | - Maintain Health and Safety at work place - Develop Basic computer operating skills - Develop Professionalism - Perform off hand grinding - Perform Surface Grinding - Perform Cylindrical and centerless grinding - Perform Tool & Cutter Grinding - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Workshop Projects-II |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. OR - Perform step milling operation on a rectangular plate as per given drawing in Annex-B. |
| Time: 5 Hrs. | During a practical assessment, under observation by an assessor, you are required to demonstrating the following criteria: |
| Minimum Evidence Required | - Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. - Cut a gear blank with Ø48 mm diameter and 20 mm thickness. - Drill Ø16 mm hole at the center of the blank as per the drawing. - Ream the hole to ensure a smooth finish. - Mount the gear blank securely on the milling machine. - Select the correct module cutter for the gear profile. - Attach the indexing head to the machine and set it for 12 divisions (30° rotation per tooth). - Index precisely for 12 teeth positioning. - Mill 12 teeth on the blank to form the gear as per the drawing. - Shape each tooth with a R20 radius. - Inspect the tooth profile and alignment for accuracy. - Clean the work area thoroughly. - Power down the machine. B. Perform step milling operation on a rectangular plate as per given drawing in Annex-B. - Install the appropriate milling cutter. - Mount the workpiece on the milling machine table. - Face mill for a length of 80 mm. - Side mill to achieve a width of 25 mm. - Face mill the workpiece for a thickness of 30 mm. - Step mill for a 40 mm depth. - Step mill for a 10 mm depth. - Step mill for a 5 mm depth. - Step mill for a depth of 60 mm. - Clear the work area. - Shut down the machine. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machinist Level-3 |
| Competency Standards | 1. Maintain Health and Safety at work place 2. Develop Basic computer operating skills 3. Develop Professionalism 4. Perform off hand grinding 5. Perform Surface Grinding 6. Perform Cylindrical and centerless grinding 7. Perform Tool & Cutter Grinding 8. Perform Milling Operations 9. Generate Gears on Hobbing Machine 10. Perform broaching operation. 11. Workshop Projects-II |
| Assessment Task | - Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. OR - Perform step milling operation on a rectangular plate as per given drawing in Annex-B. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. |  |  |
| - Cut a gear blank with Ø48 mm diameter and 20 mm thickness. |  |  |
| - Drill Ø16 mm hole at the center of the blank as per the drawing. |  |  |
| - Ream the hole to ensure a smooth finish. |  |  |
| - Mount the gear blank securely on the milling machine. |  |  |
| - Select the correct module cutter for the gear profile. |  |  |
| - Attach the indexing head to the machine and set it for 12 divisions (30° rotation per tooth). |  |  |
| - Index precisely for 12 teeth positioning. |  |  |
| - Mill 12 teeth on the blank to form the gear as per the drawing. |  |  |
| - Shape each tooth with a R20 radius. |  |  |
| - Inspect the tooth profile and alignment for accuracy. |  |  |
| - Clean the work area thoroughly. |  |  |
| - Power down the machine. |  |  |
| B. Perform step milling operation on a rectangular plate as per given drawing in Annex-B. |  |  |
| - Install the appropriate milling cutter. |  |  |
| - Mount the workpiece on the milling machine table. |  |  |
| - Face mill for a length of 80 mm. |  |  |
| - Side mill to achieve a width of 25 mm. |  |  |
| - Face mill the workpiece for a thickness of 30 mm. |  |  |
| - Step mill for a 40 mm depth. |  |  |
| - Step mill for a 10 mm depth. |  |  |
| - Step mill for a 5 mm depth. |  |  |
| - Step mill for a depth of 60 mm. |  |  |
| - Clear the work area. |  |  |
| - Shut down the machine. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machinist Level-3 |
| --- | --- |
| Competency Standards | - Maintain Health and Safety at work place - Develop Basic computer operating skills - Develop Professionalism - Perform off hand grinding - Perform Surface Grinding - Perform Cylindrical and centerless grinding - Perform Tool & Cutter Grinding - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Workshop Projects-II |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. OR B. Perform step milling operation on a rectangular plate as per given drawing in Annex-B. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| A. Perform milling operation on a blank for generating a spur gear as per drawing given in Annex-A. |  |  |  |  |  |  |
|  | Cut a gear blank with Ø48 mm diameter and 20 mm thickness. |  |  |  |  |  |
|  | Drill Ø16 mm hole at the center of the blank as per the drawing. |  |  |  |  |  |
|  | Ream the hole to ensure a smooth finish. |  |  |  |  |  |
|  | Mount the gear blank securely on the milling machine. |  |  |  |  |  |
|  | Select the correct module cutter for the gear profile. |  |  |  |  |  |
|  | Attach the indexing head to the machine and set it for 12 divisions (30° rotation per tooth). |  |  |  |  |  |
|  | Index precisely for 12 teeth positioning. |  |  |  |  |  |
|  | Mill 12 teeth on the blank to form the gear as per the drawing. |  |  |  |  |  |
|  | Shape each tooth with a R20 radius. |  |  |  |  |  |
|  | Inspect the tooth profile and alignment for accuracy. |  |  |  |  |  |
|  | Clean the work area thoroughly. |  |  |  |  |  |
|  | Power down the machine. |  |  |  |  |  |
| B. Perform step milling operation on a rectangular plate as per given drawing in Annex-B. |  |  |  |  |  |  |
|  | Install the appropriate milling cutter. |  |  |  |  |  |
|  | Mount the workpiece on the milling machine table. |  |  |  |  |  |
|  | Face mill for a length of 80 mm. |  |  |  |  |  |
|  | Side mill to achieve a width of 25 mm. |  |  |  |  |  |
|  | Face mill the workpiece for a thickness of 30 mm. |  |  |  |  |  |
|  | Step mill for a 40 mm depth. |  |  |  |  |  |
|  | Step mill for a 10 mm depth. |  |  |  |  |  |
|  | Step mill for a 5 mm depth. |  |  |  |  |  |
|  | Step mill for a depth of 60 mm. |  |  |  |  |  |
|  | Clear the work area. |  |  |  |  |  |
|  | Shut down the machine. |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |

Knowledge Assessment

| Qualification | Machinist Level-3 |
| --- | --- |
| Competency Standards | - Maintain Health and Safety at work place - Develop Basic computer operating skills - Develop Professionalism - Perform off hand grinding - Perform Surface Grinding - Perform Cylindrical and centerless grinding - Perform Tool & Cutter Grinding - Perform Milling Operations - Generate Gears on Hobbing Machine - Perform broaching operation - Workshop Projects-II |
| Candidate Details | Name: __________________________________________________________ Registration Number: __________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________ |

| Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application. |  |  |  |
| --- | --- | --- | --- |
| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
|  | Name two types of milling cutters. |  |  |  |
|  | How does climb milling differ from conventional milling? |  |  |  |
|  | Explain the purpose of an indexing head in milling. |  |  |  |
|  | What does the term grit size mean in grinding? |  |  |  |
|  | What is centerless grinding? |  |  |  |
|  | Define hob. |  |  |  |
|  | How does pull broaching differ from push broaching? |  |  |  |
|  | What is a broach tool? |  |  |  |
|  | Difference in Spur and Helical gear. |  |  |  |
|  | Define Universal Milling machine. |  |  |  |
|  | What is a grinding wheel’s bond? |  |  |  |
|  | Define grinding burn. |  |  |  |
| Feedback to the Candidate |  |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature ___________________ |  |  |  |  |  |

Annex-A

A. Perform milling operation on a blank for generating a spur gear as per drawing.

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.

Annex-B

Perform step milling operation on a rectangular plate as per given drawing.

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.