National Competency Standards Level-3 Machinist

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORDS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils, and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level 3 Machinist (Mechanical Technology). This work would not have been possible without the technical support of all the experts.

It may not be out of place to mention here that all the experts of Industry, Academia, and TVET experts of TEVTAs, BTEs, and PVTC work diligently to make this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that the development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director

NAVTTC

Table of Contents

1.	Introduction	4

1.1. Date of development	4

1.2. Date of review & validation	4

1.3. Entry requirement	4

2.	Codes of Qualifications	4

3.	Summary of Competency Standards.	5

4.	Detail of Competency Standards	6

0715MMT3501 Maintain Health and Safety at work place	6

0715MMT3502 Develop Basic computer operating skills	9

0715MMT3503 Develop Professionalism	11

0715MMT3504 Perform off hand grinding	14

0715MMT3505 Perform Surface Grinding	16

0715MMT3506 Perform Cylindrical grinding and centreless grinding	18

0715MMT3507 Perform Tool & Cutter Grinding	21

0715MMT3508 Perform Milling Operations	23

0715MMT3509 Generate Gears on Hobbing Machine	29

0715MMT3510 Perform broaching operation	32

0715MMT3511 Manufacture Mechanical Projects	34

4.	List of Tools, Equipment and Machines	37

5.	List of Consumables	69

6.	Members of Qualifications Development, Validation & Review Committee	71

- Introduction

A Machinist is a multidimensional trade person specialized in machine shop, machine and related cutting & measuring tools and equipment or the maintenance and repair of existing machinery. In order to meet the demand of machine shop, National Vocational and Technical Training Commission (NAVTTC) in collaboration with TVET Sector Support Program (TVET – SSP) has developed National Vocational Qualifications comprising generic, functional and technical competency standards for Turner-Machinist occupation. It encompasses a broad range of knowledge and skills related to machining processes, workshop techniques, quality control, safety procedures, and environmental sustainability. The demand for skilled professionals in this field is steadily growing as the industry seeks individuals who can contribute to innovation, process improvement and safe handling of equipment. It aims to establish a framework that defines the essential knowledge, skills, and attitudes required for individuals to perform effectively in this field. These standards will serve as a benchmark for training providers, employers, and learners, ensuring that the workforce is equipped with the competencies necessary to meet industry requirements and contribute to Pakistan's economic growth. Green skills incorporation in TVET aims to build capacity of skilled workforce, which is aware of environmental hazards and accordingly adopts more sustainable and environment friendly practices at the workplace.

| ## 1.1. Date of development | February 2018 |
| --- | --- |
| ## 1.2. Date of review & validation | July, 2023 |
| ## 1.3. Entry requirement | The entry for National Vocational Certificate Level 3 in Machinist is a person having Level-2 in Machinist. |

- Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| - S. C. E. D Classification |  |
| --- | --- |
| Code | Description |
| 0715MMT34 | National Vocational Certificate Level 2 Machinist |

| 0715MMT35 | National Vocational Certificate Level 3 Machinist |
| --- | --- |

- Summary of Competency Standards.

| Competency Standards | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th. | Pr. | Total |  |
| 0715MMT3501 Maintain Health and Safety at work place | 3 | Generic | 5 | 6 | 11 | 1.1 |
| 0715MMT3502 Develop Basic computer operating skills | 3 | Generic | 9 | 18 | 27 | 2.7 |
| 0715MMT3503 Develop Professionalism | 3 | Generic | 6 | 6 | 12 | 1.2 |
| 0715MMT3504 Perform off hand grinding | 3 | Technical | 8 | 12 | 20 | 2 |
| 0715MMT3505 Perform Surface Grinding | 3 | Technical | 8 | 21 | 29 | 2.9 |
| 0715MMT3506 Perform Cylindrical grinding and centerless grinding | 3 | Technical | 10 | 30 | 40 | 4 |
| 0715MMT3507 Perform Tool & Cutter Grinding | 3 | Technical | 9 | 27 | 36 | 3.6 |
| 0715MMT3508 Perform Milling Operations | 3 | Technical | 25 | 150 | 175 | 17.5 |
| 0715MMT3509 Generate Gears on Hobbing Machine | 3 | Technical | 15 | 30 | 45 | 4.5 |
| 0715MMT3510 Perform broaching operation | 3 | Technical | 15 | 30 | 45 | 4.5 |
| 0715MMT3511 Workshop Projects-II | 3 | Technical | 10 | 150 | 160 | 16 |
| Total |  |  | 120 | 480 | 600 | 60 |

- Detail of Competency Standards

## 0715MMT3501 Maintain Health and Safety at work place

Overview: This competency standard covers the skills and knowledge required to Ensure personal protective equipment (PPE), Protect Tools and Equipment, Maintain First aid Box, Maintain Fire Extinguisher, Adopt Environmental Regulation, adopt company policies and procedures, follow federal, provincial/ territorial, and municipal legislation, implement safety programmes, prepare for emergencies and respond to emergencies

| Competency Units | Performance Criteria |
| --- | --- |
| - Ensure personal protective equipment (PPE) | - Arrange PPEs as per requirement - Wear proper PPE as per nature of job - Store PPE at appropriate place after use |
| - Protect Tools and Equipment | - Ensure insulation of tools and equipment - Store tools and equipment safely - Clean tools on a regular basis before stacking |
| - Maintain First aid Box | - Ensure availability of first aid box - Check first aid box for requisite emergency medicines - Check expiry date of medicines - Perform first aid treatment against electric shock - Perform first aid treatment / bandages against minor injuries |
| - Maintain Fire Extinguisher | - Check expiry of fire extinguisher - Operate fire extinguisher - Replace fire extinguisher |
| - Adopt Environmental Regulation | - Locate applicable permits on job site - Ensure work friendly environment - Follow environmental regulations |
| - Adopt company policies and procedures | - Ensure company policy and procedures - Follow company procedures |
| - Implement safety programmes | - Take required health and safety training - Implement work place hazardous materials information system - Adopt first aid and cardio respiratory resuscitation (CPR) |
| - Prepare for emergencies | - Take emergency response training - Ensure emergency response exercises - Adopt first aid and Cardio Respiratory Resuscitation (CPR) |
| - Respond to emergencies | - Follow emergency plan - Communicate instructions - Assess risk and determine course of action - Operate emergency equipment and supplies |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Hazard and its Types

- Treatment of various hazards

- Personal Protective Equipment (PPE) and its uses

- Occupational Health & Safety Regulations.

- Knowledge and understanding of company policy and procedures.

- Use of emergency equipment

- First-Aid-Box and its uses.

- Knowledge of first-aid-box items.

- Know about the techniques for provision of first aid treatment.

- Fire Extinguisher its types and uses

- Types of fire extinguishing materials.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare a list of PPEs

- Demonstrate the use of at least one of the PPEs in front of assessor as per assessors’ directions

- Perform first aid treatment against electric shock/minor injury.

- Explain safety procedure at workplace

- Differentiate between safe and unsafe tools.

Tools and Equipment

- Steel-toed footwear,

- Safety gloves,

- Appropriate safety glasses,

- High visibility vest,

- Breathing apparatus,

- Site emergency response plan,

- Fire extinguishers,

- Fire blankets,

- Respirators, masks,

- Fire hoses,

- First aid kits, stretchers,

## 0715MMT3502 Develop Basic computer operating skills

Overview: This competency standard covers the skills and knowledge required to Operate MS word, Operate MS Excel, Operate MS Power Point, Perform Browsing and Print Document.

| Competency Units | Performance Criteria |
| --- | --- |
| - Operate MS word | P1. Perform Microsoft basic commands in MS word P2. Open File P3. Format a file - Font (Type/size/bold/Italic) - Header Footer - Page number - Insert pics / table/hyperlink P4. Save a File P5. Save a folder |
| - Operate MS Excel | P1. Perform basic commands in Microsoft MS Excel P2. Open a worksheet P3. Sum, division, multiplication, subtract, filter functions P4. If functions P5. Basic calculations P6. Table and graphs P7. Save a worksheet/folder |
| - Operate MS Power Point | P1. Prepare Microsoft power point presentation with basic commands P2. Make a power point file P3. Insert pics/table/hyperlink P4. Design a theme for slides P5. Save a power point file |
| - Perform Browsing | P1. Perform browsing on the internet as per needs P2. Perform search online on new trends in the market with the help of internet |
| - Print Document | - Select Printer - Select page setup - Print relevant pages |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic parts of computers

- Importance and uses of MS Word

- Importance and uses of MS Excel

- Importance and uses of MS Power Point

- importance of Internet.

- Use of various search engines like Google, U-tube etc.

- Printing Procedure.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate MS word

- Operate MS Excel

- Operate MS Power Point

Tools & Equipment

- Computer sets

- Multimedia

- Computer Tables

- Chairs

- Printer.

- Internet facility.

## 0715MMT3503 Develop Professionalism

Overview: This competency standard covers the skills and knowledge required to Demonstrate work ethic, resolve problems or disagreements with others, participate in professional development, Work with others, work independently, interpret documentation and Interpret rules regulation of regulatory authorities.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Demonstrate work ethics | P1. Follow principles of work ethics in all situations P2. Adopt professional behaviour |
| CU2. Resolve problems or disagreements with others | - Communicate effectively - Adopt peaceful approach - Determine cause of problem or disagreement - Discuss and resolve issues |
| CU3. Participate in professional development | - Assess own knowledge and skills - Acquire information about training opportunities - Learn through various methods, such as on job training, reading, courses and co-workers |
| CU4. Work in team | - Work as a team member to achieve common goals - Keep open mind - Participate in work place meetings - Communicate accurately and clearly - Co-ordinate job related activities - Cooperate with others |
| CU5. Work independently | P1. Confirm and clarify assignment P2. Take initiative, anticipate and prepare for next steps in job P3. Identify and resolve potential and actual problems P4. Communicate with other site personnel P5. Complete assignment |
| CU6. Interpret documentation | P1. Access and maintain documents P2. Provide complete, legible and accurate information in documents P3. Interpret equipment inspection documentation from previous shifts before conducting pre-operational inspection |
| CU7. Interpret rules and regulation of regulatory authorities | P1. Follow Health and Safety Rules P2. Ensure environmental safety P3. Follow compliance of IMEI Rules. P4. Ensure workplace safety by following safety standards P5. Ensure safety while operating electricity. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles of work ethic and expectations, such as be punctual, prepared for work, cooperative, honest, productive and respectful

- Working conditions on work station

- Legislation about harassment

- Conflict resolution techniques.

- Own role and responsibilities

- Roles and responsibilities of others in industry, such as supervisor, inspector, other trades people

- Log books, safety reports, maintenance reports, inspection reports, time cards

- Health and Safety Rules

- Environmental safety

- Workplace safety standards

- Safety while operating electricity.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate work ethic

- Work with others

- Work independently

- Interpret documentation

- Interpret rules and regulation of regulatory authorities.

## 0715MMT3504 Perform off hand grinding

Overview: This competency standard covers the skills and knowledge required to Adjust Grinding wheel, Adjust tool Rest, Prepare a single point cutting tool for Basic machining purposes, Dress the Wheel and Perform Angle Grinding for finishing

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Adjust Grinding wheel | - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. |
| CU2. Adjust tool Rest | - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that wheel is properly mounted. |
| CU3. Prepare a single point cutting tool for Basic machining purposes | P1. Select marking tools P2. Cut the material as per requirement. P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Give the angle to the tool according to cutting requirement. |
| - Dress the wheel | - Select dressing tools - Ensure that wheel is properly mounted. - Dress tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. |
| CU5. Perform Angle Grinding for finishing | P1. Select marking tools P2. Cut the material as per requirement. P3. Perform surface finishing with angle grinder machine P4. Level the surface with tri-square |

Knowledge & Understanding

- Define types of grinding wheels

- Describe basic measuring

- Describe the Grinding machine types.

- Describe clamping/holding methods.

- Define safe distance from grinding wheel for tool rest

- Describe basic measuring /Marking /cutting tools

- Knowledge of different cutting tool materials.

- Describe the different type of dressing tools.

- Describe safety during handling the angle grinder

Tool & Equipment

- Offhand Grinding Machine

- Bench vices

- Hammer

- Scriber

- Vernier caliper

- Set of spanners

- Angle Grinding Machine

## 0715MMT3505 Perform Surface Grinding

Overview: This competency standard covers the skills and knowledge required to determine job requirements and sequence of operations, Operate surface grinding and Dress the Wheel

| Competency Units | Performance Criteria |
| --- | --- |
| - Determine job requirements and sequence of operations | - Interpret and understand drawings, instructions and specifications. - Determine sequence of operations including job specifications and set-up for maximum efficiency - Select appropriate grinding wheel for job. - Mounted and positioned grinding wheel correctly. |
| - Operate surface grinding | - Select the suitable size and type of grinding wheel. - Mount the work piece over the holding devices to ensure proper clamping. - Dress the wheel with diamond tip if required. - Identify reference points on work piece before grinding. - Adjust depth of cut according to speed of machine table. - Provide coolant continuously to avoid over heating of the job. - Observe personal and workplace safety. |
| - Dress the Wheel | - Select dressing tools - Ensure that wheel is properly mounted. - Dress tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. |

Knowledge & Understanding

- Type and size of wheels and abrasive.

- Method of dressing of grinding wheel.

- Work holding methods which include:

  - Magnet Table

  - Vice

  - Angle Plate

  - Machine base

- Importance of using coolant.

- Methods and techniques for surface grinding.

- Selecting right standing position during grinding.

- Specific safety precautions and guidelines.

- Describe the different type of dressing tools.

- Describe the purposes of dressing

Tool & Equipment

- Surface Grinding

- Machine

- Holding Devices

- Wheel Dresser

- Grinding Wheels

- Wheel Dresser Stand

- Measuring Tools

- Adjustable Wrench

- Allen Key Set

## 0715MMT3506 Perform Cylindrical grinding and centreless grinding

Overview: This competency standard covers the skills and knowledge required to determine job requirements and sequence of operations, Adjust Grinding wheel, Operate Universal cylindrical grinding and Operate centre less grinding

| Competency Units | Performance Criteria |
| --- | --- |
| - Determine job requirements and sequence of operations | - Interpret and understand drawings, instructions and specifications. - Determine sequence of operations including job specifications and set-up for maximum efficiency - Select appropriate grinding wheel for job. - Mounted and positioned grinding wheel correctly. |
| - Adjust Grinding wheel | - Select the right tool for job - Select the wheel type according to material/speed. - Set the wheel on tool post - Balance the wheel - Ensure that wheel is properly mounted. |
| - Operate Universal cylindrical grinding | - Select the suitable size and type of grinding wheel. - Mount work piece according to procedure (between two centers, chuck, collet, face plate). - Ensure the grinding wheel is balanced. - Follow suitable method for universal cylindrical grinding to ensure work specifications. - Provide coolant continuously to avoid overheating of job. - Observe personal and workplace safety. |
| CU4.Operate centerless grinding | - Select the suitable size and type of grinding wheel. - Mount work piece with a blade instead of by centers or chucks. - Ensure the grinding wheel is balanced. - Follow suitable method for universal cylindrical grinding to ensure work specifications. - Apply coolant continuously to avoid overheating of job. - Observe personal and workplace safety |

Knowledge & Understanding

- Define types of grinding wheels

- Describe basic measuring

- Describe the Grinding machine types.

- Describe clamping/holding methods.

- Types of grinding.

- Types and sizes of grinding wheels.

- Procedure of mounting of work piece according to requirements which include:

    - Between Two Centers

    - Chuck

    - Collet and

    - Face Plate

- Importance of balancing the grinding wheel.

- Procedure of universal cylindrical grinding.

- Safety precautions and guidelines specific to cylindrical grinding.

- Procedure of mounting of work piece according to requirements which include:

  - Blade

  - Collet and

  - Face Plate

Tool & Equipment

- Universal cylindrical grinding Machine

- Bench vices

- Hammer

- Scriber

- Vernier caliper

- Set of spanners

- Measuring Instruments

- Grinding Wheels

- Wheel Dresser

- Dog Carrier

- Screw Wrench

- Allen key Set

- Personal Protective Equipment

## 0715MMT3507 Perform Tool & Cutter Grinding

Overview: This competency standard covers the skills and knowledge required to Adjust Grinding wheel, Adjust tool Rest and Operate the Tool and cutter grinding

| Competency Units | Performance Criteria |
| --- | --- |
| - Adjust Grinding wheel | - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. |
| - Adjust tool Rest | - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. |
| - Operate the Tool and cutter grinding | - Select the suitable size, type and shape of grinding wheel. - Mount work piece onto correct attachment for required procedure. - Adjust the attachments according to different types of tools and cutter grinding. - Follow procedure for sharpening of tools and cutter that is safe and appropriate. - Observe personal and safety precautions. |

Knowledge & Understanding

- Define types of grinding wheels

- Describe basic measuring

- Describe the Grinding machine types.

- Describe clamping/holding methods.

- Define safe distance from grinding wheel for tool rest

- Types, sizes and shapes of grinding wheels.

- Types of attachments and their use.

- Procedure of mounting of work-piece on to related attachments.

- Different tools and cutter angles.

- Procedure of sharpening of tools and cutters.

- Safety guidelines and precautions.

Tool & Equipment

- Tool and cutting grinding Machine

- Bench vices

- Hammer

- Scriber

- Vernier caliper

- Set of spanners

- Offhand Grinding Machine

- Diamond dresser tool

- Grinding attachment

- Universal bevel Protector

## 0715MMT3508 Perform Milling Operations

Overview: This competency standard covers the skills and knowledge required to Prepare Blank for Generating the Gear,  Select Tools and Equipment for Gear Cutting, Produce a square shaped work piece, Generate spur gear (Direct Indexing) On Milling Machine, Generate spur gear (Differential Indexing) On Milling Machine, Generate Helical Gear On Milling Machine, Perform slotting or grooving on work piece, Perform drilling or boring using milling machine, Milling a T slot, Bevel gear cutting on milling machine, Practice of spur rack cutting and Practice of helical rack cutting.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare Blank for Generating the Gear | - Interpret drawing and arrange the material according to job requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Check and verify the dimensions of blank for generating gear as per drawing |
| CU2. Select Tools and Equipment for Gear Cutting | - Select the material, type, shape and size of cutter(s) according to the job requirements - Arrange the measuring instruments and holding devices to attain accuracy of the work as per prescribed method |
| CU3. Produce a square shaped work piece | - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select cutters and set machine as per requirements. - Mount cutters and work piece in the machine. - Produce a part matching the process plan and the part print specifications. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. |
| CU4. Generate spur gear (Direct Indexing) On Milling Machine. | - Select gear cutter and indexing plate on a milling machine - Mount indexing plate on indexing head. - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - ENGAGE worm shaft from worm wheel - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| CU5. Generate spur gear (Differential Indexing) On Milling Machine. | - Select gear cutter and indexing plate on a milling machine - Mount indexing plate on indexing head. - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - Revolve index plate forward or backward part of turn while index crank turn to proper spacing - Choose the number close to required division that can be indexed by simple indexing - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| CU6. Generate Helical Gear on Milling Machine | - Select a indexing plate to machine a helical gear on a manual machine, - Mount gear set to engage lead screw and indexing head spindle - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| CU7. Perform slotting or grooving on work piece | - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Set the work piece in machine vice according to procedure. - Select the appropriate cutter as per specifications. - Adjust the milling cutter for slotting and grooving. - Determine the touching point of the work piece. - Produce slotting or grooving on the work piece to the required quality. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. - Observe personal and workplace safety at all time. |
| CU8. Perform drilling or boring using milling machine | - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Select drill or boring tools according to drawings. - Mount and set the required work-holding devices, work piece and cutting tools. - Adjust the RPM of machine according to the standard chart. - Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Shut down the machine at safe position after finishing the work. - Observe personal and workplace safety at all time. |
| CU9. Cut T- slots on Milling machine | - Layout position of a T slot. - Square vertical milling machine with machine table. - Mount work in milling machine. - Machine the center slot to proper depth of T slot by end mill. - Remove end mill and mount proper t slot cutter. - Machine lower part of the slot. |
| CU10. Cut Bevel gear on milling machine. | - Cut The materials to size - Cut workpiece with lathe that is shaped into a gear blank. - Cut gear with a Conflux generator. - Remove Burrs on the teeth with a deburring machine. |
| CU11. Practice of spur rack cutting. | - Hold the work piece in milling machine. - Hold milling cutter in rack milling attachment. - Hold cutter at 90 degrees for spur cutting. - Move the table for each tooth by rack indexing attachment. - Continue operation until required length is obtained. |
| CU12. Practice of helical rack cutting | - Hold the work piece in milling machine. - Hold milling cutter in rack milling attachment. - Hold cutter at some angle for helical cutting. - Move the table for each tooth by rack indexing attachment. Continue operation until required length is obtained. |

Knowledge & Understanding

- List safety hazards related with the milling machine operations.

- Use of dial indicator

- Method of mounting the cutters

- Checking of right angle with the tri- square.

- Explain square milling procedure.

- Safety guidelines and procedures.

- Safety checks for operating milling machine.

- Interpreting information given in the engineering drawings and job specifications.

- Knowledge of spur gear design

- Identifying safety hazards associated with milling machine operations.

- Quality checks procedures and techniques.

- Types of drill or boring tools and their function.

- Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools.

- Method and technique of adjusting RPM of milling machine.

- Safe Boring and milling procedures.

- Techniques of checking quality of components.

- Bevel gear design

- Spur rack and gear design

- Helical rack design

Tools & Equipment

- Measuring and Marking Tools

- Surface Table

- Machine Vice

- Tri square

- Vernier Caliper

- Dial indicator with magnet stand

- Milling cutters

- Personal Protective Equipment

- Milling machine and its accessories

- Slotting cutter

- Depth gauge

- End mil cutter

- Boring unit

- Boring tools

- Drill

- Internal Micrometer

## 0715MMT3509 Generate Gears on Hobbing Machine

Overview: This competency standard covers the skills and knowledge required to Prepare Blank for Generating the Gear, Select Tools and Equipment for Gear Cutting, Set Hobbing Machine for Operations, carry out Hobbing Operations for spur Gear Generating and Carry out Hobbing Operations for Helical Gear Generating

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare Blank for Generating the Gear | - Interpret drawing and arrange the material according to job requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Check and verify the dimensions of blank for generating gear as per drawing |
| CU2. Select Tools and Equipment for Gear Cutting | - Select the material, type, shape and size of cutter(s) according to the job requirements - Arrange the measuring instruments and holding devices to attain accuracy of the work as per prescribed method |
| CU3. Set Hobbing Machine for Operations | - Clamp the gear blank and hob cutter into their holding devices as per standard practice - Maintain safe distance between gear blank and hub cutter as per prescribed method - Adjust the revolution per minute (rpm) of hob cutter and table according to the specifications of work-piece - Adjust the parameters of speed and feed from control unit as per prescribed method |
| CU4. Carry out Hobbing Operations for spur Gear Generating | - Produce simple/angled/differential indexing and divide the gear into required number of divisions - Check the hob cutter and the gear blank that both are positioned properly and adjust them, if required - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| CU5. Carry out Hobbing Operations for Helical Gear Generating | - Check the hob cutter and the gear blank that both are positioned properly and adjust them, if required - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained |

Knowledge & Understanding

- Spur gear design

- Hobbing machine operations

- HELICAL gear design

Tool &Equipment

- Measuring and Marking Tools

- Surface Table

- Hobbing machine and its accessories

- Hobbing machine

## 0715MMT3510 Perform broaching operation

Overview: This competency standard covers the skills and knowledge required to Machining Irregular Surfaces, Make keyways and Make internal gears

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Machine Irregular Surfaces | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work |
| CU2. Make keyways | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select broaching tool and set machine according to job requirements. - Mount broaching tool and work piece in the machine. - Apply Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work. |
| CU3. Make internal gears | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Attach indexing head on the machine. - Select broaching tool and set machine according to job requirements. - Mount broaching tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work |

Knowledge & Understanding

- List safety hazards related with the Broaching machine operations.

- Use of Dial indicator

- Method of mounting of cutting tool

- Checking of angle with the bevel protector.

- V-Shape cutting procedure.

- Safety guidelines and procedures.

- Safety checks for operating Broaching machine.

- Interpreting information given in the engineering drawings and job specifications.

Tool & Equipment

- Broaching Machine

- Machine Vice

- Tri square/bevel protector

- Vernier Calliper

- Dial indicator with magnet stand

- Point cutting tools

- Personal Protective Equipment

## 0715MMT3511 Manufacture Mechanical Projects

Overview: This competency standard requires learners to adeptly manage projects by analyzing requirements, reviewing technical specifications, and confirming project selection with stakeholders. They must select appropriate materials based on detailed assessments and cost considerations, plan and optimize complex machining processes, and accurately calculate all associated costs. The process involves rigorous adherence to machining plans, effective use of tools, and thorough quality checks to ensure dimensional accuracy and compliance with standards. Each step, from material selection to final quality assurance, must be executed with precision and documented comprehensively.

| Competency Units | Performance Criteria |
| --- | --- |
| - Select The Relevant Project. | - Analyze project requirements and objectives. - Review technical drawings and specifications. - Evaluate feasibility and resources needed. - Confirm project selection with stakeholders. |
| - Identify The Right Material. | - Assess material properties relevant to the project. - Compare material options based on cost and availability. - Select material that meets technical specifications. - Document material selection process. |
| - Select The Right Machining Process and Sequence. | - Determine suitable machining processes for the material. - Plan the sequence of machining operations. - Verify compatibility of tools and equipment. - Optimize process for efficiency and quality. |
| - Calculate The Cost of Machining and Material. | - Estimate material costs based on current market prices. - Calculate machining costs including labor and equipment. - Factor in overhead and additional expenses. - Prepare a detailed cost report. |
| - Make The Project. | - Follow the planned machining process and sequence. - Use appropriate tools and techniques for each operation. - Monitor progress and adjust, as necessary. - Complete the project within the planned timeframe. |
| - Ensure Dimensional Accuracy and Quality as Per Drawing or Standard. | - Measure critical dimensions using precision instruments. - Compare measurements to the technical drawing or standard. - Identify and rectify any deviations. - Document the inspection results and quality checks. |

Knowledge & understanding

- Understanding project requirements.

- Evaluating project feasibility.

- Matching projects with skill level.

- Knowledge of material properties.

- Selecting materials based on project needs.

- Understanding material cost and availability.

- Familiarity with various machining processes.

- Determining the appropriate sequence of operations.

- Ensuring process compatibility with materials.

- Estimating material costs.

- Calculating machining and labor costs.

- Understanding overhead and miscellaneous expenses.

- Practical skills in machining and assembly.

- Following safety procedures.

- Effective use of tools and equipment.

- Interpreting technical drawings.

- Using precision measurement tools.

- Implementing quality control techniques.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard. The below-mentioned evidence should be submitted in the form of a portfolio:

- Project selection documentation.

- Material selection sheet, and material properties analysis.

- Machining process plan, sequence of operations chart.

- Document cost estimation, and breakdown of material and machining costs.

- Completed project artifacts, and photographic evidence of project stages.

- Inspection report, measurement, and quality control records, comparison with technical drawings.

- List of Tools, Equipment and Machines

| S.# | Name of Tools Items | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | All Kinds of Files | FLAT FILE Length of file (mm) = 300, Type of cut = Double cut. FLAT FILE Length of file (mm)= 300, Type of cut = Single cut FLAT FILE Length of file (mm) = 300, Type of cut= Rasp cut HALF-ROUND FILE Length of file (mm)= 300, Type of cut = Double cut ROUND FILE Length of file (mm) = 200, Type of cut= Double cut TRIANGULAR FILE Length of file (mm)=150, Type of cut = Double cut. SQUARE FILE Length of file (mm) = 200, Type of cut = Double cut. KNIFE EDGE FILE Length of file (mm) = 250, Type of cut = Double cut. NEEDLE FILE SET(Set of 6 pieces) Length of file (mm) = 200, Type of cut = Double cut. All files made of alloy steel, hardened with wooden/Plastic file handle. | 25 each |
| 2 | Aluminum Ladder – 8 To 10 Feet Height | 8 To 10 Feet Height made of best quality aluminum alloy channel with standard support spacing. | 1 No. |
| 3 | Anvil & Hammers (Assorted Range) | ANVIL Made of cast steel hardened face and horn, square hardy hole. Weight (Kg) = 50 HAMMER (BALL PEEN) Weight of hammer (Kg) = 0.5 HAMMER (CROSS PEEN) Weight of hammer (Kg) = 0.5 HAMMER (STRAIGHT PEEN) Weight of hammer (Kg) = 0.5 All hammers made of Forged steel hardened and ground peen and face. Suitable wooden handle. | 1 No. 25 each |
| 4 | Power Vice | POWER VICE (Pneumatic/Hydraulic Operated) Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm | 1 (Power Vice) |
| 5 | Bearing Pullers | Size = 4, 6 & 8 inches Made of forged steel jaws | 1 each |
| 6 | Bench Vices, Machine Vices, C-Clamps, U-Clamps, Universal Clamping Vices & Swivel Vices | BENCH VICE Made of cast steel/cast iron, hardened jaws, gun metal nut, square threaded spindle. Width of jaws (mm) = 125. Jaws opening (mm) = 150 MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm C-CLAMP Max. Opening = 8 in. Body made of cast iron U-CLAMP Max. Opening = 8 in., Body made of cast iron, UNIVERSAL CLAMPING VICES: Material = Close grain cast Iron, Jaw width = 150 mm, Jaw Height = 50 mm, Max. opening = 400 mm, Compound angle setting swivel bases SWIVEL VICES Material = Close-grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm With a swivel base able to rotate up to 360 degrees. | 25 (bench vice) 02 (M/C Vice) 15 (C-Clamp) 15 (U-Clamp) 02 (UCV) 02 (Swivel Vice) 10 (Hand Vice) |
| 7 | Cabinets & Steel Almirah (Assorted Range) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5ft Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5ft Material = MS Sheet SWG 22 | 25 each |
| 8 | Center Drills (Assorted Range) | Misc sizes ranging from 1-8 Made of High-Speed Steel | 100 each |
| 9 | Dial Indicator | DIAL INDICATOR: - For surface roughness with a magnetic stand, dial the zero setting. Range (mm) = 10 Resolution (mm) = 0.01 | 2 Nos. |
| 10 | Diamond Disc Grinding Wheels for Carbide Materials (Assorted Range) | Resin or vitrified bond grinding wheel having diameter, thickness, and bore compatible with grinding machine | 10 Nos. |
| 11 | Diamond Dressing Tip with Stand | Length: 120 mm Shank Type: Taper Shank dia.: 9mm | 5 Nos. |
| 12 | Dies & Taps Set (Assorted Range) | TAP AND DIE SET: - Consists of suitable taps and dies with tap wrenches and die stokes. System = metric. Size/Capacity (mm) = 4, 6, 8, 10, 12, 14, 16. | 2 sets |
| 13 | Drafting Tables, T-Squares & Set-Squares | Drafting Table Portable (Taper Type) Table size 32'' x 24''Tee Square 600mm Transparent Set Square Transparent medium size 300 mm | 25 Nos. 25 Nos. 25 Nos. |
| 14 | Drill Bits (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs | 3 sets |
| 15 | Drill Chuck with Key | DRILL CHUCK Three jaws hardened and ground, Key holes hardened, with Key. Capacity = 1-13 mm | 2 Nos. |
| 16 | Electric Table Lamps, Torch Lights | Electric Table Lamps Tall enough to illuminate a large area Bright, even, pleasant light Adjustable brightness and color temperature Well-built, solid, sturdy while flexible Energy Efficient LED Torch Lights Adjustable brightness More than 5 hours run time Rechargeable battery (NiCd) Typical Life: 1000 cycles Size: 10 inches Water Resistant | 2 Nos. 10 Nos. |
| 17 | Fire Extinguisher | ABC dry powder/DCP, portable, wall mounted Capacity = 8 Kg | 2 Nos. |
| 18 | First Aid Box | Suitable size having large and small gauze pads, adhesive bandages, wound cleaning agent, scissors, tweezers, adhesive tape, latex gloves, pocket mask, elastic wraps, splints | 1 No. |
| 19 | Geometry Box with Pencil, Rubber, Sharpener, Eraser & Steel Rule | Instruments Box Special Compass Set with Pencil, Rubber, Sharpener, Eraser & Steel Rule | 25 Nos. |
| 20 | Grease & Greasing Guns (Assorted Range) | GREASE GUN Suitable for soft grease or oil, metallic body funnel and pointed nozzle. Reservoir = medium size | 15 Nos. |
| 21 | Grinding Wheel Balancing Stand with Standard Accessories | Heavy-duty ball screw and linear guide Spindle motor: 1.1 kW (2800 rpm) Wheel Grinding Range(Dia.): 75-190mm Wheel Grinding Range(thickness): 25mm X-Y axes wheel stroke: 100mm Grinding Wheel motor rating: 0.5 kW | 1 No. |
| 22 | Grinding Wheels (Surface Grinding Machine, Pedestal Grinding Machine, Cylindrical Grinding Machine (Assorted Size and Shape) | Resin or vitrified bond grinding wheel having diameter, thickness and bore compatible with grinding machine | 10 Nos. |
| 23 | Hammers & Shaft Pullers | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 Shaft Pullers Sizes: 4, 6, 8 inches made of cast iron/steel | 05 Nos. 5 Nos. 5 Nos 5 Nos. 5 5 |
| 24 | Hammers (Assorted Range) | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 | 5 Nos. 5 Nos. 5 Nos. |
| 25 | Hand Hacksaws with Blades, Complete Sets (Assorted Range) with Standard Accessories | HAND HACKSAW FRAME Adjustable frame made of steel tube elliptical section. Suitable for blade 300 mm | 25 Nos. |
| 26 | Height Gauge, Step Gauge, Filler Gauge, Stubs Wire Gauge (Assorted Range) | HEIGHT GAUGE:- Made of high quality steel, rust resistant, base highly polished and lapped, main scale adjustable magnifying glass, with offset scriber carbide point. Working range (mm) = 0-300. Step Gauge(90 pcs set) FEELER GAUGEL:- No. of Leaves = 100. Thickness (mm) = 0.05 - 1 Stubs Wire Gauge hardened steel Range: 08-36 SWG | 2 Nos. 1 set 5 Nos. 5 Nos. |
| 27 | Knurling Tools (Assorted Range) | Knurling, rough, Medium, fine, revolving rollers Dimension of knurling rollers = 20 x 6 x 6 mm | 5 Nos. |
| 28 | Machine Repairing Tool Kits | Machine Repairing Tool Kits containing Open End Spanner Set, Allen Key Set, measuring tape, Screw Driver Set, LED Flashlight, Hammers, Pliers, Hacksaw with blade, Adjustable wrench, punches, utility knife, super glue, safety gloves, safety glasses, mask, ear plugs | 4 Nos. |
| 29 | Machine Vice Universal Clamping Vices | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Heigt = 50 mm Max. opening = 400 mm UNIVERSAL CLAMPING VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw Heigt = 50 mm Max. opening = 400 mm Compound angle setting swivel bases | 4 Nos. 2 Nos. |
| 30 | Maintenance Box | Made of Iron/hard plastic, shock resistant Size: 400 x 300 x 150 mm | 4 Nos. |
| 31 | Measuring Instruments, Marking Tools & Gauges (Assorted Range) | Scribers (straight and off-set), punches, Try Square, Dividers, Spirit Level, Folding Rule, Measuring Tape, inside calipers, outside calipers, odd leg calipers, steel rule, Vernier Calipers, Digital Vernier Calipers, Micrometer, Thread Pitch Gauge, v-block with clamp, Bevel Protractor, surface gauge, Layout ink/dye, Tool Grinding Gauge, plug gauges, snap gauges, ring gauges. | 2 Sets |
| 32 | Multiple Hand Tools | Hand hack saw, utility knife, scissors, pliers, c-clamp, measuring tape, steel rule, hammers, mallets, screw drivers, level, adjustable wrench, pipe wrench, spanners, Allen keys, socket wrench. | 5 Sets |
| 33 | Overall Combination | Overall Combination Arm pocket, back pocket (optional), chest pocket, elasticated waist band, hip pocket wide side access, stud fastened collar, two way zip, 65% polyester, 35% cotton | 25 Nos. |
| 34 | Personal Protective Equipment | Overalls, safety helmet, wide brimmed hat, safety footwear, safety glasses and goggles, gloves, respirators, masks, earmuffs and earpieces. | 30 Sets |
| 35 | Plug and Snape Gauge | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm Material: Tool Steel/chrome steel/carbide Sizes: 10, 20, 30 mm according to ANSI Standard | 2 Sets (Each) |
| 36 | Plug Gauge (Assorted Range) | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm | 2 Sets |
| 37 | Prick Punch (Assorted Range & Types) | PRICK PUNCH Made of high quality steel, hardened and ground point, knurled shank, point angle 60 deg. Length (mm) = 100, 125, 150 Diameter (mm) = 8, 10, 12 | 25(each) |
| 38 | Radius Gauge - Concave & Convex (Assorted Range) | Radius Gauge - Concave & Convex Individually marked, rust proof steel Range: 1-3 mm R---0.25 step & 3.5-7mm R---0.5 step No. of Plates: 30 | 5 Nos. |
| 39 | Reamers (Assorted Range) | High Speed Steel, Straight/Taper shank, straight flute, right hand cut. Sizes: 12, 16, 18, 20,22, 24 mm | 2 Sets |
| 40 | Safety Equipment - Safety Shoes, Safety Gloves, Safety Goggles, Safety Helmet and Ear Plugs | Made of plastic/ glass/leather of best quality to promise safe working in workshop and outdoor environment | 30 Nos. |
| 41 | Scientific Calculator & Multimedia Projector | Scientific Calculator 10 digits display, should be able to process mathematics and statistics functions efficiently Multimedia Projector Display: LCD/LED Pixels:1024 x 768 Brightness: 4000 ANSI lumens minimum Color System: PAL/SECAM/NTSC Computer Compatibility: UXGA/WXGA/SXGA/XGA/SVGA/VGA Projection Method: Front | 30 Nos. 1 Nos. |
| 42 | Shaper Tools (Assorted Range) | SHAPER TOOL Made of high-speed steel cobalt alloy hardened and ground 5/8” x 5/8” x 8” 3/4" x 3/4” x 8” | 10 Each |
| 43 | Smoke Detecting Alarm | Smoke Detecting Alarm (Best Quality) Able to detect smoke easily | 1 Nos. |
| 44 | Spray Guns & Oil Containers | Spray Guns Nozzle Size = 1.4 - 1.8mm Air Inlet = 1/4 inch Type of feed = Gravity Recommended Air Pressure = 2 bar Approx. Paint Capacity = 1000ml Oil Containers Transparent body having graduated marking 10 liter/quart drum body and stretch spout lid with proper sized opening. | 5 Nos. 5 Nos. |
| 45 | Standard Lathe Machine attachments | STANDARD LATHE MACHINE ATTACHMENTS 3-jaw self-centering chuck 4-jaws independent chuck Face plate with slots Square tool post Steady rest Follow rest Longitudinal stop, Mandrill , Lathe Dog Dead centers, Live Centers, Revolving Center Taper sleeve | 10 Sets |
| 46 | Steel Cabinet and Steel Almirah (03 for Each Machine) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5 Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 25 Each |
| 47 | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves etc. | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves made of high quality thermosetting and durable plastic. | 25 Each |
| 48 | Step Gauges Set | Rectangular shape, micro lapped, heat treated, high corrosion resistance steel. No of pieces per set = 103 Size (mm) = 1.01-1.49 Steps = 0.01 mm | 1 Set |
| 49 | Surface Analysis and Inspection Gauges (Assorted Range) | Type: Flat Surface Processing: Paper Finish, Grinding, Shaping, Milling | 1 No. |
| 50 | Surface Plate with Stand (Cast Iron / Graphite) 24 X 24 Inches | CAST IRON SURFACE PLATE Made of close grain cast-iron high-quality finish. Length x Width x Thickness(inch) = 24 x 24 x 1 | 2 Nos. |
| 51 | Threading Tools (Assorted Range) | Made of H.S.S. able to cut BSW and American Standard Threads, Acme thread, square thread Angle: 55 and 60 degrees | 10 Sets |
| 52 | Threads Gauge -Inches / Millimeters (Assorted Range) | Threads Gauge -Inches / Millimeters Size = 4 - 42 TPI and 0.5-10mm No of leaves = 30 Angle (degrees) = 55 & 60 | 5 Nos. |
| 53 | Turning, Parting, Grooving and Forming Tools etc. (Assorted Range) | Made of H.S.S. 200 mm long with square and rectangular cross sections | 10 Sets |
| 54 | Twist Drill Bits and Boring Bars (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs Boring Bars with holders Sizes with H.S.S. tool bits = 6 mm | 3 Sets 10 Sets |
| 55 | Wooden Cabinet, Steel Almirah | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 5 Each |
| 56 | Wooden Cabinets | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood | 5 Nos. |
| 57 | Work-Bench, Work-Table & Stools | Work-Bench L X W X H = 5 X 3 X 2.5 ft. Angle Iron used for legs 64x5mm Wooden Top 40mm thick Work-Table L X W X H = 8 X 4 X 2.5 ft. Stools Four legs made of MS pipe gauge 16SWG 2ft high, 1.25 x 1.25 ft square Wooden seat 25mm thick. | 25 Sets |
| 58 | Universal Milling Machine with Standard Accessories | Table size – 1000x260mm (minimum),Number of T-slots-3 (Minimum),T-slot size -14-16mm,Spindle to table-0-350mm,Number of spindle speeds – 12 steps, Spindle speed range – 50 to 1200 rpm, Swivel of table 45ᵒ,Spindle taper: Horizontal: ISO40,Vertical: ISO40,Table travel Vertical -400-450mm,Table Travel Cross – 280-300mm,Main spindle motor – 3-3.5KW 3Ph,Coolant pump, Machine Weight – 1100 Kg (minimum), without any attachment and accessories. Accessories: • Milling Machine vice, • Tail stock, • Milling arbor 400mmxΦ27mm, • Milling arbor 400mm x Φ22mm, • Stub arbor Φ27mm, • Stub arbor Φ22mm. Standard Accessories: Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set ,Collet holders & collets, L-Key set, Coolant System, Machine light, Centralized lubrication, Table guard,3 Axis Digital Read Out, chip tray for Base, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, • Slotting head,• Universal Head (Vertical),• Rotary Table | 2 Nos, |
| 59 | Standard Milling Machine Attachments (Sets) | Milling Machine vice, Swivel Base vice, Tailstock, Milling arbor 400mmxΦ27mm, Milling arbor 400mm x Φ22mm, Stub arbor Φ27mm, Stub arbor Φ22mm, Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier), spanner Set, Collet holders & collets, L-Key set,3 Axis Digital Read Out, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, Slotting head, Universal Head (Vertical). | 2 Nos. |
| 60 | File Brush | 3 inches | 25 Nos. |
| 61 | Digital Vernier Caliper | 160mm | 02 Nos |
| 62 | Counter sink | 18x60⁰, 12.5x50⁰ | 5 set |
| 63 | Counter Bore | 6, 10, 12 mm | 5 set |
| 64 | Digital Micro Mete | 0-25 mm | 02 Nos |
| 65 | Digital Micrometer | 25-50 mm | 02 Nos |
| 66 | Chisel Flat, Groove, Crosscut | 8 inches | 5 Each |
| 67 | Number Punch | 4 mm | 02 Sets |
| 68 | Letter Punch | 4 mm | 02 Sets |
| 69 | Lathe Machine (with standard accessories) | Center height – 160-200 mm, Maximum swing over bed – 320-400mm, Maximum swing over saddle – 180-190mm, Maximum swing in gap – 460-470mm, Length of gap – 200-250mm, Distance between centers - 750mm, Spindle bore – 38mm, Spindle taper – MT5, Tailstock taper – MT3, Bed width – 180-190mm, Number of spindle speeds – 8, Range of spindle speeds – 70-2000 rpm, Metric threads (26 pitches) – 0.4-7mm, Imperial threads – (34 pitches) – 4-60 TPI, Lead screw dia./pitch – 22/3mm, Travel of compound slide: 75mm, Travel of cross slide – 130mm, Tailstock quill travel – 100mm, Main Motor – 1.5-2 KW 3phase, Coolant Motor, Tool Section – 16mm, Net Weight of machine – 525 to 550 Kg without any attachment and accessories. Standard Accessories: 160mm 3 jaw chuck(Universal), 200mm 4 jaw chuck(Independent), 250mm face plate, Fixed steady, Traveling steady, Electrically interlocked chuck guard, 4 way indexing tool post stand, Tool post chip guard, Rear splash guard, Spindle foot brake, Thread dial indicator, Coolant system, Low volt machine light, Tool box(Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier). | 10 Nos. |
| 70 | Pedestal Grinder with Tools, Cutting Angle Support | Ball bearing mounted spindle, suitable for tool grinding, protection glass/clear plastic eye shield with lamp, grinding wheel = 250 x 30 x 25 mm, Spindle speed = 2500-3000 rpm, Motor rating 3 hp/440V/50 Hz = 600–750-Watt, Quick angle setting of = 30-90 degree. Attachment: Wheel dresser star=Standard type / Diamond, Complete in all respects. | 2 Nos. |
| 71 | Surface Grinding Machine | Working surface of Table (W x L) 250 x 500mm Minimum, Table travel (y,x)-250 x 550mm Minimum, Minimum distance spindle centre-table-400mm, Magnetic Chuck-size 200 x 450mm, Longitudinal feeds 5-25m/min, Automatic Cross Feed of Saddle 0.5-10mm, Grinding wheel size- 180 x 13 x 31.75mm, Speed-2800 rpm Minimum, Spindle motor -1.5-2 KW, 3phase, Hydraulic pump motor-0.75 KW, Coolant pump motor, Elevating motor, Cross feed motor, Total rated power- 3KW, 3phase, Maximum table load incl.chuck-120 Kg, Net weight of machine-750 Kg Minimum. STANDARD ACCESSORIES: Wheel balancing base, Wheel balancing arbor, Grinding wheel, Diamond dresser fit on the top of machine spindle grinding wheel, Toolbox (Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier), Low volt machine light, Built in demagnetizer, Coolant system, 200 x 450mm electro-magnetic chuck. | 2 Nos. |
| 72 | Cylindrical Grinding Machine | Distance between center = 500-600mm, Center height = 120-150 mm, Max dia. grind 100mm, Spindle taper MT4, Maximum transverse Travel = 120-150mm, Swiveling angle 0-10˚, Spindle speed = 2500-3000 RPM, Tail stock taper MT4, Motor 4Kw, Internal Grinding motor 1-1.25Kw, Work head motor 0.5-1Kw, Hydraulic longitudinal feed, Wheel head manual feed, Wheel head & Work head (Swivel Type). | 2 Nos. |
| 73 | Tool/Cutter Grinding Machine | Suitable to grind all types of tool and cutters, Clamping area of table (mm) =820x 140, Longitudinal table travel (mm) =450, Lateral table travel (mm) = 225,Distance between centers (mm) = 600-650,Height of center (mm) =125-130,Spindle speed (rpm) =2800- 5600,Motor power 3ph = 1 kW, Table angle setting (degrees) =±450,Grinding head tilting on vertical axis (degrees) = ±3600,Grinding head tilting on horizontal axis (degrees) = ±150. Attachments: Face milling cutter grinding attachment (mm) = 450, Radius grinding attachment (mm) = 0-25,Cylindrical grinding attachment (mm) = 250,Internal grinding attachment (mm) =12-50,Indexing attachment = 01,Extended grinding wheel spindle (mm) = 75,Universal vice (mm) = 100x25x75,Drill point grinding attachment (mm) = 3-25,Lathe and planer tool grinding attachment (mm) =10-38,Set of spring collets (mm) = 6, 8, 10, 12 ,16, 20. Accessories : Work head = 1,Left hand tail stock = 1,Right hand tail stock = 1,Diamond wheel dresser = 1,Setoff wheel guards = 1,Universal tooth rest = 1,Center height gauge = 1.Collect wrench = 1,Ejector rod = 1,T- wrench = 1,Pin wrench = 1,Grinding wheel sleeves = 5,Set of grinding wheels different shapes = 7,Spindle extension 100 mm =1,Belt for spindle drive = 1,Suitable tool cabinet = 1,Instruction / operation manual, complete in all respect, COMPLETE AND COMPATIBLE IN ALL RESPECTS, OR EQUIVALENT OF BEST QUALITY. | 1 No. |
| 74 | Centerless Grinding Machine along with Standard Accessories | Speed = 20-220 RPM (Step less Variable), Grinding Wheel = Diameter - 350 mm x 100 Width, Bore Size = 127mm,Grinding Capacity = Dia. - 2mm (min), 70mm (max),Machine Weight = 2200 KG (Approx.),Coolant Pump Motor = 0.12 KW, Regulating Wheel Motor = 0.75 KW DC, Width = 100 mm, Regulating Wheel = Diameter - 230 mm, Swivel Angle = Swivel of grinding dresser - +/- 5 Degree, Regulating Swivel Angle = +/- 5 Degree Horizontal, Dresser Operation = Grinding & Regulating Wheel - Hydraulics, Power Requirement = Grinding Wheel Motor - 5.5 KW, Power Pack Pump = 0.18 KW. | 1 No. |
| 75 | Pillar Drilling Machines | PILLAR DRILL MACHINE: - V-belt drive with 12 spindle speeds, table raise/lower by means of gear and rack. Chuck capacity (mm) = 16 Depth of feed (mm) = 80 Distance pillar to spindle center(mm) = 300 Table size (mm) = 230 x 240. Spindle speeds (rpm) = 220 – 3000. Spindle Morse taper number = MT-2. Motor 3-Ph./440V/50Hz = 0.75KW. Drill Press Vice = 100 mm. Drill chuck with arbor = 1-16mm. Magnet stator switch of on/off | 1 No. |
| 76 | Power Hacksaw Machine | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm,Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm,Width of stand = 350mm,Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY. | 2 No. |
| 77 | Shaper Machine with Standard Accessories | Stroke length = 400-450mm, No. of speeds =2-8, Strokes per mint = 20-130, Vertical Travel of tool slide=150mm or greater, Table automatic feed (5)=0.2 to 2 mm/stroke, Motor rating 3 ph/440v/50 Hz =1.25-1.5 KW, Shaper Vice (Swivel Base)=250-300 mm width of jaws, Note: - Along with all standard accessories machine handles, set of spanner. Ellen Key set and complete in all respect. | 2 No. |
| 78 | Work Bench and Stools (Set) | Work-Bench L X W X H = 5 X 3 X 2.5 ft. Angle Iron used for legs 64x5mm Wooden Top 40mm thick Stools Four legs made of MS pipe with wall thickness 3mm, 2ft high 1.25 x 1.25 ft square Wooden seat 200mm thick. | 5 Nos. |
| 79 | Machine Vices, Swivel Vices (Each) | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm SWIVEL VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm With swivel base able to rotate up to 360 degrees. | 2 Nos. |
| 80 | Measuring and Marking Tools (Assorted Range) (Sets) | Scribers (straight and off-set), punches, Try Square, Dividers, Spirit Level, Folding Rule, Measuring Tape, inside calipers, outside calipers, odd leg calipers, steel rule, Vernier Calipers, Digital Vernier Calipers, Micrometer, Thread Pitch Gauge, v-block with clamp, Bevel Protractor, surface gauge, Layout ink/dye. | 2 Nos. |
| 81 | Work Holding Devices and Attachments | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm | 2 Nos. |
|  |  | SWIVEL VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm With swivel base able to rotate up to 360 deg | 4 Nos. |
|  |  | C-CLAMP Max. Opening = 8 in. Body made of cast iron | 10 Nos. |
|  |  | U-CLAMP Max. Holding capacity = 200mm | 10 Nos. |
| 82 | Plug and Snape Gauges (Sets) | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm Material: Tool Steel/chrome steel/carbide Sizes: 10, 20, 30 mm according to ANSI Standard | 2 Nos. |
| 83 | Twist Drill Bits and Boring Bars (Assorted Range) | TWIST DRILL SET (set): - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs | 3 Nos. |
|  |  | Boring Bars Miscellaneous Sizes with H.S.S. tool bits (set) | 10 Nos. |
| 84 | Side and Face Cutters, End Mill Cutters, Ball Mill Cutters, Shell-End Mill Cutters, Slotting Cutters, Grooving Cutters etc. (Assorted Range) Each | Side and Face Cutters = 100 x 8 x 27, 100 x 10 x 27, 100 x 12 x 27, 100 x 14 x 27 End Mill Cutters = 5, 6, 8, 10, 12 , 14 , 16 Ball Mill Cutters = 5, 6, 8, 10, 12 , 14 , 16 Shell-End Mill Cutters = 40 x 40 x 27, 60 x60 x 27, 100 x 50 x 27 Slotting Cutters = 80 x 3 x 27, 80 x 5 x 27 Grooving Cutters = 63 x 2 x 22 Made of H.S.S. and cutter bore(22 or 27mm) compatible with milling arbor diameter. | 10 Nos. |
| 85 | Common Kinds and Sizes Of Files (Assorted Range) | FLAT FILE Length of file (mm) = 300, Type of cut = Double cut. FLAT FILE Length of file (mm)= 300, Type of cut = Single cut FLAT FILE Length of file (mm) = 300, Type of cut= Rasp cut HALF ROUND FILE Length of file (mm)= 300, Type of cut = Double cut ROUND FILE Length of file (mm) = 200, Type of cut= Double cut TRIANGULAR FILE Length of file (mm)=150, Type of cut = Double cut. SQUARE FILE Length of file (mm) = 200, Type of cut = Double cut. KNIFE EDGE FILE Length of file (mm) = 250, Type of cut = Double cut. NEEDLE FILE SET(Set of 6 pieces) Length of file (mm) = 200, Type of cut = Double cut. All files made of alloy steel, hardened with wooden/Plastic file handle. | 25 Nos. |
| 86 | Steel Cabinet and Steel Almirah (03 for Each Machine) Each | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5 Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 25 Nos. |
| 87 | Hammers (Assorted Range) Each | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 | 5 Nos. |
| 88 | Personal Protective Equipment (Sets) | Overalls, safety helmet, wide brimmed hat, safety footwear, safety glasses and goggles, gloves, respirators, masks, earmuffs and earpieces. | 30 Nos. |
| 89 | First Aid Box | Suitable size having large and small gauze pads, adhesive bandages, wound cleaning agent, scissors, tweezers, adhesive tape, latex gloves, pocket mask, elastic wraps, splints | 1 Nos. |
| 90 | Maintenance Tool Kit, General Repairing Tool Kit & Hands Tool Kit (Sets) | Machine Repairing Tool Kits containing Open End Spanner Set, Allen Key Set, measuring tape, Screw Driver Set, LED Flashlight, Hammers, Pliers, Hacksaw with blade, Adjustable wrench, punches, utility knife, super glue, safety gloves, safety glasses, mask, ear plugs | 2 Nos. |
| 91 | Radius Gauge - Concave & Convex (Assorted Range) | Radius Gauge - Concave & Convex Individually marked, rust proof steel Range: 1-3 mm R---0.25 step & 3.5-7mm R---0.5 step No. of Plates: 30 | 5 Nos. |
| 92 | Dial Indicator with Magnet Stand | DIAL INDICATOR: - For surface roughness with magnetic stand, dial zero setting. Range (mm) = 10 Resolution (mm) = 0.01 | 2 Nos. |
| 93 | Involute Gear Cutter Sets | H.S.S. M1 PA14, H.S.S. M1.75 PA14, H.S.S. M2 PAPA14, H.S.S. M2.5 PA14 08 cutters in each module (1 Set (Each Module) | 1 Nos. |
| 94 | Hob Cutters Sets | H.S.S. M1 PA20, H.S.S. M1.75 PA20, H.S.S. M2 PA20, H.S.S. M2.5 PA20 (1 Set (Each Module) | 1 Nos. |
| 95 | Tooth Vernier | TOOTH VERNIER: - Suitable to measure gear tooth thickness in module system. Horizontal beam (mm)= 100 Vertical beam (mm) = 80 LC (mm) = 0.02 | 2 Nos. |
| 96 | Lathe Chucks and Steady Rests (Assorted Range) Sets | 3-jaw self-centering chuck = 160 mm 4-jaws independent chuck = 200 mm Steady rest | 2 Nos. |
| 97 | Turning, Parting, Grooving and Forming Tools etc. (Assorted Range) Sets | Made of H.S.S. 200 mm long with square and rectangular cross sections | 10 Nos. |
| 98 | Universal Milling Machine with Standard Accessories | Table size – 1000x260mm (minimum),Number of T-slots-3 (Minimum),T-slot size -14-16mm,Spindle to table-0-350mm,Number of spindle speeds – 12 steps, Spindle speed range – 50 to 1200 rpm, Swivel of table 45ᵒ,Spindle taper: Horizontal: ISO40,Vertical: ISO40,Table travel Vertical -400-450mm,Table Travel Cross – 280-300mm,Main spindle motor – 3-3.5KW 3Ph,Coolant pump, Machine Weight – 1100 Kg (minimum), without any attachment and accessories. Accessories: • Milling Machine vice, • Tail stock, • Milling arbor 400mmxΦ27mm, • Milling arbor 400mm x Φ22mm, • Stub arbor Φ27mm, • Stub arbor Φ22mm. Standard Accessories: Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set ,Collet holders & collets, L-Key set, Coolant System, Machine light, Centralized lubrication, Table guard,3 Axis Digital Read Out, chip tray for Base, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, • Slotting head,• Universal Head (Vertical),• Rotary Table | 6 Nos. |
| 99 | Power Hacksaw Machine with Blade | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm,Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm,Width of stand = 350mm,Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY | 1 Nos. |
| 100 | Standard Milling Machine Attachments (Sets) | Milling Machine vice, Swivel Base vice, Tail stock, Milling arbor 400mmxΦ27mm,Milling arbor 400mm x Φ22mm,Stub arbor Φ27mm,Stub arbor Φ22mm,Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set, Collet holders & collets, L-Key set,3 Axis Digital Read Out, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, Slotting head, Universal Head (Vertical). | 6 Nos. |
| 101 | Boring Head with Boring Tools (Sets) | Compatible with vertical milling machine able to perform boring operation using various boring tools | 2 Nos. |
| 102 | Gear Generator Machine, Gear Hobbing Machine along with Standard Accessories | Vertical differential type machine capable to cut spur gears, helical gears and worm gears, heaving even, odd, prime number of teeth. Built in coolant and lubrication pump, Maximum module for gears (mm) = 6,Maximum diameter of gear hob wit out outer support (mm) = 400-500,Maximum diameter of helical gears hobbed (mm) = 370,Hob diameter (mm) = 120,Hob arbor diameter (mm) = 22, 27, 32.Work mandrel diameter (mm) = 30,Spindle speeds (8) rpm = 50-300,Hob down feed per rev. of job (mm) = 0.24-4,Main drive motor 3Ph 1420 rpm = 3kw,With all standard accessories, tool kit, tool cabinet, operation manual, Maintenance manual etc, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY. | 2 Nos. |
| 103 | Lathe Machine along with Standard Accessories | Centre height – 160-200 mm, Maximum swing over bed – 320-400mm,Maximum swing over saddle – 180-190mm,Maximum swing in gap – 460-470mm,Length of gap – 200-250mm,Distance between centers - 750mm,Spindle bore – 38mm,Spindle taper – MT5,Tailstock taper – MT3,Bed width – 180-190mm,Number of spindle speeds – 8,Range of spindle speeds – 70-2000 rpm, Metric threads (26 pitches) – 0.4-7mm,Imperial threads – (34 pitches) – 4-60 TPI, Lead screw dia./pitch – 22/3mm,Travel of compound slide : 75mm,Travel of cross slide – 130mm,Tailstock quill travel – 100mm,Main Motor – 1.5-2 KW 3phase,Coolant Motor ,Tool Section – 16mm,Net Weight of machine – 525 to 550 Kg without any attachment and accessories, Standard Accessories,160mm 3 jaw chuck(Universal),200mm 4 jaw chuck(Independent),250mm face plate, Fixed steady, Traveling steady, Electrically interlocked chuck guard,4 way indexing tool post stand, Tool post chip guard, Rear splash guard, Spindle foot brake, Thread dial indicator, Coolant system, Low volt machine light, Tool box(Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier). | 6 Nos. |
| 104 | Power Hacksaw Machine along with Standard Accessories | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm, Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm, Width of stand = 350mm, Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, OR EQUIVALENT OF BEST QUALITY. | 2 Nos. |
| 105 | Pedestal Grinder with Tools Cutting Angle Support | Ball bearing mounted spindle, suitable for tool grinding, protection glass/clear plastic eye shield with lamp, grinding wheel = 250 x 30 x 25 mm, Spindle speed = 2500-3000 rpm, Motor rating 3 ph/440V/50 Hz = 600–750-Watt, Quick angle setting of = 30-90 degree. Attachment: Wheel dresser star=Standard type / Diamond, Complete in all respect. | 2 Nos. |

- List of Consumables

| Sr. No | Name | Qty. Required |
| --- | --- | --- |
| 1 | Cleaning Supplies (E.G., Wipes, Disinfectants) | Sufficient Supply |
| 2 | Safety Posters | 10-15 Nos. |
| 3 | Markers And Highlighters | 30 Nos. |
| 4 | Compostable Bags | 10 Rolls |
| 5 | First Aid Kit | 10 Nos. |
| 6 | Fire Extinguishers | 5 Nos. |
| 7 | Rags And Wipes | 40 Kg |
| 8 | Gloves | 100 Pairs |
| 9 | First Aid Supplies | 5 Kits |
| 10 | Masking Tape | 50 Rolls |
| 11 | WD-40 or Equivalent Lubricant | 5 Cans |
| 12 | Paper Towels | 50 Rolls |
| 13 | Lubricating Oil | 100 Liters |
| 14 | Grease | 3 Kg |
| 15 | Machine Oil | 50 Liters |
| 16 | Trash Bags | 50 Nos. |
| 17 | Disposable Gloves | 100 Pairs |
| 18 | Cleaning Detergent | 3 Kg |
| 19 | Safety Goggles | 50 Nos. |
| 20 | Kerosene Oil | 20 Liter |
| 21 | Lubricating Oil | 70 Liters |
| 22 | Sandpaper | 150 Sheets |
| 23 | Polishing Compound | 5 Nos. |
| 24 | Polishing Cloths | 50 Nos. |
| 25 | Ear Protection | 25 Nos. |
| 26 | Safety Gloves | 50 Pairs |
| 27 | Hand Sanitizer | 10 Bottles |
| 28 | Disposable Masks | 50 Nos. |
| 29 | Hand Grinding Disk | 30 Nos. |
| 30 | Twist Drill Set | 10 Set |
| 31 | File Brush | 10 Nos. |
| 32 | Grinding Wheels | 5 Nos. |
| 33 | Cutting Oil | 50 Liter |
| 34 | Aprons | 30 Nos. |
| 35 | Surface Grinding Wheels | 2 Nos. |
| 36 | Cylindrical Grinding Wheels | 2 Nos. |
| 37 | Tool/Cutter Grinding Wheels Different Types of Sets | 2 Sets. |
| 38 | Paper Towels | 50 Nos. |
| 39 | Rags And Wipes | 10 Kg |
| 40 | Carbide Tools of Different Shapes and Sizes | 50 Nos. |
| 41 | Tool Bits of Different Shapes and Sizes | 50 Nos. |
| 42 | Chips Cleaning Brushes | 50 Nos. |
| 43 | Shaper Cutting Tools of Different Types and Shapes | 25 Nos. |
| 44 | Tool Bits of Different Sizes and Shapes | 100 Nos. |
| 45 | Milling Inserts Different Types | 100 Nos. |
| 46 | Milling Cutters of Different Types and Shape | 100 Nos. |

- Members of Qualifications Development, Validation & Review Committee

The Qualifications Development, Validation & Review Committees consisted of following members:

| S. # | Name |  | Designation | Organization |
| --- | --- | --- | --- | --- |
| 1. | Dr. Mirza Nadeem Baig |  | Principal Engineer | Pakistan Welding, Islamabad |
| 2. | Muhammad Asad Saeed |  | Engineer | Infinity Engineering |
| 3. | Furqan-Ur- Rahman |  | Director | Pak Industries |
| 4. | Muhammad Imran Ihsan |  | In-Charge (CNC Machine Shop) | Light Engineering, Gujranwala |
| 5. | Umar Hayat |  | Executive Maintenance & Project | Atlas Honda Ltd. |
| 6. | Muhammad Younas |  | Ex. Assistant Manager | Staff Training Collage, Lahore |
| 7. | Hafiz Muhammad Saqib |  | Assistant Manager | Dynamic Tooling Service |
| 8. | Ihssan Ali |  | Production Operator | Dic. Color Comfort by Chemistry |
| 9. | Shahzad Zafar |  | Facilitator (Ex. Assistant Manager) | Staff Training Collage, Lahore |
| 10 | Muhammad Ramzan |  | Sales Engineer Dynamic | Tooling Service |
| Qualifications Validation Committee |  |  |  |  |
| 1. |  | M. Aslam Khattak | Dy. General Manager | HMC, Taxila |
| 2. |  | Shamshad Gill | Junior Officer (Mech) | HMC, Taxila |
| 3. |  | Muhammad Imran | In-Charge, (CNC Shop) | Light Engineering, Gujranwala |
| 4. |  | Muhammad Younas | Shift In-Charge | Sultan Traders, Ferozpur Road, Lahore |
| 5. |  | Abdul Redman | Assistant Manager | Millet Tractors Ltd. Lahore |
| 6. |  | Sadam Hussain | Trainer | PITAC, Lahore |
| 7. |  | Nawaz Malik | In-Charge Mech. W. shop | Kohinoor Textile Mills Limited |
| 8. |  | M. Ali Akhter | Assistant Manager | Kohinoor Textile Mills Limited |
| 9. |  | M. Tayyab Murtaza | CNC Shop Supervisor | Tool Dies & Molds Center, Gujranwala |
| 10 |  | Gul Naseem Khan | Instructor | SVTI Abbottabad |
| 11 |  | Shahzad Zafar | Ex. Assistant Manager | GSTC, Gulberg, Lahore |

| Members of Qualifications Review Committee |  |  |  |
| --- | --- | --- | --- |
| 1 | Mr. Fayyaz Khan | Sr. Instructor Machinist | PVTC Lahore |
| 2 | Mr. Sajid Raza Zaidi | Lecturer Mechanical | NUTECH Islamabad |
| 3 | Mr. Safdar Ali | Director Technology | Pakistan Air Force (PAF) |
| 4 | Mr. Athar Ali Soomro | Senior Instructor Machinist | Sindh - TEVTA |
| 5 | Mr. Faheem Naeem | Instructor Machinist | Punjab - TEVTA |
| 6 | M. Dawood ul Hassan | Jr. Instructor Mechanical | Punjab - TEVTA |
| 7 | Mr. Sajid Mushtaq | Instructor Machine-Shop | Punjab - TEVTA |