|  | # Assessment Evidence Guide # for # “Mason Helper Level 2” Carryout Plaster Work on Brick Masonry (Formative Assessment) |
| --- | --- |
|  | 21th to 25th June 2021 |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: National Vocational Certificate Level -2 in Construction (Mason Helper) |  | CS Code: | Level: 2 | Version: 01 |
| --- | --- | --- | --- | --- |
| Competency Standard Title: Carryout Plaster Work on Brick Masonry Maintain Personal Health, Hygiene and Safety Perform Basic Communication Carryout General Maintenance Perform Basic Green Skills for Construction |  | Assessment Date (DD/MM/YY): Assessment Time: 5 hrs |  |  |
| Candidate Details | Name: Registration/Roll Number: |  |  |  |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment): Assessment Task 1: Candidate is required to perform plastering on vertical surface of wall as per instructions given by assessor (for reference see ANNEX-I) Assessment Task 2: Candidate is required to perform plastering on ceiling of slab as per instructions given by assessor. And complete: - Knowledge assessment test (Written or Oral) - Portfolios at the time of assessment (if any) |  |  |  |
| Minimum Evidence Required | During a practical assessment, under observation by an assessor, you will complete: Assessment Task 1 Performance criteria 1:Select material as per specifications Performance criteria 2:Select tools according to the task Performance criteria 3:Fix level pegs on wall with reference to the off lines Performance criteria 4:Prepare mortar according to the given ratio Performance criteria 5:Rack the joints Performance criteria 6:Chip off and clean the wall Performance criteria 7:Apply mortar with trowel/float on wall surface within the level pegs Performance criteria 8:Remove extra mortar from off line of pegs Performance criteria 9:Fill depression of surface with additional mortar Performance criteria 10:Level the plaster surface with straight edge Performance criteria 11:Finish the plaster surface with float Performance criteria 12:Apply foam for smooth finish Performance criteria 13:Perform curing after specified time Performance Criteria 14:Interpret work processes and procedures correctly Performance Criteria 15: Select personal protective equipment in terms of type and quantity according to work orders. Performance Criteria 16: Wear, adjust, and maintain personal protective equipment to ensure correct fit and optimum protection in compliance with company procedures. Performance Criteria 17:Comply with Health, hygiene and safety precautions before starting work Performance Criteria 18:Deal with resolvable problems according to prescribed procedures Performance Criteria 19:Carefully listen and note down the instructions of Supervisor Performance Criteria 20:Apply appropriate methods and techniques for cleanliness and maintenance of machines & tools Performance Criteria 21:Fill the checklist for daily cleanliness of the workplace Performance Criteria 22:Identify various types of construction waste Performance Criteria 23:Apply waste reduction techniques during construction process Performance Criteria 24:Ensure appropriate storage of water |  |  |  |
|  | Assessment Task 2 Performance criteria 1:Select appropriate material as per specifications Performance criteria 2:Select appropriate tools according to the task Performance criteria 3:Fix level pegs on ceiling with reference to the off lines Performance criteria 4:Prepare mortar according to the given ratio Performance criteria 5:Chip off and clean the ceiling Performance criteria 6:Apply cement slurry on ceiling Performance criteria 7:Apply mortar with trowel/float on wall surface within the level pegs Performance criteria 8:Remove extra mortar from off line of pegs Performance criteria 9:Fill depression of surface with additional mortar Performance criteria 10:Level the plaster surface with straight edge Performance criteria 11:Finish the plaster surface with float Performance criteria 12:Apply foam for smooth finish Performance criteria 13:Perform curing after specified time Performance Criteria 14: Identify risk to Health, hygiene and safety Performance Criteria 15: Maintain cleanliness and hygiene as per organizational policy Performance Criteria 16: Follow organizational Health, hygiene and safety guidelines during work Performance Criteria 17: Report unresolved problems to immediate supervisor Performance Criteria 18: Place the tools equipment etc at their prescribed place after completion of work Performance Criteria 19: Carry out the instructions of the supervisor Performance Criteria 20: Respond appropriately to safety hazards on all tools and machines Performance Criteria 21: Categorize reusable waste Performance Criteria 22: Dispose unusable waste as per set standards Performance Criteria 23:Avoid wastage of water |  |  |  |

Continued on following page

Assessors Judgment Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |  |  |  |  |  |  |  |
| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |  |
| Activity |  | Method |  |  |  |  | Result |  |
| Nature of Activity |  | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |  |

| Each Assessment Task (with performance criteria) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assessment Task 1 |  |  | Description of assessment task 1 Candidate is required to perform plastering on vertical surface of wall as per instructions given by assessor (for reference see ANNEX-I) |  |  |  |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select material as per specifications |  |  |  |  |  |
|  | Select tools according to the task |  |  |  |  |  |
|  | Fix level pegs on wall with reference to the off lines |  |  |  |  |  |
|  | Prepare mortar according to the given ratio |  |  |  |  |  |
|  | Rack the joints |  |  |  |  |  |
|  | Chip off and clean the wall |  |  |  |  |  |
|  | Apply mortar with trowel/float on wall surface within the level pegs |  |  |  |  |  |
|  | Remove extra mortar from off line of pegs |  |  |  |  |  |
|  | Fill depression of surface with additional mortar |  |  |  |  |  |
|  | Level the plaster surface with straight edge |  |  |  |  |  |
|  | Finish the plaster surface with float |  |  |  |  |  |
|  | Apply foam for smooth finish |  |  |  |  |  |
|  | Perform curing after specified time |  |  |  |  |  |
|  | Interpret work processes and procedures correctly |  |  |  |  |  |
|  | Select personal protective equipment in terms of type and quantity according to work orders. |  |  |  |  |  |
|  | Wear, adjust, and maintain personal protective equipment to ensure correct fit and optimum protection in compliance with company procedures. |  |  |  |  |  |
|  | Comply with Health, hygiene and safety precautions before starting work |  |  |  |  |  |
|  | Deal with resolvable problems according to prescribed procedures |  |  |  |  |  |
|  | Carefully listen and note down the instructions of Supervisor |  |  |  |  |  |
|  | Apply appropriate methods and techniques for cleanliness and maintenance of machines & tools |  |  |  |  |  |
|  | Fill the checklist for daily cleanliness of the workplace |  |  |  |  |  |
|  | Identify various types of construction waste |  |  |  |  |  |
|  | Apply waste reduction techniques during construction process |  |  |  |  |  |
|  | Ensure appropriate storage of water |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

| Assessment Task 2 |  |  | Description of assessment task 2 Candidate is required to perform plastering on ceiling of slab as per instructions given by assessor |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select appropriate material as per specifications |  |  |  |  |  |
|  | Select appropriate tools according to the task |  |  |  |  |  |
|  | Fix level pegs on ceiling with reference to the off lines |  |  |  |  |  |
|  | Prepare mortar according to the given ratio |  |  |  |  |  |
|  | Chip off and clean the ceiling |  |  |  |  |  |
|  | Apply cement slurry on ceiling |  |  |  |  |  |
|  | Apply mortar with trowel/float on wall surface within the level pegs |  |  |  |  |  |
|  | Remove extra mortar from off line of pegs |  |  |  |  |  |
|  | Fill depression of surface with additional mortar |  |  |  |  |  |
|  | Level the plaster surface with straight edge |  |  |  |  |  |
|  | Finish the plaster surface with float |  |  |  |  |  |
|  | Apply foam for smooth finish |  |  |  |  |  |
|  | Perform curing after specified time |  |  |  |  |  |
|  | Identify risk to Health, hygiene and safety |  |  |  |  |  |
|  | Maintain cleanliness and hygiene as per organizational policy |  |  |  |  |  |
|  | Follow organizational Health, hygiene and safety guidelines during work |  |  |  |  |  |
|  | Report unresolved problems to immediate supervisor |  |  |  |  |  |
|  | Place the tools equipment etc at their prescribed place after completion of work |  |  |  |  |  |
|  | Carry out the instructions of the supervisor |  |  |  |  |  |
|  | Respond appropriately to safety hazards on all tools and machines |  |  |  |  |  |
|  | Categorize reusable waste |  |  |  |  |  |
|  | Dispose unusable waste as per set standards |  |  |  |  |  |
|  | Avoid wastage of water |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |  |

ANNEX-I