|  | Assessment Evidence Guide For “PLUMBER” Level-2 Formative Assessment 0732BCE1506 Perform Sanitary Work |
| --- | --- |
|  | Date 04-09 November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | PLUMBER LEVEL-2 |
| --- | --- |
| Competency Standard | 0732BCE1506 Perform Sanitary Work |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Excavate according to plan - Mark out installation points for fixtures - Install wash basin & wall shower according to plan - Install WC (Asian & European) according to given drawing - Install sink & water filter as per given plan ACTIVITY - Install an Asian Water Closet (WC) with a proper drainage connection to a manhole, ensuring a secure connection to the drainage system with the P-Trap properly installed |
| Time: 4 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Select the correct tools and materials for the WC, P-Trap, and drainage system. - Level the WC and P-Trap to ensure they are properly aligned for efficient operation. - Laying drainage pipe with the correct slope for proper wastewater flow to the manhole to maintain self-cleaning velocity, - Verify the dimensions from the back and side walls to ensure proper spacing for the WC and P-Trap. - Install the P-Trap securely to the WC outlet and connect it to the drainage pipe without leaks. - Use sealing materials around the P-Trap and WC connections to ensure a watertight seal. - Check the stability of the WC, ensuring it is securely fastened and properly positioned. - Test for leaks and drainage by flushing the WC and inspecting all connections. - Ensure compliance with plumbing codes for proper dimensions, slopes, and fittings. - Inspect the drainage pipe for correct slope to avoid blockages or backflow. - Perform a final inspection to check for leaks, stability, and proper functionality. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | PLUMBER LEVEL-2 |
| Competency Standard | 0732BCE1506 Perform Sanitary Work |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Excavate according to plan - Mark out installation points for fixtures - Install wash basin & wall shower according to plan - Install WC (Asian & European) according to given drawing - Install sink & water filter as per given plan ACTIVITY - Install an Asian Water Closet (WC) with a proper drainage connection to a manhole, ensuring a secure connection to the drainage system with the P-Trap properly installed |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the correct tools and materials for the WC, P-Trap, and drainage system. |  |  |
| - Level the WC and P-Trap to ensure they are properly aligned for efficient operation. |  |  |
| - Laying drainage pipe with the correct slope for proper wastewater flow to the manhole to maintain self-cleaning velocity, |  |  |
| - Verify the dimensions from the back and side walls to ensure proper spacing for the WC and P-Trap. |  |  |
| - Install the P-Trap securely to the WC outlet and connect it to the drainage pipe without leaks. |  |  |
| - Use sealing materials around the P-Trap and WC connections to ensure a watertight seal. |  |  |
| - Check the stability of the WC, ensuring it is securely fastened and properly positioned. |  |  |
| - Test for leaks and drainage by flushing the WC and inspecting all connections. |  |  |
| - Ensure compliance with plumbing codes for proper dimensions, slopes, and fittings. |  |  |
| - Inspect the drainage pipe for correct slope to avoid blockages or backflow. |  |  |
| - Perform a final inspection to check for leaks, stability, and proper functionality. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | PLUMBER LEVEL-2 |
| --- | --- |
| Competency Standard | 0732BCE1506 Perform Sanitary Work |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Excavate according to plan - Mark out installation points for fixtures - Install wash basin & wall shower according to plan - Install WC (Asian & European) according to given drawing - Install sink & water filter as per given plan ACTIVITY - Install an Asian Water Closet (WC) with a proper drainage connection to a manhole, ensuring a secure connection to the drainage system with the P-Trap properly installed |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select the correct tools and materials for the WC, P-Trap, and drainage system. |  |  |  |  |  |
|  | Level the WC and P-Trap to ensure they are properly aligned for efficient operation. |  |  |  |  |  |
|  | Laying drainage pipe with the correct slope for proper wastewater flow to the manhole to maintain self-cleaning velocity, |  |  |  |  |  |
|  | Verify the dimensions from the back and side walls to ensure proper spacing for the WC and P-Trap. |  |  |  |  |  |
|  | Install the P-Trap securely to the WC outlet and connect it to the drainage pipe without leaks. |  |  |  |  |  |
|  | Use sealing materials around the P-Trap and WC connections to ensure a watertight seal. |  |  |  |  |  |
|  | Check the stability of the WC, ensuring it is securely fastened and properly positioned. |  |  |  |  |  |
|  | Test for leaks and drainage by flushing the WC and inspecting all connections. |  |  |  |  |  |
|  | Ensure compliance with plumbing codes for proper dimensions, slopes, and fittings. |  |  |  |  |  |
|  | Inspect the drainage pipe for correct slope to avoid blockages or backflow. |  |  |  |  |  |
|  | Perform a final inspection to check for leaks, stability, and proper functionality. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | PLUMBER LEVEL-2 |
| --- | --- |
| Competency Standard | 0732BCE1506 Perform Sanitary Work |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Write down any three Kitchen fixtures. |  |  |
|  | Write down any three Washroom fixtures. |  |  |
|  | Define self cleaning velocity? |  |  |
|  | What is the function of P Trap? |  |  |
|  | PVC stand for what? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________