Assessment Evidence Guide For Solar PV System Technician Level-2 (Summative Assessment) Date: 04-09 November, 2024 Lahore

National Vocational & Technical Training Commission Islamabad

Instruction Sheet for the Candidate

| Qualification | Solar PV System Technician |
| --- | --- |
| Competency Standard | Module 0713E&E3401: Follow Occupational Health and Safety practices Module 0713E&E3402: Demonstrate Basic Communication Skills Module 0713E&E3403: Apply Basic Electricity Concept Module 0713E&E3404: Perform Single phase/three phase Electrical Wiring System Module 0713E&E3405: Introduction to Solar PV System Module 0713E&E3406: Design of Solar PV System Module 0713E&E3407: Installation of Solar PV System Module 0713E&E3408: Maintenance of Solar PV System Module 0713E&E3409: Operation of Solar PV System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | The candidate is required to complete the given task by following the operational health and safety at workplace within time frame (for practical demonstration & assessment): Task 1: - Installation of Room Wiring according to the layout diagram - Design and complete installation of Solar PV System according to the electrical load of room (Annex. 1) (OR) Task 2: Design and install a Solar PV System of up to 5 KVA (Annex. 1) |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Task 1 (a): - Put on Personal protective equipment (PPE) and appropriate clothing - Identify & Handle tools and equipment according to the practical assessment task - Select wiring materials and accessories according to the circuit diagram. - Make the circuit on exercise board according to installation diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the open /short circuit & polarity Test before connection with the main supply. - Make connection with the main supply. - Check the function of circuit after connection with main supply. Task 1 (b) - Temporarily mount Solar PV modules (Panels) securely on specific site. - Connect the solar PV modules in Series/Parallel based on system design. - Install and wire the Hybrid Solar inverter, battery storage and charge controller according to the safety and regulatory standard. - Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. - Make the connection of the Load with Hybrid solar invertor - Make the connection of Solar PV modules with Hybrid Solar Invertor according to the input voltage range of Hybrid Solar Invertor. - Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. - Measure earth resistance of the earthing system. (OR) Task 2 - Put on Personal protective equipment (PPE) and appropriate clothing. - Size Solar PV System including Solar PV Modules, Battery, Inverter and controller according to the system design of 5 KVA. - Temporarily mount Solar PV Modules (Panels) securely on specific site. - Set the direction and tilt angle of solar PV Modules. - Connect the solar PV Modules in Series/ Parallel according to the system design. - Install and wire the Hybrid Solar inverter, battery storage and charge controller according to the safety and regulatory standard. - Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. - Make the connection of the Load with Hybrid solar invertor - Make the connection of Solar PV modules with Hybrid Solar Invertor according to the input voltage range of Hybrid Solar Invertor. - Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Solar PV System Technician |
| Competency Standard | Module 0713E&E3401: Follow Occupational Health and Safety practices Module 0713E&E3402: Demonstrate Basic Communication Skills Module 0713E&E3403: Apply Basic Electricity Concept Module 0713E&E3404: Perform Single phase/three phase Electrical Wiring System Module 0713E&E3405: Introduction to Solar PV System Module 0713E&E3406: Design of Solar PV System Module 0713E&E3407: Installation of Solar PV System Module 0713E&E3408: Maintenance of Solar PV System Module 0713E&E3409: Operation of Solar PV System |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | The candidate is required to complete the given task by following the operational health and safety at workplace within time frame (for practical demonstration & assessment): Task 1: - Installation of Room Wiring according to the layout diagram - Design and complete installation of Solar PV System according to the electrical load of room (Annex. 1) (OR) Task 2: Design and install a Solar PV System of up to 5 KVA (Annex. 1) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| Task 1 (a) |  |  |
| - Put on Personal protective equipment (PPE) and clothing. |  |  |
| - Identify and handle tools and equipment according to the practical assessment task. |  |  |
| - Select wiring materials and accessories according to the circuit diagram. |  |  |
| - Make the circuit on exercise board according to installation diagram. |  |  |
| - Install accessories according to layout diagram. |  |  |
| - Lay wires in duct/pipe according to layout diagram. |  |  |
| - Make connections according to wiring diagram. |  |  |
| - Check the open /short circuit & polarity Test before connection with the main supply. |  |  |
| - Make connection with the main supply. |  |  |
| - Check the function of circuit after connection with main supply. |  |  |
| Task 1 (b) |  |  |
| - Temporary mount the Solar PV Modules |  |  |
| - Connect the solar PV modules in series/Parallel. |  |  |
| - Make the connection of the battery with the Hybrid solar invertor. |  |  |
| - Make the connection of the solar PV modules with Hybrid solar invertor according to the input operating voltage range of solar invertor. |  |  |
| - Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. |  |  |
| - Make the connection of the Load with Hybrid solar invertor |  |  |
| - Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. |  |  |
| - Measure earth resistance of the earthing system. |  |  |
| (OR) Task 2 |  |  |
| - Put on Personal protective equipment (PPE) and appropriate clothing. |  |  |
| - Size Solar PV System including Solar PV Modules, Battery, Inverter and controller according to the system design of 5 KVA. |  |  |
| - Temporarily mount Solar PV Modules (Panels) securely on specific site. |  |  |
| - Set the direction and tilt angle of solar PV Modules. |  |  |
| - Connect the solar PV Modules in Series/ Parallel according to the system design. |  |  |
| - Install and wire the Hybrid Solar inverter, battery storage and charge controller according to the safety and regulatory standard. |  |  |
| - Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. |  |  |
| - Make the connection of the Load with Hybrid solar invertor |  |  |
| - Make the connection of Solar PV modules with Hybrid Solar Invertor according to the input voltage range of Hybrid Solar Invertor. |  |  |
| - Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Solar PV System Technician |
| --- | --- |
| Competency Standard | Module 0713E&E3401: Follow Occupational Health and Safety practices Module 0713E&E3402: Demonstrate Basic Communication Skills Module 0713E&E3403: Apply Basic Electricity Concept Module 0713E&E3404: Perform Single phase/three phase Electrical Wiring System Module 0713E&E3405: Introduction to Solar PV System Module 0713E&E3406: Design of Solar PV System Module 0713E&E3407: Installation of Solar PV System Module 0713E&E3408: Maintenance of Solar PV System Module 0713E&E3409: Operation of Solar PV System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | The candidate is required to complete the given task by following the operational health and safety at workplace within time frame (for practical demonstration & assessment): Task 1: - Installation of Room Wiring according to the layout diagram - Design and complete installation of Solar PV System according to the electrical load of room (Annex. 1) (OR) Task 2: Design and install a Solar PV System of up to 5 KVA (Annex. 1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| Task 1 (a) |  |  |  |  |  |  |
|  | Put on Personal protective equipment (PPE) and clothing. |  |  |  |  |  |
|  | Identify & handle tools and equipment according to the practical assessment task |  |  |  |  |  |
|  | Select wiring materials and accessories according to the circuit diagram. |  |  |  |  |  |
|  | Make the circuit on exercise board according to installation diagram. |  |  |  |  |  |
|  | Install accessories according to layout diagram. |  |  |  |  |  |
|  | Lay wires in duct/pipe according to layout diagram. |  |  |  |  |  |
|  | Make connections according to wiring diagram. |  |  |  |  |  |
|  | Check the open /short circuit & polarity Test before connection with the main supply. |  |  |  |  |  |
|  | Make connection with the main supply. |  |  |  |  |  |
|  | Check the function of circuit after connection with main supply. |  |  |  |  |  |
| Task 1 (b) |  |  |  |  |  |  |
| 1. | Temporary mount the Solar PV Modules |  |  |  |  |  |
| 2. | Connect the solar PV modules in series/Parallel. |  |  |  |  |  |
| 3. | Make the connection of the battery with the Hybrid solar invertor. |  |  |  |  |  |
| 4. | Make the connection of the solar PV modules with Hybrid solar invertor according to the input operating voltage range of solar invertor. |  |  |  |  |  |
| 5. | Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. |  |  |  |  |  |
| 6. | Make the connection of the Load with Hybrid solar invertor |  |  |  |  |  |
| 7. | Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. |  |  |  |  |  |
| 8. | Measure earth resistance of the earthing system. |  |  |  |  |  |
| (OR) Task 2 |  |  |  |  |  |  |
| 1. | Put on Personal protective equipment (PPE) and appropriate clothing. |  |  |  |  |  |
| 2. | Size Solar PV System including Solar PV Modules, Battery, Inverter and controller according to the system design of 5 KVA. |  |  |  |  |  |
| 3. | Temporarily mount Solar PV Modules (Panels) securely on specific site. |  |  |  |  |  |
| 4. | Set the direction and tilt angle of solar PV Modules. |  |  |  |  |  |
| 5. | Connect the solar PV Modules in Series/ Parallel according to the system design. |  |  |  |  |  |
| 6. | Install and wire the Hybrid Solar inverter, battery storage and charge controller according to the safety and regulatory standard. |  |  |  |  |  |
| 7. | Make the connection of the WAPDA supply/Main supply with Hybrid solar invertor. |  |  |  |  |  |
| 8. | Make the connection of the Load with Hybrid solar invertor |  |  |  |  |  |
| 9. | Make the connection of Solar PV modules with Hybrid Solar Invertor according to the input voltage range of Hybrid Solar Invertor. |  |  |  |  |  |
| 10. | Measure Voltage, Current, Power, of solar PV modules and Hybrid solar invertor with help of meters. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | 0713E&E34 Solar PV System Technician _Level-2 |
| --- | --- |
| Competency Standard | Module 0713E&E3401: Follow Occupational Health and Safety practices Module 0713E&E3402: Demonstrate Basic Communication Skills Module 0713E&E3403: Apply Basic Electricity Concept Module 0713E&E3404: Perform Single phase/three phase Electrical Wiring System Module 0713E&E3405: Introduction to Solar PV System Module 0713E&E3406: Design of Solar PV System Module 0713E&E3407: Installation of Solar PV System Module 0713E&E3408: Maintenance of Solar PV System Module 0713E&E3409: Operation of Solar PV System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the difference between series and parallel connection? |  |  |
|  | Write down different types of Solar PV Modules? |  |  |
|  | Describe the function of charge controller? |  |  |
|  | What is MPPT? |  |  |
|  | What is meant by dry battery? |  |  |
|  | What is earthing in Solar PV System? |  |  |
|  | What is DC Earthing? |  |  |
|  | What is AC Earthing? |  |  |
|  | Explain the function of On-Grid System? |  |  |
|  | What is Off-Grid System? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________

Task 1

(Annex. 1)

- Installation of Room Wiring according to the layout diagram

- Design and complete installation of Solar PV System according to the electrical load of room

(OR)

Task 2

(Annex. 1)

Design and install a Solar PV System of up to 5 KVA.

# Tools, Equipment Required

The tools and equipment required for this competency standard are provided below:

| SR. NO | NAME | QTY. REQUIRED |
| --- | --- | --- |
| 1 | Safety shoes | 10 |
| 2 | Safety goggles | 10 |
| 3 | Safety Helmet | 25 |
| 4 | Safety Belt | 05 |
| 5 | First Aid Box | 1 |
| 6 | Fire Extinguisher | 02 |
| 7 | AVO Meter | 05 |
| 8 | Combination Plier. | 25 |
| 9 | Nose plier. | 25 |
| 10 | Wire Stripper | 25 |
| 11 | Soldering Iron | 05 |
| 12 | Set of Screw Drivers | 25 |
| 13 | Ring Spanner Set | 02 |
| 14 | Tacho Meter-Digital | 02 |
| 15 | Earth resistance tester (Digital) | 02 |
| 16 | Continuity & Insulation tester | 02 |
| 17 | IR Temperature Gun | 02 |
| 18 | Digital Multimeter | 05 |
| 19 | Cable Knife | 25 |
| 20 | Phase Sequence Meter | 02 |
| 21 | Clamp-On Meter (AC &DC) | 05 |
| 22 | Lux Meter | 02 |
| 23 | Lugs Punching Machine (Hydraulic and Manual) | 01 |
| 24 | Battery Tester | 02 |
| 25 | Micro Meter mm | 03 |
| 26 | Hydrometer | 05 |
| 27 | Disk Grinder Machine | 02 |
| 28 | Hammer | 05 |
| 29 | Hacksaw cutter | 05 |
| 30 | Battery Charger | 02 |
| 31 | Bench Vice | 02 |
| 32 | Chisel | 02 |
| 33 | Digital Oscilloscope | 01 |
| 34 | 3- Phase energy meter (Green Meter) | 01 |
| 35 | Energy meter Digital Single Phase | 03 |
| 36 | Charge Controller PWM 40 amp | 01 |
| 37 | Charge Controller MPPT 50 amp | 01 |
| 38 | Solar Hybrid Inverter 4KW MPPT | 01 |
| 39 | Solar Hybrid Inverter 6KW MPPT | 01 |
| 40 | Solar OFF-Grid Inverter MPPT 3KW | 01 |
| 41 | Solar ON-Grid Inverter MPPT 10 KKW | 01 |
| 42 | Inverter 1500-watt sine wave | 02 |
| 43 | Lead Acid Battery -12 V, 150 AH | 04 |
| 44 | DRY Battery- 12 V, 100AH | 02 |
| 45 | Tall tubular Battery -12 V, 180AH | 02 |
| 46 | Solar PV Module Mono PERC Half Cut P-Type, 580 W | 10 |
| 47 | Solar PV Module Mono PERC Half Cut N-Type, 580 W | 10 |
| 48 | Solar panel monocrystalline | 02 |
| 49 | Solar panel thin film | 02 |
| 50 | DC power Supply 12-to-24-volt | 01 |
| 51 | Solar panel Structure (L2, L3) | 10 |
| 52 | Solar PV Tester | 01 |
| 53 | Three Phase Induction Motor | 01 |
| 54 | Voltmeter (Analog & Digital) | 05 |
| 55 | Ampere Meter (Analog & Digital) | 05 |
| 56 | Watt Meter (Analog & Digital) | 02 |
| 57 | VAR Meter | 02 |
| 58 | Digital Power Factor Meter | 02 |
| 59 | Analog Multimeter | 02 |
| 60 | Energy meter Digital- Three phase | 01 |
| 61 | Safety gloves with leather upper cover (pair) | 10 |
| 62 | Spanner set (Open end) | 02 |
| 63 | Ratcheted Set | 02 |
| 64 | SWG Wire Gage | 05 |
| 65 | Level | 05 |
| 66 | Screw Driver Set Cardless (battery operated) | 01 |
| 67 | Printer | 01 |
| 68 | Computer | 03 |
| 69 | Multimedia Projector | 01 |
| 70 | Rotary Change Over 2P 50A | 2 |
| 71 | Rotary Change Over 4P 100A | 2 |
| 72 | Electronic Relay 0-20Amp | 2 |