|  | Assessment Evidence Guide For National Vocational Certificate in Welder Level-2 Summative Assessment |
| --- | --- |
|  | Date 04th - 09th November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate in Welder Level 02 |
| --- | --- |
| Competency Standard | - 0715MMT3701 Introduction to Work Ethics - 0715MMT3702 Perform Basic Communication Skills - 0715 MMT3703 Maintain Safety at Workplace - 0715 MMT3704 Prepare Materials for Welding - 0715 MMT3705 Carry Out Gas Welding and Gas Cutting Operations - 0715 MMT3706 Carry Out Shielded Metal Arc Welding (SMAW) in different Positions - 0715 MMT3707 Carry Out Gas Metal Arc Welding (GMAW) in different Positions - 0715 MMT3708 Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions - 0715 MMT3709 Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions - 0715 MMT3710 Carry Out Submerged Arc Welding (SAW) - 0715 MMT3711 Develop entrepreneurship skills |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): Activity-01: Perform Gas Welding on Lap / T – joint (Material: M.S Plate, Size: 200mm x 25mm x 3mm) (See Annex-1) OR Activity 02: Perform Welding to make Single V-Butt joint in 1G/2G/3G Position using SMAW/GMAW/GTAW/FCAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) (See Annex-2) |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Activity 01: Perform Gas Welding on Lap / T – joint (Material: M.S Plate, Size: 200mm x 25mm x 3mm) - Selects the correct M.S. plates (200mm x 25mm x 3mm) and ensures they meet specifications. - Fit up the plates in the correct position for a Lap or T-joint configuration as per task instructions. - Use fixtures for base materials to prevent movement during welding, ensuring accurate alignment and joint stability. - Prepares the gas welding equipment by selecting appropriate torch tips, adjusting flame settings, and ensuring the correct gas pressure according to the plate thickness. - Apply required PPE’s and follow other safety precautions for safe welding process. - Adjusts the gas torch to achieve a neutral flame suitable for M.S. plate welding. - Maintains a consistent distance and angle between the torch and the material to avoid excessive heat buildup or undercutting. - Initiates the weld at the correct starting point, progressing in a smooth and controlled manner along the joint. - Uses a steady and consistent weaving or circular motion as required, achieving even heat distribution and penetration across the joint. - Ensures adequate penetration and fusion between the plates, with no visible gaps or separation. - Maintains consistent bead width, height, and reinforcement along the joint for structural integrity. - Inspects the completed joint for structural soundness, uniformity, and smoothness. - Removes any slag or oxide layers to reveal a clean weld finish. - Identifies any weld defects and, if necessary, repair the weld to meet quality standards. - Cleans the workspace and stores all equipment and materials properly, following safety and maintenance procedures. OR Activity -02: Perform Welding to make Single V-Butt joint in 1G/2G/3G Position using SMAW/GMAW/GTAW/FCAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) - MS plate (200mm x 38mm x 10mm) is prepared and aligned correctly for a single V-butt joint with appropriate root gap and angle as per the welding procedure specification (WPS). - Use fixture for the work piece to securely clamped in the designated position (1G / 2G / 3G) to ensure stability during welding. - Welding equipment (SMAW / GMAW / GTAW / FCAW) is set up and calibrated as per the WPS, including amperage, voltage, and gas flow (if applicable). - The welding surfaces are cleaned to remove rust, oil, and other contaminants that could affect weld quality. - The root face and root opening are accurately maintained, meeting WPS requirements. - V-groove edges are prepared with the correct angle, ensuring uniformity and consistent penetration. - The arc length is consistently controlled to prevent excessive spatter and to achieve a stable, smooth arc throughout the weld. - The initial root pass and subsequent filler passes are executed with uniform bead appearance, proper penetration, and fusion without any undercut or overlap. - Appropriate heat input is maintained to prevent distortion and ensure good fusion between the base materials, with interpass temperature control as required by the WPS. - The welding is performed in the correct 1G/2G/3G position, maintaining electrode angle and travel speed suited to each position's demands. - Weld bead is inspected for uniform width, height, and smoothness without visible defects such as cracks, porosity, or inclusions. - The root pass demonstrates full penetration, visible on the underside of the weld, where required. - Any observed defects (e.g., lack of fusion, undercut) are promptly corrected with appropriate repair techniques. - Slag from SMAW/FCAW is removed after each pass to prevent inclusions in the weld bead. - The final weld is cleaned and visually inspected for quality assurance. - The joint is checked for dimensional accuracy, alignment, and overall structural integrity in line with the WPS and inspection standards. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate in Welder Level-02 |
| Competency Standard | - 0715MMT3701 Introduction to Work Ethics - 0715MMT3702 Perform Basic Communication Skills - 0715 MMT3703 Maintain Safety at Workplace - 0715 MMT3704 Prepare Materials for Welding - 0715 MMT3705 Carry Out Gas Welding and Gas Cutting Operations - 0715 MMT3706 Carry Out Shielded Metal Arc Welding (SMAW) in different Positions - 0715 MMT3707 Carry Out Gas Metal Arc Welding (GMAW) in different Positions - 0715 MMT3708 Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions - 0715 MMT3709 Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions - 0715 MMT3710 Carry Out Submerged Arc Welding (SAW) - 0715 MMT3711 Develop entrepreneurship skills |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Activity -01: Perform Gas Welding on Lap / T – joint (Material: M.S Plate, Size: 200mm x 25mm x 3mm) (See Annex-1) OR Activity -02: Perform Welding to make Single V-Butt joint in 1G/2G/3G Position using SMAW/GMAW/GTAW/FCAW (Material: M.S Plate, Size: 200mmx38mmx10mm) (See Annex-2) |

I can……………….

| Performance Criteria (Activity -1) | Yes | No |
| --- | --- | --- |
| - Selects the correct M.S. plates (200mm x 25mm x 3mm) and ensures they meet specifications. |  |  |
| - Fit up the plates in the correct position for a Lap or T-joint configuration as per task instructions. |  |  |
| - Use fixtures for base materials to prevent movement during welding, ensuring accurate alignment and joint stability. |  |  |
| - Prepares the gas welding equipment by selecting appropriate torch tips, adjusting flame settings, and ensuring the correct gas pressure according to the plate thickness. |  |  |
| - Apply required PPE’s and follow other safety precautions for safe welding process. |  |  |
| - Adjusts the gas torch to achieve a neutral flame suitable for M.S. plate welding. |  |  |
| - Maintains a consistent distance and angle between the torch and the material to avoid excessive heat buildup or undercutting. |  |  |
| - Initiates the weld at the correct starting point, progressing in a smooth and controlled manner along the joint. |  |  |
| - Uses a steady and consistent weaving or circular motion as required, achieving even heat distribution and penetration across the joint. |  |  |
| - Ensures adequate penetration and fusion between the plates, with no visible gaps or separation. |  |  |
| - Maintains consistent bead width, height, and reinforcement along the joint for structural integrity. |  |  |
| - Inspects the completed joint for structural soundness, uniformity, and smoothness. |  |  |
| - Removes any slag or oxide layers to reveal a clean weld finish. |  |  |
| - Identifies any weld defects and, if necessary, repair the weld to meet quality standards. |  |  |
| - Cleans the workspace and stores all equipment and materials properly, following safety and maintenance procedures. |  |  |

OR

| Performance Criteria (Activity -2) | Yes | No |
| --- | --- | --- |
| - MS plate (200mm x 38mm x 10mm) is prepared and aligned correctly for a single V-butt joint with appropriate root gap and angle as per the welding procedure specification (WPS). |  |  |
| - Use fixture for the work piece to securely clamped in the designated position (1G / 2G / 3G) to ensure stability during welding. |  |  |
| - Welding equipment (SMAW / GMAW / GTAW / FCAW) is set up and calibrated as per the WPS, including amperage, voltage, and gas flow (if applicable). |  |  |
| - The welding surfaces are cleaned to remove rust, oil, and other contaminants that could affect weld quality. |  |  |
| - The root face and root opening are accurately maintained, meeting WPS requirements. |  |  |
| - V-groove edges are prepared with the correct angle, ensuring uniformity and consistent penetration. |  |  |
| - The arc length is consistently controlled to prevent excessive spatter and to achieve a stable, smooth arc throughout the weld. |  |  |
| - The initial root pass and subsequent filler passes are executed with uniform bead appearance, proper penetration, and fusion without any undercut or overlap. |  |  |
| - Appropriate heat input is maintained to prevent distortion and ensure good fusion between the base materials, with interpass temperature control as required by the WPS. |  |  |
| - The welding is performed in the correct 1G/2G/3G position, maintaining electrode angle and travel speed suited to each position's demands. |  |  |
| - Weld bead is inspected for uniform width, height, and smoothness without visible defects such as cracks, porosity, or inclusions. |  |  |
| - The root pass demonstrates full penetration, visible on the underside of the weld, where required. |  |  |
| - Any observed defects (e.g., lack of fusion, undercut) are promptly corrected with appropriate repair techniques. |  |  |
| - Slag from SMAW/FCAW is removed after each pass to prevent inclusions in the weld bead. |  |  |
| - The final weld is cleaned and visually inspected for quality assurance. |  |  |
| - The joint is checked for dimensional accuracy, alignment, and overall structural integrity in line with the WPS and inspection standards. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate in Welder Level 02 |
| --- | --- |
| Competency Standard | - 0715MMT3701 Introduction to Work Ethics - 0715MMT3702 Perform Basic Communication Skills - 0715 MMT3703 Maintain Safety at Workplace - 0715 MMT3704 Prepare Materials for Welding - 0715 MMT3705 Carry Out Gas Welding and Gas Cutting Operations - 0715 MMT3706 Carry Out Shielded Metal Arc Welding (SMAW) in different Positions - 0715 MMT3707 Carry Out Gas Metal Arc Welding (GMAW) in different Positions - 0715 MMT3708 Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions - 0715 MMT3709 Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions - 0715 MMT3710 Carry Out Submerged Arc Welding (SAW) - 0715 MMT3711 Develop entrepreneurship skills |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Activity -01: Perform Gas Welding on Lap / T – joint (Material: M.S Plate, Size: 200mm x 25mm x 3mm) (See Annex-1) OR Activity -02: Perform Welding to make Single V-Butt joint in 1G/2G/3G Position using SMAW/GMAW/GTAW/FCAW (Material: M.S Plate, Size: 200mmx38mmx10mm) (See Annex-2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: (Activity -1) |  |  |  | Yes | No | Remarks |
|  | Selects the correct M.S. plates (200mm x 25mm x 3mm) and ensures they meet specifications. |  |  |  |  |  |
|  | Fit up the plates in the correct position for a Lap or T-joint configuration as per task instructions. |  |  |  |  |  |
|  | Use fixtures for base materials to prevent movement during welding, ensuring accurate alignment and joint stability. |  |  |  |  |  |
|  | Prepares the gas welding equipment by selecting appropriate torch tips, adjusting flame settings, and ensuring the correct gas pressure according to the plate thickness. |  |  |  |  |  |
|  | Apply required PPE’s and follow other safety precautions for safe welding process. |  |  |  |  |  |
|  | Adjusts the gas torch to achieve a neutral flame suitable for M.S. plate welding. |  |  |  |  |  |
|  | Maintains a consistent distance and angle between the torch and the material to avoid excessive heat buildup or undercutting. |  |  |  |  |  |
|  | Initiates the weld at the correct starting point, progressing in a smooth and controlled manner along the joint. |  |  |  |  |  |
|  | Uses a steady and consistent weaving or circular motion as required, achieving even heat distribution and penetration across the joint. |  |  |  |  |  |
|  | Ensures adequate penetration and fusion between the plates, with no visible gaps or separation. |  |  |  |  |  |
|  | Maintains consistent bead width, height, and reinforcement along the joint for structural integrity. |  |  |  |  |  |
|  | Inspects the completed joint for structural soundness, uniformity, and smoothness. |  |  |  |  |  |
|  | Removes any slag or oxide layers to reveal a clean weld finish. |  |  |  |  |  |
|  | Identifies any weld defects and, if necessary, repair the weld to meet quality standards. |  |  |  |  |  |
|  | Cleans the workspace and stores all equipment and materials properly, following safety and maintenance procedures. |  |  |  |  |  |
| OR |  |  |  |  |  |  |
| During the practical assessment, candidate demonstrated the following: (Activity -2) |  |  |  | Yes | No | Remarks |
|  | MS plate (200mm x 38mm x 10mm) is prepared and aligned correctly for a single V-butt joint with appropriate root gap and angle as per the welding procedure specification (WPS). |  |  |  |  |  |
|  | Use fixture for the work piece to securely clamped in the designated position (1G / 2G / 3G) to ensure stability during welding. |  |  |  |  |  |
|  | Welding equipment (SMAW / GMAW / GTAW / FCAW) is set up and calibrated as per the WPS, including amperage, voltage, and gas flow (if applicable). |  |  |  |  |  |
|  | The welding surfaces are cleaned to remove rust, oil, and other contaminants that could affect weld quality. |  |  |  |  |  |
|  | The root face and root opening are accurately maintained, meeting WPS requirements. |  |  |  |  |  |
|  | V-groove edges are prepared with the correct angle, ensuring uniformity and consistent penetration. |  |  |  |  |  |
|  | The arc length is consistently controlled to prevent excessive spatter and to achieve a stable, smooth arc throughout the weld. |  |  |  |  |  |
|  | The initial root pass and subsequent filler passes are executed with uniform bead appearance, proper penetration, and fusion without any undercut or overlap. |  |  |  |  |  |
|  | Appropriate heat input is maintained to prevent distortion and ensure good fusion between the base materials, with underpass temperature control as required by the WPS. |  |  |  |  |  |
|  | The welding is performed in the correct 1G/2G/3G position, maintaining electrode angle and travel speed suited to each position's demands. |  |  |  |  |  |
|  | Weld bead is inspected for uniform width, height, and smoothness without visible defects such as cracks, porosity, or inclusions. |  |  |  |  |  |
|  | The root pass demonstrates full penetration, visible on the underside of the weld, where required. |  |  |  |  |  |
|  | Any observed defects (e.g., lack of fusion, undercut) are promptly corrected with appropriate repair techniques. |  |  |  |  |  |
|  | Slag from SMAW/FCAW is removed after each pass to prevent inclusions in the weld bead. |  |  |  |  |  |
|  | The final weld is cleaned and visually inspected for quality assurance. |  |  |  |  |  |
|  | The joint is checked for dimensional accuracy, alignment, and overall structural integrity in line with the WPS and inspection standards. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate in Welder Level 02 |
| --- | --- |
| Competency Standard | - 0715MMT3701 Introduction to Work Ethics - 0715MMT3702 Perform Basic Communication Skills - 0715 MMT3703 Maintain Safety at Workplace - 0715 MMT3704 Prepare Materials for Welding - 0715 MMT3705 Carry Out Gas Welding and Gas Cutting Operations - 0715 MMT3706 Carry Out Shielded Metal Arc Welding (SMAW) in different Positions - 0715 MMT3707 Carry Out Gas Metal Arc Welding (GMAW) in different Positions - 0715 MMT3708 Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions - 0715 MMT3709 Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions - 0715 MMT3710 Carry Out Submerged Arc Welding (SAW) - 0715 MMT3711 Develop entrepreneurship skills |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | What is Work Ethics? |  |  |  |
|  | Name two common hazards in welding and how to avoid them? |  |  |  |
|  | Name two types of flames and their uses in gas welding. |  |  |  |
|  | Why is shielding gas used in GMAW? |  |  |  |
|  | What are the main differences between FCAW and GMAW? |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |

(Annex-1)

LAP Joint

T –Joint

OR

(Annex-2)