National Vocational Qualification Level-2 in Welder (06 Months)

CURRICULUM

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORD

The National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils, and trade associations who dedicated their time and expertise for the development of the National Vocational Qualification for 
Level-2 Welder (Mechanical Technology). This work would not have been possible without the technical support of these experts.

It is worth mentioning that all the experts from Industry, Academia, and TVET experts from TEVTAs, BTEs, and PVTC worked diligently to make this qualification valuable and error-free, for which all credit goes to them. However, NAVTTC accepts responsibility for any errors and omissions that may still be present in the qualification document.

The development of Skill Standards is a dynamic and ongoing process, and the competency standards need periodic review and updating due to constant technological advancements, developments in scientific knowledge and growing experience of implementation at the grassroots level as well as industry demands. The NAVTTC, Government of Pakistan, will ensure that the qualifications remain up-to-date with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	5

2.	Purpose of this training programme	5

3.	Overall objectives of the training course	6

4.	Competencies to be gained after completion of course	7

5.	Job opportunities available immediately and in the future	7

6.	Trainee entry level	7

7.	Minimum qualification of trainer	7

8.	Recommended trainer trainee ratio	8

9.	Medium of instruction	8

10.	Curriculum Development committee	8

11.	Curriculum Review committee	8

12.	Duration of the course (total time, theory & practical)	9

13.	Summary of competency standards	10

14.	Summary – overview of the curriculum	10

15.	Core Modules	13

Module 01- 0715MMT3701 - Introduction to Work Ethics	13

Module 02 - 0715MMT3702 - Perform Basic Communication Skills	14

Module 03 - 0715MMT3703 - Maintain Safety at Workplace	18

Module 04 - 0715MMT3704 - Prepare Material for Welding	22

Module 05 - 0715MMT3705 - Carry Out Gas Welding And Gas Cutting Operations	26

Module 06 - 0715MMT3706 - Carry out Shielded Metal Arc Welding (SMAW) in different Positions	31

Module 07 - 0715MMT3707 - Carry Out Gas Metal Arc Welding (GMAW) in Different Positions	37

Module 08 - 0715MMT3708 - Carry out Gas Tungsten Arc Welding (GTAW) in different Positions	43

Module 09 - 0715MMT3709 - Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions	50

Module 10 - 0715MMT3710 - Carry out Submerged Arc Welding (SAW)	56

Module 11 - 0715MMT3711 - Develop Entrepreneurship Skills	61

# Introduction

A welder is a skilled tradesman who specializes in joining materials together or fills and repairs holes on metal constructions. Welders work on all types of industrial, manufacturing and construction applications; some even work underwater to repair oil rig foundations, ship hulls and other types of sub-aquatic structures. Skilled welders know the welding specifications of many types of materials. Apprenticeship or education/certification enables the learners to get familiar with advanced welding techniques. Through the application of these techniques and skills, they may weld manually or use machines to weld metal components.

Welders typically work from drawings or specifications, than use their knowledge of base metals and joining techniques to select the appropriate material for the job. They cut, position, and tack weld the material/s in preparation for one of the many welding processes. The difficulty of the job depends on the types of materials and welding positions. Regardless of the type of welding process, welders are exposed to intense and blinding heat and radiations and must take special care to ensure their own safety and the safety of those around them. Welders wear special gloves and aprons to prevent sparks and flame from burning their clothes and skin. In addition to taking safety precautions, welders also maintain their equipment and work with various power tools to prepare materials for welding. The specific job duties of a welder vary depending on the skills of the welder and the industry in which he works. Due to universal need for their skills, welders are in high demand not only nationally but also internationally.

In order to meet the domestic and worldwide demand for welders, National Vocational and Technical Training Commission (NAVTTC) in collaboration with TVET Sector Support Programme (TVET-SSP) has developed national vocational qualifications comprising of generic, functional and technical competency standards for welder occupation. To facilitate the process of developing national qualifications for welder, a Qualification Development Committee (QDC) was established under NVQF Operational Manual-1.

# Purpose of this training programme

The purpose of this training programme is to provide the Green skills to the trainees in the field of Mechanical and convert it into value added product, which is acceptable by International market and fit-in a skilled graduate into National Vocational Qualification Framework for his / her vertical career progression and qualification equivalencies at par with acceptable international standards.

To set high professional standards for welder’s job. These national qualifications will support training providers in enhancing the quality of training and assessment in Pakistan. The specific objectives of developing these qualifications are as under:

- Improve the overall quality of training delivery and setting national benchmarks for training of welders in the country

- Provide flexible pathways and progressions to learners enabling them to receive relevant, Up-to-date and recent skills

- Provide basis for competency-based assessment, which is recognized and  accepted by employers

- Establish a standardized and sustainable system of training for welders in the country.

Green skills incorporation in TVET aims to build capacity of skilled workforce, which is aware of environmental hazards and accordingly adopts more sustainable and environment friendly practices at the workplace. The workforce that possesses competencies and relevant knowledge & understanding oriented to green occupations is more employable and become an asset for businesses, enterprises and communities in developing green economies that result in improved human well-being and reducing environmental risks.

# Overall objectives of the training course

This objectives of the training course is to developed by considering the demands of skilled and qualified welders for both domestic and international markets. The primary objectives of this training programme are to:

- Develop and enhance skill level of the incumbent in the industry.

- Impart training and provide the industry a workforce with recognized and certified job knowledge, skills and attitude.

- Reduce unemployment and poverty in the society.

- Provide opportunity to those who want to equip themselves with such knowledge and skills which shall be helpful for their employment after completing this training.

- Enable the trainees to start their own business with professional approach.

- Establish coordination among employers, workers and government agencies relating to human resource development programs.

- Provide basis of technical and vocational training reflecting the requirements of the industry.

- Capacity building of the workforce and trainers in modern competency based trainings, methodologies and processes as envisaged under NVQF.

- Competencies to be gained after completion of course

After completing this course, the trainee will be capable of performing Competency standards, which are benchmarks for the performance, cover the commercial aspects of a welder’s job. Required skills, underpinning knowledge and attitudes expected of a welder have been incorporated in these competency standards while setting standards for the performance of a welder.

# Job opportunities available immediately and in the future

Job opportunities available immediately and in the future for the successful trainees.

| - Steel manufacturing industry - Steel manufacturing industry - Fertilizer industry - Chemical industry - Sugar industry - Cement industry - Thermal power plants - National Scientific and Engineering commission, Pakistan - Pakistan International Airlines (PIA) - Water and Power Development Authority (WAPDA) - Automobile industry - Local metal fabrication shops - Training Institutes | - Nuclear power plants - Industrial projects - PN Shipyards - Pak Railways - Pakistan Ordnance Factory, Wah Cantt. - Heavy Mechanical Complex-1, Taxila - Heavy Mechanical Complex-3, Taxila - Heavy Forge and Foundry, Taxila - Pakistan Atomic Energy Commission - Tri-forces of Pakistan - Tractor and Agricultural Equipment Industry - Local industry - TEVTAs - Self-employment etc. |
| --- | --- |

# Trainee entry level

Entry requirements for this qualification is minimum Primary Preferably Middle Certificate.

# Minimum qualification of trainer

D.A.E / B. Tech. / B.E. / G-II with 3 years’ experience in the field of welding.

# Recommended trainer trainee ratio

The recommended trainer and trainee ratio per class is 1:25

# Medium of instruction

Urdu / English

# Curriculum Development committee

The following members participated in the Curriculum development workshop from 13 to 14 Feb 2018, in Lahore:

| S.No. | Name | Organization |
| --- | --- | --- |
| 1. | Dr. Mirza Nadeem Baig | Pakistan Welding Institute (DACUM Facilitator) |
| 2. | Ms. Saba Sadiq | DESCON Technical Institute, Lahore |
| 3. | Mr. Basit Ali | Infinity College of Engineering, Lahore |
| 4. | Mr. M. Aslam Khatak | Heavy Mechanical Complex, Taxila |
| 5. | Mr. Malik M. Nazir Awan | Pakistan Welding Institute, Islamabad |
| 6. | Mr. Aziz Ullah Khan | Technical Support Organization, Chashma Nuclear Power Generating Station, Kundian, Mianwali |
| 7. | Mr. Azhar Iqbal | Heavy Mechanical Complex-3, Taxila |
| 8. | Mr. Awais Tanoli | Heavy Mechanical Complex-3, Taxila |

# Curriculum Review committee

The following members participated in the Curriculum Review workshop from 02 to 07 Sep 2024, in Lahore:

| Sr. No | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Mr. Ishfaq Hussain Shah | Instructor (Welding) GTTI Mianwali | P-TEVTA, Lahore |
| 2 | Engr. Hassan Ali | Instructor (Mechanical) GTVC(B), Mardan | KP-TEVTA, Peshawar |
| 3 | Engr. Kishore Kumar | Assistant Professor GCT Hyderabad | S-TEVTA, Karachi |
| 4 | Mr. Hafiz Muhammad Farooq | Trade Instructor (Welding) GTTC Darya Khan | P-TEVTA, Lahore |
| 5 | Syed Salman Nasir Ali Shah | DACUM Facilitator | P-TEVTA, Lahore |

CONVENERS:

| Sr. No | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Mr. Muhammad Ishaq | Deputy Director | NAVTTC, Islamabad |
| 2 | Mr. Muhammad Asim | Deputy Director | NAVTTC, Islamabad |
| 3 | Mr. Shams Shahbaz | Assistant Director Operation II | P-TEVTA, Lahore |
| 4 | Engr. Hafiz Waqas Hashmi | Assistant Director Curriculum | P-TEVTA, Lahore |

# Duration of the course (total time, theory & practical)

| Module. | Competency Standards | Theory Hours | Practical Hours | Total Contact Hours | Credit Hours |
| --- | --- | --- | --- | --- | --- |
| 0715MMT3701 | Introduction to Work Ethics | 3 | 0 | 3 | 0.3 |
| 0715MMT3702 | Perform Basic Communication Skills | 15 | 36 | 51 | 5.1 |
| 0715MMT3703 | Maintain Safety at Workplace | 15 | 21 | 36 | 3.6 |
| 0715MMT3704 | Prepare Materials for Welding | 15 | 36 | 51 | 5.1 |
| 0715MMT3705 | Carry Out Gas Welding and Gas Cutting Operations | 18 | 87 | 105 | 10.5 |
| 0715MMT3706 | Carry Out Shielded Metal Arc Welding (SMAW) in different Positions | 18 | 114 | 132 | 13.2 |
| 0715MMT3707 | Carry Out Gas Metal Arc Welding (GMAW) in different Positions | 18 | 114 | 132 | 13.2 |
| 0715MMT3708 | Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions | 18 | 114 | 132 | 13.2 |
| 0715MMT3709 | Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions | 15 | 54 | 69 | 6.9 |
| 0715MMT3710 | Carry Out Submerged Arc Welding (SAW) | 15 | 48 | 63 | 6.3 |
| 0715MMT3711 | Develop Entrepreneurship Skills | 10 | 16 | 26 | 2.6 |
| Total |  | 160 | 640 | 800 | 80 |

# Summary of competency standards

The proposed curriculum is composed of 11 modules that will be covered in 800 hours. This course should be delivered in a Six months period. The distributions of contact hours (practical & theory) are:

- Theory: (20%) Practical (80%)

- Theory: 160 hours

- Practical:  640 hours

- Total: 800 hours

# Summary – overview of the curriculum

| Module Title | Learning Units | Theory Days / hours | Work place Days / hours | Total Time frame of modules |
| --- | --- | --- | --- | --- |
| 0715MMT3701 Introduction to Work Ethics | Understand the Concept of Work Ethics. Recognize the Importance of Professionalism. Acknowledge the Impact of Work Ethics on Job Performance. | 03 | 00 | 03 |
| 0715MMT3702 Perform Basic Communication Skills | - Perform Work in Team Deal with Clients in Workshop/Industry Perform Basic IT Skills Conduct Mock Interview | 15 | 36 | 51 |
| 0715MMT3703 Maintain Safety at Workplace | - Apply Personal Protective Equipment (PPE’s) in Workshop - Handle Hazardous Materials as per SOP’s - Operate Fire Extinguisher as per given Instructions - Apply Emergency Response in Workshop | 15 | 21 | 36 |
| 0715MMT3704 Prepare Materials for Welding | - Measure and Mark Material/s as per Drawing / Job Requirement - Cut and Prepare Edge/s of Base Materials - Prepare Welding Consumables as per Job Requirement - Fit-up Base Materials as per Job Requirement | 15 | 36 | 51 |
| 0715MMT3705 Carry Out Gas Welding and Gas Cutting Operations | - Perform Flame Making on Gas Welding Torch - Perform Puddle Formation and Straight Bead on Base Metal - Carry Out Gas Cutting on Base Metal - Carry out Soldering and Brazing on Base Metal - Perform Project Work as per Instructions | 18 | 87 | 105 |
| 0715MMT3706 Carry Out Shielded Metal Arc Welding (SMAW) in different Positions | - Prepare Welding Machine and accessories for SMAW - Perform Striking and Blind Bead on Base Metal - Perform Fillet Welds on Mild Steel Plates / Pipes - Perform Groove Welds on Mild Steel Plates / Pipes - Perform Post Welding Procedures | 18 | 114 | 132 |
| 0715MMT3707 Carry Out Gas Metal Arc Welding (GMAW) in different Positions | - Prepare Welding Machine and Accessories for GMAW - Perform Blind Bead on Base Metal - Perform Fillet Welds on Mild Steel Plates / Pipes - Perform Groove Welds on Mild Steel Plates / Pipes - Perform Post Welding Procedures | 18 | 114 | 132 |
| 0715MMT3708 0715WEL07 Carry Out Gas Tungsten Arc Welding (GTAW) in different Positions | - Prepare Welding Machine and Accessories for GTAW - Perform Blind Bead on Base Metal - Perform Fillet Welds on Mild Steel Plates / Pipes - Perform Groove Welds on Mild Steel Plates / Pipes - Perform Post Welding Procedures | 18 | 114 | 132 |
| 0715MMT3709 Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions | - Prepare Welding Machine and Accessories for FCAW - Perform Fillet Welds on Mild Steel Plates. - Perform Groove Welds on Mild Steel Plates. - Perform Post Welding Procedures | 15 | 54 | 69 |
| 0715MMT3710 Carry Out Submerged Arc Welding (SAW) | - Prepare Welding Machine and Accessories for SAW - Perform Fillet Welds on Mild Steel Plates - Perform Groove Welds on Mild Steel Plates - Perform Post Welding Procedures | 15 | 48 | 63 |
| 0715MMT3711 Develop Entrepreneurship Skills | - Concept of Entrepreneurship Project Preparation & Marketing Analysis - Investment Procurement | 10 | 16 | 26 |
|  | - Total Hours | 160 | 640 | 800 |

# Core Modules

# 0715MMT3701 - Introduction to Work Ethics

Objective: This module covers the knowledge and  skills required to install a strong understanding of the importance of professional behaviour, responsibility, and integrity in the workplace. Learners will recognize how ethical conduct influences safety, teamwork, and overall job performance, ensuring they meet industry standards and contribute positively to their work environment.

Duration: 03 Hours			Theory: 03 Hours				Practice: 0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Understand work ethics | Trainee will be able to: - Understand the Concept of Work Ethics. - Recognize the Importance of Professionalism. - Acknowledge the Impact of Work Ethics on Job Performance. | - Definition and importance of Work Ethics. - Professionalism in the Workplace. - Impact of Work Ethics on Job Performance. - Job Roles and Career Pathways. | Theory: 03Hrs. Practical: 0Hrs. Total: 03Hrs. | - White board marker - White board - Multimedia /Projector - Relevant videos/ photos | - Class Room |

# 0715MMT3702 - Perform Basic Communication Skills

Objective: This module covers the knowledge and  skills required to apply communication skills at workplace in accordance with the organization’s guidelines and procedures. You are expected to work in a team to achieve common organizational goals and avoid conflicts. This competency standard would also enable you to use basic computer skills to communicate effectively and prepare work related documents.

Duration: 51 Hours			Theory: 15 Hours				Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform Work in Team | Trainee will be able to: - Treat team members with respect and maintain positive relationship to achieve common organizational goals. - Listen to instructions carefully and follow them. - Adopt communication skills appropriate to work activities and company procedures. - Identify problems and resolve them through discussion and mutual agreement. | - Introduction to Team Roles and Responsibilities - Importance of Listening to Instructions. - Introduction to communication skills. - Language for effective communication. - Introduction to Team Performance. - Sources of conflict and their resolutions. Practical Activity: Make a team of 3-5 trainees: - Each team is tasked with designing a layout for a new welding workstation for five No’s of SMAW machines and two pedestal grinding machines. Considering the availability of space, equipment, and safety. - Discuss the problems raised during work in team. - Make a report for the Workshop Incharge. | Theory: 03Hrs. Practical: 05Hrs. Total: 08Hrs. | - White board marker - White board - Multimedia /Projector - Visual Aids | - Class Room - Workshop |
| Deal with Clients in Workshop/ Industry | Trainee will be able to: - Collect and confirm work requirements from clients using Appropriate communication Procedures. - Provide clear information to clients about work requirements including costs and time needed to accomplish the task. - Negotiate with clients regarding wages, time, labor requirements etc. | - Introduction and scope of Client Communication. - Client Needs and requirements. - Types of Information to Collect. - Customer Service Skills. - Techniques for negotiating project terms Practical Activity: Make a team of 3-5 trainees: - Observe a role play - Scenario of team where client’s interactions are essential and discuss appropriate responses and actions for making two No’s of Tables. | Theory: 03Hrs. Practical: 05Hrs. Total: 08Hrs. | - Data Sheets - Visual Aids | - Class Room - Workshop |
| Perform Basic IT Skills | Trainee must be able to: - Create folders and files and learn major commands of operating system/windows. - Type text and use major commands such as printing, editing, creating tables, header footer, footnotes, table of contents and page number etc. - Make the document as per work specifications and client’s requirement. - Use internet for sending/receiving emails and connecting through social or other media. | - Introduction to the Operating System. - Major windows commands. - Introduction to M.S Word. - Major commands used in M.S Word document. - CV making and Application Writing in M.S Word. - Introduction to Internet browsers. - Sending/receiving of emails. - Overview of social media. Practical Activity: - Trainees to design the CV / Application by using M.S Word Software. - Please make sure to shut down the computers after completion of practical activity. - Switch off the lights and fans of the lab. | Theory: 05Hrs. Practical: 21Hrs. Total: 26Hrs. | - Computer with relevant accessories. - M.S Word Software | - Class Room - Workshop |
| Conduct Mock Interview | Trainee must be able to - Preparation for the Interview as per Job requirement. - Set Up Professional Appearance and Etiquette for interview. - Presentation of Skills and Experience during Interview. | - Purpose of Mock Interview. - Preparation for Mock Interview. - Develop Interview Questions. - Conducting the Mock Interview. - Introduction to Interview Skills. Practical Activity: Conduct and observe mock interview for “welder job” by creating groups where trainees can practice answering questions in a simulated interview environment. | Theory: 03Hrs. Practical: 05Hrs. Total: 08Hrs. | - White board marker - White board - Questions sheet | - Class Room - Workshop |

# 0715MMT3703 - Maintain Safety at Workplace

Objective: This module covers the knowledge and  skills required to maintain safety at workplace in accordance with the organization’s approved guidelines and procedures. You will be expected to identify and use Personal Protective Equipment (PPE) according to the job requirement and potential hazards at workplace. The underpinning knowledge regarding OSH will be sufficient to provide the basis for your work.

Duration: 36 Hours			Theory: 15 Hours			Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Apply Personal Protective Equipment (PPE’s) in Workshop | Trainee will be able to: - Arrange the required personal protective equipment - Check functional condition of PPE’s - Wear personal protective equipment - Store PPE at correct place after use. | - Introduction to Personal Protective Equipment (PPE’s). - Types and importance of PPE’s. - Inspection and Maintenance of PPE’s. - PPE’s Selection and Adaptation. - Importance of Proper Storage of PPE’s. - Disadvantages/ Consequences of Improper PPE’s Storage. Practical Activity: - Practice of trainees to correctly put on and take off PPE’s for performing welding (i.e. Helmet, Apron, Shoes, and Gloves). - Please make sure to safely store the PPE’s in the assigned place/rack. | Theory: 06Hrs. Practical: 06Hrs. Total: 12Hrs. | - Welding helmets - Safety goggles - Apron - Safety shoes - Safety gloves - Respirators - Safety helmets | - Class Room - Workshop |
| Handle Hazardous Materials as per SOP’s | Trainee will be able to: - Identify Hazardous Materials and Interpret Safety Data Sheets (SDS). - Comply Safe Handling Procedures and storage Protocols - Identify Spill and Leak Procedures and Waste Disposal of Hazardous Materials | - Introduction to Hazardous Materials. - Common hazardous materials in welding shop. - Significance and structure of Safety Data Sheets (SDS). - Standard Operating Procedures (SOPs) in Safe Handling and Storage Procedures. - Spill and Leak Management. - Waste Disposal Procedures. Practical Activity: Make a team of 3-5 trainees: - Trainees to Identify and categorize hazardous materials commonly used in welding, including lifting, transferring, and transporting. | Theory: 03Hrs. Practical: 06Hrs. Total: 09Hrs. | - SOPs Charts - Regulatory Charts - Safety Data Sheets(SDS) - Visual Aids | - Class Room - Workshop |
| Operate Fire Extinguisher as per given Instructions | Trainee must be able to: - Select fire extinguisher based on the type and size of the fire. - Check expiry of fire extinguisher - Apply appropriate fire extinguishing technique such as using the PASS method (Pull, Aim, Squeeze, Sweep) for extinguishing the small fire. | - Introduction to Fire Safety. - Types of Fire Extinguishers. - Maintenance of Fire Extinguishers. - PASS Technique of Fire Extinguishers. Practical Activity: - Engage the trainees in simulated fire situations where they must choose the correct extinguisher and operate it according to the given instructions. | Theory: 03Hrs Practical: 03Hrs Total: 06Hrs | - Fire Extinguisher - Visual Aids | - Class Room - Workshop |
| Apply Emergency Response in Workshop | Trainee must be able to - Take emergency response training - Assess risk and determine course of action - Operate emergency equipment and supplies - Use First Aid Box in emergency as per set standards. | - Introduction to Workshop Hazards. - Emergency Response Protocols. - Potential risks associated with welding tasks. - Safety measures and actions to mitigate risks. - Emergency Communication. - Evacuation Procedures. - First aid box and their uses. - First Aid in the Workshop. Practical Activity: Make a team of 3-5 trainees: - Practice using first aid supplies correctly (e.g., applying bandages, using burn gel). | Theory: 03Hrs. Practical: 06Hrs. Total: 09Hrs. | - First Aid Box With necessary medicines, i.e. bandages, pyodine, sunny plast, burn gel etc. - Hearing Protection - Respiratory Mask - First Aid Box - Stretcher - Fire Buckets - Fire Extinguisher | - Class Room - Workshop |

# 0715MMT3704 - Prepare Material for Welding

Objective: This module covers the knowledge and  skills required to prepare materials for a specific job. The standard covers specific knowledge of marking the material as per drawing/job requirement, setting up cutting equipment, cutting and preparing edges of base materials, cleaning surfaces and edges, preparing welding consumables and fitting up base materials.

Duration:  51 Hours			Theory: 15 Hours			Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Measure and Mark Material/s as per Drawing / Job Requirement | Trainee will be able to: - Select and obtain required material/s as per job. - Select correct measuring tools as per job requirements. - Select correct marking tools as per job requirements. - Mark the area to be cut as per drawing / job requirements. | - Materials Properties and specifications. - Purpose of measurement. - Types of Measuring Tools. - Tool Calibration and Maintenance. - Marking tools and their purposes. - Criteria for selecting marking tools. - Techniques for marking materials. - Introduction to drawings and specifications. - Transfer measurements from drawing to work piece. - Mark the specified area to be cut as per drawing. Practical Activity: Trainees to perform measurement on Steel foot rule, steel tape, Vernier caliper, Vernier Height gauge. | Theory: 04Hrs. Practical: 06Hrs. Total: 10Hrs. | - Steel foot rule - Steel Tape - Vernier Caliper - Vernier Height Gauge - Scriber | - Class Room - Workshop |
| Cut and Prepare Edge/s of Base Materials | Trainee will be able to: - Select correct cutting equipment as per job requirements. - Set-up cutting equipment as per manufacturer’s Instructions / job requirements. - Cut the base material as per job specifications and dimensions provided in the drawing. - Prepare edges of the base materials as per Drawing / WPS. - Check dimensions of the prepared edges as per Drawing / WPS. - Clean the edges of the base materials as per job requirements. | - Types of cutting equipment. - Set up cutting equipment. - Appropriate cutting method for base material. - Techniques for edges preparation of base materials. - Check dimensions and measurement results. - Edge cleaning requirements. - Techniques for edge cleaning. Practical Activity: Trainees to perform practice on M.S plate (Dimension: 80mm x 60mm x 15mm) including Filing, Marking, Center Punching, Drilling, Counter Sinking, Counter Boring, Threading, Sawing operations. | Theory: 04 Hrs. Practical: 18Hrs. Total: 22 Hrs. | - Pedestal Drill Machine with accessories - Flat File - Line Scriber - Center Punch - Hammer - Counter Sink - Counter Bores - Tap & Die Set - Hand Hack Saw | - Class Room - Workshop |
| Prepare Welding Consumables as per Job Requirement | Trainee must be able to: - Select correct welding consumables as per job requirements / WPS. - Prepare consumables in accordance with required Specifications. - Dispose Defective Consumables in accordance with Green practices i.e. Recycle, Reuse defective metal pieces. | - Introduction to welding consumables. - Types and specifications of welding consumables - Introduction to green practices - Recycle and reuse defective metal pieces. Practical Activity: - Trainees to prepare base metal from M.S plate as per instructions/drawing including Leveling, Mill scale, Squaring, Beveling. - Make Recycling bins from scrap metal pieces. - Make scrapers from metal sheets. | Theory: 04Hrs. Practical: 06Hrs. Total: 10Hrs. | - Flat File - Hammer - Hand shear - Scriber - Try square - Hand Grinder - Beveling Machine | - Class Room - Workshop |
| Fit-up Base Materials as per Job Requirement | Trainee must be able to - Select required tools and equipment to fit-up base materials. - Tack weld joint/s as per drawing / welding procedure Specifications (WPS). - Verify root gap as per drawing / welding procedure Specifications (WPS). - Verify alignment as per applicable code / standard. | - Tools and equipment for fit-up. - Work area and materials for tack welding. - Welding techniques for tacking. - Importance of root gap - Tools and Equipment for Measuring Root Gap - Importance of welding alignment. - Alignment verification Practical Activity: - Trainees to perform Tacking and checking of root gap of base metal prepared from M.S plate. | Theory: 03Hrs. Practical: 06Hrs. Total: 09Hrs. | - Hammer - Rivet Gun - Bevel Protractor - Welding Gauges | - Class Room - Workshop |

# 0715MMT3705 - Carry Out Gas Welding And Gas Cutting Operations

Objective: This module covers the knowledge and  skills required to This competency standard is designed for gain hands-on experience in striking arcs, creating clean cuts, and producing quality welds, along with understanding the importance of maintenance and troubleshooting techniques. Upon completion, participants will be proficient in executing gas welding and cutting operations to industry standards.

Duration:  105 Hours			Theory: 18 Hours			Practice: 87 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform Flame Making on Gas Welding Torch | Trainee will be able to: - Inspect welding torch and components for damage or wear. - Identify Gas cylinders and Acetylene cylinders. - Make Weld Flames on Welding/Cutting Torch. - Adjust the flame to achieve a neutral, carburizing, or oxidizing flame as required. | - Introduction to gas welding torch. - Components and functions of gas welding torch. - Purpose of gas cylinders in welding. - Comparison between Oxygen and Acetylene cylinders. - Handling and Storage of Gas Cylinders - Types of welding flames: - Ignite welding/cutting torch. - Flame adjustment techniques. - Impact of flame type on weld quality. Practical Activity: Trainees to perform the flame making on a gas welding torch to achieve the desired flame (Neutral, Oxidizing, and Carburizing) characteristics for Oxy-Acetylene gas welding. | Theory: 04Hrs. Practical: 09Hrs. Total: 13Hrs. | - Oxy-Acetylene Gas Welding Plant with all Accessories. - Gas Welding Table with Fire Brick - Tip Cleaner - Spark Lighter | - Class Room - Workshop |
| Perform Puddle Formation and Straight Bead on Base Metal | Trainee will be able to: - Demonstrate strike arc consistently on base metal. - Maintain stable weld puddle by controlling the arc length, travel speed, and heat input. - Identify and address common weld defects, making necessary adjustments to techniques. | - Base metal for welding operation - Strike and Control the Arc - Concept of welding puddle in gas welding. - Factors affect weld puddle stability. - Common welding defects. - Techniques for preventing weld defects. Practical Activity: Trainees to perform puddle formation on M.S plate of size 3mm thick. | Theory: 04Hrs. Practical: 18Hrs. Total: 22Hrs. | - Oxy-Acetylene Gas Welding Plant with all Accessories. - Gas Welding Table with Fire Brick - Tip Cleaner - Spark Lighter - Hammer - Center Punch - Line Scriber - Flat File | - Class Room - Workshop |
| Carry Out Gas Cutting on Base Metal | Trainee must be able to: - Ignite and Adjust Welding torch / Cutting torch. - Perform Oxy-Acetylene Gas Cutting M.S Plate. - Follow operational Instructions of cutting torch - Identify and prepare base metal for cutting. - Identify and address common defects, such as beveling or excessive dross. | - Inspection of welding/cutting torch - Oxy-Acetylene gas cutting. - Oxy-Acetylene gas cutting operation on MS Plate - Operational Instructions of cutting torch. - Base metal properties - Base Metal Selection for Cutting - Common welding defects. - Techniques to address beveling or excessive dross issues. Practical Activity: Trainees to perform gas cutting on M.S plate. (Dimensions: 150mm x 75mm x 12mm) or as per instructions. - Collect and properly dispose of or recycle welding wastes, such as metal scraps and used consumables. | Theory: 04Hrs. Practical: 12Hrs. Total: 16Hrs. | - Oxy-Acetylene Gas Cutting Plant with all Accessories. - Gas Cutting Table with Fire Brick - Tip Cleaner - Spark Lighter - Hand Vice - Leather Gloves - Leather Apron - Safety Shoes | - Class Room - Workshop |
| Carry out Soldering and Brazing on Base Metal | Trainee must be able to: - Select correct soldering and brazing equipment and materials (solder, flux, brazing rods). - Perform Soldering on different Metals as per requirement and standard procedures. - Perform Brazing on different Metals as per requirement and standard procedures. - Inspect the soldered or brazed joints for uniformity, smoothness, and the absence of defects such as voids or cracks. | - Soldering and brazing equipment - Solder / flux materials and brazing rods. - Introduction to Soldering - Types of metals used in soldering - Soldering techniques - Brazing principles - Material identification and suitability for brazing - Brazing procedures on metals - Common defects in joints - Inspection tools and equipment - Corrective measures for defective joints Practical Activity: Trainees to perform Butt/Lap/T joints by soldering and brazing on mild steel (M.S) plates (specified thickness and dimensions as per instructions), demonstrating proper techniques, safety measures, and inspection methods. - Collect and properly dispose of or recycle welding wastes, such as metal scraps and used consumables. | Theory: 03Hrs. Practical: 24Hrs. Total: 27Hrs. | - Oxy-Acetylene Gas Welding Plant with all Accessories - Gas Welding Table with Fire Brick - Soldering Iron 450 Watt With Stand - Brass Filler Rod - Leather Gloves - Gas Welding Goggle - Safety Shoes - Leather Apron - Tong - Tip Cleaner - Wire Brush - Spark Lighter - Hand Vice | - Class Room - Workshop |
| Perform Project Work as per Instructions | Trainee must be able to: - Perform Puddle formation from M.S Plate. - Perform Brazing on Lap/Butt joint from M.S Plate. | - Introduction to puddle formation - Tools and equipment for puddle formation. - Introduction to Lap and Butt joint. - Brazing procedure on Lap/Butt joint from M.S plate Practical Activity: - Trainees to perform the gas welding on Lap/Butt/T joint from M.S plate and M.S pipe considering appropriate procedure. | Theory: 03Hrs. Practical: 24Hrs. Total: 27Hrs. | - Oxy-Acetylene Gas Welding Plant with all Accessories. - Gas Welding Table with Fire Brick - Tip Cleaner - Spark Lighter - Hand Vice - Leather Gloves - Leather Apron - Safety Shoes | - Class Room - Workshop |

# 0715MMT3706 - Carry out Shielded Metal Arc Welding (SMAW) in different Positions

Objective: This module covers the knowledge and  skills required to This Competency Standard is designed to gain basic knowledge and skills required to perform Shielded Metal Arc Welding (SMAW) operations in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G), Overhead (4F, 4G) and Pipe Welding (5F, 5G, 6G) positions at workplace. The standard covers specific knowledge of performing Shielded Metal Arc Welding (SMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in positions of plate/Pipe. The standard also covers post welding Procedures comprising cleaning, measuring, inspecting and repairing welds at workplace.

Duration:  	132 Hours			Theory: 18 Hours			Practice: 114 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Welding Machine and Accessories for SMAW | Trainee must be able to: - Identify welding requirements, welding procedure specifications and/or technical drawings. - Prepare SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions. - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions. - Set polarity indicated in the welding procedure specifications. | - Job requirements as per WPS/drawings - Introduction and types of SMAW power source. - SMAW power source accessories - Current setting as per job requirement - Welding output current AC/DC - Classification of SMAW electrodes - Polarity, Types and uses Practical Activity: Trainees to identify the parts and accessories of the SMAW machine. | Theory: 04Hrs. Practical: 03Hrs. Total: 07Hrs. | - SMAW power source with all accessories - Mild steel plates - SMAW electrodes - Electrode backing oven - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - MS wire brush - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Lights / - Emergency - Lights | - Class Room - Workshop |
| Perform Striking and Blind Bead on Base Metal | Trainee will be able to: - Use the correct striking technique to avoid sticking the electrode to the base metal. - Adjust the arc length as needed to avoid excessive spatter and achieve a smooth weld bead. - Produce uniform bead with consistent width and height along the weld path. | - Principles of arc striking and blind bead welding. - Techniques for striking an arc. - Concept of arc length in welding. - Relationship between arc length, spatter, and bead quality. - Factors effecting a uniform weld bead. Practical Activity: Trainees to perform striking of electrode on base metal and make blind bead on M.S plate. Select welding rods and other materials that have lower environmental impact. | Theory: 04Hrs. Practical: 18Hrs Total: 22Hrs. | - SMAW power source with all accessories - Mild steel plates - SMAW electrodes - Electrode backing oven - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Chipping hammer - MS wire brush - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables | - Class Room - Workshop |
| Perform Fillet Welds on Mild Steel Plates / Pipes | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures and according to given requirements - Make Project work as per instruction. | - Welding parameters’ adjustment. - Effects of Arc Length - Striking the arc and maintaining the arc gap - Standard procedure for fillet weld on mild steel plate and pipe in 1F, 2F, 3F, 4F and 5F positions. - Visual Inspection - Acceptance criteria for welding defects Practical Activity: Make a fillet weld (Butt/Lap/T-joint) in 1F, 2F, 3F, 4F and 5F positions with SMAW Process. | Theory: 04Hrs. Practical: 45Hrs. Total: 49Hrs. | - Mild steel plates - SMAW electrodes - Electrode backing oven - Grinder - Angle cutting Machine/ Cut off Machine - Cutting discs - Grinding discs - Chipping hammer - MS wire brush - File set - Tongs - Combination Plier - Grip Plier/Burner Plier - Ear plugs - Fume extractors - Exhaust fans - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Fire Extinguishers - Cotton gloves - Leather apron - Welding gloves - Welding helmet - Safety goggles - Safety helmet - Safety shoes - Set of nose pliers - Set of screw drivers | - Class Room - Workshop |
| Perform Groove Welds on Mild Steel Plates / Pipes | T Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures and according to given requirements - Deposit root pass, filling passes and capping pass as per welding procedure specifications/job requirements. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Welding parameters’ adjustment. - Gap between electrode and base metal. - Standard procedure for groove weld on mild steel plate and pipe in 1G, 2G, 3G, 4G, 5G and 6G positions. - Root face, root gap and root pass - Hot pass - Filling pass - Capping pass - Weaving and stringer technique - Carry out visual Inspection - Acceptance criteria for welding defects Practical Activity: - Make a groove weld (Butt-joint) in 1G, 2G, 3G, 4G, 5G and 6G positions with SMAW process | Theory: 03Hrs. Practical: 45Hrs. Total: 48Hrs. | - Mild steel plates - SMAW electrodes - Electrode backing oven - Grinder - Angle cutting Machine/ Cut off Machine - Cutting discs - Grinding discs - Chipping hammer - MS wire brush - File set - Tongs - Combination Plier - Grip Plier/Burner Plier - Ear plugs - Fume extractors - Exhaust fans - WPS/ instruction sheet - Welding tables - Jigs and fixtures | - Class Room - Workshop |
| Perform Post Welding Procedures | Trainee must be able to: - Inspect weld visually and mark any visual defects, as required. - Carry out repair work in accordance with approved procedures, as required. - Maintain and store tools / equipment / consumable materials in accordance with organization guidelines. - Apply 6 R’s as per Green practices | - Introduction to welding defects - Types of welding defects, causes and remedies - Methods of welding repairing. - Clean the work area - Occupational safety practice at the workplace - Organization’s/ workshop guidelines for storing tools, equipment and consumable materials. Practical Activity: Trainees to perform visual inspection of the welded job, and mark any visual defects if occurred. | Theory: 03Hrs. Practical: 03Hrs. Total: 06Hrs. | - Penetrants spray - Inspection tools | - Class Room - Workshop |

# 0715MMT3707 - Carry Out Gas Metal Arc Welding (GMAW) in Different Positions

Objective: This module covers the knowledge and  skills required to perform Gas Metal Arc Welding (GMAW) operations in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G), Overhead (4F, 4G) and Pipe Welding (5F, 5G, 6G) positions at workplace. The standard covers specific knowledge of performing Gas Metal Arc Welding (GMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in positions of plate/Pipe. The standard also covers post welding Procedures comprising cleaning, measuring, inspecting and repairing welds at workplace.

Duration:  132 Hours			Theory: 18 Hours			Practice: 114 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Welding Machine and Accessories for GMAW | Trainee will be able to: - Identify welding requirements, welding procedure specifications and/or technical drawings. - Prepare GMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions. - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions. - Set polarity indicated in the welding procedure specifications. | - Welding requirements as per WPS/technical drawings - Introduction to GMAW power source - GMAW power source accessories - Current and voltage setting as per job requirement - MIG wire as per job requirement - Polarity and its types - Uses of polarity Practical Activity: Trainees to identify the parts and accessories of the GMAW machine. | Theory: 04Hrs. Practical: 03Hrs. Total: 07Hrs. | - GMAW power source with all accessories - Mild steel plates - GMAW electrodes - Electrode backing oven - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Beveling machine - Chipping hammer - MS wire brush - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Lights/ Emergency lights | - Class Room - Workshop |
| Perform Blind Bead on Base Metal | Trainee will be able to: - Adjust arc length as needed to avoid excessive spatter and achieve a smooth weld bead. - Produce uniform bead with consistent width and height along the weld path. | - Arc length in welding. - Relationship between arc length, spatter, and bead quality. - Factors for uniform weld bead. - Concept of maintaining width and height throughout the weld path. Practical Activity: Trainees to perform striking of electrode on base metal and make blind bead on M.S plate. | Theory: 03Hrs. Practical: 18Hrs. Total: 21Hrs. |  | - Class Room - Workshop |
| Perform Fillet Welds on Mild Steel Plates / Pipes | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current and Voltage setting as per job requirement - Effects of Arc Length - Striking the arc and maintain the arc gap. - Introduction to Fillet weld on mild steel in 1F and 2F positions - Terms used in making project work - Visual Inspection - Acceptance criteria for welding defects Practical Activity: - Make a fillet weld (T-joint) in 1F and 2F positions with GMAW process. - Set the wire feed speed correctly to minimize wastage. Use only the amount of welding wire needed for the job. | Theory: 04Hrs. Practical: 45Hrs. Total: 49Hrs. |  | - Class Room - Workshop |
| Perform Groove Welds on Mild Steel Plates / Pipes | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures and according to given requirements - Deposit root pass, filling passes and capping pass as per welding procedure specifications/job requirements. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects - Make Project work as per instruction. | - Welding parameters - Effects of Arc Length - Striking of arc on base metal - Maintaining the arc gap - Overview of groove weld on mild steel plate and pipe in 1G, 2G, 3G, 4G, 5G and 6G positions. - Root face, root gap and root pass - Hot pass, Filling pass and Capping pass. - Visual Inspection - Criteria for welding defects - Weaving and stringer technique Practical Activity: Make a groove weld (Butt-joint) in 1G, 2G, 3G, 4G, 5G and 6G positions with GMAW process | Theory: 04Hrs. Practical: 45Hrs. Total: 49Hrs. |  | - Class Room - Workshop |
| Perform Post Welding Procedures | Trainee must be able to: - Carry out finishing work of welds following standard procedures. - Inspect weld visually and mark any visual defects, as required. - Carry out repair work in accordance with approved procedures, as Required. - Maintain and store tools/equipment/ consumable materials in accordance with organization Guidelines. - Apply 6 R’s as per Green practices. | - Introduction to welding defects - Types of welding defects, causes and remedies - Types of visual defects. - Methods for welding repair - Occupational safety practice at the workplace - Organization’s/ workshop guidelines for storing tools, equipment and consumable materials Practical Activity Trainees to perform visual inspection of the welded job, and mark any visual defects if occurred. | Theory: 03Hrs. Practical: 03Hrs. Total: 06Hrs. |  | - Class Room - Workshop |

# 0715MMT3708 - Carry out Gas Tungsten Arc Welding (GTAW) in different Positions

Objective: This module covers the knowledge and  skills required to This Competency Standard is designed to gain basic knowledge and skills required to perform Gas Tungsten Arc Welding (GTAW) operations in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G), Overhead (4F, 4G) and Pipe Welding (5F, 5G, 6G) positions at workplace. The standard covers specific knowledge of performing Gas Tungsten Arc Welding (GTAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in positions of plate/Pipe. The standard also covers post welding Procedures comprising cleaning, measuring, inspecting and repairing welds at workplace.

Duration:  132 Hours			Theory: 18 Hours			Practice: 114 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Welding Machine and Accessories for GTAW | Trainee must be able to: - Identify welding requirements, welding procedure specifications and/or technical drawings. - Prepare GTAW welding machine in accordance with welding procedure specifications/ manufacturer instructions. - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions. - Set polarity indicated in the welding procedure specifications. | - Job requirements in workshop - Introduction to GTAW power source - GTAW power source accessories - Current setting - Cored/tubular wire as per job requirement - Define of Polarity - Types of polarity and its uses Practical Activity: - Trainees to identify the parts and accessories of the GTAW machine. | Theory: 04Hrs. Practical: 03Hrs. Total: 07Hrs. | - GTAW power source with all accessories - Mild steel plates - GTAW electrodes - Electrode backing oven - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - MS wire brush - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Lights/ Emergency lights | - Class Room - Workshop |
| Perform Blind Bead on Base Metal | Trainee will be able to: - Adjust arc length as needed to avoid excessive spatter and achieve a smooth weld bead. - Produce uniform bead with consistent width and height along the weld path. | - Concept of arc length in welding. - Relationship between arc length, spatter, and bead quality. - Factors that contribute to producing a uniform weld bead. Practical Activity: - Trainees to perform striking of electrode on base metal and make blind bead on MS plate. | Theory: 03Hrs. Practical: 18Hrs. Total: 21Hrs. | - Mild steel plates - GTAW electrodes - Electrode backing oven - Grinder - Angle cutting Machine/ Cut off Machine - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - MS wire brush - File set - Tongs - Combination Plier - Grip Plier/Burner Plier - Ear plugs - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Fire Extinguishers - Cotton gloves - Leather apron - Welding gloves - Welding helmet - Safety goggles - Safety helmet - Safety shoes - Set of nose pliers - Set of screw drivers | - Class Room - Workshop |

| Perform Fillet Welds on Mild Steel Plates / Pipes | Trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures and according to given requirements. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current and Voltage setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Overview of Fillet weld on mild steel in 1F and 2F positions - Terms in making project work Visual Inspection Acceptance criteria for welding defects Practical Activity: - Make a fillet weld (T-joint) in 1F and 2F positions with GTAW process. - Use the correct size and type of tungsten electrode for the job. Sharpen electrodes properly to avoid wastage and ensure optimal performance. | Theory: 04Hrs. Practical: 45Hrs. Total: 49Hrs. |  | - Class Room - Workshop |
| --- | --- | --- | --- | --- | --- |
| Perform Groove Welds on Mild Steel Plates / Pipes | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in different positions on Mild steel plates/pipes as per standard procedures. - Deposit root pass, filling passes and capping pass as per welding procedure specifications/job requirements. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Introduction of groove weld on mild steel plate and pipe in 1G, 2G, 3G, 4G, 5G and 6G positions. - Root face, root gap and root pass - Hot pass, Filling pass and Capping pass - Weaving and stringer technique - Visual Inspection - Acceptance criteria for welding defects Practical Activity: - Make a groove weld (Butt-joint) in 1G, 2G, 3G, 4G, 5G and 6G positions with GTAW process. | Theory: 04Hrs. Practical: 45Hrs. Total: 49Hrs. |  | - Class Room - Workshop |
| Perform Post Welding Procedures | Trainee must be able to: - Carry out finishing work of welds following standard procedures. - Inspect weld visually and mark any visual defects, as required. - Carry out repair work in accordance with approved procedures, as Required. - Maintain and store tools/equipment/ consumable Materials in accordance with organization guidelines. - Apply 6 R’s as per Green practices | - Introduction to welding defects - Types of welding defects, causes and remedies - Types of visual defects. - Methods for welding repair - Occupational safety practice at the workplace - Organization’s / workshop guidelines for storing tools, equipment and consumable materials Practical Activity: - Trainees to perform visual inspection of the welded job, and mark any visual defects if occurred. | Theory: 03Hrs. Practical: 03Hrs. Total: 06Hrs. |  | - Class Room - Workshop |

# 0715MMT3709 - Carry out Flux Cored Arc Welding (FCAW) in Flat (1F, 1G) and Horizontal (2F, 2G) Positions

Objective:This module covers the knowledge and skills required to prepare perform Flux Cored Arc Welding (FCAW) operations at workplace. The standard covers specific knowledge of performing Flux Cored Arc Welding (FCAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in Flat (1F, 1G) and Horizontal (2F, 2G) Positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace.

Duration: 69 Hours			Theory: 15 Hours			Practice: 54 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Welding Machine and Accessories for FCAW | Trainee will be able to: - Identify welding requirements from the job, welding procedure specifications and/or technical drawings. - Prepare FCAW welding machine in accordance with welding procedure specifications/manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Set polarity indicated in the welding procedure specifications. | - Welding requirements as per WPS/technical drawings - Introduction to FCAW power source - FCAW power source accessories - Current and voltage setting as per job requirement - Flux-Cored wire Selection as per job requirement - Define of Polarity - Types of polarity and its uses Practical Activity: Trainees to identify the parts and accessories of the GTAW machine. | Theory: 04Hrs. Practical: 03Hrs. Total: 07Hrs. | - FCAW power source with all accessories - Mild steel plates - FCAW electrodes - Electrode backing oven - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - MS wire brush - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Lights/ Emergency lights | Class Room and workshop |
| Perform Fillet Welds on Mild Steel Plates | Trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in Flat (1F) and Horizontal (2F) positions following standard procedures. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current and Voltage setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Standard procedure used to fillet weld on mild steel in 1F and 2F positions - Terms in making project work - Visual Inspection - Criteria for welding defects Practical Activity: - Make a fillet weld (T-joint) in 1F and 2F positions with FCAW process. Use the appropriate type and size of flux-cored wire for the welding job to minimize waste and ensure effective use of materials. | Theory: 04Hrs. Practical: 24Hrs Total: 28Hrs | - Mild steel plates - FCAW electrodes - Electrode backing oven - Grinder - Angle cutting Machine/ Cut off Machine - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - MS wire brush - File set - Tongs - Combination Plier - Grip Plier/Burner Plier - Ear plugs - Fume extractors - Exhaust fans - Pencil Grinder - WPS/ instruction sheet - Welding tables - Jigs and fixtures - Fire Blankets - Fire Extinguishers - Cotton gloves - Leather apron - Welding gloves - Welding helmet - Safety goggles - Safety helmet - Safety shoes - Set of nose pliers - Set of screw drivers | Class Room and workshop |
| Perform Groove Welds on Mild Steel Plates. | Trainee must be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld. - Maintain gap between electrode and base metal as per standard practices. - Carry out welding in Flat (1G) And Horizontal (2G) positions following standard procedures. - Deposit root pass, filling passes and capping pass as per welding procedure specifications/job requirements. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Introduction to Groove weld on mild steel plate and pipe in 1G and 2G positions. - Proper Fit up as per requirement - Root face, root gap and root pass - Hot pass, filling pass and Capping pass - Weaving and stringer technique - Visual Inspection - Acceptance criteria for welding defects Practical Activity: Make a groove weld (Butt-joint) in 1G and 2G positions with FCAW process | Theory: 04Hrs. Practical: 24Hrs. Total: 28Hrs. |  | Class Room and workshop |
| Perform Post Welding Procedures | Trainee must be able to: - Carry out finishing work of welds following standard procedures. - Inspect weld visually and mark any visual defects, as required. - Carry out repair work in accordance with approved procedures, as required. - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines. - Apply 6 R’s as per Green practices. | - Introduction to welding defects - Types of welding defects, causes and remedies - Types of visual defects. - Methods for welding repai - Occupational safety practice at the workplace - Organization’s/workshop guidelines for storing tools, equipment and consumable materials Practical Activity: Trainees to perform visual inspection of the welded job, and mark any visual defects if occurred. | Theory: 03Hrs. Practical:03Hrs. Total: 06Hrs. |  | Class Room and workshop |

0715MMT3710 - Carry out Submerged Arc Welding (SAW)

Objective: This module covers the knowledge and skills required to gain basic knowledge and skills required to perform Submerged Arc Welding (SAW) operations at workplace. The standard covers specific knowledge of performing Submerged Arc Welding (SAW)) by selecting and setting up welding equipment, installing consumables, adjusting and welding parameters and making fillet and groove welds at different positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace

Duration: 63 Hours			Theory: 15 Hours			Practice: 48 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare Welding Machine and Accessories for SAW | Trainee will be able to: - Identify welding requirements from the job, welding procedure specifications and/or technical drawings. - Prepare SAW welding machine in accordance with welding procedure specifications/manufacturer instructions. - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions. - Set polarity indicated in the welding procedure specifications. | - Job requirements as per WPS/technical drawings - Introduction to SAW power source and its types - SAW power source accessories - Current and voltage setting as per job requirement - Selection of cored/tubular wire as per job requirement - Define of Polarity - Types of polarity and its uses Practical Activity: Trainees to identify the parts and accessories of the SAW machine. | Theory: 04Hrs. Practical: 03Hrs. Total: 07Hrs. | - SAW power source with all accessories - Wire spool for SAW - Granular Flux - Mild steel plates - M.S Pipe - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - Pencil Grinder - WPS/ instruction sheet | Class Room and workshop |
| Perform Fillet Welds on Mild Steel Plates | Trainee will be able to: - Adjust welding parameters (current, voltage, wire feed speed, welding speed etc.) as per welding procedure specifications/job requirements to produce acceptable Weld. - Maintain gap between electrode and base metal as per standard practices - Carry out welding in 1F position following standard procedures. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current and Voltage setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Introduction of fillet weld on mild steel in 1F and 2F positions - Visual Inspection - Acceptance criteria for welding defects Practical Activity: - Make a fillet weld (T-joint) in 1F and 2F positions with SAW process. Set the correct welding parameters (voltage, current, and travel speed) based on the material and thickness to optimize energy use and prevent overheating. | Theory: 04Hrs. Practical: 24Hrs. Total: 28Hrs. | - SAW power source with all accessories - Wire spool for SAW - Granular Flux - Mild steel plates - M/S Pipe - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Bevelling machine - Chipping hammer - Pencil Grinder - WPS/ instruction sheet | Class Room and workshop |
| Perform Groove Welds on Mild Steel Plates. | Trainee must be able to: - Adjust welding parameters (current, voltage, wire feed Speed, Welding speed etc.) as per welding procedure Specifications / job Requirements to produce acceptable Weld. - Maintain gap between electrode and base metal as Per standard practices. - Carry out welding in 1G position following standard procedures - Deposit root pass, filling passes and capping pass as per welding procedure specifications/job requirements. - Make Project work as per instruction. - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Current setting as per job requirement - Effects of Arc Length - Striking the arc and maintaining the arc gap - Overview of groove weld on mild steel plate and pipe in 1G position. - Proper Fit up as per requirement - Root face, root gap and root pass - Hot pass, filling pass and Capping pass - Weaving and stringer technique - Visual Inspection - Acceptance criteria for welding defects Practical Activity: Make a groove weld (Butt-joint) in 1G and 1G positions with SAW process | Theory: 04Hrs. Practical: 18Hrs. Total: 22Hrs. | - SAW power source with all accessories - Wire spool for SAW - Granular Flux - Mild steel plates - M.S Pipe - Angle cutting Machine/ Cut off Machine - Grinder - Cutting discs - Grinding discs - Beveling machine - Chipping hammer - Pencil Grinder WPS/ instruction sheet | Class Room and workshop |
| Perform Post Welding Procedures | Trainee must be able to: - Carry out finishing work of welds following standard procedures. - Inspect weld visually and mark any visual defects, as required. - Carry out repair work in accordance with approved procedures, as required. - Maintain and store tools / equipment / consumable materials in accordance with organization guidelines. - Apply 6 R’s as per Green practices | - Introduction to welding defects - Types of welding defects, causes and remedies - Types of visual defects. - Methods for welding repair - Occupational safety practice at the workplace - Organization’s / workshop guidelines for storing tools, equipment and consumable materials Practical Activity: Trainees to perform visual inspection of the welded job, and mark any visual defects if occurred. | Theory: 03Hrs. Practical: 03Hrs. Total: 06Hrs. |  | Class Room and workshop |

# 0715MMT3711 - Develop Entrepreneurship Skills

Objective:  This module is designed to equip Welder with the skill and knowledge required to apply entrepreneurship skills, concept, market analysis and investment.

Duration:  26 Hours			Theory: 10 Hours			Practice: 16 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interpret Entrepreneurship | - Understand Entrepreneur- Entrepreneurship - Enterprises: conceptual issue Entrepreneurship vs. management, Entrepreneurial motivation. - Understand the functions of entrepreneurs in relation to the enterprise and economy. - Identify source of business ideas | - Entrepreneurship - Difference between enterprise and entrepreneurship | Theory: 04Hrs. Practical: 05Hrs. Total: 09Hrs. | - Project Proposals | Class room and Lab |
| - Illustrate Project Preparation & Marketing Analysis | - Understand the qualities of a good entrepreneur, SWOT, and risk analysis. - Develop communication skills between buyer and seller. - Keep record of each business deal (email, etc.). | - SWOT analysis - Communications skills related to buyer and seller - Record keeping for business deal Practical Activity: - Perform SWOT analysis of the given project | Theory: 03Hrs. Practical:05Hrs. Total: 08Hrs. | - SWOT analysis of previous projects - Record Registers | Class room and Lab |
| - Interpret Investment Procurement | - Develop project, feasibility, legal formalities. - Understand investment procedure - Loan procurement - Banking processes. | - Process for developing a project feasibility, and legal formalities - Procedures of loan procurement for investment Practical Activity: - Prepare a project proposal for acquiring loan | Theory: 03Hrs. Practical:06Hrs. Total: 09Hrs. | - Project feasibility reports - Loan forms | Class room and Lab |