|  | Assessment Evidence Guide For National Vocational Certificate in Welder Level-3 Summative Assessment |
| --- | --- |
|  | Date 04th - 09th November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): Activity-02: Perform Groove Welding to make Butt joint in 3G Position using SMAW/GMAW/FCAW/GTAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) (See Annex-2) |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Activity-02: Perform Groove Welding to make Butt joint in 3G Position using SMAW/GMAW/FCAW/GTAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) - Ensure all welding tools, equipment, and personal protective equipment (PPE) are selected and inspected according to standard operating procedures. - Verify the M.S. plate dimensions (200mm x 38mm x 10mm) and check for cleanliness, free of rust, oil, or other contaminants. - Set up the work piece in the 3G (vertical) position, securely clamping or fixing it to prevent movement during welding. - Select the appropriate welding process (SMAW/GMAW/FCAW/GTAW) based on task requirements and adjust machine settings for current, voltage, and polarity according to the material thickness and electrode or filler specifications. - Confirm that the electrodes or filler wires are suitable for the material and process, ensuring compatibility with the Mild Steel (M.S.) plate. - Prepare the edge bevels on the M.S. plate as per specifications to achieve the required groove angle and root gap. - Position and align plates with the specified root opening, ensuring proper alignment to prevent excessive distortion. - Begin with tack welds to secure the plates, ensuring they are evenly spaced and free from defects such as porosity or cracks. - Initiate welding with a steady hand, controlling the arc length and speed to achieve a uniform bead profile. - Control heat input to prevent excessive penetration or undercut, maintaining a consistent arc length in the 3G vertical position. - For thicker sections, apply multiple passes, ensuring each layer is uniform and adequately fused to the previous pass. - Clean each pass by removing slag and spatter before proceeding with the next layer, especially when using processes like SMAW or FCAW. - Inspect the completed weld for uniformity, smoothness, and continuity along the weld path, with no visible defects such as cracks, porosity, undercut, or incomplete fusion. - Ensure the weld profile meets dimensional requirements for width, height, and penetration. - Remove any slag, spatter, and other residues from the work piece to leave a clean, finished surface. - Conduct a final inspection, ensuring compliance with quality standards and welding specifications for the 3G butt joint. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate in Welder Level-03 |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Activity-02: Perform Groove Welding to make Butt joint in 3G Position using SMAW/GMAW/FCAW/GTAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) (See Annex-2) |

I can……………….

| Performance Criteria (Activity-2) | Yes | No |
| --- | --- | --- |
| - Ensure all welding tools, equipment, and personal protective equipment (PPE) are selected and inspected according to standard operating procedures. |  |  |
| - Verify the M.S. plate dimensions (200mm x 38mm x 10mm) and check for cleanliness, free of rust, oil, or other contaminants. |  |  |
| - Set up the work piece in the 3G (vertical) position, securely clamping or fixing it to prevent movement during welding. |  |  |
| - Select the appropriate welding process (SMAW/GMAW/FCAW/GTAW) based on task requirements and adjust machine settings for current, voltage, and polarity according to the material thickness and electrode or filler specifications. |  |  |
| - Confirm that the electrodes or filler wires are suitable for the material and process, ensuring compatibility with the Mild Steel (M.S.) plate. |  |  |
| - Prepare the edge bevels on the M.S. plate as per specifications to achieve the required groove angle and root gap. |  |  |
| - Position and align plates with the specified root opening, ensuring proper alignment to prevent excessive distortion. |  |  |
| - Begin with tack welds to secure the plates, ensuring they are evenly spaced and free from defects such as porosity or cracks. |  |  |
| 9. Initiate welding with a steady hand, controlling the arc length and speed to achieve a uniform bead profile. |  |  |
| - Control heat input to prevent excessive penetration or undercut, maintaining a consistent arc length in the 3G vertical position. |  |  |
| - For thicker sections, apply multiple passes, ensuring each layer is uniform and adequately fused to the previous pass. |  |  |
| - Clean each pass by removing slag and spatter before proceeding with the next layer, especially when using processes like SMAW or FCAW. |  |  |
| - Inspect the completed weld for uniformity, smoothness, and continuity along the weld path, with no visible defects such as cracks, porosity, undercut, or incomplete fusion. |  |  |
| - Ensure the weld profile meets dimensional requirements for width, height, and penetration. |  |  |
| - Remove any slag, spatter, and other residues from the work piece to leave a clean, finished surface. |  |  |
| - Conduct a final inspection, ensuring compliance with quality standards and welding specifications for the 3G butt joint. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Activity-02: Perform Groove Welding to make Butt joint in 3G Position using SMAW/GMAW/FCAW/GTAW (Material: M.S Plate, Size: 200mm x 38mm x 10mm) (See Annex-2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: (Activity-2) |  |  |  | Yes | No | Remarks |
|  | Ensure all welding tools, equipment, and personal protective equipment (PPE) are selected and inspected according to standard operating procedures. |  |  |  |  |  |
|  | Verify the M.S. plate dimensions (200mm x 38mm x 10mm) and check for cleanliness, free of rust, oil, or other contaminants. |  |  |  |  |  |
|  | Set up the work piece in the 3G (vertical) position, securely clamping or fixing it to prevent movement during welding. |  |  |  |  |  |
|  | Select the appropriate welding process (SMAW/GMAW/FCAW/GTAW) based on task requirements and adjust machine settings for current, voltage, and polarity according to the material thickness and electrode or filler specifications. |  |  |  |  |  |
|  | Confirm that the electrodes or filler wires are suitable for the material and process, ensuring compatibility with the Mild Steel (M.S.) plate. |  |  |  |  |  |
|  | Prepare the edge bevels on the M.S. plate as per specifications to achieve the required groove angle and root gap. |  |  |  |  |  |
|  | Position and align plates with the specified root opening, ensuring proper alignment to prevent excessive distortion. |  |  |  |  |  |
|  | Begin with tack welds to secure the plates, ensuring they are evenly spaced and free from defects such as porosity or cracks. |  |  |  |  |  |
|  | 9. Initiate welding with a steady hand, controlling the arc length and speed to achieve a uniform bead profile. |  |  |  |  |  |
|  | Control heat input to prevent excessive penetration or undercut, maintaining a consistent arc length in the 3G vertical position. |  |  |  |  |  |
|  | For thicker sections, apply multiple passes, ensuring each layer is uniform and adequately fused to the previous pass. |  |  |  |  |  |
|  | Clean each pass by removing slag and spatter before proceeding with the next layer, especially when using processes like SMAW or FCAW. |  |  |  |  |  |
|  | Inspect the completed weld for uniformity, smoothness, and continuity along the weld path, with no visible defects such as cracks, porosity, undercut, or incomplete fusion. |  |  |  |  |  |
|  | Ensure the weld profile meets dimensional requirements for width, height, and penetration. |  |  |  |  |  |
|  | Remove any slag, spatter, and other residues from the work piece to leave a clean, finished surface. |  |  |  |  |  |
|  | Conduct a final inspection, ensuring compliance with quality standards and welding specifications for the 3G butt joint. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | What factors affect the cost estimation of a welding job? |  |  |  |
|  | What are the main differences between SMAW and GMAW |  |  |  |
|  | What are the challenges of welding in the overhead position (4G) compared to flat welding? |  |  |  |
|  | What is slag, and why is it important to remove it after welding? |  |  |  |
|  | What is spot welding, and what types of joints is it typically used for? |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |

(Annex-2)

Groove Butt Joint