|  | Assessment Evidence Guide For National Vocational Certificate in Welder Level-3 Summative Assessment |
| --- | --- |
|  | Date 04th - 09th November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): Activity-03: Perform Spot welding to make Lap joint (Material: M.S Sheet, Size: 100mm x 50mm x 20SWG) (See Annex-3) |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Activity-03: Perform Spot welding to make Lap joint (Material: M.S Sheet, Size: 100mm x 50mm x 20SWG) - Select and confirm the M.S. sheet meets required specifications (100mm x 50mm x 20 SWG). - Clean the surfaces to be joined to ensure they are free of contaminants, such as grease, rust, or dust that could affect weld quality. - Position the sheets to achieve the specified overlap for a lap joint. - Select and set up the appropriate spot-welding machine based on the sheet thickness and material. - Adjust the electrode pressure, current, and welding time according to the standard welding procedure for 20 SWG mild steel. - Confirm electrode alignment to ensure consistent contact with the material during welding. - Position the electrodes correctly over the lap joint area to achieve a centered weld. - Apply controlled pressure and initiate the spot-welding cycle, ensuring a smooth start and proper fusion between sheets. - Verify that the weld time is consistent with the procedure to prevent overheating or insufficient bonding. - Inspect the spot weld visually for uniformity, smoothness, and correct weld nugget formation. - Check for any visible defects, such as porosity, cracking, or burn-through, that indicate improper welding parameters. - Perform a pull test or shear test on the joint to verify adequate weld strength as per the standards for 20 SWG mild steel. - Clean any oxidation or slag around the weld area to improve joint appearance and prevent corrosion. - Ensure the lap joint dimensions meet the specified tolerances and no misalignment occurred during welding. - Follow all safety procedures, including wearing appropriate PPE (e.g., gloves, welding helmet, safety glasses). - Ensure that the work area is free from flammable materials and that proper ventilation is maintained during welding. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate in Welder Level-03 |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Activity-03: Perform Spot welding to make Lap joint (Material: M.S Sheet, Size: 100mm x 50mm x 20SWG) (See Annex-3) |

I can……………….

| Performance Criteria (Activity-3) | Yes | No |
| --- | --- | --- |
| - Select and confirm the M.S. sheet meets required specifications (100mm x 50mm x 20 SWG). |  |  |
| - Clean the surfaces to be joined to ensure they are free of contaminants, such as grease, rust, or dust that could affect weld quality. |  |  |
| - Position the sheets to achieve the specified overlap for a lap joint. |  |  |
| - Select and set up the appropriate spot welding machine based on the sheet thickness and material. |  |  |
| - Adjust the electrode pressure, current, and welding time according to the standard welding procedure for 20 SWG mild steel. |  |  |
| - Confirm electrode alignment to ensure consistent contact with the material during welding. |  |  |
| - Position the electrodes correctly over the lap joint area to achieve a centered weld. |  |  |
| - Apply controlled pressure and initiate the spot-welding cycle, ensuring a smooth start and proper fusion between sheets. |  |  |
| Verify that the weld time is consistent with the procedure to prevent overheating or insufficient bonding. |  |  |
| - Inspect the spot weld visually for uniformity, smoothness, and correct weld nugget formation. |  |  |
| - Check for any visible defects, such as porosity, cracking, or burn-through, that indicate improper welding parameters. |  |  |
| - Perform a pull test or shear test on the joint to verify adequate weld strength as per the standards for 20 SWG mild steel. |  |  |
| - Clean any oxidation or slag around the weld area to improve joint appearance and prevent corrosion. |  |  |
| - Ensure the lap joint dimensions meet the specified tolerances and no misalignment occurred during welding. |  |  |
| - Follow all safety procedures, including wearing appropriate PPE (e.g., gloves, welding helmet, safety glasses). |  |  |
| - Ensure that the work area is free from flammable materials and that proper ventilation is maintained during welding. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Activity-03: Perform Spot welding to make Lap joint (Material: M.S Sheet, Size: 100mm x 50mm x 20SWG) (See Annex-3) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: (Activity-3) |  |  |  | Yes | No | Remarks |
|  | Select and confirm the M.S. sheet meets required specifications (100mm x 50mm x 20 SWG). |  |  |  |  |  |
|  | Clean the surfaces to be joined to ensure they are free of contaminants, such as grease, rust, or dust that could affect weld quality. |  |  |  |  |  |
|  | Position the sheets to achieve the specified overlap for a lap joint. |  |  |  |  |  |
|  | Select and set up the appropriate spot-welding machine based on the sheet thickness and material. |  |  |  |  |  |
|  | Adjust the electrode pressure, current, and welding time according to the standard welding procedure for 20 SWG mild steel. |  |  |  |  |  |
|  | Confirm electrode alignment to ensure consistent contact with the material during welding. |  |  |  |  |  |
|  | Position the electrodes correctly over the lap joint area to achieve a centered weld. |  |  |  |  |  |
|  | Apply controlled pressure and initiate the spot-welding cycle, ensuring a smooth start and proper fusion between sheets. |  |  |  |  |  |
|  | Verify that the weld time is consistent with the procedure to prevent overheating or insufficient bonding. |  |  |  |  |  |
|  | Inspect the spot weld visually for uniformity, smoothness, and correct weld nugget formation. |  |  |  |  |  |
|  | Check for any visible defects, such as porosity, cracking, or burn-through, that indicate improper welding parameters. |  |  |  |  |  |
|  | Perform a pull test or shear test on the joint to verify adequate weld strength as per the standards for 20 SWG mild steel. |  |  |  |  |  |
|  | Clean any oxidation or slag around the weld area to improve joint appearance and prevent corrosion. |  |  |  |  |  |
|  | Ensure the lap joint dimensions meet the specified tolerances and no misalignment occurred during welding. |  |  |  |  |  |
|  | Follow all safety procedures, including wearing appropriate PPE (e.g., gloves, welding helmet, safety glasses). |  |  |  |  |  |
|  | Ensure that the work area is free from flammable materials and that proper ventilation is maintained during welding. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate in Welder Level 03 |
| --- | --- |
| Competency Standard | - 0715MMT3801 Perform Basic Calculation and Estimation for Welding Work - 0715MMT3802 Prepare Welding Procedure Specification (WPS) - 0715MMT3803 Carry Out Shielded Metal Arc Welding (SMAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3804 Carry Out Gas Metal Arc Welding (GMAW) In Vertical (3F,3G) &Overhead(4F,4G) Positions - 0715MMT3805 Carry Out Flux Cored Arc Welding (FCAW) In Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3806 Carry Out Gas Tungsten Arc Welding (GTAW) in Vertical (3F,3G) & Overhead (4F,4G) Positions - 0715MMT3807 Carry Out Spot Welding on Mild Steel Sheet |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | What factors affect the cost estimation of a welding job? |  |  |  |
|  | What are the main differences between SMAW and GMAW |  |  |  |
|  | What are the challenges of welding in the overhead position (4G) compared to flat welding? |  |  |  |
|  | What is slag, and why is it important to remove it after welding? |  |  |  |
|  | What is spot welding, and what types of joints is it typically used for? |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |

(Annex-3)

Spot Weld (Lap Joint)